# 3GPP TSG-RAN WG4 Meeting # 111						R4-2409095

# Fukuoka, Japan, 20th  –24th  May, 2024

Source:		Ericsson

Title:			A-IoT general overview

Agenda item:		10.13.1

Document for:		Approval

# Introduction

In this paper, we present our thoughts on some of general issues for A-IoT RAN4 study.

# Discussion

In WF [1], the spectrum deployment mode is stated below:

## Topic 2-3: Spectrum deployment mode

Issue 2-3-2: Priorities of spectrum deployment mode for co-existence evaluation

Agreement:

- Prioritize the following spectrum deployment mode for RAN4 co-existence evaluation

  - A-IoT is located within a NR transmission bandwidth configuration

  - A-IoT which is operating indoor shares in-band spectrum with outdoor macro BS

This means the A-IoT will be deployed as stand-alone mode. Compared to NB-IoT BS which has guard-band, in-band and stand-alone mode for LTE, this seems not in line with the SID objective:

- Deployment Scenarios with the following characteristics, referenced to the tables in Clause 4.2.2 of TR 38.848:

- Deployment scenario 1 with Topology 1

  - Basestation and coexistence characteristics: Micro-cell, co-site

- Deployment scenario 2 with Topology 2 and UE as intermediate node, under network control

  - Basestation and coexistence characteristics: Macro-cell, co-site

  - The location of intermediate node is indoor

- FR1 licensed spectrum in FDD.

- Spectrum deployment in-band to NR, in guard-band to LTE/NR, in standalone band(s).

- Traffic types DO-DTT, DT, with focus on rUC1 (indoor inventory) and rUC4 (indoor command).

- From RAN#104, the study will assess whether the harmonized air interface design (per bullet ‘A’ above) can address the DO-A (Device-originated autonomous) use case, only to identify which part(s) of the harmonized air interface design (per bullet ‘A’ above) is/are not sufficient for the DO-A use case.

Prioritize the stand-alone deployment is not in line with SID objective.

The prioritization of the stand-alone A-ioT deployment is proposed in the coexisting study and one can argue whether such prioritization is only from coexisting perspective or from the SID scope perspective, as in obiective, the “co-site” is listed in Topology 1. We prefer to keeping the SID objective or now and the coexisting of legacy LTE/NR should be within the coexisting scope with possible additional consideration (e.g the isolation distance between LTE/NR network for coexisting purpose).

Keep the original deployment mode scope in SID and study further the coexisting of A-IoT and legacy NR/LTE.

Another aspect is that RAN1 and RAN4 work progress dependency. As RAN4 coexisting/RF study has dependency on the RAN1 progress, so there is a need to understand better how to proceed RAN4 work considering RAN1 progress and understand whose responsibility of these issue. This may need an issue list dependent on RAN1 work. Below we list some issues:

- SNR for baseline operation:  In legacy coexisting study, the baseline scenario is without the aggressor network and the SNR to operate A-IoT should be known.  In legacy coexisting study, the throughput is used and a table of SINR to throughput is used so the SNR operation is given. For UL receiving at A-IoT BS, such SINR operation point relating to both the RF isolation capability (CW transmission leakage to receiving band) and may be baseband capability to further attenuate the CW signal. Maybe the RF part could be discussed in RAN4, but baseband part may rely on RAN1.

- Channel BW definition:  In RAN4, it is strictly relating to the occupied bandwidth requirement, and it is not clear if RAN1 will define it or RAN4 should study the channel definition as it relates to RF requirement compliant.

Further clarify the RAN4 coexisting dependency on RAN1 on the issues above.

# Conclusions

In this contribution, we present our overview on the A-IoT general issues with below proposal:

Observation 1 Prioritize the stand-alone deployment is not in line with SID objective.

Proposal-1: Keep the original deployment mode scope in SID and study further the coexisting of A-IoT and legacy NR/LTE.

Proposal-2: Further clarify the RAN4 coexisting dependency on RAN1 on the issues above.

# References

- R4-2406714, WF on Ambient IoT in NR, CMCC