3GPP TSG-RAN WG4 Meeting #111                                        R4-2409177

Fukuoka, Japan, May 20 - 24, 2024

Source: 	Huawei, HiSilicon

Title: 	On modification of existing BS RF requirements for SBFD

Agenda Item:	10.12.2.1

Document for:	Approval

Introduction

During RAN4#110bis, BS RF requirements for SBFD have been discussed. Following conclusions are reproduced from [1].

- Work Plan (RAN4 core part) - Agreement: - Approve the work plan with the note as follows: - Note: During certain particular BS RF requirement is specified, FFS if the additional coexistence simulation to Rel-18 RAN4 co-existence study is required. Table 1: WI plan for Evolution of NR duplex operation (RAN4 Part) Schedule and TU; Work plan. April, 2024 RAN4#110bis (0.25 TU); RAN4 (core) - Start discussion on BS RF requirements for SBFD operation at gNB - Discuss on SBFD operation impact on existing BS RF requirements - Discuss on the necessity of introducing new requirements for SBFD operation, identified in study item. May, 2024 RAN4#111 (0.25 TU); RAN4 (core) - Continue discussion on BS RF requirements for SBFD operation at gNB - Continue discussion on SBFD operation impact on existing BS RF requirements - Continue discussion on the necessity of introducing new requirements for SBFD operation, identified in study item. -- Omit the rest of work plan for simplicity -- - SBFD frequency-domain configuration in RAN4 specification - Agreement: - It is within RAN4 scope to study/specify the limitation or restriction on the size of subband/guardband, by taking account different feasible BS/UE implementations. - FFS how RAN4 specification captures the subband configurations - FFS the necessity of standardize the guardband; - If needed, FFS the sizes of guardband in RAN4 shall be decided. - SBFD operation in BS configured with multi-carriers - Agreement: - Rel-19 requirement shall consider both (1) single carrier operation for SBFD (2) SBFD operates in only one BS carrier, and legacy TDD operates in other intra-band BS carrier(s) contiguous or non-contiguous to the SBFD carrier. - Co-location requirement and reference antenna - Way forward: - Continue the discussion on TX IMD requirement for SBFD, based on the existing assumption for co-location reference antenna. - The assumption of co-location reference antenna could be revised based on the outcome from BS RF enh. WI.

In this contribution, we would like to share our views regarding this topic.

Discussion

As one of the hot issues that have been discussed last meeting, whether any BS types need to be deprioritized/precluded for future discussion for SBFD, we think WA BS should be equally treated based on the operators’ demands and some interested vendors’ feedback. Besides, the support of SBFD operation would be up to declaration but not mandatory.

Proposal 1: Not to consider precluding or deprioritizing WA BS type from future discussion on SBFD.

In the rest of this contribution, our thinking regarding SBFD BS RF requirements mainly based on the outcomes of Rel-18 SI on SBFD will be provided.

Impact on existing BS requirements for SBFD operation

Tx aspect

Output power

It was agreed to have different output power declaration for normal DL symbols/slots and SBFD DL symbols/slots, respectively. The accuracy requirement for TRP/EIRP and conducted power shall be the same for normal DL symbols/slots and SBFD DL symbols/slots.

Observation 1: For output power:

- Different conducted power and EIRP/TRP declaration for normal DL symbols/slots and SBFD DL symbols/slots should be allowed. The accuracy requirement for conducted power and EIRP/TRP should be the same for normal DL symbols/slots and SBFD DL symbols/slots

Output power dynamics

RAN4 agreed to reuse the existing RE power control dynamic range requirements for the BS capable of SBFD. Besides, the output power dynamic range requirement for BS capable of SBFD will be defined as ratio of the declared rated output power with all DL sub-band RBs active for SBFD (maximum) and the same single RB power as non-SBFD (minimum).

Observation 2: For output power dynamics:

- Existing RE power control dynamic range requirements will be reused

- The total dynamic range requirement is applicable for SBFD-capable BS during normal DL symbols/slots

- The output power dynamic range will be defined as the ratio of the declared rated output power with all DL sub-band RBs active for SBFD (maximum) and the same single RB power as non-SBFD (minimum)

Transmit ON/OFF power and Tx signal quality

RAN4 conclusion on transmit ON/OFF power requirement is that it is not applicable within SBFD time slot, while all existing requirements for frequency error, EVM and time alignment error should be applied to SBFD symbols/slots.

Observation 3: For transmit ON/OFF power and Tx signal quality:

- Transmit ON/OFF power requirement is not applicable within SBFD time slot, while all existing requirements for frequency error, EVM and time alignment error should be applied to SBFD symbols/slots

Unwanted emissions

While it was agreed to still apply the existing OBW requirement for the whole BS channel bandwidth in SBFD symbols/slots, both ACLR and OBUE requirements have also been agreed to be defined from the edge of the whole carrier instead of sub-band for SBFD DL symbols/slots. In order to guarantee feasible co-existence performance between different SBFD BSs operating in adjacent frequency, ACLR and especially OBUE requirements for SBFD capable BS should be further discussed.

Observation 4: For unwanted emissions:

- For OBUE requirement, the RF bandwidth edge from which OBUE is defined is the edge of the carrier (same for both SBFD and non-SBFD symbols/slots)

- For transmitter spurious emission requirement, all the existing requirements shall also be applied to SBFD-capable BS in SBFD symbols/slots. The requirement of protection of the BS receiver of own or different BS is not applicable for TDD operation

Proposal 2: Further discuss how to define OBUE requirement for SBFD capable BS in order to ensure feasible co-existence performance for SBFD BSs operating in adjacent frequency.

Rx aspect

Likewise, potentially impacted BS Rx requirements can be treated in similar manner.

Reference sensitivity and OTA sensitivity

During Rel-18 SI, value range [0.5~1] dB has been considered for the allowed sensitivity degradation due to self-interference. RAN4 may need to continue the work from e.g. whether to consider interferences other than self-interference for OTA sensitivity.

Proposal 3: Based on the Rel-18 SI outcome i.e. ~1dB sensitivity degradation due to self-interference, further discuss whether to consider interference sources other than self-interference for OTA sensitivity.

In-band selectivity and blocking

We reproduce some related conclusions from TR 38.858 for better illustration.

- ACS requirement and the interference level shall be determined by RAN4 co-existence study, and for the definition of ACS requirement: - Conducted ACS: Take the existing wanted signal of ACS requirement by using the existing reference sensitivity level. - OTA ACS: The OTA sensitivity degradation shall be taken into account to determine the level of wanted signal and interference signal mean power. - In-band blocking requirement and the interference level shall be determined by RAN4 co-existence study, and for the definition of In-band blocking requirement: - Conducted In-band blocking: Take the existing wanted signal of In-band blocking requirement by using the existing reference sensitivity level. - OTA In-band blocking: The OTA sensitivity degradation shall be taken into account to determine the level of wanted signal and interference signal mean power. - For in-band selectivity and blocking, the requirements shall be defined out of the BS channel bandwidth instead of uplink subband bandwidth.

In our view, in order to guarantee feasible co-existence performance between different BSs operating in adjacent frequency, in-band blocking requirement for SBFD should be further discussed since the blocker outside of carrier could be DL transmission from other BS, which is the UL transmission from other UEs that had been considered for legacy requirement. Whether the blocker signal level can be scenario specific can be further discussed as well.

Proposal 4: Further discuss how to define Rx blocking requirement for SBFD capable BS due to the fact that blocker could be DL transmission from other BSs but not only UEs. Whether it can be scenario specific can be further discussed as well.

Conclusion

In this contribution we discussed on the BS RF requirement for SBFD. We have the following observations and proposals:

Observation 1: For output power:

- Different conducted power and EIRP/TRP declaration for normal DL symbols/slots and SBFD DL symbols/slots should be allowed. The accuracy requirement for conducted power and EIRP/TRP should be the same for normal DL symbols/slots and SBFD DL symbols/slots

Observation 2: For output power dynamics:

- Existing RE power control dynamic range requirements will be reused

- The total dynamic range requirement is applicable for SBFD-capable BS during normal DL symbols/slots

- The output power dynamic range will be defined as the ratio of the declared rated output power with all DL sub-band RBs active for SBFD (maximum) and the same single RB power as non-SBFD (minimum)

Observation 3: For transmit ON/OFF power and Tx signal quality:

- Transmit ON/OFF power requirement is not applicable within SBFD time slot, while all existing requirements for frequency error, EVM and time alignment error should be applied to SBFD symbols/slots

Observation 4: For unwanted emissions:

- For OBUE requirement, the RF bandwidth edge from which OBUE is defined is the edge of the carrier (same for both SBFD and non-SBFD symbols/slots)

- For transmitter spurious emission requirement, all the existing requirements shall also be applied to SBFD-capable BS in SBFD symbols/slots. The requirement of protection of the BS receiver of own or different BS is not applicable for TDD operation

Proposal 1: Not to consider precluding or deprioritizing WA BS type from future discussion on SBFD.

Proposal 2: Further discuss how to define OBUE requirement for SBFD capable BS in order to ensure feasible co-existence performance for SBFD BSs operating in adjacent frequency.

Proposal 3: Based on the Rel-18 SI outcome i.e. ~1dB sensitivity degradation due to self-interference, further discuss whether to consider interference sources other than self-interference for OTA sensitivity.

Proposal 4: Further discuss how to define Rx blocking requirement for SBFD capable BS due to the fact that blocker could be DL transmission from other BSs but not only UEs. Whether it can be scenario specific can be further discussed as well.

- References

- R4-2406107, “Way Forward for [110bis][313] NR_duplex_evo”, Samsung, 3GPP TSG-RAN4 Meeting #110bis, Changsha, China, April 15-19, 2024.