3GPP TSG-RAN WG4 Meeting # 111						R4-2409190

Fukuoka Meeting, May 20th – May 24th, 2024

Source:	Nokia

Title:	On UE RF specifications table improvement

Agenda item:	12.1.1.1

Document for:	Discussion

Introduction

During RAN#103 a WF was agreed to improve the RAN4 specifications with relevance for this agenda focus on the UE RF specification in TS 38.101-1, 38.101-2 and 38.101-3. The RAN WF is included below for reference [1]:

- The RAN4 Rel-19 specifications are expected to be available by December 2024. - RAN4 will organize the discussions for improving the specifications in Q2 and Q3 2024 in RAN4 meeting(s), and report to RAN#104 and RAN#105 - Focus on 38.133 and 38.101-1/38.101-2/38.101-3, not covering other specifications in this RAN task - Motivation of the work: - Try to improve the above specifications for Rel-19 for 5G in the short term - Try to conclude on guidance including the structure, drafting rule to ensure the quality of specifications for UE RF and RRM. - Set up one dedicated agenda to collect the input from companies for specification improvement - Companies are expected to point out the key issues and also provide the concrete solutions. - No corresponding CR is expected before September - Schedule the specific time slot for the single discussions on the specification improvement in RAN4 main session starting from April - Identify the key issues and root reasons behind - Summarize the candidate solutions for the next action - Further discuss and decide how to capture the outcome of this RAN task in RAN#105

At RAN4#110bis the RAN WF above was further discussed and relate to UE RF a RAN4 WF [2] was agreed include the following way-forward:

<Way forward/Agreement>: - Following proposals to be discussed in UE RF Spec specification improvement agenda: Proposals in R4-2405036 - Proposal 1: RAN4 to discuss if information/requirements spread across multiple tables in the TS can be merged into a single table. - Proposal 3: RAN4 to discuss if the listing of UE relaxation per band combination can be collected into a single table.

This contribution will further discuss the proposals in [3] for improving the RAN4 UE RF specification.

Discussion

Length/size of tables related to band combinations

Listing of supported band combinations

When opening RAN4 UE specification TS 38.101-1 it is evident that the dominant part of the specification content is tables listing band combinations. The first tables list simply what DL and UL configurations are supported and as one moves on in the specification additional information is added on e.g. supported channel bandwidths within the listed DL and UL bands. Listing the same band combinations multiple times just adding additional information is needed simply because it is not currently practical to condense all the information and requirements for a single DL configuration into a single table.

Currently it is not possible to condense all the information and requirements for a single DL configuration into a single table.

As a result, RAN#102 commissioned an activity to identify a new methodology with a small working party in conjunction with ETSI MCC to handle capturing a large number of band combinations. The status of this work was last presented at RAN#103 in [4]. The intention of this work is in long term to shift the listing of the band combinations from the Word-based specification to a database managed by MCC and then simply reference from the written Word-based specification.

The long-term goal is to move the listing of band combinations to a database managed by MCC.

However, due to the complexity of the information related to the band combinations, this may take significant time and RAN4 can in the meantime investigate whether additional simplification and removal of redundancy can be achieved.

Definition of UE relaxations per band combination

A starting point to reduce the length and number of tables for band combinations is, as suggested in [3], to investigate if some of the information/requirements now spread across multiple tables listing band combinations can be merged into a single table listing the band combination.

Multiple tables are now listing band combinations meaning that there are numerous long tables in the specification.

UE relaxations per band combination, e.g. ΔTIB,c and ΔRIB,c and even MSD in many cases, have the same value dependent on the type of issue, e.g. harmonics, creating the need for the relaxation. Even so, these relaxation values are currently listed per band combination in multiple tables for each type of issue.

Currently the RAN4 UE RF specification has separate tables for each UE relaxation type, e.g. MSD due to harmonica mixing issues.

A suggestion is to list relaxation per UE issue/type in a single table and then list all the types of issues for the specific band combinations in list of supported band combinations. By grouping a list of issues per band combination would provide an overview of the individual band combinations combined need for UE relaxation.

Providing a list of supported band combinations together with their “issues” requiring relaxation would provide an overview instead of spreading the information over multiple tables in the specification.

In a practical example for TS 38.101-1 only targeting Clause 7 this would mean introducing the following 8 as shown in the following in Table 1-8. The Reasoning for the values proposed in these tables is given in Annex A which shows the statistics and investigations conducted for the currently defined UE relaxations in TS 38.101-1.

With the agreement of the LowerMSD capability signaling only a worst-case MSD value is required for each MSD type and order, including band-group relations Table 1 to Table 8 in the following has been made using the highest MSD for each MSD type and MSD order. If the MSD is expected to be much better than specified in the generic requirements it is expected that UE vendors signal the improved MSD Class for the combination.

Annex A show the statistics and investigations conducted for the currently defined UE relaxations in TS 38.101-1 clause 7.

Table 1: Band group definition

| FR1 band group range |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name | FR1-1 | FR1-2 | FR1-3 | FR1-4 | FR1-5 |
| Range (MHz) | 600-1000 | 1400-2200 | 2300-2700 | 3300-5000 | 5150-7125 |

Table 2: Uplink harmonic requirements.

| PC3 |  | PC2 |  | PC1.5 |  |
| --- | --- | --- | --- | --- | --- |
| Order | MSD | Order | MSD | Order | MSD |
| UL2 | 23.9dB | UL2 | 27.1dB | UL2 | 32.3dB |
| UL3 | 9dB | UL3 | N/A | UL3 | N/A |
| UL4 | 11dB | UL4 | N/A | UL4 | N/A |
| UL5 | 10.5dB | UL5 | N/A | UL5 | N/A |

Table 3: Harmonic mixing requirements PC3. UHB = UHB + VHB

| PC3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Order | MB/LB | HB/LB | HB/MB | HB/HB | UHB/LB | UHB/MB | UHB/HB | UHB/UHB |
| UL1/DL2 | 3.7dB | N/A | 6.7dB | N/A | N/A | 8.3dB | N/A | N/A |
| UL1/DL3 | 26.5dB | 24.3dB | N/A | N/A | N/A | 13.5dB | N/A | N/A |
| UL1/DL4 | N/A | 5.7dB | N/A | N/A | 5.7dB | N/A | N/A | N/A |
| UL1/DL5 | N/A | N/A | N/A | N/A | 31dB | N/A | N/A | N/A |
| UL2/DL3 | N/A | N/A | N/A | N/A | N/A | N/A | 10.4dB | 22.6dB |
| UL3/DL4 | N/A | N/A | N/A | 4.5dB | N/A | N/A | N/A | N/A |
| UL4/DL3 | N/A | N/A | N/A | N/A | N/A | N/A | 8.3dB | N/A |

Table 4: Harmonic mixing requirements PC2/PC1.5. UHB = UHB + VHB

| PC2/PC1.5 |  |  |  |
| --- | --- | --- | --- |
| Order | UHB/LB | UHB/MB | UHB/HB |
| UL1/DL2 | N/A | 9.2dB/11.9dB | N/A |
| UL1/DL4 | 8.1dB/10.7dB | N/A | N/A |
| UL1/DL5 | 34dB/37dB | N/A | N/A |
| UL2/DL3 | N/A | N/A | 13.2dB/16.1dB |

Table 5: Harmonic mixing requirements PC5. UHB = UHB + VHB

| PC5 |  |  |
| --- | --- | --- |
| Order | UHB/HB | UHB/UHB |
| UL1/DL2 | 5.3dB | N/A |
| UL2/DL3 | N/A | 20.3dB |

Table 6: IMD requirements for PC3-PC5.

| 2 CA IMD |  |  |  | 3 CA IMD |  |
| --- | --- | --- | --- | --- | --- |
| Order | MSD | 2CA (NC_ULCA) | MSD | Order | MSD |
| IMD2 | 30dB | IMD4 | 8.6dB | IMD2 | 30dB |
| IMD3 same b.gr | 25dB | IMD5 same b.gr | 22dB | IMD3 same b.gr | 25dB |
| IMD3 diff. b.gr | 15dB | IMD5 diff. b.gr | 17.5dB | IMD3 diff b.gr. | 17.5dB |
| IMD4 | 8.6dB | IMD7 same b.gr | 8.5dB | IMD4 | 12dB |
| IMD5 | 5.5dB | IMD7 diff. b.gr | 3.2dB | IMD5 | 6dB |
|  |  | IMD9 | 2.8dB | IMD7 | 2.5dB |

Table 7: IMD requirements for PC2.

| 2 CA IMD |  |  |  | 3 CA IMD |  |
| --- | --- | --- | --- | --- | --- |
| Order | MSD | 2CA (NC_ULCA) | MSD | Order | MSD |
| IMD2 | 36dB | IMD4 | 18.6dB | IMD2 | 36.5dB |
| IMD3 same b.gr | 25dB | IMD5 same b.gr | 32.5dB | IMD3 same b.gr | 26.5dB |
| IMD3 diff. b.gr | 15dB | IMD5 diff. b.gr | 22dB | IMD3 diff b.gr. | 22dB |
| IMD4 | 19dB | IMD7 same b.gr | 13.5dB | IMD4 | 22dB |
| IMD5 | 14dB | IMD7 diff. b.gr | 9dB | IMD5 | 21dB |
|  |  | IMD9 | 3dB | IMD7 |  |

Table 8: IMD requirements for PC1.5.

| 2 CA IMD |  |  |  | 3 CA IMD |  |
| --- | --- | --- | --- | --- | --- |
| Order | MSD | 2CA (NC_ULCA) | MSD | Order | MSD |
| IMD2 | 37.5dB | IMD4 | N/A | IMD2 | N/A |
| IMD3 same b.gr | N/A | IMD5 same b.gr | N/A | IMD3 same b.gr | N/A |
| IMD3 diff. b.gr | N/A | IMD5 diff. b.gr | N/A | IMD3 diff b.gr. | N/A |
| IMD4 | 26.5dB | IMD7 same b.gr | N/A | IMD4 | N/A |
| IMD5 | 30dB | IMD7 diff. b.gr | N/A | IMD5 | N/A |
|  |  | IMD9 | N/A | IMD7 | N/A |

Referring to the relaxations listed in Table 2-8, sometimes dependent on the band group listed in Table 1, it would be possible to list all e.g. 2 band combinations as shown in Table 9 below. Note that the development of the IMD test point equations is still ongoing, hence shown in gray.

Table 9: 2 UL CA configurations with MSD requirements.

| NR CA band combination (CA_nX-nY) | NR Rx band (nZ) | Uplink Harmonic | Harmonic Mixing | Cross Band Interf. | Order | NC ULCA | IMD test point equation UL Band(x,y) to DL Band(z) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1-n3 | n1 |  |  |  | IMD3 same |  | I2*nX-nYI=nZ |
| CA_n1-n3 | n3 |  |  | 1,>2 |  |  |  |
| CA_n1-n8 | n1 |  |  |  | IMD4 |  | I2*nX-2*nYI=nZ |
| CA_n1-n28 | n1 | UL3 |  |  |  |  |  |
| CA_n1-n38 | n1 |  |  | >2 |  |  |  |
| CA_n1-n38 | n38 |  |  | >2 |  |  |  |
| CA_n1-n40 | n1 |  |  | >2 |  |  |  |
| CA_n1-n40 | n40 |  |  | >2 |  |  |  |
| CA_n1-n41 | n41 |  |  | >2 |  |  |  |
| CA_n1-n46 | n1 |  |  |  | IMD5 |  | InY-4*nXI=nZ |
| CA_n1-n77 | n1 |  |  |  | IMD24 | IMD5 diff | InY-nXI=nZ; nY-2*2CCBW=nZ |
| CA_n1-n77 | n77 | UL2 |  |  |  |  |  |
| CA_n1-n78 | n1 |  |  |  | IMD4 | IMD7 | I3*nX-nYI=nZ; nY-3*2CCBW=nZ |
| CA_n1-n102 | n1 |  | UL1/DL3 |  | IMD3 diff |  |  |
| CA_n1-n105 | n105 |  | UL1/DL3 |  | IMD3 diff |  |  |
| CA_n2-n38 | n2 |  |  | >2 |  |  |  |
| CA_n2-n41 | n2 |  |  | >2 |  |  |  |
| CA_n2-n48 | n2 |  |  |  | IMD4 |  |  |
| CA_n2-n48 | n48 | UL2 |  |  |  |  |  |
| CA_n2-n66 | n2 |  |  |  | IMD3 same |  |  |
| CA_n2-n66 | n66 |  |  |  | IMD5 |  |  |
| CA_n2-n71 | n2 | UL3 |  |  |  |  |  |
| CA_n2-n71 | n71 |  | UL1/DL3 |  |  |  |  |
| CA_n2-n772,3 | n2 |  | UL1/DL2 |  | IMD24, IMD4 | IMD7 |  |
| CA_n2-n772,3 | n77 | UL2 |  |  |  |  |  |
| CA_n2-n78 | n2 |  |  |  | IMD24 |  |  |
| CA_n2-n78 | n78 | UL2 |  |  |  |  |  |
| CA_n3-n5 | n3 |  |  |  | IMD4 |  |  |
| CA_n3-n5 | n5 |  |  |  | IMD2 |  |  |
| CA_n3-n7 | n3 |  |  | >2 |  |  |  |
| CA_n3-n7 | n7 |  |  |  | IMD4 |  |  |
| CA_n3-n8 | n3 |  |  |  | IMD5 |  |  |
| CA_n3-n8 | n8 |  |  |  | IMD4 |  |  |
| CA_n3-n18 | n3 |  |  |  | IMD4 |  |  |
| CA_n3-n20 | n3 |  |  |  | IMD4 |  |  |
| CA_n3-n20 | n20 |  |  |  | IMD4 |  |  |
| CA_n3-n26 | n3 |  |  |  | IMD4 |  |  |
| CA_n3-n26 | n26 |  | UL1/DL2 |  | IMD24 |  |  |
| CA_n3-n34 | n3 |  |  | >2 |  |  |  |
| CA_n3-n38 | n3 |  |  |  | IMD4 |  |  |
| CA_n3-n41 | n3 |  |  | >2 | IMD3 diff, IMD4 | IMD9 |  |
| CA_n3-n41 | n41 |  |  | >2 |  |  |  |
| CA_n3-n74 | n74 |  |  | >2 |  |  |  |
| CA_n3-n75 | n75 |  |  | >2 |  |  |  |
| CA_n3-n772,3 | n3 |  |  |  | IMD24, IMD4 | IMD7 |  |
| CA_n3-n772,3 | n77 | UL2 |  |  |  |  |  |
| CA_n3-n782,3 | n3 |  |  |  | IMD24, IMD4 | IMD7 |  |
| CA_n3-n782,3 | n78 | UL2 |  |  |  |  |  |
| CA_n5-n7 | n5 |  |  |  | IMD3 diff |  |  |
| CA_n5-n14 | n5 |  |  |  | IMD3 same |  |  |
| CA_n5-n14 | n5 |  |  |  | IMD3 same |  |  |
| CA_n5-n41 | n28 |  |  | 2 |  |  |  |
| CA_n5-n41 | n5 |  | UL1/DL3 |  | IMD3 diff |  |  |
| CA_n5-n41 | n41 | UL3 |  |  |  |  |  |
| CA_n5-n66 | n5 |  |  |  | IMD24 |  |  |
| CA_n5-n71 | n5 |  |  | >2 |  |  |  |
| CA_n5-n71 | n71 |  |  | >2 |  |  |  |
| CA_n5-n772,3 | n5 |  |  |  | IMD24, IMD4 | IMD4 |  |
| CA_n5-n772,3 | n77 | UL4, UL5 |  |  |  |  |  |
| CA_n5-n78 | n5 |  |  |  | IMD4 | IMD4 |  |
| CA_n5-n78 | n78 | UL4 |  |  |  |  |  |
| CA_n7-n8 | n7 | UL3 |  |  |  |  |  |
| CA_n7-n26 | n7 |  |  |  | IMD5 |  |  |
| CA_n7-n26 | n26 |  |  |  | IMD3 diff |  |  |
| CA_n7-n40 | n7 |  |  | >2 | IMD3 same |  |  |
| CA_n7-n40 | n40 |  |  | >2 |  |  |  |
| CA_n7-n464 | n7 |  |  |  | IMD24 |  |  |
| CA_n7-n464 | n46 |  | UL1/DL2 |  |  |  |  |
| CA_n7-n66 | n7 |  |  |  | IMD4 |  |  |
| CA_n7-n71 | n7 | UL4 |  |  |  |  |  |
| CA_n7-n77 | n7 |  |  | >2 | IMD4 | IMD5 diff |  |
| CA_n7-n78 | n7 |  |  | >2 |  |  |  |
| CA_n7-n105 | n7 | UL3 |  |  |  |  |  |
| CA_n8-n20 | n8 |  |  |  | IMD3 same |  |  |
| CA_n8-n20 | n20 |  |  |  | IMD3 same |  |  |
| CA_n8-n41 | n8 |  |  |  | IMD3 diff |  |  |
| CA_n8-n41 | n41 | UL3 |  |  |  |  |  |
| CA_n8-n77 | n77 | UL4 |  |  |  |  |  |
| CA_n8-n782 | n8 |  |  |  | IMD4 |  |  |
| CA_n8-n782 | n78 | UL4 |  |  |  |  |  |
| CA_n8-n792,3 | n8 |  | UL1/DL5 |  | IMD5 |  |  |
| CA_n8-n792,3 | n79 | UL5 |  |  |  |  |  |
| CA_n12-n48 | N48 | UL5 |  |  |  |  |  |
| CA_n12-n66 | n66 | UL3 |  |  |  |  |  |
| CA_n12-n71 | n12 |  |  | 2,>2 |  |  |  |
| CA_n12-n772,3 | n12 |  | UL1/DL5 |  |  |  |  |
| CA_n12-n772,3 | n77 | UL5 |  |  |  |  |  |
| CA_n12-n78 | n12 |  | UL1/DL5 |  |  |  |  |
| CA_n12-n78 | n78 | UL5 |  |  |  |  |  |
| CA_n13-n772,3 | n13 |  | UL1/DL5 |  |  |  |  |
| CA_n13-n772,3 | n77 | UL5 |  |  |  |  |  |
| CA_n14-n772,3 | n14 |  | UL1/DL5 |  |  |  |  |
| CA_n14-n772,3 | n77 | UL5 |  |  |  |  |  |
| CA_n18-n28 | n28 |  |  | 1,2 |  |  |  |
| CA_n18-n41 | n18 |  | UL1/DL3 |  |  |  |  |
| CA_n18-n77 | n18 |  |  |  | IMD4, IMD5 |  |  |
| CA_n18-n77 | n77 | UL5 |  |  |  |  |  |
| CA_n18-n78 | n18 |  |  |  | IMD4 |  |  |
| CA_n20-n40 | n20 |  | UL1/DL3 |  |  |  |  |
| CA_n20-n78 | n20 |  |  |  | IMD4 |  |  |
| CA_n20-n78 | n78 | UL4 |  |  |  |  |  |
| CA_n24-n77 | n24 |  |  |  | IMD4 |  |  |
| CA_n25-n38 | n25 |  |  | >2 |  |  |  |
| CA_n25-n41 | n25 |  |  | >2 |  | IMD7 |  |
| CA_n25-n48 | n25 |  |  |  | IMD4 |  |  |
| CA_n25-n66 | n25 |  |  |  | IMD3 same |  |  |
| CA_n25-n66 | n66 |  |  |  | IMD3 same, IMD5 |  |  |
| CA_n25-n71 | n25 | UL3 |  |  |  |  |  |
| CA_n25-n71 | n71 |  | UL1/DL3 |  |  |  |  |
| CA_n25-n772,3 | n25 |  | UL1/DL2 |  | IMD24, IMD4 | IMD7 |  |
| CA_n25-n772,3 | n77 | UL2 |  |  |  |  |  |
| CA_n25-n78 | n25 |  |  |  | IMD24 | IMD7 |  |
| CA_n25-n78 | n78 | UL2 |  |  |  |  |  |
| CA_n26-n29 | n29 |  |  | >2 |  |  |  |
| CA_n26-n48 | n26 |  | UL1/DL4 |  |  |  |  |
| CA_n26-n66 | n26 |  |  |  | IMD24 |  |  |
| CA_n26-n70 | n26 |  |  |  | IMD24 |  |  |
| CA_n26-n77 | n26 |  | UL1/DL4 |  |  |  |  |
| CA_n26-n78 | n26 |  | UL1/DL4 |  | IMD4 |  |  |
| CA_n26-n78 | n78 | UL4 |  |  |  |  |  |
| CA_n28-n40 | n28 |  | UL1/DL3 |  |  |  |  |
| CA_n28-n50 | n28 |  |  |  | IMD2, IMD4 |  |  |
| CA_n28-n50 | n50 | UL2 |  |  |  |  |  |
| CA_n28-n71 | n28 |  |  | >2 |  |  |  |
| CA_n28-n71 | n71 |  |  | 2 |  |  |  |
| CA_n28-n74 | n28 |  |  |  | IMD2, IMD4 |  |  |
| CA_n28-n74 | n74 | UL2 |  |  | IMD4, IMD5 |  |  |
| CA_n28-n75 | n75 | UL2 |  |  |  |  |  |
| CA_n28-n772,3 | n28 |  |  |  | IMD2, IMD5 | IMD4 |  |
| CA_n28-n772,3 | n77 | UL5 |  |  |  |  |  |
| CA_n28-n782 | n28 |  |  |  | IMD5 |  |  |
| CA_n28-n782 | n78 | UL5 |  |  |  |  |  |
| CA_n28-n94 | n94 | UL2 |  |  |  |  |  |
| CA_n29-n48 | n29 |  | UL1/DL5 |  |  |  |  |
| CA_n29-n71 | n29 |  |  | 2 |  |  |  |
| CA_n29-n772,3 | n29 |  | UL1/DL5 |  |  |  |  |
| CA_n30-n66 | n66 |  |  | >2 |  |  |  |
| CA_n30-n772,3 | n30 |  | UL2/DL3 |  | IMD4 | IMD7 |  |
| CA_n34-n41 | n34 |  |  | >2 |  |  |  |
| CA_n34-n41 | n41 |  |  | >2 |  |  |  |
| CA_n38-n66 | n66 |  |  | >2 |  |  |  |
| CA_n38-n78 | n38 |  |  | >2 |  |  |  |
| CA_n38-n78 | n78 |  |  | >2 |  |  |  |
| CA_n39-n41 | n39 |  | UL3/DL4 | >2 |  |  |  |
| CA_n39-n41 | n41 |  |  | >2 |  |  |  |
| CA_n40-n41 | n40 |  |  | 2 |  |  |  |
| CA_n40-n41 | n41 |  |  | 2 |  |  |  |
| CA_n40-n772 | n40 |  | UL2/DL3 |  |  |  |  |
| CA_n40-n772 | n77 |  |  | >2 |  |  |  |
| CA_n40-n78 | n40 |  | UL2/DL3 | >2 |  |  |  |
| CA_n41-n48 | n41 |  |  | >2 |  |  |  |
| CA_n41-n48 | n48 |  | UL4/DL3 | >2 |  |  |  |
| CA_n41-n66 | n66 |  |  | >2 |  | IMD5 same |  |
| CA_n41-n70 | n70 |  |  | >2 |  |  |  |
| CA_n41-n71 | n41 | UL4 |  |  |  |  |  |
| CA_n41-n71 | n71 |  |  |  | IMD4 |  |  |
| CA_n41-n772 | n41 |  | UL2/DL3 | >2 |  |  |  |
| CA_n41-n772 | n77 |  |  | >2 |  | IMD9 |  |
| CA_n41-n78 | n41 |  | UL2/DL3 | >2 |  | IMD9 |  |
| CA_n41-n78 | n78 |  |  | >2 |  |  |  |
| CA_n46-n484 | n46 |  |  | >2 |  |  |  |
| CA_n46-n484 | n48 |  | UL2/DL3 | >2 |  |  |  |
| CA_n46-n77 | n46 |  | UL2/DL3 |  |  | IMD5 diff, IMD6, IMD7 |  |
| CA_n46-n77 | n77 |  |  | >2 |  |  |  |
| CA_n46-n784 | n46 |  | UL2/DL3 | >2 |  | IMD6, IMD7 |  |
| CA_n46-n784 | n78 |  |  | >2 |  |  |  |
| CA_n48-n66 | n48 | UL2 |  |  |  |  |  |
| CA_n48-n66 | n66 |  |  |  | IMD5 |  |  |
| CA_n48-n70 | n70 |  |  |  | IMD24 |  |  |
| CA_n48-n964 | n48 |  | UL2/DL3 | >2 |  |  |  |
| CA_n48-n964 | n96 |  |  | >2 |  |  |  |
| CA_n66-n71 | n66 |  |  |  | IMD4 |  |  |
| CA_n66-n77 | n66 |  |  |  | IMD2, IMD5 | IMD5 diff, IMD7 |  |
| CA_n66-n77 | n77 | UL2 |  |  |  |  |  |
| CA_n66-n78 | n66 |  |  |  | IMD5 | IMD7 |  |
| CA_n66-n78 | n78 | UL2 |  |  |  |  |  |
| CA_n66-n85 | n66 | UL3 |  |  | IMD4 |  |  |
| CA_n67-n78 | n67 |  | UL1/DL5 |  |  | IMD4 |  |
| CA_n70-n71 | n70 |  |  |  | IMD4 |  |  |
| CA_n70-n71 | n71 | UL3 |  |  |  |  |  |
| CA_n70-n77 | n70 | UL2 |  |  | IMD2, IMD5 |  |  |
| CA_n70-n78 | n70 | UL2 |  |  | IMD2, IMD5 |  |  |
| CA_n71-n77 | n71 |  |  |  | IMD5 | IMD4 |  |
| CA_n71-n78 | n71 |  |  |  | IMD5 |  |  |
| CA_n71-n85 | n71 |  |  | >2 |  |  |  |
| CA_n71-n85 | n85 |  |  | 1,2 |  |  |  |
| CA_n77-n85 | n77 | UL5 |  |  |  |  |  |
| CA_n77-n85 | n85 |  | UL1/DL5 |  | IMD5 |  |  |
| CA_n78-n79 | n78 |  |  | >2 |  |  |  |
| CA_n78-n79 | n79 |  |  | >2 |  |  |  |
| CA_n78-n92 | n78 | UL4 |  |  |  |  |  |
| CA_n78-n94 | n78 | UL4 |  |  |  |  |  |
| CA_n78-n102 | n102 |  |  |  |  | IMD4 |  |
| CA_n78-n105 | n78 | UL5 |  |  |  |  |  |
| CA_n78-n105 | n105 |  |  |  | IMD5 |  |  |
| Note 2: Power class 2 supported. Note 3: Power class 1.5 supported. Note 4: Power class 5 supported. |  |  |  |  |  |  |  |

It should be noted that the information now captured in Table 1-9 currently in TS 38.101-1 within Clause 7 takes up 23 pages meaning if RAN4 adopts this approach 18 could be saved. If RAN4 than also incorporates the ΔRIB,c values into the table additionally 3 pages can be saved in the specification just in Clause 7.

RAN4 could reduce the length of TS 38.101-1 by 21 pages using the approach presented here.

Given the number of saved pages together with the possibility of providing an overview of UE relaxations per band combinations it is suggested that RAN4 proceeds developing the presented unified table approach.

RAN4 shall further develop the unified table approach for UL configurations and adopt this in the specification.

Additionally it can be investigated of the band combinations lister per DL configuration can be simplified in similar manner as by merging the information in Clause 5 for DL interruption with the relaxations listed in Clause 6, i.e. ΔTIB,c, per band combination.

RAN4 shall further investigate whether a unified tabled can be developed for DL configurations.

Ordering of band combinations in the specification tables

While reviving the tables listing band combinations it is also suggested to spend effort on correcting the ordering of the listed band combinations as there now are multiple mistakes.

Currently there are multiple mistakes in the ordering of the listed band combinations in the specification.

It is understood that some effort is already employed to correct this both as maintenance and by the BigCR authors of the different band combination baskets. However, at each iteration/release of the specification the sometimes-multiple colliding CRs make it very difficult not to introduce new mistakes. A suggestion for a solution is to select a single CR responsible author per band combination table in the specification. This may currently be difficult with the way the different band combinations are split with multiple having to add into the same table.

This is related to the organization of Rel-19 band combination baskets which is further discussed in [5]. Which uses the principal that there should be no band combination tables which is covered by multiple baskets, i.e. WIDs.

The organization of Rel-19 band combination baskets can mitigate some of the issues with the listing/ordering of band combinations in the specification.

Conclusion

In this contribution, we discuss suggestions for improving the RAN4 UE RF specification in TS 38.101-1, 38.101-2 and 38.101-3. The following Observations and Proposals were made:

Observation 1: Currently it is not possible to condense all the information and requirements for a single DL configuration into a single table.

Observation 2: The long-term goal is to move the listing of band combinations to a database managed by MCC.

Observation 3: Multiple tables are now listing band combinations meaning that there are numerous long tables in the specification.

Observation 4: Currently the RAN4 UE RF specification has separate tables for each UE relaxation type, e.g. MSD due to harmonica mixing issues.

Observation 5: Providing a list of supported band combinations together with their “issues” requiring relaxation would provide an overview instead of spreading the information over multiple tables in the specification.

Observation 6: Annex A show the statistics and investigations conducted for the currently defined UE relaxations in TS 38.101-1 clause 7.

Observation 7: RAN4 could reduce the length of TS 38.101-1 by 21 pages using the approach presented here.

Proposal 1: RAN4 shall further develop the unified table approach for UL configurations and adopt this in the specification.

Proposal 2: RAN4 shall further investigate whether a unified tabled can be developed for DL configurations.

Observation 8: Currently there are multiple mistakes in the ordering of the listed band combinations in the specification.

Observation 9: The organization of Rel-19 band combination baskets can mitigate some of the issues with the listing/ordering of band combinations in the specification.

Annes A – Study of MSD values in TS 38.101-1

The move to simplify the tables comes from numerous meetings and review of band combinations that have clearly shown the trend of re-using MSD values determined in similar existing band combinations, and if not directly found there’s a mentioning most often of the relation in terms of band groups. Unfortunately, the re-use has also led to several MSD values based on mistakes in past submissions, which will continue to pollute the specifications unless we agree to have a baseline of MSD value requirements as captured in this document.

The following analysis shows all uplink harmonic, harmonic mixing and IMD cases of NR (No SUL) and shows with charts how it can be determined that mistakes are carried forward.

Based on the acceptance to eliminate the identified mistakes, the charts in the following documentation serves for determining the MSD value of the particular MSD type and order as captured in the beginning of the document.

As a simplification step it is proposed to eliminate the description of test frequencies and setup, but instead insert the equation as identified in the MSD analysis for IMD, which determines the direct hit. The equation shall serve RAN5 in terms of determining the relation between band nX (first in the CA declaration) and band nY (second in the CA declaration), which according to the equation must satisfy band nZ, which is either related to band nX or band nY (a duplex offset added to the uplink frequency for FDD). There’s no IMD on TDD bands in 2 CA combinations.

UL harm (UL2/DL1) (Table 7.3A.4-1)

2

1

CA_n2-n48 (R4-1908094)

[Number 1]

Table 6.x.1.5-1: Reference sensitivity exceptions due to UL harmonic for NR CA FR1

| MSD due to harmonic exception for the DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
|  |  | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB |
| n2 | n481, 2 | 27.1 | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.94 | 16.14 | 14.84 | 14.34 | 13.84 |
|  | n483 | 1.9 | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |

Another CA_n25-n48 should have been equal to CA_n2-n48 as also captured in 38.717-02-1

As can be seen in the co-existence studies in 6.8.1.3 there are 2nd harmonics issues from n25 UL into the n48 DL. MSD values are same as for CA_n2A-n48A and need to be inserted into the 38.101-1 tables below.

CA_n25-n48 need to be added in NOTE 1 of Table 7.3A.4-1.

| MSD due to harmonic exception for the DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
|  |  | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB |
| n25 | n481, 2 | 27.1 | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.912 | 16.112 | 14.812 | 14.312 | 13.812 |
|  | n483 | 1.9 | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |

This should be corrected to 23.9dB.

CA_n48-n66 (R4-1908095)

Table 6.32.1.5-1: Reference sensitivity exceptions due to UL harmonic for NR CA FR1

| MSD due to harmonic exception for the DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
|  |  | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB |
| n66 | n481, 2 | 27.1 | 23.9 | 22.1 | 20.9 |  |  | 17.9 | 16.94 | 16.14 | 14.84 | 14.34 | 13.84 |
|  | n483 | 1.9 | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |

This should be corrected to 23.9dB.

CA_n28-n75(R4-2216662)

[Number 2]

Same as CA_n28-n94 (R4-2217040)

Reference sensitivity exceptions of CA_n28-n75 due to UL harmonic for the CA combination are defined in TS38.101-1 and are given in the tables below.

Table 5.12.1.5-1: Reference sensitivity exceptions due to UL harmonic for NR CA FR1

| MSD due to harmonic exception for the DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
|  |  | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB |
| n28 | n751,2 | 28.1 | 25.3 | 24.0 | 22.8 | 21.8 | 21.0 | 19.7 | 18.7 |  |  |  |  |

Table 6.X.1.5-1: Reference sensitivity exceptions and uplink/downlink configurations due to UL harmonic from a PC3 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n28 | n94 | 5 | 15 | 12 (RBstart=0) | 5 | 28.1 | NOTE 2 | UL2/DL1 direct-hit |
| n28 | n94 | 10 | 15 | 50 (RBstart=0) | 50 | 18.7 | NOTE 2 | UL2/DL1 direct-hit |

No arguments or reason why 28.1dB in 5MHz.

UL harm (UL3/DL1) (Table 7.3A.4-1)

2

1

CA_n8-n41 (R4-1814650)

[Number 1]

6.1.1.5		REFSENS requirements

MSD needs to be considered for Band n41 due to 3rd harmonic interference from Band n8.-> R4-1904973

The reference sensitivity exceptions for CA_n8-n41 are proposed in Table 6.9.1.5-1 with the uplink configurations specified in Table 6.9.1.5-2.

Table 6.9.1.5-1: MSD due to UL harmonic for CA_n8-n41

| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n8 | n418,9 | N/A | 13 | 11.3 | 10.1 |  |  | 7.0 | 6.1 | 5.5 | 4.3 | 3.9 | 3.5 |

Suspecting a mistake.

CA_n7-n105 (R4-2306514)

[Number 2]

Based on the co-existence studies n105 uplink harmonic hits n7 downlink. The value is based on DC_7A_n71A, which has similar harmonic issue. -> R4-1900907

There’s a mistake here, because it is not UL3/DL1 but UL4/DL1

Reference sensitivity exceptions for DC_7_n71 are proposed in Table 2-3 with uplink configuration specified in Table 2-4.

Table 2-3: MSD due to UL harmonic for EN-DC in NR FR1

| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n71 | 76,7 | 14.6 | 11.7 | 10.1 | 9 |  |  |  |  |  |  |  |  |

UL harm (UL4/DL1) (Table 7.3A.4-1)

1

CA_n7-n71 (R4-1900907)

[Number 1]

Reference sensitivity exceptions for DC_7_n71 are proposed in Table 2-3 with uplink configuration specified in Table 2-4.

Table 2-3: MSD due to UL harmonic for EN-DC in NR FR1

| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n71 | 76,7 | 14.6 | 11.7 | 10.1 | 9 |  |  |  |  |  |  |  |  |

Will be same as the rest of the UL4/DL1 with 10MHz value. Expecting the current value to be a mistake.

UL harm (UL5/DL1) (Table 7.3A.4-1)

1

CA_n8-n79 (R4-1807126)

[Number 1]

MSD values for Band n79 due to 5th harmonic of Band n8 in CA_n8A-n79A are captured in Table 8.x.4-1 and Table 8.x.4-2.

Table 8.x.4-1: MSD due to harmonic issue for CA_n8-n79

|  | MSD due to harmonic exception for the DL band |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
|  |  | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB |
| n8 | n791,2 |  |  |  |  |  |  | [6.8] | 6.2 | [5.6] | 4.9 | 4.4 |

No reason why this LB – UHB combination deviates from all other LB -UHB UL5/DL1 combination.

Harm. Mixing (UL1/DL2) (Table 7.3A.4-1)

Harm. Mixing (UL1/DL3) (Table 7.3A.4-1)

1

CA_n28-n40 (R4-2005053)

[Number 1]

Additional REFSENS requirements are needed to be defined in tables below in TS 38.101-1 due to 3rd harmonic DL mixing from band n28 UL affecting band n40 DL. Values are reused from CA_28-40-> R4-2208450

No reason why MSD is as found here. Suspecting this is a mistake.

Harm. Mixing (UL1/DL4) (Table 7.3A.4-1)

Harm. Mixing (UL1/DL5) (Table 7.3A.4-1)

Harm. Mixing (UL2/DL3) (Table 7.3A.4-1)

Harm. Mixing (UL3/DL4) (Table 7.3A.4-1)

Harm. Mixing (UL4/DL3) (Table 7.3A.4-1)

2CA – IMD2 (Table 7.3A.5-1)

2

1

CA_n28-n50 (R4-1904562)

[Number 1]

Analysis not in usual level plan style without supporting details of RF block diagram and filter attenuation specifications.

| Component | IMD2(dBm) |  |
| --- | --- | --- |
|  | Main_@Antenna | Div_@Antenna |
| n28 PA Forward mixing interference | -72.3 |  |
| n50 PA Forward mixing interference | -91.7 |  |
| n28 PA Reserve mixing interference | -96.9 |  |
| n50 PA Reserve mixing interference | -123.1 |  |
| n28 Duplexer | -124.4 |  |
| n28 LNA via filters | -75.7 |  |
| n28 LNA via PCB coupling | -89.3 |  |
| Diplexer (L-M) | -73.4 | -104.2 |
| Antenna switch | -84.6 | -102.1 |
| interference From main path |  | -83.6 |
| Total | -68.6 | -83.3 |

CA_n66-n77 (R4-2008354)

[Number 2]

CA_n70-n77 and CA_n70-n78 re-use CA_n66-n77.

Based on above coexistence study, two-tone 2nd order IMD products may fall into the own Rx Band of Band n66. And two-tone 5th order IMD products may fall into Rx Band of Band n66. The MSD for IMD2 and IMD5 are ??? (Are what?)

Table 6.x.2.2-1 lists the MSD required for the dual connectivity configuration due to IMD2 and IMD5.

Table 6.x.2.2-1: MSD due to IMD issue

| Operating Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n66A-n77A | n66 | 1775 | 5 | 25 | 2175 | 31 | FDD | IMD2 |
|  | n77 | 3950 | 10 | 50 | 3950 | N/A | TDD | N/A |
|  | n66 | 1730 | 5 | 25 | 2130 | 5.0 | FDD | IMD5 |
|  | n77 | 3660 | 10 | 50 | 3660 | N/A | TDD | N/A |

Below is CA_n1A-n77A (same DL band) CA_n3A-n77A (Same UL band):

| CA_n1-n77 | 1 | 1950 | 5 | 25 | 2140 | 29.8 | FDD | IMD24 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n77 | 4090 | 10 | 50 | 4090 | N/A | TDD | N/A |
| CA_n3-n77 | n3 | 1740 | 5 | 25 | 1835 | 26 | FDD | IMD24 |
|  | n77 | 3575 | 10 | 50 | 3575 | N/A | TDD | N/A |

It seems questionable that CA_n66-n77 is that much higher and a correct value would be between 26dB and 29.8dB.

Table 10: 2CA IMD2 common requirements

| 2CA | MSD |
| --- | --- |
| IMD2 | 30dB |

1

2

2CA – IMD3 (Table 7.3A.5-1)

CA combinations of the same bandgroup

Same bandgroup relies on isolation and filtration in minimum frequency offset, which gives less attenuation of aggressor.

CA combinations between bandgroups

Different bandgroups has more attenuation due to frequency offset and potentially advantages from antenna isolation, which gives more attenuation of aggressor than those of same bandgroup.

Table 11: 2CA IMD3 common requirements

| 2CA | MSD |
| --- | --- |
| IMD3 same bandgroud | 25dB |
| IMD3 different bandgroup | 15dB |

2CA – IMD4 (Table 7.3A.5-1)

7

11

10

9

8

2

2

7

6

5

4

33

2

1

CA_n2-n48 (R4-1908094)

[Number 1]

Table 6.x.2.2-1 lists the MSD required due to 4th IMD for the dual uplink configuration.

Table 6.x.2.2-1: MSD due to IMD issue

| Operating band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | Operating band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n2A-n48A CA_n2A-n48C | n2 | 1852.5 | 5 | 25 | 1932.5 | [12] | FDD | IMD4 |
|  | n48 | 3625 | 20 | 100 | 3625 | N/A | TDD | N/A |

Brackets indicate that this could have been up for discussion. There’s no argument why 12dB was used.

CA_n3-n5 (R4-2114845)

[Number 2]

Frequencies and MSD value are same as in DC_3_n5 -> R4-1904440

MSD values can be reused from LTE CA_3A-5A -> Introduced in 36.101 v12.6.0 but untraceable how the value is determined (draftCR R4-147978). It is refenced for CA_3-18 and CA-3-26, both also found in the table above.

CA_n3-n20 (R4-2201559)

[Number 3]

Values are reused from CA_3A-20A and DC_3A-n20A. -> Lost in same way as [2] above introduced in 36.101 v12.6.0

CA_n7-n66 (R4-1910199)

[Number 4]

Hence the MSD value for CA_4A-7A can be re-used for CA_n7-n66. -> Lost in same way as [2] above introduced in 36.101 v12.6.0

CA_n12-n66 (R4-2109401)

[Number 5]

The MSD values can be reused from DC_3-n5. . -> Lost in same way as [2] above introduced in 36.101 v12.6.0

CA_n25-n48 (R4-2107700)

[Number 6]

Values from CA_n2A-n48A is reused -> R4-1908094 (Number 1: above of section 4.14)

CA_n28-n74 (R4-2109876)

[Number 7]

We use main/diversity antenna topology as RF front-end architecture in our analysis.

Table 6 MSD requirements for DC_3-20_n8 with UL band 20 and n8

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n28-n74 | n28 | 709 | 5 | 25 | 764 | N/A | N/A |
|  | n74 | 1466 | 5 | 25 | 1514 | 14.6 | IMD4 |

Not everything is clear in this analysis and it has missed the IMD4 into n28.

Marking as a mistake.

CA_n66-n71 (R4-2002635)

[Number 8]

Based on the table above, there is IMD4 falling on top of n66 DL. There is pretty similar combination in EN-DC, DC_66A_n71A where MSD is defined for B66. Because the IMD mechanisms and frequencies are exactly the same, we propose to reuse that MSD number for n66-> R4-1804890

R4-1804890

RAN4#86 had a contribution on DC_66A_(n)71B with an explanation of the value.

A combination of Band 66 and band 71 have similarities to CA_1A-8A and CA_3A-5A. Both these are a low band plus a high band, and both have IMD4 affecting the high band. CA_3A-5A has 4 dB and CA_1A-8A has 6 dB as MSD, and for a combination of band 66 and band 71 the MSD is set to an average value of 5 dB.

Table 6.107.5-1 lists the MSD required for the dual connectivity configuration DC_66A_(n)71B.

Table 6.107.5-1: MSD for the DC_66A_(n)71B configuration

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_66A_(n)71B | 66 | 1750 | 5 | 25 | 2150 | 5 | FDD | IMD4 |
|  | n71 | 678 | 10 | 10 (RBstart =0) | 632 | N/A |  | N/A |

CA_n66A-n85 (R4-2219112)

As seen in the coexistence study 4th IMD product might fall in own Rx band why MSD is needed for 2 band UL. Values from CA_n66A-n71A is reused due to similar frequency range.

CA_n70A-n71(R4-2002635)

Based on the table above, there is IMD4 falling on top of n70 DL. There is pretty similar combination in EN-DC, DC_66A_n71A with same IMD source (2*66 UL-2*71UL=66 DL) where MSD is defined for B66. Because the IMD mechanisms are exactly the same, we propose to reuse that MSD number for n70 recognizing MSD for n70 is not fully optimized as in CA_n70A-n71A MSD hits only partially on top of n70 while in DC_66A_n71A there is full hit.

CA_n26-n78 (R4-2215019)

[Number 9]

reused from DC_26_n77 -> Not found in 37.716-11-11, 37.717-11-11 or 37.718-11-11.

CA_n26-n77 (R4-2217034)

Values are therefore marked as N/A as in the current version of 38.101-1 (v17.6.0).

Table 5.x.2.3-1: MSD due to IMD issue

| Operating band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n26-n7713 | n26 | N/A | N/A | N/A | N/A | N/A | FDD | IMD4 |
|  | n77 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
|  | n26 | N/A | N/A | N/A | N/A | N/A | FDD | IMD5 |
|  | n77 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
| NOTE 13: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |

Why not use CA_n5A-n77A? -> Suggest 8.3dB

CA_n20-n78 ()

[Number 10]

Not found in 38.716-02-00, 38.717-02-01 or 38.718-02-01. Suspecting mistake or re-use of value in CA_n26-n77.

38.718-02-01 does have the NC ULCA IMD4 case as 8.6dB

IMD4 MSD value band n20 is reused from CA_n5-n77, CA_n67-n78 and CA_n71-n77.

Table 5.69.1.5-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n20-n78 | n20 | N/A | 5 | N/A | 800 | 8.6 | FDD | IMD415 |
|  | n7812 | 3350 | 10 | 1 RBSTART=25 | 3350 | N/A | TDD | N/A |
|  |  | 3750 | 10 | 1 RBSTART=25 | 3750 |  |  |  |
| NOTE 12: This band supports intra-band non-contiguous uplink configuration. NOTE 15: This band is subject to IMD6 also which MSD is not specified |  |  |  |  |  |  |  |  |

Marking combination as a mistake.

CA_n41-n71 (R4-2002638)

[Number 11]

MSD value reused from DC_71A_n38A in table below and proposed to be included in TS 38.101-1. -> R4-2001529

Table 7.3A.5-1: 2DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR Band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| CA_n41A-n71A | n41 | 2614 | 5 | 25 | 2614 | N/A | N/A |
|  | n71 | 665 | 5 | 25 | 619 | 11 | IMD4 |

R4-2001529

MSD value reused from 20_n77, 20_n78 and 26_n77. -> Back to same issues as [10] and [9]

Marking as a mistake.

Table 12: 2CA IMD4 common requirements

| 2CA | MSD |
| --- | --- |
| IMD4 | 8.5dB |

2CA – IMD5 (Table 7.3A.5-1)

3 (NC ULCA)

2 (NC ULCA)

1(NC ULCA)

CA_n1-n77 (R4-2010539)

[Number 1]

Table 6.x.2.3-1 lists the MSD required due to 2nd and 4th  IMD for the dual uplink configuration. We use same MSD as DC_1_n77 specified in TS 38.101-3. Note that no MSD for 3rd, 4th and 5th IMD to n77 is specified in TS 38.101-3, and then we follow the result.

| Operating band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | Operating band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1A-n77A | 1 | 1950 | 5 | 25 | 2140 | 29.8 | FDD | IMD24 |
|  |  |  |  |  |  | 32.5 5 |  |  |
|  | n77 | 4090 | 10 | 50 | 4090 | N/A | TDD | N/A |
| CA_n1A-n77A | 1 | 1950 | 5 | 25 | 2140 | 8.0 | FDD | IMD44 |
|  |  |  |  |  |  | 10.75 |  |  |
|  | n77 | 3710 | 10 | 50 | 3710 | N/A | TDD | N/A |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. NOTE 5: Applicable only if operation with 4 antenna ports is supported in the band with EN-DC configured. |  |  |  |  |  |  |  |  |

No IMD5 here, but the Note 4. -> R4-2313802

Based on calculation, IMD 5, IMD6 and IMD7 of UL CA_n77(2A) fall into DLn1. Possible problematic cases are:

- IMD5

As such, the lowest odd and even order cases are IMD5 and IMD6, but only IMD5 is recommended to be specified. The MSD exception is defined as in Table below.

Table 7.3A.5-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC3 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1A-n77A12 | n1 | N/A | 5 | N/A | 2130 | 17 | FDD | IMD5X |
|  | n7712 | 3310 | 10 | 1 RBSTART=25 | 3310 | N/A | TDD | N/A |
|  |  | 3900 | 10 | 1 RBSTART=25 | 3900 | N/A | TDD | N/A |
| NOTE 12: This band supports intra-band non-contiguous uplink configuration. NOTE X: This band is subject to IMD7 also which MSD is not specified. |  |  |  |  |  |  |  |  |

This makes it a NC ULCA.

CA_n41-n66 (R4-2108117)

[Number 2]

Submitted through a CR:

Inter-band combinations have been introduced in release 17 with intra-band UL CA as UL configuration without considering potential interfrence from the IMDs of the two CC allocation. Revision of R4-2107982 with adtion of omitted “IMD7” to CA_n3-n77(2A) case

No analysis and no arguments for 22-33.5dB?

This seems like a rushed case and suggests mistakes (R4-2111016 suggests 22dB).

CA_n66-n77 (R4-2220563)

[Number 3]

The MSD value is taken from the agreed WF.

| Operating band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n66-n77 | n66 | N/A | 5 | N/A | 2197.5 | 15 | FDD | IMD513 |
|  | n7712 | 3305 | 10 | 1 (RBstart=0) | 3305 | N/A | TDD | N/A |
|  |  | 3855 | 10 | 1 (RBstart=8) | 3855 |  |  |  |

Table 13: 2CA IMD5 common requirements

| 2CA | MSD | 2 CA (NC_ULCA) | MSD |
| --- | --- | --- | --- |
| IMD5 | 5.5dB | IMD5 same bgr | 22dB |
|  |  | IMD5 diff. bgr | 17.5dB |

2CA – IMD7 (Table 7.3A.5-1)

1(NC ULCA)

CA_n25-n41 (R4-2108117)

[Number 1]

Submitted through a CR:

Inter-band combinations have been introduced in release 17 with intra-band UL CA as UL configuration without considering potential interfrence from the IMDs of the two CC allocation. Revision of R4-2107982 with adtion of omitted “IMD7” to CA_n3-n77(2A) case

From R4-2107926:

This is the only IMD7 (same band group) case there is so far. Value is found ok.

2CA – IMD9 (Table 7.3A.5-1)

Only reference is CA_n41-n77 (adjacent bandgroups)

3CA – IMD2 (Table 7.3A.5-2)

11

10

9

8

7

6

5

4

3

2

1

CA_n1-n41-n79 (R4-2212449)

[Number 1]

Table 5.x.2.2-1 lists the MSD required for the dual connectivity configuration for the cases that IMD interference fall into the own 3rd Rx frequency band.

Table 5.x.2.2-1: MSD for the CA configuration

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n41-n79 | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n41 | 2530 | 10 | 50 | 2530 | N/A | TDD | N/A |
|  | n79 | 4500 | 40 | 216 | 4500 | 19.0 | TDD | IMD21 |
|  | n1 | 1970 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n79 | 4500 | 40 | 216 | 4500 | N/A | TDD | N/A |
|  | n41 | 2530 | 10 | 50 | 2530 | 29.4 | TDD | IMD21 |
|  | n41 | 2530 | 10 | 50 | 2530 | N/A | TDD | N/A |
|  | n79 | 4690 | 40 | 216 | 4690 | N/A | TDD | N/A |
|  | n1 | 1970 | 5 | 25 | 2160 | 29.9 | FDD | IMD21 |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

MSD in n79 for IMD2 at 19.0 not explained. Suspecting a mistake.

CA_3-7-20 (R4-69AH-0076)

[Number 2]

LTE combination hard to track MSD. Suspecting mistake.

CA_3-7-28 (R4-2113582)

[Number 3]

Referencing DC_3-28_n7 (->R4-1914702)

R4-1914702

Co-existence analysis from DC_28_n7 shows that there is IMD2 impact from DC_28_n7 UL to Band 3 DL. MSD values are reused from DC_3A-7A_n28A and CA_3A-7A-28A with CA_7A-28A as uplink.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A-28A_n7A DC_3C-28A_n7A DC_3A-3A-28A_n7A DC_3A-28A_n7B DC_3C-28A_n7B DC_3A-3A-28A_n7B | 3 | 1737.5 | 5 | 25 | 1832.5 | 26.0 | IMD2 |
|  | 28 | 710.5 | 5 | 25 | 765.5 | N/A | N/A |
|  | n7 | 2543 | 10 | 50 | 2663 | N/A | N/A |
| DC_3A-28A_n7A DC_3C-28A_n7A DC_3A-3A-28A_n7A DC_3A-28A_n7B DC_3C-28A_n7B DC_3A-3A-28A_n7B | 3 | 1747 | 5 | 25 | 1842 | N/A | N/A |
|  | 28 | 741 | 5 | 25 | 796.0 | 20.0 | IMD2 |
|  | n7 | 2543 | 5 | 25 | 2663 | N/A | N/A |

This has an MSD value of 26dB, which makes sense. Why it is 16.5dB today seems to be an error.

CA_n3-n7-n105 (R4-2308103)

[Number 4]

Reused from CA_n3-n7-n28 (3)

CA_n3-n40-n77 (R4-2301078)

[Number 5]

MSD values from CA_n1-n41-n79 are reused. (1)

CA_n5-n78-n79 (R4-2319664)

[Number 6]

Table 5.x.2.2-1 lists the MSD required for the cases that IMD interference fall into the own 3rd Rx frequency band. The MSD values for CA_n28-n46-n78 and CA_n28-n78-n79 are reused. (7)

CA_n28-n77-n79 (R4-2107726)

[Number 7]

From the MSD analysis of DC_28-42_n79. -> R4-1804543

R4-1804543

6.x.5 MSD

TBD

R4-1808114

Table 6.128.5-1 lists the required MSD levels for the dual connectivity configuration.

Table 6.128.5-1: MSD for the DC configuration

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA/NR DC |  | EUTRA/NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode |  |
| DL Configuration | UL Configuration |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| DC_28A-42A_n79A | DC_28A_n79A | 28 | 730 | 5 | 25 | 785 | - | FDD | - |
|  |  | 42 | 3420 | 5 | 25 | 3420 | 15.3 | TDD | IMD3 |
|  |  | n79 | 4880 | 40 | 216 | 4880 | - |  | - |
|  | DC_42A_n79A | 28 | 745 | 5 | 25 | 800 | 16.2 | FDD | IMD2 |
|  |  | 42 | 3597.5 | 5 | 25 | 3597.5 | - | TDD | - |
|  |  | n79 | 4420 | 40 | 216 | 4420 | - |  | - |

No explanation why 16.2dB for this IMD2 product. Suspecting an error.

CA_n28-n78-n79 (R4-2200355)

[Number 8]

Same MSD apply with CA_n28_n77-n79 (7)

CA_n34-n41-n79 (R4-2320102)

[Number 9]

Based on co-existence studies additional MSD is needed to be defined, shown in table 5.x.2.2-1.

Table 5.x.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n34-n41-n79 | n34 | 2020 | 5 | 25 | 2020 | N/A | N/A |
|  | n41 | 2660 | 5 | 25 | 2660 | N/A | N/A |
|  | n79 | 4680 | 40 | 216 | 4680 | 19.3 | IMD2 |
|  | n34 | 2020 | 5 | 25 | 2020 | N/A | N/A |
|  | n41 | 2550 | 5 | 25 | 2550 | 27.2 | IMD2 |
|  | n79 | 4570 | 40 | 216 | 4570 | N/A | IN/A |
|  | n34 | 2015 | 5 | 25 | 2015 | 7.5 | IMD5 |
|  | n41 | 2515 | 5 | 25 | 2515 | N/A | N/A |
|  | n79 | 4780 | 40 | 216 | 4780 | N/A | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

No explanation why 19.3dB for this IMD2 product. Suspecting an error.

CA_n3-n7-n67 (R4- 2303599)

[Number 10]

MSD value from CA_n3-n7-n28 is reused (3)

Not a single case of the outliers has been identified in the attempt to understand why the MSD value is as specified. The amount of re-use expose how errors and mistakes easily scales inside the specification work.

CA_n2A-n48A-n66A(R4-1908724)

[Number 11]

CA_n2A-n48A-n66A from LTE 32dB (36.716-03-02, R4-1903035 (TBD), R4-1908724 has 28.3dB)

CA_n2A-n66A-n77A from DC_2A_n66A-n78A(R4-2015081), (doesn’t hit n2!) -> mistake

CA_n25A-n48A-n66A (Same as CA_n2A-n48A-n66A)

CA_n25A-n66A-n77A (Same as CA_n25A-n66A-n77A)

Marking these as a mistake.

3CA – IMD3(Table 7.3A.5-2)

17 (x2)

16

15

11

14

13

12

10

9 (x2)

7 (x2): LB-LB

8

6

5:LB-LB

4

3

2

1

CA_n1-n7-n40 (R4-2209942)

[Number 1]

re-used from similar IMD product in DC_1A-7A_n40A. (->R4-2005146)

MSD is derived from similar combinations already captured for CA in 36.101 and EN-DC in 38.101-3. In this case CA_1A-7A-40A, DC_7_n40 and DC_7A-40A_n1A.

Only CA_1A-7A-40A can have the value for n7 of the mentioned references. Therefore, this lead ends at this (R4-162331) and no information here shows the reason for the MSD value.

| Channel bandwidth |  |  |  |  |  |  |  |  | Applicable active UL band |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |  |
| CA_1A-7A-40A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 1 |
|  | 7 |  |  |  | -94.7 | -92.9 | -91.7 |  |  |
|  | 40 |  |  | -92.6 | -90.5 | -89.2 | -88.1 | TDD |  |
| CA_1A-7A-40A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD | 7 |
|  | 7 |  |  |  | -94.7 | -92.9 | -91.7 |  |  |
|  | 40 |  |  | -96 | -93.3 | -91.7 | -90.6 | TDD |  |
| CA_1A-7A-40A | 1 |  |  | -91.7 | -89.5 | -87.9 | -86.9 | FDD | 40 |
|  | 7 |  |  |  | -94 | -92.4 | -91.2 |  |  |
|  | 40 |  |  | -99.2 | -96.2 | -94.4 | -93.2 | TDD |  |

CA_n1-n41-n77 (R4-2201042)

[Number 2]

reused from DC_1A-41A_n77A and DC_1A_n41A-n77A (-> R4-2014073)

values can reuse the values for DC_1A_n41A-n78A. (->R4-2003940)

This is the wrong value (19.6 != 22.4). The TP contains a complete MSD calculation.

Table 3 MSD calculation for victim Rx of band n78

| Component | IMD3(dBm) |  |
| --- | --- | --- |
|  | Main_@Antenna | Div_@Antenna |
| B1 PA Forward mixing interference | -94.0 |  |
| n41 PA Forward mixing interference | -64.0 |  |
| B1 PA Reserve mixing interference | -98.0 |  |
| n41 PA Reserve mixing interference | -67.5 |  |
| Diplexer (M-M) | -117 |  |
| n78 LNA via filters | -73.5 |  |
| n78 LNA via PCB coupling | -104.0 |  |
| Diplexer (L-H) | -111 | -141.5 |
| Antenna switch | -82 | -106 |
| interference From main path |  | -72.0 |
| Total | -62.0 | 72.0 |

Table 4 MSD requirements for DC_1_n41-n78 with UL band 1 and n41

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_1_n41-n78 | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | n41 | 2650 | 10 | 25 | 2650 | N/A | N/A |
|  | n78 | 3330 | 10 | 50 | 3330 | 22.4 | IMD3 ¦2*fn41-fB1¦ |

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A_n41A-n78A | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |
|  | n41 | 2515 | 10 | 50 | 2515 | 12 | IMD4 |
|  | n78 | 3410 | 10 | 50 | 3410 | N/A | N/A |
|  | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | n41 | 2650 | 10 | 25 | 2650 | N/A | N/A |
|  | n78 | 3330 | 10 | 50 | 3330 | 22.4 | IMD3 ¦2*fn41-fB1¦ |
|  | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | n41 | 2520 | 10 | 25 | 2520 | N/A | N/A |
|  | n78 | 3390 | 10 | 50 | 3390 | 10.1 | IMD4 ¦fn41-3*fB1¦ |
|  | 1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | n41 | 2520 | 10 | 50 | 2520 | N/A | N/A |
|  | n78 | 3620 | 10 | 50 | 3620 | 3.8 | IMD5 ¦3*fn41-2*fB1¦ |

Strangely it diverts from CA_n3A-n41A-n78A, which would have a band 3 PA of same data as used for band 1. CA_n3-n41-n78 has 16.8dB MSD.

CA_n1-n77-n79 (R4-2010580)

[Number 3]

-	3rd, 4th and 5th order IMD generated by dual uplink of CA_n77-n79 may fall into part of own band n1.

=><NOTE: There is no related studies on this IMD issue in past. MSD values will be updated. >

Table 5.1.x.4-1: MSD for the CA configuration

| NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_n1A-n77A-n79A | n1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n77 | 3410 | 10 | 50 | 3410 | N/A | N/A |
|  | n79 | 4870 | 40 | 216 | 4870 | 15.9 | IMD3 |
|  | n1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n77 | 4670 | 40 | 216 | 4670 | N/A | N/A |
|  | n79 | 3490 | 10 | 50 | 3490 | 4.6 | IMD5 |
|  | n1 | 1950 | 5 | 25 | 2140 | TBD | IMD3 |
|  | n77 | 3400 | 10 | 50 | 3400 | N/A | N/A |
|  | n79 | 4660 | 40 | 216 | 4660 | N/A | N/A |

R4-2101198

1. Regarding MSD value due to IM3 for CA_n1-n77-n79, average the newly provided value with two values provided at the last meeting and revise it as the latest value.

2. Modify the MSD value due to IM3 for CA_n1-n78-n79 to be the same as CA_n1-n77-n79.

| CA_n1A-n77A-n79A | n1 | 1950 | 5 | 25 | 2140 | 16.0 | FDD | IMD31,2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n77 | 3400 | 10 | 50 | 3400 | N/A | TDD | N/A |
|  | n79 | 4660 | 40 | 216 | 4660 | N/A | TDD | N/A |

This is the correct value in R4-2101198 supported by also (R4-2101158 @16.8dB). Blind track after that – no reason why MSD value is 6dB today.

CA_n3-n5-n7 (R4-2113579)

[Number 4]

MSD value n5 are derived from DC_3A-18A_n41A. (-> R4-2100345)

Table 5.1.x.4-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n5-n7 | n3 | 1780 | 5 | 25 | 1875 | N/A | FDD | N/A |
|  | n5 | 845 | 5 | 25 | 890 | N/A | FDD | N/A |
|  | n7 | 2505 | 10 | 50 | 2625 | 30.0 | FDD | IMD24 |
|  | n3 | 1720 | 5 | 25 | 1815 | N/A | FDD | N/A |
|  | n5 | 835 | 5 | 25 | 880 | 19.0 | FDD | IMD3 |
|  | n7 | 2560 | 10 | 50 | 2680 | N/A | FDD | N/A |
| NOTE 4: This band is subject to IMD3 also which MSD is not specified. |  |  |  |  |  |  |  |  |

R4-2100345

Table 5.x.4-1 shows the required MSD:

Table 5.x.4-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| E-UTRA and NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL Configuration | EUTRA and NR band | UL Fc (MHz) | UL BW (MHz) | UL CLRB | DL Fc (MHz) | DL BW (MHz) | MSD (dB) | IMD order |
| DC_3-18_n41 | 18 | 820 | 5 | 25 | 865 | 5 | N/A | N/A |
|  | n41 | 2670 | 5 | 25 | 2670 | 5 | N/A | N/A |
|  | 3 | 1755 | 5 | 25 | 1850 | 5 | 28.8 | IMD2 |
|  | 3 | 1725 | 10 | 50 | 1820 | 10 | N/A | N/A |
|  | n41 | 2565 | 10 | 50 | 2685 | 10 | N/A | N/A |
|  | 18 | 830 | 5 | 25 | 875 | 5 | 19.0 | IMD3 |

No reason why MSD value is 19dB today, but the UL BW has changed without the MSD value changing.

CA_n5-n28-n105 (R4-2313378)

[Number 5]

LB-LB combination. These suffer from difficulty in isolation at small frequency distances at same front-end modules. Value of 25dB is justified.

CA_n5-n78-n79 (R4-2319664)

[Number 6]

The MSD values for CA_n28-n46-n78 and CA_n28-n78-n79 are reused. -> R4-2200355

Table 5.x.2.2-1: 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n5-n78-n79 | n5 | 846 | 5 | 25 | 891 | N/A | FDD | N/A |
|  | n78 | 3790 | 10 | 50 | 3790 | N/A | TDD | N/A |
|  | n79 | N/A | 40 | N/A | 4363 | 26.2 | TDD | IMD2 |
|  | n5 | 827 | 5 | 25 | 872 | N/A | FDD | N/A |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | TDD | N/A |
|  | n79 | N/A | 40 | N/A | 4959 | 22 | TDD | IMD3 |
|  | n5 | 827 | 5 | 25 | 872 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3593 | 26.9 | FDD | IMD2 |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | FDD | N/A |
|  | n5 | 827 | 5 | 25 | 872 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3326 | 17 | FDD | IMD3 |
|  | n79 | 4980 | 40 | 216 | 4980 | N/A | FDD | N/A |
|  | n5 | N/A | 5 | N/A | 880 | 16.2 | FDD | IMD2 |
|  | n78 | 3550 | 10 | 50 | 3550 | N/A | FDD | N/A |
|  | n79 | 4430 | 40 | 216 | 4430 | N/A | FDD | N/A |
|  | n5 | N/A | 5 | N/A | 875 | 3 | FDD | IMD5 |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | FDD | N/A |
|  | n79 | 4520 | 40 | 216 | 4520 | N/A | FDD | N/A |

R4-2200355

-	2nd, 3rd and 5th order IMD generated by dual uplink of CA_n28-n78 may fall into part of own band n79.

This MSD value is the average of the analysis results of the two companies. Also, we define only the highest MSD, and omit the MSDs due to IM3 and IM5

- 2nd and 3rd order IMD generated by dual uplink of CA_n28-n79 may fall into part of own band n78

This MSD value is the average of the analysis results of the two companies. Also, we define only the highest MSD, and omit the MSD due to IM3.

Table 5.1.6.4-1: MSD for the CA configuration

| NR Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| CA_n28A-n78A-n79A | n28 | 740 | 5 | 25 | 795 | N/A | N/A |
|  | n78 | 3700 | 10 | 50 | 3700 | N/A | N/A |
|  | n79 | 4440 | 40 | 216 | 4440 | 26.2 | IMD21,3,4 |
|  | n28 | 740 | 5 | 25 | 795 | N/A | N/A |
|  | n78 | 3700 | 10 | 50 | 3700 | 26.9 | IMD23,4 |
|  | n79 | 4440 | 40 | 216 | 4440 | N/A | N/A |
|  | n28 | 745 | 5 | 25 | 800 | 16.2 | IMD21 |
|  | n78 | 3620 | 10 | 50 | 3620 | N/A | N/A |
|  | n79 | 4420 | 40 | 216 | 4420 | N/A | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: This band is subject to IMD4 also which MSD is not specified. NOTE 3: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. NOTE 4: This band is subject to IMD3 also which MSD is not specified. |  |  |  |  |  |  |  |

Similar combinations can be found suggesting that there has been an analysis of the issue in IMD2 when the two bands n78 and n79 in close vicinity creates products in combination with a low band.

| DC_19A_n78A-n79A | 19 | 835 | 5 | 25 | 880 | N/A | N/A |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | n78 | 3680 | 10 | 50 | 3680 | N/A | N/A |
|  | n79 | N/A | 40 | N/A | 4515 | 29.3 | IMD2 |
|  | 19 | 835 | 5 | 25 | 880 | N/A | N/A |
|  | n79 | 4550 | 40 | 216 | 4550 | N/A | N/A |
|  | n78 | N/A | 10 | N/A | 3715 | 28.8 | IMD2 |

Marking as a mistake on MSD value for IMD3

CA_n8-n20-n28 (R4-2306468)

[Number 7]

LB-LB-LB combination, so there’s two MSD values here. These suffer from difficulty in isolation at small frequency distances at same front-end modules in the low bands. Values of 24.3dB and 24dB are justified.

CA_n8-n40-n78 (R4-2210403)

[Number 8]

They are re-used from similar IMD product in DC_8A-40A_n78A for IMD in band n8 and n40. For IMD2 MSD in band n78, similar value is used as from DC_20A_n41A-n78A (->R4-2010559)

R4-2010559

As stated in 5.x.3, for MSD requirement caused by IMDs is specified below accordingly.

Table 7.3B.2.3.5.2-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_8A-40A_n78A | 8 | 905 | 5 | 25 | 950 | 30.5 | IMD2 |
|  | 40 | 2380 | 5 | 25 | 2380 | N/A | N/A |
|  | n78 | 3330 | 10 | 50 | 3330 | N/A | N/A |
| DC_8A-40A_n78A | 8 | 890 | 5 | 25 | 935 | 19.8 | IMD3 |
|  | 40 | 2320 | 5 | 25 | 2320 | N/A | N/A |
|  | n78 | 3705 | 10 | 50 | 3705 | N/A | N/A |
| DC_8A-40A_n78A | 8 | 910 | 5 | 25 | 955 | N/A | N/A |
|  | 40 | 2395 | 5 | 25 | 2395 | 28 | IMD2 |
|  | n78 | 3305 | 10 | 50 | 3305 | N/A | N/A |

No reason for 19.8dB found. Comparing to DC_5A-40A_n78A the MSD would be 15.2dB, where the test is defined just 55MHz lower in downlink test frequency.

| DC_5A-40A_n78A DC_5A-40C_n78A | 5 | 835 | 5 | 25 | 880 | 15.2 | IMD3 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_5A-40A_n78C | 40 | 2310 | 5 | 25 | 2310 | N/A | N/A |
| DC_5A-40C_n78C | n78 | 3740 | 10 | 50 | 3740 | N/A | N/A |

Marking as a mistake.

CA_n25-n71-n77 (R4-2016761, R4-2106496)

[Number 9]

MSD values n77 are derived from DC_2A_n71A-n78A.

R4-2102052

Based on Table 5.3-1 in TR 37.717-11-21, there is IMD3 produced by Band 2 and NR band n71 that impact the reference sensitivity of NR band n78.

The required MSD levels and test configuration are shown in the following Table.

Table 6.69.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for DC in NR FR1 (three bands)

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A_n71A-n78A | 2 | IMD3 | ¦fB2 +2*fn71¦ | 1907.5 | 5 | 25 | 1987.5 | 5 | N/A |
|  | n71 |  |  | 695.5 | 5 | 25 | 649.5 | 5 |  |
|  | n78 |  |  | 3305 | 10 | 50 | 3305 | 10 | 8.0 |

There is no argumentation why 8dB was applied.

CA_n28-n39-n79 (R4-2314784)

[Number 10]

Based on co-existence studies additional MSDs are needed to be defined, shown in table 5.x.2.2-1

| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplexer Mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n28-n39-n79 | n28 | 715 | 5 | 25 | 770 | N/A | FDD | N/A |
|  | n39 | 1902.5 | 5 | 25 | 1902.5 | N/A | TDD | N/A |
|  | n79 | 4520 | 40 | 216 | 4520 | 6.7 | TDD | IMD3 |

There is no argumentation why 6.7dB was applied.

CA_n28-n40-n77 (R4-2301079)

[Number 11]

MSD values from CA_n28-n40-n78 are reused.(See CA_n28-n40-n78, no IMD3 defined for this case)

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n28-n40-n77 | n28 | 745.5 | 5 | 25 | 800.5 | 11 | FDD | IMD31 |
|  | n40 | 2302.5 | 5 | 25 | 2302.5 | N/A | TDD | N/A |
|  | n77 | 3795 | 10 | 50 | 3795 | N/A | TDD | N/A |

CA_n28-n40-n78 (R4-2119753)

[Number 12]

IMD3 MSD of DC_28_40_n78 is reused for bands n28 and n40 and IMD3 MSD of CA_n12-n30-n77 for  n77 is reused for n78. -> R4-2105315

R4-2105315

MSD value for 28 is chosen as in DC_20A-40A_n78A -> R4-2103006

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_28A-40A_n78A DC_28A-40C_n78A | 28 | N/A | 5 | 25 | 800.5 | 11 | IMD3 |
|  | 40 | 2302.5 | 5 | 25 | 2302.5 | N/A | N/A |
|  | n78 | 3795 | 10 | 50 | 3795 | N/A | N/A |

R4-2103006

It is proposed to re-use IMD3 MSD value from DC_8-40-n78 which is very similar -> R4-2011618

R4-2011618

The requirements are captured as 19.8dB and not 11dB. The requirements remain in v18.4.0.

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_8A-40A_n78A | 8 | 890 | 5 | 25 | 935 | 19.8 | IMD3 |
|  | 40 | 2320 | 5 | 25 | 2320 | N/A | N/A |
|  | n78 | 3705 | 10 | 50 | 3705 | N/A | N/A |

The number of 11dB has been wrongly set in all combinations traversing back towards CA_n28-n40-n77.

CA_n28-n41-n79 (R4-2112859)

[Number 13]

The MSD values caused by the intemodulation are defined as:

Table 5.3.x.4-1 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n28-n41-n79 | n28 | 725 | 5 | 25 | 780 | 13.0 | FDD | IMD31 |
|  | n41 | 2600 | 10 | 50 | 2600 | N/A | TDD | N/A |
|  | n79 | 4600 | 40 | 216 | 4600 | N/A | TDD | N/A |
|  | n28 | 720 | 5 | 25 | 780 | N/A | FDD | N/A |
|  | n41 | 2600 | 10 | 50 | 2600 | N/A | TDD | N/A |
|  | n79 | 4480 | 40 | 216 | 4600 | 10.1 | TDD | IMD32 |
|  | n28 | 735 | 5 | 25 | 790 | N/A | FDD | N/A |
|  | n41 | 2645 | 10 | 50 | 2645 | 10.4 | TDD | IMD4 |
|  | n79 | 4850 | 40 | 216 | 4850 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

There is no argumentation why 10.1dB was applied.

CA_n28-n46-n78 (R4- 2118543)

[Number 14]

Table 5.1.x.4-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n28A-n46A-n78A CA_n28A-n46C-n78A CA_n28A-n46D-n78A | n28 | 710 | 5 | 25 | 765 | N/A | FDD | N/A |
|  | n46 | 5170 | 20 | 100 | 5170 | N/A | FDD | N/A |
|  | n78 | 3750 | 10 | 50 | 3750 | 17 | TDD | IMD31 |
|  | n28 | 725 | 5 | 25 | 780 | 16 | FDD | IMD3 |
|  | n46 | 5900 | 20 | 100 | 5900 | N/A | FDD | N/A |
|  | n78 | 3340 | 10 | 50 | 3340 | N/A | TDD | N/A |
|  | n28 | 740 | 5 | 25 | 795 | N/A | FDD | N/A |
|  | n46 | 5900 | 20 | 100 | 5900 | 22 | TDD | IMD31,2 |
|  | n78 | 3320 | 10 | 50 | 3320 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD4 also which MSD is not specified. NOTE 2: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

There is no argumentation why 22dB was applied.

Marking as a mistake.

CA_n28-n78-n102 (R4-2310354)

[Number 15]

MSD values from CA_n28-n46-n78 are reused .(Same as number 14)

Marking as a mistake.

CA_n41-n77-n79 (R4-2212456)

[Number 16]

Table 5.x.2.2-1 lists the MSD required for the dual connectivity configuration for the cases that IMD interference fall into the own 3rd Rx frequency band.

Table 5.x.2.2-1: MSD for the CA configuration

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n41-n77-n79 | n77 | 3600 | 10 | 50 | 3600 | N/A | TDD | N/A |
|  | n79 | 4600 | 40 | 216 | 4600 | N/A | TDD | N/A |
|  | n41 | 2600 | 10 | 50 | 2600 | 10.7 | TDD | IMD31,2 |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. NOTE 2: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |  |

There is no argumentation why 10.7dB was applied.

CA_n66-n77-n85 (R4-2306524)

[Number 17]

The MSD values for CA_n12-n66-n77 and are reused.

Table 5.x.2.2-1: MSD for the CA configuration

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode | Source of IMD |
| CA_n66-n77-n85 | n66 | 1720 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n77 | 4180 | 10 | 50 | 4180 | N/A | TDD | N/A |
|  | n85 | 710 | 5 | 25 | 740 | 23.5 | FDD | IMD35 |
|  | n66 | 1724 | 5 | 25 | 2124 | 21.4 | FDD | IMD3 |
|  | n77 | 3540 | 10 | 50 | 3540 | N/A | TDD | N/A |
|  | n85 | 708 | 5 | 25 | 738 | N/A | FDD | N/A |
| NOTE 5: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |

But it is not the case that they are reused. The MSD should have been 15.2dB and 13.2dB.

Marking as a mistake

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination |  | NR band | UL Fc (MHz) |  | UL/DL BW (MHz) | UL LCRB |  | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n12-n66-n77 | n12 |  | N/A | 5 |  |  | N/A | 740 | 15.2 | FDD | IMD35 |
|  | n66 |  | 1720 | 5 |  |  | 25 | 2120 | N/A | FDD | N/A |
|  | n77 |  | 4180 | 10 |  |  | 50 | 4180 | N/A | TDD | N/A |
|  | n12 |  | 707 | 5 |  |  | 25 | 737 | N/A | FDD | N/A |
|  | n66 |  | N/A | 5 |  |  | N/A | 2126 | 13.2 | FDD | IMD3 |
|  | n77 |  | 3540 | 10 |  |  | 50 | 3540 | N/A | TDD | N/A |
|  | n12 |  | 704 | 5 |  |  | 25 | 734 | N/A | FDD | N/A |
|  | n66 |  | 1723 | 5 |  |  | 25 | 2123 | N/A | FDD | N/A |
|  | n77 |  | N/A | 10 |  |  | N/A | 4150 | 16.0 | TDD | IMD31,2,5 |

3CA – IMD4(Table 7.3A.5-2)

9

8

7

6

5

4

3

2

1

CA_n1-n5-n79 (R4-2319660)

[Number 1]

The MSD values for CA_n1A-n28A-n79A and CA_n3-n28-n79 are reused -> R4-2200206

R4-2200206

For the exception, the same exception values in DC_1A-28A_n79A and DC_1A_n28A-n79A are used-> R4-1804532

R4-1804532

6.x.5	MSD

TBD. -> R4-1806666

R4-1806666

| DC_1A-28A_n79A | 1 | IMD3 | ¦2*fB1 –fn79¦ | 1930 | 5 | 25 | 2120 | 5 | 1.3 | N/A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n79 |  |  | 4648 | 40 | 216 | 4648 | 40 |  |  |
|  | 28 |  |  | 733 | 5 | 25 | 788 | 5 |  | 15.2 |
|  | 1 | IMD4 | ¦2*fB1 –2*fn79¦ | 1925 | 5 | 25 | 2115 | 5 | 0.9 | N/A |
|  | n79 |  |  | 4980 | 40 | 216 | 4980 | 40 |  |  |
|  | 28 |  |  | 740 | 5 | 25 | 795 | 5 |  | 10.0 |
|  | 28 | IMD4 | ¦3*fB28 –fn79¦ | 745.5 | 5 | 25 | 800.5 | 5 | 0 | N/A |
|  | n79 |  |  | 4420 | 40 | 216 | 4420 | 40 |  |  |
|  | 1 |  |  | 1977.5 | 5 | 25 | 2167.5 | 5 |  | 1.2 |

No reason but seems to be due to band edge (near-miss).

CA_n1-n28-n79 (R4-2200206)

[Number 2]

Same as Number 1 (R4-1806666)

CA_n3-n78-n105 (R4-2310360)

[Number 3]

It is not possible to select frequency points that combine to a direct hit in the victim RX band. It is therefore omitted (3795MHz – 3*666MHz = 1797MHz, which is 11MHz below the centre frequency of the lowest n3 DL channel)

Table 5.x.2.3-1: MSD due to IMD issue

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n78-n105 | n3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | n78 | 3740 | 10 | 50 | 3740 | 17.3 | TDD | IMD44 |
|  | n105 | 670 | 5 | 25 | 619 | N/A | FDD | N/A |

There is no argumentation why 17.3dB but explained this is not a direct hit. This is a mistake to declare 17.3dB and looking at CA_n66-n71-n78 IMD4 it has 9dB, which could have been used.

| CA_n66-n71-n78 | n66 | 1720 | 5 | 25 | 2120 | N/A | FDD | N/A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | n71 | 668 | 5 | 25 | 622 | N/A | FDD | N/A |
|  | n78 | N/A | 10 | N/A | 3724 | 9 | TDD | IMD41 |

Marking as a mistake.

CA_n5-n25-n66(R4-2011677)

[Number 4]

IMD4 issue is similar to CA_2A-5A-66A, the same MSD value is reused. -> R4-164189-> R4-2107209

R4-2107209

MSD is based on already completed EN-DC_2A-66A_n5A -> R4-1906426

R4-1906426

The MSD value is based on predefined value for LTE CA_2A-2A-5A-66A-66A / CA_2A-5A_66A -> R4-2003244

R4-2003244

Summary of interference studies

Table 1 in [1] shows 3 bands DL with 2 bands UL CA band combinations with self-interference problems in Rel-16 for RAN4 #94-e online meeting.

Table 1: Summary of self-interference analysis for 3 bands DL with 2 bands UL CA

| Downlink CA configuration | Uplink CA configuration | Harmonic relation to 3rd band without uplink | Intermodulation to 3rd band without uplink | Interference due to small frequency separation | MSD (Maximum Sensitivity Degradation) |
| --- | --- | --- | --- | --- | --- |
| CA_2A-2A-5A-66A-66A CA_2A-5A-66A, CA_2A-5A-66B, CA_2A-5A-66C, CA_2A-5B-66A, CA_2A-5B-66B, CA_2A-5B-66C, CA_2A-2A-5A-66A, CA_2A-2A-5A-66B, CA_2A-2A-5A-66C, CA_2A-5A-66A-66A, CA_2A-5B-66A-66A | CA_2A-5A | - | 4th IMD | - | 7.2 dB |

No reason why 7.2dB has been determined. This document contains also source for many other similar combinations as CA_2A-13A-66A, CA_n2A-n5A-n66A

CA_n8-n39-n79(R4-2308177)

[Number 5]

MSD requirements are reused form DC_8_n39-n79.-> R4-2110455

Table 5.x.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n8-n39-n79 | n8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | n39 | 1890 | 10 | 50 | 1890 | N/A | N/A |
|  | n79 | 4680 | 40 | 216 | 4680 | 15.9 | IMD3 |
|  | n8 | 890 | 5 | 25 | 935 | N/A | N/A |
|  | n39 | 1890 | 10 | 50 | 1890 | N/A | N/A |
|  | n79 | 4560 | 40 | 216 | 4560 | 12.1 | IMD4 |
|  | n8 | 897.5 | 5 | 25 | 942.5 | N/A | N/A |
|  | n39 | 1907.5 | 10 | 50 | 1907.5 | 13.8 | IMD4 |
|  | n79 | 4600 | 40 | 216 | 4600 | N/A | N/A |
|  | n8 | 895 | 5 | 25 | 940 | 15.1 | IMD3 |
|  | n39 | 1900 | 10 | 50 | 1900 | N/A | N/A |
|  | n79 | 4740 | 40 | 216 | 4740 | N/A | N/A |

R4-2110455

6.x.5	MSD

Based on co-existence studies additional MSD is needed to be defined, shown in table 6.x.5-1.

Table 6.x.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_8A_n39A-n79A | 8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | n39 | 1890 | 10 | 50 | 1890 | N/A | N/A |
|  | n79 | 4680 | 40 | 216 | 4680 | 15.9 | IMD3 |
| DC_8A_n39A-n79A | 8 | 890 | 5 | 25 | 935 | N/A | N/A |
|  | n39 | 1890 | 10 | 50 | 1890 | N/A | N/A |
|  | n79 | 4560 | 40 | 216 | 4560 | 12.1 | IMD4 |
| DC_8A_n39A-n79A | 8 | 897.5 | 5 | 25 | 942.5 | N/A | N/A |
|  | n39 | 1907.5 | 10 | 50 | 1907.5 | 13.8 | IMD4 |
|  | n79 | 4600 | 40 | 216 | 4600 | N/A | N/A |

There is no argumentation why 13.8dB was applied.

Suspecting and marking as a mistake.

CA_n13-n25-n66(R4-2103093)

[Number 6]

reuse the value of DC_2A-13A_n66A -> Same source as Number 5(R4-2011677)

CA_n28-n39-n79 (R4-2314784)

[Number 7]

Based on co-existence studies additional MSDs are needed to be defined, shown in table 5.x.2.2-1.:

Table 5.x.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplexer Mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n28-n39-n79 | n28 | 727.5 | 5 | 25 | 782.5 | N/A | FDD | N/A |
|  | n39 | 1902.5 | 5 | 25 | 1902.5 | N/A | TDD | N/A |
|  | n79 | 4980 | 40 | 216 | 4980 | 4.0 | TDD | IMD41 |
|  | n28 | 725 | 5 | 25 | 780 | 8.5 | FDD | IMD4 |
|  | n39 | 1900 | 5 | 25 | 1900 | N/A | TDD | N/A |
|  | n79 | 4920 | 40 | 216 | 4920 | N/A | TDD | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

n79 IMD4 is a band edge case, so the 4dB is considering only partial interference.

CA_n39-n40-n79 (R4-2010644)

[Number 8]

The required MSD is shown in the table 5.1.x.4-1, where the MSD of EN-DC_39A_n40A-n79A are re-used.-> R4-2003797

Table 5.1.x.4-1: MSD due to IMD4

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n39A-n40A-n79A | n39 | 1917.5 | 5 | 25 | 1917.5 | N/A | TDD | N/A |
|  | n40 | 2302.5 | 5 | 25 | 2302.5 | N/A | TDD | N/A |
|  | n79 | 4980 | 40 | 216 | 4980 | 5.8 | TDD | IMD4 |

R4-2003797

The required MSD is shown in the following table.

Table 6.73.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_39A_n40A-n79A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_39A_n40A-n79A | 39 | IMD4 | ¦fB39-3*fn40¦ | 1917.5 | 5 | 25 | 1917.5 | N/A |
|  | n40 |  |  | 2302.5 | 5 | 25 | 2302.5 |  |
|  | n79 |  |  | 4980 | 40 | 216 | 4980 | 5.8 |

n79 IMD4 is a band edge case, so the 5.8dB is considering only partial interference.

CA_n48-n66-n71 (R4-2114865)

[Number 9]

In the case of IMD4 in n48 at upper band edge the requirement is relaxed by 3dB compared to CA_n5-n66-n77

Table 5.1.x.4-1: MSD for the CA configuration

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n48A-n66A-n71A | n48 | 3552.5 | 10 | 50 | 3552.5 | N/A | TDD | N/A |
|  | n66 | 1761.5 | 5 | 25 | 2161.5 | 14.4 | FDD | IMD3 |
|  | n71 | 695.5 | 5 | 25 | 649.5 | N/A | FDD | N/A |
|  | n48 | 3695 | 10 | 50 | 3695 | 5.2 | TDD | IMD4 |
|  | n66 | 1712.5 | 5 | 25 | 2112.5 | N/A | FDD | N/A |
|  | n71 | 665.5 | 5 | 25 | 619.5 | N/A | FDD | N/A |

n48 IMD4 is a band edge case, so the 5.2dB is considering only partial interference.

3CA – IMD5(Table 7.3A.5-2)

5

4

3

2

1

1: CA_n1-n3-n40 (R4-2301075)

[Number 1]

MSD value from CA_1-3-40 is reused. -> R4-166912

Table 5.x.2.2-1: 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n3-n40 | n1 | 1950 | 5 | 25 | 2140 | N/A | FDD | N/A |
|  | n3 | 1735 | 5 | 25 | 1830 | N/A | FDD | N/A |
|  | n40 | 2380 | 5 | 25 | 2380 | 8.0 | TDD | IMD5 |

R4-166912

Table 1 shows the merged MSD results from interested companies [1][2][3][4]. The required MSD levels are derived by taking average values of proposed MSD levels from interested companies. If there are outliers, the sources for differences will be discussed and applicability for averaging considered case by case.

Table 1 MSD results for 3DL/2UL CA UE with self-interference issues

| DL CA | UL CA | IMD |  | Qualcomm | LGE | Huawei | Avg. MSD [dB] | Proposed MSD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B1+B3+B40 | B1 | IMD5 | 3*fB1 -2*fB3 | 15.6 | 0.5 |  | 8.05 | [8.0] |
|  | B3 |  |  |  |  |  |  |  |

This seems like it could be challenged.

It is the same value as the IMD4 for CA_n1-n3-n40, which is 8dB, and the CA_n1-n3-41 has 5dB MSD, which seems more like the right value also considering the rest of the IMD5 cases.

Marking as a mistake.

CA_n1-n5-n7 (R4-2112753)

[Number 2]

MSD value n5 are derived from CA_1A-5A-7A.-> R4-155377

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n5-n7 | n1 | 1968 | 5 | 25 | 2158 | N/A | FDD | N/A |
|  | n7 | 2512 | 10 | 50 | 2632 | N/A | FDD | N/A |
|  | n5 | 835 | 5 | 25 | 880 | 1.0 | FDD | IMD5 |

R4-155377

Table 1 MSD results for 3DL/2UL CA UE with self-interference issues

| CA configuration | Uplink CA configuration | Order | Qualcomm (R4-153360) |  | Intel (R4-154121) |  | LG (R4-154885) |  | Nokia (Shared offline) |  | Avg |  | Proposed MSD [dB] | Delta |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | CF [dB] | MSD [dB] | CF [dB] | MSD [dB] | CF [dB] | MSD [dB] | CF [dB] | MSD [dB] | CF [dB] | MSD [dB] |  |  |
| CA_1A_5A_7A | CA_1A_7A | IMD5 | 8.5 | 0 | 4.3 | 2.3 | 4.37 | 0.22 |  | 0.6 | 5.72 | 0.78 | 1.0 | 2.30 |
| CA_3A_7A_20A | CA_3A_7A | IMD2 | 2.8 | 7.5 | 2.4 | 12.3 | 2.63 | 16.1 |  | 7 | 2.61 | 10.73 | [10.5] | 9.10 |
|  | CA_3A_20A | IMD3 | 0.1 | 7.4 | 0.8 | 8.1 | 1.2 | 10.3 |  | 7 | 0.70 | 8.20 | 8.0 | 3.30 |
|  |  | IMD2 | 1.3 | 27.9 | 0.1 | 26.7 | 0.29 | 23.8 |  |  | 0.56 | 26.13 | 26.0 | 4.10 |

This number of 1dB has been suggested between 0dB (R4-153360)

Covers also:

CA_n1-n7-n8(R4-21209941) re-used from similar IMD product in CA_n1A-n5A-n7A falling into band n5A.-> R4-155377

CA_n1-n7-n105 (R4-2317692) For MSD value capturing the IMD5 in n105 a similar IMD5 case is found in CA_n1-n7-n8.-> R4-21209941 -> R4-155377

CA_n3-n40-n41(R4-2000475)

The required MSD are shown in the following table.

Table 5.1.2.5-1 MSD exception for 3DL/2UL NR CA configuration

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-40A-n41A | n3 | 1747.5 | 5 | 25 | 1842.5 | 1.0 | FDD | IMD5 |
|  | n40 | 2347.5 | 5 | 25 | 2347.5 | N/A | TDD | N/A |
|  | n41 | 2600 | 10 | 50 | 2600 | N/A | TDD | N/A |

There is no argumentation why 1dB was applied.

CA_n2-n66-n77 (R4-2015081)

[Number 3]

IMD issue is the same as DC_2A_n66A-n78A, these MSD values are reused.

Table 5.X.4.4-1: MSD for the CA configuration

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n2A-n66A-n77A | n2 | 1880 | 5 | 25 | 1960 | 2.1 | FDD | IMD5 |
|  | n66 | 1760 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n77 | 3620 | 10 | 50 | 3620 | N/A | TDD | N/A |

The referenced combination cannot serve this number, since the DC combination cannot have uplink in both NR bands. Looking instead in R4-1903647.

R4-1903647

- 5th order IMD generated by dual uplink of Band 66 + Band n78 may fall into own Rx of band 2

Table5.1.33.4-3 propose the required MSD levels for the DC configuration.

Table 5.1.33.4-3: Proposed MSD requirements

| E-UTRA and NR Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | EUTRA and NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_2A-66A_n78A DC_2A-66A-66A_n78A | 2 | 1880 | 5 | 25 | 1960 | [2.1] | FDD | IMD5 ¦2*fBn78-3*fB66¦ |
|  | 66 | 1760 | 5 | 25 | 2160 | N/A | FDD | N/A |
|  | n78 | 3620 | 10 | 52 | 3620 | N/A | TDD | N/A |

This TP contains data on used components and isolation/attenuation assumptions. Using these values we get 4.5dB and not 2.1dB, since the MSD is dominated by the LNA iP5 of -10dBm and the PCB isolation of 50dB.

Covers also:

CA_n25-n66-n77(R4-2103100)

IMD issues are similar to CA_n2A-n66A-n77A, the same MSD are used for CA_n25A-n66A-n77A

CA_n66-n70-n78(R4-2205694)

MSD values for DL n70 can be derived from CA_n2-n66-n77

CA_n34-n39-n40 (R4-2317706)

[Number 4]

Based on co-existence studies on 5.x.2.1, additional MSD is needed to be defined, shown in table 5.x.2.2-1.

Table 5.x.2.2-1: 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n34-n39-n40 | n34 | 2022.5 | 5 | 25 | 2022.5 | N/A | TDD | N/A |
|  | n39 | 1882.5 | 5 | 25 | 1882.5 | N/A | TDD | N/A |
|  | n40 | N/A | 5 | N/A | 2302.5 | 2.4 | TDD | IMD5 |

No explanation why 2.4dB IMD5, but it is a band edge case (2022.5 and 1882.5), so the 2.4dB is considering only partial interference.

CA_n34-n41-n79 (R4-2320102)

[Number 5]

Based on co-existence studies additional MSD is needed to be defined, shown in table 5.x.2.2-1.

Table 5.x.2.2-1: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n34-n41-n79 | n34 | 2020 | 5 | 25 | 2020 | N/A | N/A |
|  | n41 | 2660 | 5 | 25 | 2660 | N/A | N/A |
|  | n79 | 4680 | 40 | 216 | 4680 | 19.3 | IMD2 |
|  | n34 | 2020 | 5 | 25 | 2020 | N/A | N/A |
|  | n41 | 2550 | 5 | 25 | 2550 | 27.2 | IMD2 |
|  | n79 | 4570 | 40 | 216 | 4570 | N/A | IN/A |
|  | n34 | 2015 | 5 | 25 | 2015 | 7.5 | IMD5 |
|  | n41 | 2515 | 5 | 25 | 2515 | N/A | N/A |
|  | n79 | 4780 | 40 | 216 | 4780 | N/A | N/A |
| NOTE 1: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

There is no argumentation why 7.5dB was applied. There’s no DC counterpart. Suspecting an error.

Marking as a mistake.

3CA – IMD7(Table 7.3A.5-2)

- References

- RP-240782, Moderator's summary on RAN4 spec quality, RAN4 chair (Huawei).

- R4-2406709, WF on UERF_Spec_Improvement, Qualcomm

- R4-2405036, On UE RF specifications Improvement, Nokia

- RP-240263, Progress update on CA framework overhaul, Nokia

- R4-2409191, On RAN4 basket WI work planning, Nokia