3GPP TSG-RAN WG4 Meeting # 111						R4-2409196

Fukuoka Meeting, May 20th – May 24th, 2024

Title:	TP to TR 38.850 to add HP-NRCA n1-n3

Source:	Nokia, BT plc

Agenda item:	6.18.2

Document for:	Approval

# 1	Introduction

## This contribution is a text proposal for TR38.850 for adding PC2 of n3 in CA_n1A-n3A.

This contribution is a text proposal to introduce PC2 n3 for DL CA_n1A-n3A.

There is no cross-band isolation MSD for PC3 n3 for DL CA_n1A-n3A and there’s no uplink harmonic of harmonic mixing MSD.

************************************* Start of TP*****************************************

## 5.x	CA_n1-n3

### 5.x.1	UE maximum output power

Table 5.5A.3.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n3A | n38 CA_n1A-n3A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n1 | 5, 10, 15, 20 | 2 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40 |  |
|  |  | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 8: Minimum requirements for Power Class 2 are applicable for this uplink combination with 1Tx antenna connector in each band or single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination NOTE 10: Only single uplink carriers with power class other than PC3 are listed. |  |  |  |  |

### 5.x.2	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 5.x.2.0	General

For PC3, CA_ n1A-n3A has no cross-band isolation MSD for UL n3.

For PC3, CA_ n1A-n3A has no uplink harmonic or harmonic mixing MSD for UL n3.

#### 5.x.2.1	Reference sensitivity requirements with PC2 on n3 without TxD

For CA_ n1A-n3A, this would follow already specified reference sensitivity requirements.

************************************* End of TP******************************************