3GPP TSG-RAN WG4 Meeting # 111						R4-2409202

Fukuoka Meeting, May 20th – May 24th, 2024

Title:	TP to TR 38.850 to add HP-NRCA n3-n7-n28

Source:	Nokia, BT plc

Agenda item:	6.18.2

Document for:	Approval

# 1	Introduction

## This contribution is a text proposal for TR38.850[1] for adding PC2 of n3 and n7 in CA_n3A-n7A-n28A.

************************************* Start of TP*****************************************

## 5.x	CA_n3-n7-n28

### 5.x.1	UE maximum output power

Table 5.x.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n7A-n28A | n37 n77 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  | n37 n77 CA_n3A-n7A CA_n3A-n28A CA_n7A-n28A | n3 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 2 |
|  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  | n28 | 5, 10, 15, 20 |  |
| NOTE 7: Power Class 2 is allowed for this uplink combination or single uplink carrier in this downlink/uplink combination |  |  |  |  |

### 5.x.2	Reference sensitivity requirements

For single UL PC2 n3 in CA_n3A-n7A:

-    The harmonic uplink and mixing interference of n7 does not fall into Rx frequencies of n3.

-    Cross band isolation interference of PC2 n7 does not fall into n3.

For single UL PC2 n7 in CA_n3A-n7A:

-    The harmonic uplink and mixing interference of n3 does not fall into Rx frequencies of n7.

-    Cross band isolation interference of PC2 n3 does not fall into n7.

For single UL PC2 n3 in CA_n3A-n28A:

-    The harmonic uplink and mixing interference of n3 does not fall into Rx frequencies of n28.

-    Cross band isolation interference of PC2 n3 does not fall into n28.

For single UL PC2 n7 in CA_n7A-n28A:

-    The harmonic uplink and mixing interference of n7 does not fall into Rx frequencies of n28.

-    Cross band isolation interference of PC2 n7 does not fall into n28.

#### 5.x.2.1	Reference sensitivity requirements with PC2 on n3 and n7 without TxD

Void

#### 5.x.2.2	Reference sensitivity requirements with PC2 on n3 and n7 with TxD

Void

### 5.3.4	∆TIB and ∆RIB values

Not applicable

************************************* End of TP******************************************

# References

[1] 3GPP TR38.850 “Rel-18 High power UE (power class 2) for a single FR1 NR FDD band in UL of NR intra-band and inter-band CA/DC combinations with y bands downlink (y=1,2,3,4,5,6) and x bands uplink (x=1); (Release 18)”, v1.3.0