3GPP TSG-RAN WG4 Meeting #111	R4-2409275

Fukuoka, Japan, 20 – 24 May, 2024

Title: 	On performance requirements for RedCap positioning

Source: 	Huawei, HiSilicon

Agenda item:	7.12.2.4

Document for:	Discussion

Introduction

RRM performance requirements for RedCap positioning are discussed in RAN4#110-bis, and the outcomes are captured in [1]. Based on [1], further discussions are needed for the following issues.

- Measurement accuracy

- RRM test case

In this paper, we will provide our views on RRM performance requirements for RedCap positioning.

Discussion

Measurement accuracy

RAN4 needs to decide on the BW grouping for accuracy requirements considering the BW limit of RedCap UE. The max CH BW that can be supported by RedCap UE is 20MHz in FR1 and 100MHz in FR2. The max Tx BW configuration corresponding to different CH BWs are copied in Table 1 and Table 2.

Table 1: max Tx BW configuration for different CH BWs in FR1

| SCS (kHz) | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 15 | 25 | 52 | 79 | 106 | 133 | 160 | 188 | 216 | 242 | 270 | N/A | N/A | N/A | N/A | N/A |
| 30 | N/A | 11 | 24 | 38 | 51 | 65 | 78 | 92 | 106 | 119 | 133 | 162 | 189 | 217 | 245 | 273 |
| 60 | N/A | N/A | 11 | 18 | 24 | 31 | 38 | 44 | 51 | 58 | 65 | 79 | 93 | 107 | 121 | 135 |

Table 2: max Tx BW configuration for different CH BWs in FR2

| SCS (kHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB |
| 60 | 66 | 132 | 264 | N.A |
| 120 | 32 | 66 | 132 | 264 |

For non-FH case, we suggest to define the BW grouping based on BW grouping in existing requirements, and the BW groups exceeding the max CH BW of RedCap UE should be removed. Besides, the “≥” should be removed for the max RB numbers. For example for 60kHz in FR1, we have one BW group ≥24 RB in existing requirements, but 24 RB is already the max PRS BW for RedCap UE, so the “≥” is removed.

Proposal 1: Define accuracy requirements for non-FH case based on following BW groups.

- 15kHz: ≥24, ≥[52] and [104] RBs

- 30kHz: ≥24 and [48] RBs

- 60kHz (FR1): [24] RBs

- 60kHz (FR2): ≥24, ≥[64] and [132] RBs

- 120kHz: ≥32 and [64] RBs

For 2RX, the existing accuracy numbers for the corresponding BW groups can be re-used. For 1RX, RAN4 needs to derive accuracy numbers from simulation results.

Proposal 2: For accuracy requirements for non-FH case,

- 2RX: re-use existing accuracy numbers for the corresponding BW groups

- 1RX: derive accuracy numbers from simulation results

For FH case, RAN4 agreed to define accuracy requirements based on combination (total measured PRS BW during all hops, per-hop BW). We suggest to define the requirements based on the simulation assumption [2] (related parameters are copied blow). This, combined with Table 1 and Table 2, gives following BW groups.

| UE measurement bandwidth in each hop | 15kHz = 10MHz 30kHz = 20MHz 60kHz (FR1) = 20MHz 60kHz (FR2) = 50MHz 120kHz = 100MHz |
| --- | --- |
| UE total measurement bandwidth after all hops | 15kHz = 50MHz (6 hops) 30kHz = 100MHz (6 hops) 60kHz (FR1) = 100MHz (6 hops) 60kHz (FR2) = 200MHz (5 hops) 120kHz = 400MHz (5 hops) |

Proposal 3: Define accuracy requirements for FH case based on following BW groups.

- 15kHz: per-hop BW ≥ 52 RB, total BW = 268 RB

- 30kHz: per-hop BW ≥ 51 RB, total BW = 272 RB

- 60kHz (FR1): per-hop BW ≥ 24 RB, total BW = 132 RB

- 60kHz (FR2): per-hop BW ≥ 66 RB, total BW = 264 RB

- 120kHz: per-hop BW ≥ 66 RB, total BW = 264 RB

In core part RAN4 agreed on the minimum PRS BW expected to be measured with Rx hopping (BWtotal).

- The minimum PRS BW expected to be measured with Rx hopping in RRC_INACTIVE/IDLE state is given by - minBWPRS,Nhops∙BWhop-BWoverlap∙Nhops-1 where - BWPRS is determined by the minimum among the configured PRS BW, UE capability reported as [component 1 of the FG 41-5-1], and the total BW of all hops requested by LMF. - BWhop is the BW per hop signalled in the UE capability. - BWoverlap is the minimum hop overlap signalled in the UE capability. - Nhops is the number of Rx hops measured by the UE in RRC_INACTIVE/IDLE state within a single time window.

In order to apply the accuracy requirements, the PRS configuration should allow UE to do enough number of hops such that the resulted BWtotal is no less than the total BW in Proposal 3. This mainly relates to provision of enough number of repetitions. This should be captured as a condition for the accuracy requirements.

Proposal 4: For accuracy requirements for non-FH case, derive accuracy numbers from simulation results. The requirements apply provided that the BWtotal defined in clause core requirements is no less than the total BW in Proposal 3.

RRM test case

For RRM testing, RAN4 has defined two sets of PRS RMCs, one with small BW and one with large BW.

- 15kHz: 24 RB and 104 RB

- 30kHz: 24 RB and 132 RB

- 120kHz: 32 RB and 128 RB

For measurement delay TCs, for non-FH case the RMC with small BW can be re-used; for FH case new RMC with large BW needs to be defined. Existing RMC with large BW are without repetition as existing accuracy requirements for large PRS BW do not require repetition, but repetition is needed for FH.

Proposal 5: Use the following PRS RMC for measurement delay TCs

- Non-FH: small BW (existing RMC)

- FH: large BW with repetitions (new RMC)

For measurement accuracy TCs, in existing TCs both PRS BW are used (in different sub-tests) to verify UE performance with small and large PRS BWs. For non-FH case we can follow same principle, and a new PRS RMC can be defined with medium BW for 30kHz and 120kHz, e.g. 48 RB and 64 RB (for 15kHz existing RMC with 104 RB can still be used). For FH case, following Proposal 3 only one RMC for the large BW is needed, and the same RMC is also used for the delay TCs.

Proposal 6: Use the following PRS RMC for measurement accuracy TCs

- Non-FH (2 BWs): small BW (existing RMC) and medium BW (104/48/64 RB for 15/30/120kHz)

- FH (1 BW): large BW with repetitions (new RMC)

In RAN4#109, the BW in test configuration for positioning TCs in R16 and R17 were updated. Based on TE vendors’ feedback, BW in test configuration for TCs are considered as both cell and UE CH BW, and PRS BW cannot be larger than it. For FH TCs, the PRS BW needs to be larger than the UE CH BW, so approach in existing TCs does not work. RAN4 needs to discuss handle PRS BW for FH TCs, e.g. two options are listed for Proposal 7. Besides, RAN4 also needs to clarify the definition of Io in the TCs, i.e. over which symbols and BWs Io is calculated.

Proposal 7: Discuss how to handle PRS BW for FH TCs and clarify the definition of Io.

- Option 1: consider the BW in test configuration as UE BW and cell BW can be larger

- Option 2: consider PRS are transmitted from non-serving cells (the cell BW of the non-serving cells need to be defined), and BW in test configuration is only for serving cell.

Conclusions

In this paper we provided our views on RRM performance requirements for RedCap positioning.

Proposal 1: Define accuracy requirements for non-FH case based on following BW groups.

- 15kHz: ≥24, ≥[52] and [104] RBs

- 30kHz: ≥24 and [48] RBs

- 60kHz (FR1): [24] RBs

- 60kHz (FR2): ≥24, ≥[64] and [132] RBs

- 120kHz: ≥32 and [64] RBs

Proposal 2: For accuracy requirements for non-FH case,

- 2RX: re-use existing accuracy numbers for the corresponding BW groups

- 1RX: derive accuracy numbers from simulation results

Proposal 3: Define accuracy requirements for FH case based on following BW groups.

- 15kHz: per-hop BW ≥ 52 RB, total BW = 268 RB

- 30kHz: per-hop BW ≥ 51 RB, total BW = 272 RB

- 60kHz (FR1): per-hop BW ≥ 24 RB, total BW = 132 RB

- 60kHz (FR2): per-hop BW ≥ 66 RB, total BW = 264 RB

- 120kHz: per-hop BW ≥ 66 RB, total BW = 264 RB

Proposal 4: For accuracy requirements for non-FH case, derive accuracy numbers from simulation results. The requirements apply provided that the BWtotal defined in clause core requirements is no less than the total BW in Proposal 3.

Proposal 5: Use the following PRS RMC for measurement delay TCs

- Non-FH: small BW (existing RMC)

- FH: large BW with repetitions (new RMC)

Proposal 6: Use the following PRS RMC for measurement accuracy TCs

- Non-FH (2 BWs): small BW (existing RMC) and medium BW (104/48/64 RB for 15/30/120kHz)

- FH (1 BW): large BW with repetitions (new RMC)

Proposal 7: Discuss how to handle PRS BW for FH TCs and clarify the definition of Io.

- Option 1: consider the BW in test configuration as UE BW and cell BW can be larger

- Option 2: consider PRS are transmitted from non-serving cells (the cell BW of the non-serving cells need to be defined), and BW in test configuration is only for serving cell.

Reference

- R4-2406383, WF on RedCap positioning and PRS/SRS bandwidth aggregation, Ericsson

- R4-2314460, Simulation assumptions for RedCap positioning with FH, Ericsson