3GPP TSG-RAN WG4 Meeting #111	R4-2409281

Fukuoka, Japan, 20 – 24 May, 2024

Title: 	Discussion on remaining issues in RRM requirements for MUSIM

Source: 	Huawei, HiSilicon

Agenda item:	7.15.1

Document for:	Discussion

Introduction

Maintenance of RRM core requirements for MUSIM gaps are discussed in RAN4#110-bis, and the outcomes are captured in [1]. Based on [1], further discussions are needed for following remaining issues.

- Mandatory MUSIM gap patterns or constraints on MUSIM gap request from UE side

- Collision with union of MUSIM gap and MG

In this paper, we will provide our views on RRM core requirements for MUSIM gaps.

Discussion

Mandatory MUSIM gap patterns or constraints on MUSIM gap request from UE side

Issue 1-1-1: Mandatory MUSIM gap patterns or constraints on MUSIM gap request from UE side - Proposals - P1: No need to introduce mandatory MUSIM gap patterns and constraints on MUSIM gap request from UE side (Apple oppo xiaomi Huawei MTK) - P2: Define 1 or 2 mandatory MUSIM gap patterns, as minimum the UE shall support MUSIM gap 6ms MGL and 160ms MGRP (Nokia ZTE) - P3: UE support at least one MUSIM gap pattern within a subset of MUSIM gap patterns and UE shall know the preferred MUSIM gap patterns from NW before UE requesting the MUSIM gaps.(Ericsson) - P4: For compromise, when UE requests more than one periodic MUSIM gaps, at least one MUSIM gap has a MGRP larger than x ms where x could be 1280 (vivo ZTE) - P5: Discuss whether an LS is needed to RAN5 confirming that RAN4 assumption is reasonable. (Nokia) Recommendations:

The issue has been discussed in Rel-17, and there was no consensus. Our view is still that no need to define mandatory MUSIM gap patterns. Gap pattern to use for MUSIM is up to UE to request which is further depending on NW B configuration, and it is not the case that all NW B operations can be done with a single MUSIM gap pattern. In addition, and RAN2 has agreed that NW cannot configure a different gap pattern than what UE requests, so we do not see the need to define mandatory gap patterns for MUSIM.

We also do not support to define constraints on MUSIM gap request from UE side. It cannot really address the inconsistence between MUSIM gap pattern requested by the UE and that supported by NW A. On the other hand, it restricts the flexibility on how UE would use the MUSIM gaps for NW B. It is noted that NW A always has the option to not configure one or more MUSIM gaps requested by the UE, if it considers small MGRP of MUSIM gaps will cause too much impact to NW A.

Proposal 1: RAN4 not to introduce mandatory MUSIM gap patterns, or define constraints on MUSIM gap request from UE side.

Collision with union of MUSIM gap and MG

Issue 1-1-2: Scenarios for the case where the MO to be measured without MG have to be measured in the associated MG - Proposals - P1: When UE performs a measurement without gap which is partially overlapping with the MG but fully overlapping with the union of the NW-A’s gap and MUSIM gaps, UE shall perform the measurement within MG. (Apple oppo Ericsson vivo Huawei) - P1-1: RAN4 shall clarify which gap to be used for this measurement, especially when association between MO and MG is not supported (UE doesn’t support R17 concurrent gaps). (Apple) Recommendations: FFS

For L3 measurement outside MG, Kp is defined to account for the collision between SMTC or CSI-RS with MG or MUSIM gap. For example, below is a copy of Kp definition for SSB based intra-frequency outside MG in 9.2.5.1. One remaining issue is what happens when SMTC is fully overlapping with union of MUSIM gap and MG, resulting in Navailable = 0.

Kp is the scaling factor for an SSB frequency layer to be measured without GAP. Kp = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows: - For a window W of duration max(SMTC period, xRP_max), where xRP_max is the maximum xRP across all configured per-UE measurement GAPs, periodic MUSIM gaps, and/or per-FR measurement GAPs within the same FR as the SSB frequency layer, and starting from the beginning of any SMTC occasion: - Ntotal is the total number of SMTC occasions within the window, including those overlapped with GAP and MUSIM gap occasions within the window, and - Navailable is the number of SMTC occasions that are not overlapped with any non-dropped GAP or non-dropped MUSIM gap occasions within the window W, after accounting for measurement GAP and MUSIM gap collisions by applying the collision rules for GAP and MUSIM gap in section 9.1.8.3 and 9.1.10.6, respectively. Kp = 1 when Navailable = 0. - xRP = MGRP when configured GAP is activated Pre-MG or MG, and xRP = VIRP when configured GAP is NCSG. Requirements in this clause do not apply when Navailable = 0 due to fully overlapping between SMTC occasions and MUSIM gap occasions within the window W. Editor Note: FSS for the case when Navailable = 0 due to fully overlapping between SMTC occasions and the union of MUSIM gap and measurement gap occasions within the window W.

In R17 con-MG, similar scenario is discussed where SMTC is fully overlapping with union of con-MGs, and the agreement is that UE shall measure the MO in the associated MG. It is straightforward to follow the same principle when SMTC is fully overlapping with union of MUSIM gap and MG, i.e. the MO should be measured in the associated MG.

It is noted that following the new structure of spec, the requirements for such a scenario should be defined in 9.2.6 rather than 9.2.5, and the applicability should be defined in 9.2.1.

Proposal 2: The MO that can be measured without MG should be measured in the associated MG if the SMTC is fully overlapping with union of MUSIM gap and MG.

Conclusions

In this paper we provided our views on RRM core requirements for MUSIM gaps.

Proposal 1: RAN4 not to introduce mandatory MUSIM gap patterns, or define constraints on MUSIM gap request from UE side.

Proposal 2: The MO that can be measured without MG should be measured in the associated MG if the SMTC is fully overlapping with union of MUSIM gap and MG.

Reference

- R4-2406435, WF on NR Dual TxRx Multi-SIM, Moderator (vivo)