3GPP TSG-RAN WG4 Meeting # 111	R4-2409428

Fukuoka City, Fukuoka , Japan, 20th – 24th May, 2024

Agenda Item:	10.5.2

Source: 		Qualcomm Incorporated

Title: 	On FR2 OTA test method on STxMP

Document for:	Approval

# Introduction

The new study on FR2 OTA testing enhancements was approved in RAN #103 meeting with the following objectives [1]. In RAN4#110bis, the WF on NR FR2 OTAin [2] was approved.

In this paper, we provide our views on FR2 OTA test method on transmission with multi-panel (STxMP) testing.

# 2. Discussion

## 2.1 General

Based on the agreements on work scope in [2], RAN4 will focus on the test method for configured transmitted power requirements in this SI. The compatibility with Maximum Output power and MPR/A-MPR could be considered for future proofing.

For the configured transmitted power requirement specified in section 6.2K.4 in [3], there are three target test cases:

- Peak EIRP PUMAX,f,c,k: Measure peak EIRP per TCI

- Aggregated EIRP PUMAX,f,c: Measure aggregated EIRP over all the directions shall satisfy the regulatory limit EIRPmax

- Aggregated TRP	PTMAX,f,c : Integration of aggregated EIRP.

### 6.2K.4 Configured transmitted power for simultaneous transmission to multiple directions A UE configured for simultaneous transmission to multiple directions can configure two maximum output powers. The configured UE maximum output power PCMAX,f,c,k for each of joint/UL TCI states k (k=0,1) indicated for simultaneous transmission to multiple directions of carrier f and serving cell c is defined as that available to the reference point of a given transmitter branch that corresponds to the reference point of the higher-layer filtered RSRP measurement as specified in TS 38.215 [11]. The configured UE maximum output power PCMAX,f,c,k shall be set such that the corresponding measured peak EIRP PUMAX,f,c,k is within the following bounds PPowerclass + ΔPIBE – MAX(MAX(MPRf,c,k, A- MPRf,c,k,) + ΔMPRSTxMP + ΔMBP,n, P-MPRf,c,k) – MAX{T(MAX(MPRf,c,k, A- MPRf,c,k,) + ΔMPRSTxMP), T(P-MPRf,c,k)} – ΔTSTxMP ≤ PUMAX,f,c,k ≤ EIRPmax and PUMAX,f,c, the corresponding measured peak EIRP for carrier f of a serving cell c, aggregated over all indicated joint/UL TCI states in a given direction, satisfies over all directions PUMAX,f,c ≤ EIRPmax while the corresponding measured total radiated power PTMAX,f,c is bounded by PTMAX,f,c ≤ TRPmax with PPowerclass the UE minimum peak EIRP as specified in sub-clause 6.2K.1, EIRPmax the applicable maximum EIRP as specified in sub-clause 6.2.1, MPRf,c,k as specified in sub-clause 6.2K.2 , A-MPRf,c,k as specified in sub-clause 6.2K.3, ΔMBP,n the peak EIRP relaxation as specified in clause 6.2.1 and TRPmax the maximum TRP for the UE power class as specified in sub-clause 6.2.1. ΔPIBE is 1.0 dB if UE declares support for mpr-PowerBoost-FR2-r16, UL transmission is QPSK, MPRf,c = 0 and when NS_200 applies and the network configures the UE to operate with mpr-PowerBoost-FR2-r16 otherwiseΔPIBE is 0.0 dB. ΔMPRSTxMP is [3.0] dB if two TCI states are indicated for simultaneous transmission to multiple directions, 0.0 dB otherwise. ΔTSTxMP is specified in sub-clause 6.2K.1. The requirement is verified in beam peak direction. P-MPRf,c,k is the power management maximum output power reduction P-MPRf,c for each of indicated joint/UL TCI states. P-MPRf,c is defined in clause 6.2.4. The tolerance T(ΔP) for applicable values of ΔP (values in dB) is specified in Tables 6.2.4-1 and 6.2.4-2.

To measure the EIRP per TCI, it is necessary to distinguish EIRP from two TCIs and record EIRP separately [4]. Per the feedback from TE vendors in RAN4#110bis, TE can distinguish between EIRP from two TCIs with two layers transmission. FFS on the side condition such as SNR, power imbalance.

Issue 2-2-1: TE capability to measure/distinguish EIRP per TCI - Proposals - Proposal 1: TE vendors to provide the feedback on the capability of measure/distinguish EIRP per TCI in STxMP testing. - Proposal 1a: Reuse the same solution to address beam overlap issue in Multi-Rx also for Multi-Tx, i.e., configure rank 2 PUSCH transmission in EIRP measurement of STxMP - Agreement: - TE vendors confirmed the capability of distinguishing EIRP per TCI with two layers transmission under SDM scheme - The feasibility of this approach under all signal level conditions (e.g. SNR, power imbalance) is FFS

For SNR side condition, per our understanding, it is to define the ratio of received power signal to noise floor in each TE’s receiver. Since the measurement is for peak EIRP and EIRP value per TCI should be similar to legacy EIRP measurement, there should be no issue for signal level of SNR side condition.

Proposal 1: For TE’s capability on distinguishing EIRP per TCI with two layers transmission, no issue for signal level of SNR condition is identified.

For power imbalance, it is not clear whether the power imbalance is to define the difference between received power from a first TCI and a second TCI. It is proposed to clarify the definition of power imbalance.

Proposal 2: To clarify the definition of power imbalance condition for EIRP per TCI measurement.

The UE Beamlock test function was specified in [6] to make the UE lock the UE antenna pattern once it forms a beam towards the SS. While Beamlock test function was assuming only one beam is formed. For STxMP, it is necessary to make sure UE Beamlock test function can lock UE two antenna patterns simultaneously.

Proposal 3: Send a LS to RAN5 extending current UE Beamlock test function to support locking two antenna patterns simultaneously.

## 2.2 EIRP PUMAX,f,c,k testing

With the assumption that TE has the capability of distinguishing EIRP per TCI, the remaining testability issues for peak EIRP PUMAX,f,c,k testing are listed as follows:

- Issue#1: How to select the proper AoA pair

- Issue#2: Measurement grid

For Issue#1 in peak EIRP per TCI measurement, since it is UE’s minimum EIRP requirement, it is straightforward to follow the same approach as Multi-Rx, i.e., UE to declare one AoA pair from the set of {30deg, 60deg, 90deg, 120deg, 150deg} and proper UE orientation listed in [3].

Proposal 4: Consider UE declaration approach for AoA pair selection from the set of {30deg, 60deg, 90deg, 120deg, 150deg} and proper UE orientation listed in [3] for EIRP per TCI measurement for peak EIRP PUMAX,f,c,k testing.

Proposal 5: FFS on whether declared AoA pair and UE orientation for STxMP and Multi-Rx are same or not.

For Issue#2 in peak EIRP per TCI measurement, as concluded in Multi-Rx test setup [7], the constant-step measurement grid should be used. For multi-Rx PC3, the step size is 15deg which is same as the step size of legacy EIS measurement in [3]. Additionally, based on the measurement grid study for multi-Rx in [7], there is not much difference between 10deg and 15deg. Therefore, it is reasonable to reuse the same step size of legacy EIRP measurement for peak EIRP PUMAX,f,c,k testing. That says for PC1 and PC5, the step size for peak EIRP PUMAX,f,c,k testing is 15deg.

Proposal 6: Constant-step measurement grid should be used for peak EIRP PUMAX,f,c,k testing.

Proposal 7: 15deg is used as step size for peak EIRP PUMAX,f,c,k testing.

## 2.2 Aggregated EIRP PUMAX,f,c testing

For PUMAX,f,c testing, the main issue is how to make sure aggregated EIRP over all the directions satisfy the regulatory limits, EIRPmax.

From regulatory PoV, to verify UE’s aggregated EIRP and TRP, exhaustive search for all the possible AoA separations shall be measured but it is very time consuming and infeasible as there are only several AoA pairs supported by current setup.

Observation 1: From regulatory PoV, exhaustive search for EIRP per TCI with all the possible AoA separations and UE orientations shall be measured but it is very time consuming and infeasible as there are only several AoA pairs supported by current test setup.

Therefore, it is suggested RAN4 to study and select the worst case of AoA pair from the set of {30deg, 60deg, 90deg, 120deg, 150deg} and potential UE orientation listed in [3].

Proposal 8: RAN4 to study and select worst case of AoA pair from the set of {30deg, 60deg, 90deg, 120deg, 150deg} and potential UE orientation listed in [3] for PUMAX,f,c testing.

In [5], an alternative test method for aggregated EIRP PUMAX,f,c testing is proposed, i.e., if the peak EIRP of single carrier is smaller than EIRPmax – 3dB, the corresponding EIRPmax verification can be skipped.

Proposal 9: The test method for aggregated EIRP PUMAX,f,c testing, i.e., if the peak EIRP of single carrier is smaller than EIRPmax – 3dB, the corresponding EIRPmax verification can be skipped, can be adopted.

Similar as EIRP PUMAX,f,c,k testing, constant-step measurement grid should be used. The step size for PC1 and PC5 legacy EIRP testing should be used for aggregated EIRP PUMAX,f,c testing.

Proposal 10: Constant-step measurement grid should be used for aggregated EIRP PUMAX,f,c testing.

Proposal 11: 15deg is used as step size for aggregated EIRP PUMAX,f,c testing.

## 2.2 Aggregated TRP PTMAX,f,c

For Aggregated TRP PTMAX,f,c, testing, the issue is similar as aggregated EIRP PUMAX,f,c testing. With the worst-case conclusion in section 2.2, aggregated TRP can be calculated by integrating EIRP per TCI.

Observation 2: With the worst-case conclusion in section 2.2, aggregated TRP can be calculated by integrating EIRP per TCI.

Proposal 12: The worst-case conclusion for aggregated EIRP PUMAX,f,c testing should apply for aggregated TRP PTMAX,f,c, testing.

Proposal 13: The test method for aggregated TRP PTMAX,f,c testing, i.e., if the TRP of single carrier is smaller than TRPmax – 3dB, the corresponding TRPmax verification can be skipped, can be adopted.

# 3.	Conclusion

In this paper, we provide our views on test methodology for STxMP and have the following observations and proposals:

Proposal 1: For TE’s capability on distinguishing EIRP per TCI with two layers transmission, no issue for signal level of SNR condition is identified.

Proposal 2: To clarify the definition of power imbalance condition for EIRP per TCI measurement.

Proposal 3: Send a LS to RAN5 extending current UE Beamlock test function to support locking two antenna patterns simultaneously.

Proposal 4: Consider UE declaration approach for AoA pair selection from the set of {30deg, 60deg, 90deg, 120deg, 150deg} and proper UE orientation listed in [3] for EIRP per TCI measurement for peak EIRP PUMAX,f,c,k testing.

Proposal 5: FFS on whether declared AoA pair and UE orientation for STxMP and Multi-Rx are same or not.

Proposal 6: Constant-step measurement grid should be used for peak EIRP PUMAX,f,c,k testing.

Proposal 7: 15deg is used as step size for peak EIRP PUMAX,f,c,k testing.

Observation 1: From regulatory PoV, exhaustive search for EIRP per TCI with all the possible AoA separations and UE orientations shall be measured but it is very time consuming and infeasible as there are only several AoA pairs supported by current test setup.

Proposal 8: RAN4 to study and select worst case of AoA pair from the set of {30deg, 60deg, 90deg, 120deg, 150deg} and potential UE orientation listed in [3] for PUMAX,f,c testing.

Proposal 9: The test method for aggregated EIRP PUMAX,f,c testing, i.e., if the peak EIRP of single carrier is smaller than EIRPmax – 3dB, the corresponding EIRPmax verification can be skipped, can be adopted.

Proposal 10: Constant-step measurement grid should be used for aggregated EIRP PUMAX,f,c testing.

Proposal 11: 15deg is used as step size for aggregated EIRP PUMAX,f,c testing.

Observation 2: With the worst-case conclusion in section 2.2, aggregated TRP can be calculated by integrating EIRP per TCI.

Proposal 12: The worst-case conclusion for aggregated EIRP PUMAX,f,c testing should apply for aggregated TRP PTMAX,f,c, testing.

Proposal 13: The test method for aggregated TRP PTMAX,f,c testing, i.e., if the TRP of single carrier is smaller than TRPmax – 3dB, the corresponding TRPmax verification can be skipped, can be adopted.

# 4.	References

- RP-240856, “New SID: Study on NR FR2 OTA (Over the Air) testing enhancement Phase 3,” Qualcomm Incorporated

- R4-2406087, Way Forward for [110bis][339] NR_FR2_OTA, Qualcomm

- TS 38.101-2

- R4-2405394, Discussion on FR2 OTA test method on STxMP, Qualcomm

- R4-2404672, Discussion on the sTxMP OTA test, vivo

- TS 38.509

- TR 38.871