3GPP TSG-RAN WG4 Meeting #111                                                       R4- 2409466
Fukuoka, Japan, 20th – 24th May, 2024

Agenda item:	4.2

Source:	Vodafone, Orange, Telecom Italia, Deutsche Telekom

# Title:	Consideration of multiple carrier operation for n100/n101

Document for:	Approval

# Background

In RAN4#110-bis meeting, NR channel BW less than 5MHz for TN (NR channel BW less than 5MHz for FR1 Phase 2; [1]), was discussed and the following ECC Decision (20)02 [2] was highlighted:

Decides 4. that CEPT administrations wishing to allow multiple carriers using wideband technologies (i.e., LTE or NR, including NB-IoT) or higher e.i.r.p. for RMR BS than stated in the technical conditions should consider the implementation of a coordination procedure or other mitigation measures;

While the “higher e.i.r.p” case of the above referred Decides 4 in ECC(20)02 has been already addressed in Rel-17 versions of TS 38.104 and TS 38.141-1, the “multiple carriers” case was not reflected.

It was agreed in the WF that in the RAN4#111meeting, RAN4 will reflect the “multiple carriers using wideband technologies” wording and related considerations on “coordination procedure or other mitigation measures” from Decides 4 in ECC(20)02, into Rel-17 versions of TS 38.104 and TS 38.141-1, where the exact wording to be implemented in related CRs is FFS.

# Discussion

Multi-carrier simulations were not covered in the ECC reports 313, 314, 318 for n100 and n101 bands. Only single carriers were used to identify worst-case scenarios. Multiple simultaneously operated DL carriers could potentially generate higher amount of interference towards victim users. The ECC decision (20)02 [2] addresses the concern of multiple simultaneously operated DL carriers, where the “coordination procedure or other mitigation measures” wording was captured as protective measure to prevent any potential harmful interference issues which were not fully analysed for multi-carrier scenario beforehand during technical studies.

Proposal 1: Add multi-carrier clarification to Rel-17 BS specifications TS 38.104 and TS 38.141-1, wishing to allow multiple carriers shall implement coordination procedure or other mitigation measures.

# Conclusions

Based on the discussion above, the following proposal formulated:

Proposal 1: Add multi-carrier clarification to Rel-17 BS specifications TS 38.104 and TS 38.141-1, wishing to allow multiple carriers shall implement coordination procedure or other mitigation measures.

CR for TS 38.104 [3].

CR for TS 38.141-1 [4].

# References

- RP -240832 WI resulting from discussion on NR channel BW less than 5MHz for TN, NR channel BW less than 5MHz for FR1 Phase 2, WID, RAN#103.

- ECC Decision (20)02 Harmonised use of the paired frequency bands 874.4-880.0 MHz and 919.4-925.0 MHz and of the unpaired frequency band 1900-1910 MHz for Railway Mobile Radio (RMR), 20.11.2020.

- R4-2409448 CR to TS 38.104: Clarification on multiple carrier operation for n100/n101, Rel-17

- R4-2409465 CR to TS 38.141-1: Clarification on multiple carrier operation for n100/n101, Rel-17