3GPP TSG-RAN WG4 Meeting #111             	R4-2409481

Fukuoka, Japan, 20th-24th May, 2024

Agenda item:	7.19.3.2

Source:	Samsung

Title:	Simulation results on BS demodulation requirements for Rel-18 FeMIMO

Document for:	Discussion

- Introduction

In the last RAN4 meeting, the test setup of BS demodulation performance part for FeMIMO was under discussion. The related agreement was captured in the WF [1] as following

- Requirement for Rel-18 DMRS - Adding a new clause to TS 38.141-1 for the applicability of PUSCH with Rel-18 DMRS - RAN4 shall add a new clause to TS 38.141-1 for the applicability of PUSCH with Rel-18 DMRS, assign the new clause to clause number 8.1.2.1.10 and use the proposed wording within this contribution for the new clause. - Unless otherwise stated, a BS declare to support PUSCH enhanced DMRS (see D.xxx in table 4.6-1) and pass the requirement defined in 8.2.X, can also consider the tests defined in 8.2.1 with same configurations besides DM-RS ports as passed. - Adding a new manufacturer declaration to Table 4.6-1 in TS 38.141-1 - RAN4 need to introduce new declaration “Declaration of support for enhanced DMRS as specified in TS 38.211 [17]”. - Minimum requirements for tests need to be defined for Rel-18 DMRS - Option 1: reuse the legacy values for Rel-15 DMRS - Option 2: use the new simulation results for Rel-18 DMRS - Companies are encouraged to provide simulation results in the May meeting

In this contribution, the review on remaining issue of BS demodulation requirement with Rel-18 DMRS

# 2	Discussion

Requirement Definition

Regarding the minimum requirement of PUSCH with Rel-18 DMRS, whether to reuse the legacy requirement of Rel-15 or new requirement based on the new simulation results of Rel-18 DMRS patten, there is no conclusion yet. Compare to Rel-15 DMRS, FD-OCC with 4 was introduce for Rel-18 DMRS pattern, which will have impact on the channel estimation performance. From the baseband processing perspective, since the process is different, we think the requirement should be based on the new simulation results, instead of reusing the legacy requirement. While considering the minor performance difference between Rel-15 DMRS and Rel-18 DMRS, we are ok to reuse the legacy values for Rel-18 DMRS requirement for moving forward, aiming to reduce the simulation effort

Proposal 1: Define PUSCH requirement with Rel-18 DMRS based new simulation results. Reusing the legacy values of Rel-15 DMRS can be accepted as compromise.

# 3	Simulation Results

In this section, the ideal and impairment simulation results are provided based on agreed simulation assumption as following

Table 3-1: simulation assumption for PUSCH with Rel-18 DMRS port

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {8}, {8,9} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
|  | DM-RS type | enhanced-dmrs-Type_r18 |
| Time domain | PUSCH mapping type | A and B |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
| assignment | Frequency hopping | Disabled |
| TPMI index for 2Tx two-layer spatial multiplexing transmission |  | 0 |
| TPMI index for 4Tx four-layer spatial multiplexing transmission |  | 0 |
| Code block group based PUSCH transmission |  | Disabled |
| NOTE 1: The same requirements are applicable to FDD and TDD with different UL-DL pattern. |  |  |

Table 3-2:  initial simulation results for PUSCH with Rel-18 DMRS port

| Test Cases | SCS&BW | RB | MCS | Channel | Mapping Type | Length of symbol | Numb of Tx and Rx | Number of DMRS | SNR @70% with Rel-18 DMRS ports (ideal) | SNR @70% with Rel-18 DMRS ports (impairment) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 15KHz 5MHz | 25 | 16 | TDLC300-100 | A | 14 | 1Tx2Rx | 1+1 (2,11) | 8.7 | 10.7 |
| 2 | 30KHz 10MHz | 24 | 16 | TDLC300-100 | A | 14 | 1Tx2Rx | 1+1 (2,11) | 8.7 | 10.7 |
| 3 | 15KHz 5MHz | 25 | 16 | TDLC300-100 | A | 14 | 2Tx2Rx | 1+1 (2,11) | 16.8 | 18.8 |
| 4 | 30KHz 10MHz | 24 | 16 | TDLC300-100 | A | 14 | 2Tx2Rx | 1+1 (2,11) | 18.7 | 20.7 |
| 5 | 15KHz 5MHz | 25 | 16 | TDLC300-100 | B | 14 | 1Tx2Rx | 1+1 (2,11) | 8.7 | 10.7 |
| 6 | 30KHz 10MHz | 24 | 16 | TDLC300-100 | B | 14 | 1Tx2Rx | 1+1 (2,11) | 8.8 | 10.8 |
| 7 | 15KHz 5MHz | 25 | 16 | TDLC300-100 | B | 14 | 2Tx2Rx | 1+1 (2,11) | 16.8 | 18.8 |
| 8 | 30KHz 10MHz | 24 | 16 | TDLC300-100 | B | 14 | 2Tx2Rx | 1+1 (2,11) | 18.7 | 20.7 |

# 4	Conclusion

In this contribution, the view on the remaining issue of PUSCH requirement with Rel-18 DMRS. And the ideal and impairment simulation results are provided for requirement derivation.

Proposal 1: Define PUSCH requirement with Rel-18 DMRS based new simulation results. Reusing the legacy values of Rel-15 DMRS can be accepted as compromise.

# References

R4-2406111, WF on NR_MIMO_evo_DL_UL_demod