3GPP TSG-RAN WG4 Meeting#111                              R4-2409600

Fukuoka City, Fukuoka , Japan, 20th – 24th May, 2024

Agenda item:	10.8.3

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on RF requirements for ATG BS in Rel-19

Document for:	Approval

Introduction

In the RAN#103 meeting, new WID on Enhancements for Air-to-ground network for NR has been approved and it’s expected to start the normative work from this April meeting. During last RAN4 meeting, we have reached good progress for RF requirement for Rel-19 ATG BS supporting CA. In this contribution, we would like to share some further views on the remaining issue.

Issue 1-1: TAE requirements for ATG BS supporting intra-band contiguous CA and inter-band CA - Way Forward - To reuse the existing NR BS TAE requirements - Issue 1-2: How to handle CA TAE requirements for ATG BS in TS38.104 - Way Forward - Option 1: Remove the sentence “There is no TAE requirement for ATG BS” in 6.5.3.1 and 9.6.3 - Option 2: Remove the sentence “There is no TAE requirement for ATG BS” in 6.5.3.1 and 9.6.3, then add the following in 6.5.3.2 and 9.6.3.2, - For ATG BS, for intra-band contiguous carrier aggregation, without MIMO, TAE shall not exceed 260ns. - For ATG BS, inter-band carrier aggregation, without MIMO, TAE shall not exceed 3µs. - Option 3: Find a place to add a general clarification that for ATG, intra-band contiguous CA and inter-band CA are supported but DL MIMO is not supported. - Issue 1-3: RF requirements other than TAE for ATG BS supporting intra-band contiguous CA and inter-band CA - Online agreements: - To reuse the same requirements as defined for the legacy NR BS RF CA requirements.

Discussion

In the last RAN4 meeting, we have reached the consensus on RF requirement for ATG BS supporting intra-band contiguous CA and inter-band CA. Regarding how to capture it in the specification, at least from our understanding, option 1, 2 and 3 is not exclusive for each other.

Firstly, the note for TAE requirement for single carrier operation without MIMO of ATG BS in Rel-18 is not applicable anymore in Rel-19 specification.

Secondly, it’s clearly captured in the WID that ATG BS is supposed to support the intra-band contiguous CA and inter-band DL CA without MIMO transmission. If we just reuse the existing TAE requirement for ATG BS, this will cause some confusions since the wording of TAE requirements in the specification includes both with or without MIMO. Therefore option 2 is somehow preferred and make the specification more clear.

Thirdly, as we discussed during the last RAN4 meeting, for other RF requirement needs to be reused for ATG BS e.g. aggregated measurement bandwidth for OFF power or OBW and CACLR requirement etc. In other words, the option 3 is also essential for other requirements.

Based on the above considerations, we propose to consider both option 2 and option 3.

Proposal 1: to consider both option 2 and option 3 in order to cover all relevant RF requirement for Rel-19 ATG BS and leave DL MIMO as implementation.

Conclusions

In this contribution, we shared some further views on the RF requirements for ATG BS supporting CA in Rel-19 and proposals are made as following:

Proposal 1: to consider both option 2 and option 3 in order to cover all relevant RF requirement for Rel-19 ATG BS and leave DL MIMO as implementation.

References

- RP-240839,New WID on Enhancements for Air-to-ground network for NR, Approved.

- R4-2321028,CR for TS 38.104 on adding RF requirements for ATG BS, ZTE, Agreed

[3] R4-2406089, Way Forward for [110bis][312] NR_ATG_enh, Approved.