3GPP TSG-RAN WG4 Meeting#111                              R4-2409604

Fukuoka City, Fukuoka , Japan, 20th – 24th May, 2024

Agenda item:	10.2.2

Source: 	ZTE Corporation,Sanechips

Title: 	Further discussion on Expected EIRP mask for upper 6GHz

Document for:	Approval

Introduction

In the RAN#103 meeting, new WID for Rel-19 BS RF evolution has been approved and it’s expected to start the normative work from this April meeting. In last RAN4 meeting, we have some initial discussions on EIRP mask requirement for U6GHz and reached some consensus in [3]. In this contribution, we would like to share further views on the remaining issues for the expected EIRP mask requirement for 6425-7125MHz.

- Issue 2-1: The applicability of band for the EIRP mask requirement - Agreement: - The applicable band is n104 - Further discuss the applicable freq range within band n104. - Issue 2-2: The naming for this requirement Agreement: - Use OTA spatial emission above horizon as stating point; - Issue 2-3: New definition for vertical and horizontal for U6GHz EIRP mask Agreement: - To use the following figure as stating point; - Issue 2-5: Release independent or not Agreement: - Regardless the BS declared the supported release version, BS has to fulfill the regulatory requirement. - Open issues: - Issue 2-4: Which sub-clause to capture the requirement the requirement - Option 1: clause 9.2 for BS EIRP - Option 2: clause 9.7 for unwanted emission mask - Option 3: add new clause for EIRP emission mask

Discussion

- 2.1. General aspects for expected EIRP mask for U6GHz

In last RAN4 meeting, we have some initial discussion on the applicability of expected EIRP mask for U6GHz, it was agreed to apply for the band n104 only and not related with band n96 or band n102. Another question was raised whether the applicable frequency range should be restricted within 6425-7075MHz of band n104 due to the incumbent FSS service existing only at that spectrum. However according to the ITU-R RESOLUTION COM4/7 (WRC-23) , the regulatory requirement is defined for 6425-7125MHz. In addition, not only the protection for  FSS (Earth-to-space) should be considered, but also the coexistence between IMT and the fixed service should be considered. In other words, the expected EIRP mask requirement should be applicable for the whole frequency range of band n104.

d) that studies have shown that co-channel coexistence between IMT and the fixed service can be achieved but would require site-by-site coordination if IMT and the fixed service are deployed in the same or adjacent geographical areas; that, in order to ensure protection for the FSS (Earth-to-space), and taking into account considering d), the level of expected e.i.r.p. spectral density emitted by an IMT base station as a function of the vertical angle above the horizon shall not exceed the following values (No. 21.5 does not apply)

Figure 1. the illustration of coexistence RATs for U6GHz

Proposal 1: the expected EIRP mask requirement apply for whole frequency range of band n104.

Regarding the naming for this expected EIRP mask requirement, from our understanding, OTA spatial emission above horizon looks good to indicate both of the purpose of this requirement as restricting OTA spatial emissions and also indicate the spatial domain above the horizon.

Proposal 2: use the OTA spatial emission above horizon as the name of the expected EIRP mask requirement.

Regarding the new definition for vertical and horizontal for U6GHz EIRP mask, from our understanding, there would be at least the following coordinate system to be considered in RAN4;

- The declared coordinate system orientation (D.2) and its corresponding reference coordinate system captured in clause 4.14 of TS 38.141-2.

- The coordinate system of test system;

- The coordinate system of RF requirement for vertical and horizontal domain.

Figure 2. the illustration of the required coordinate system for BS core requirement and conformance testing

As shown in the above figure, the coordinate system for U6GHz EIRP mask is different from the reference coordinate system as captured in TS 38.141-2 with the declared mechanical down-tilt. In addition, the θ in the reference coordinate system is defined as positive along the down-tilt angle, while the θ in the coordinate system for U6GHz is positive along the up-tilt angle according to the ITU-R regulatory requirements.

Based on the above considerations, we propose to define the coordinate system for expected EIRP mask for U6GHz as following:

OTA spatial emission requirement over the horizon is defined in the elevation angle above the horizon towards the sky. To be able to meet the OTA spatial emission requirement over the horizon, the reference coordinate system for EEIRP is required.

The coordinate system for OTA spatial emission over horizon is created of a Cartesian coordinate system with rectangular axis (x, y, z) and spherical angles () irrespective of BS mechanical down-tilt as showed in figure 4.xxx-1.

Figure 3: Reference coordinate system for OTA spatial emission above horizon

is the angle in the x/y plane, between the x-axis and the projection of the radiating vector onto the x/y plane and is defined between -180° and +180°, inclusive.  is the angle between the projection of the vector in the x/y plane and the radiating vector and is defined between 0 and +90°, inclusive. Note that  is defined as positive along the up-tilt angle.

Proposal 3: define the reference coordinate system for OTA spatial emission over the horizon as following

The coordinate system for OTA spatial emission over horizon is created of a Cartesian coordinate system with rectangular axis (x, y, z) and spherical angles () irrespective of BS mechanical down-tilt as showed in figure 4.xxx-1.

Figure 3: Reference coordinate system for OTA spatial emission above horizon

is the angle in the x/y plane, between the x-axis and the projection of the radiating vector onto the x/y plane and is defined between -180° and +180°, inclusive.  is the angle between the projection of the vector in the x/y plane and the radiating vector and is defined between 0 and +90°, inclusive. Note that  is defined as positive along the up-tilt angle.

- 2.2. Core requirement

The expected EIRP mask requirement has been specified in Resolution COM4/7 (WRC-23), most relevant part with the core requirement of U6GHz BS EIRP mask is extracted as following:

Figure 4. Illustration of horizontal (azimuth) angle, vertical (elevation) angle and beamforming directions

Proposal 4:

To ensure protection for the FSS (Earth-to-space), the Expected EIRP of NR BS operating in band n104 shall not exceed the limits in Table 9.9-1.

Table 99: Expected EIRP limits.

| Vertical angle range θL ≤ θ < θH (vertical angle θ above the horizon) | Expected e.i.r.p. (dBm/MHz) |
| --- | --- |
| 0 ≤ θ < 5 | 27 |
| 5 ≤ θ < 10 | 23 |
| 10 ≤ θ < 15 | 19 |
| 15 ≤ θ < 20 | 18 |
| 20 ≤ θ < 30 | 16 |
| 30 ≤ θ < 60 | 15 |
| 60 ≤ θ ≤ 90 | 15 |
| - Note 1: The reference coordinate system for OTA spatial emission requirement is defined in clause 4. 10. - Note 2: The requirement is defined within specific vertical angle range above the horizon and over horizontal angles from −180° to +180 °. Note 3: NR BS shall comply with the specified limits on expected e.i.r.p. spectral density for all mechanical tilts with which it can be deployed |  |

In addition, in last RAN4 meeting, there were some discussions where to capture the core requirement for OTA spatial emission requirements. From our understanding, Unwanted emissions consisting of so-called out-of-band emissions and spurious emissions according to ITU definitions ITU-R SM.329 might be not appropriate place to put this requirement since this clause is targeted for out of band emissions instead of within wanted carriers. In other words, to have new clause e.g. Clause 9.9 OTA spatial emission above the horizon should be one better structure for it.

Proposal 5: have new clause 9.9 for OTA spatial emission above the horizon for band n104.

- 2.3. Effective EIRP calculation

Regarding the effective EIRP calculation, we need to discuss the following issues:

- EIRP measurement grid for EEIRP calculation;

- EEIRP calculation over the discrete spatial samples, the existing EEIRP profile is calculated from an integration over continuous average EIRP pattern which could be updated accordingly for discrete spatial samples.

- 2.3.1. Measurement grid size

- Firstly, the reference angular step size shown as following is based on Rayleigh resolution for spatial domain to avoid the aliasing similar as Nyquist sampling in the time domain. For EIRP mask over different elevation bins, the purpose and principle should be considered to get the sufficient spatial information for the whole spatial EIRP mask. In other words, the legacy reference angular step approach could be reused for EIRP measurement over certain elevation bins without any aliasing problem and only focus on the upper hemispheres.

The reference angular steps Δϕref and Δθref in degrees should be updated according to elevation bins firstly since the step size for elevation angle is 5 degree from 0 degree to 20 degree.

Δϕref= min⁡(180∘πλDcyl,5∘)

Δθref=⁡min(180∘πλD,5∘)

or

∆θref=min(180°πarcsin⁡(λDz),5°)

Δϕref=min(180°πarcsin⁡(λDy),5°)

- There are also some potential concerns on the difference between main beam or sidelobe beams in nature, or peak direction misalignment between main beam or sidelobe/grating lobe beams. From our understanding, it’s not necessary to find out the peak directions for Rayleigh based measurement grid approach in the past. In other words, to reuse the legacy measurement grid for EIRP measurement above upper hemisphere space should be okay.

- Proposal 6: use the legacy measurement grid with updated maximum step size for elevation angle 0 to 20 degree at least for EIRP measurement on upper hemisphere or above the horizon, then use these measured spatial EIRP value to calculate the EEIRP mask.

Δϕref= min⁡(180∘πλDcyl,5∘)

Δθref=⁡min(180∘πλD,5∘)

or

∆θref=min(180°πarcsin⁡(λDz),5°)

Δϕref=min(180°πarcsin⁡(λDy),5°)

- 2.3.2. Effective EIRP calculation

The existing EEIRP profile is calculated from an integration over continuous average EIRP pattern shown as following:

EIRPθL , θH=12πsinθH−sinθLθLθH−ππEIRPθ,φ cosθ dφ dθ

As we mentioned in clause 2.3.1, to reuse the same measurement grid e.g. M sampling grid over azimuth plane and N sampling grid over the elevation plane (over the range of [0 to π/2]) and azimuth ϕ (over the range of [-π to π]), and with N and M points respectively, this average e.i.r.p. can be approximated to

EIRPθL , θH=12πsinθH−sinθLθLθH−ππEIRPθ,φ cosθ ∆φ ∆θ  ,

where

dφ≈∆φ=2πM

dθ≈∆θ=πN

Note: N sampling grid in elevation angle should be based on the whole elevation range instead of upper hemisphere.

Therefore, the overall equation for EEIRP is

EEIRPθL , θH=πMNsinθH−sinθLn=nLnHm=1MEIRPθn,φm cosθn  ,

where

θn	is the discrete elevation sampling angles between the elevation bins

φm is the discrete azimuth sampling angles ranging from −180° to +180 °

nL	is the lowest elevation sampling angles within the θL , θH bounding range

nH	is the highest elevation sampling angles within the θL , θH bounding range

- Proposal 7:

EEIRPθL , θH=πMNsinθH−sinθLn=nLnHm=1MEIRPθn,φm cosθn  ,

where

θn	is the discrete elevation sampling angles between the elevation bins

φm is the discrete azimuth sampling angles ranging from −180° to +180 °

nL	is the lowest elevation sampling angles within the θL , θH bounding range

nH	is the highest elevation sampling angles within the θL , θH bounding range

- 2.4. Mechanical downtilt and beam declarations

In addition, in the ITU-R resolution requirements, the EEIRP mask requirement is applicable for any mechanical down-tilt which is also reasonable since NR BS deployed in different scenarios might be configured with different down-tilt. If NR BS is capable to be deployed in lots of different scenarios, then NR BS should fulfill the EEIRP mask requirement at any mechanical down-tilt. In short, the concept proposed in [3] seems more comprehensive solutions to consist of both coverage angular range and steering ranges for each mechanical down-tilt.

Table 2.4-1: Manufacturer declarations

| CAR ID | Parameter | Description | Example values |
| --- | --- | --- | --- |
| 1 | qmin | Minimum angle along q axis | 0 degrees |
|  | qmax | Maximum angle along q axis | 6 degrees |
|  | jmin | Minimum angle along j axis | -60 degrees |
|  | jmax | Maximum angle along j axis | 60 degrees |
|  | Δk | Test beam power back-off | 0, 0, 0, 0, 0, 0, 0, 0, 0 dB |
| 2 | qmin | Minimum angle along q axis | 0 degrees |
|  | qmax | Maximum angle along q axis | 12 degrees |
|  | jmin | Minimum angle along j axis | -60 degrees |
|  | jmax | Maximum angle along j axis | 60 degrees |
|  | Δk | Test beam power back-off | 0, 0, 0, 0, 0, 0, 0, 0, 0 dB |
| ⋮ | ⋮ | ⋮ | ⋮ |
| Q | qmin | Minimum angle along q axis | 0 degrees |
|  | qmax | Maximum angle along q axis | 15 degrees |
|  | jmin | Minimum angle along j axis | -60 degrees |
|  | jmax | Maximum angle along j axis | 60 degrees |
|  | Δk | Test beam power back-off | 2, 2, 2, 2, 2, 2, 2, 2, 2 dB |

Proposal 8: to consider the manufacture declarations in Table 2.4-1 for different mechanical down-tilt and its the corresponding coverage angular range.

In addition, the direction for each test beam within certain CAR should be declared by vendors accordingly. In addition, the number of test beams of each CAR should be further discussed to ensure the measurement EEIRP mask with sufficient confidence interval e.g. 95%. Meanwhile the testing time for this EEIRP mask requirement should be considered as well in case of too long measurement time for the conformance testing of this EEIRP requirement.

Conclusions

In this contribution, we shared some initial views on the EIRP mask requirement for band n104 and proposals are made as following:

Proposal 1: the expected EIRP mask requirement apply for whole frequency range of band n104.

Proposal 2: use the OTA spatial emission above horizon as the name of the expected EIRP mask requirement.

Proposal 3: define the reference coordinate system for OTA spatial emission over the horizon as following

The coordinate system for OTA spatial emission over horizon is created of a Cartesian coordinate system with rectangular axis (x, y, z) and spherical angles () irrespective of BS mechanical down-tilt as showed in figure 3.

Figure 3: Reference coordinate system for OTA spatial emission above horizon

is the angle in the x/y plane, between the x-axis and the projection of the radiating vector onto the x/y plane and is defined between -180° and +180°, inclusive.  is the angle between the projection of the vector in the x/y plane and the radiating vector and is defined between 0 and +90°, inclusive. Note that  is defined as positive along the up-tilt angle.

Proposal 4:

To ensure protection for the FSS (Earth-to-space), the Expected EIRP of NR BS operating in band n104 shall not exceed the limits in Table 9.9-1.

Table 9.9-1: Expected EIRP limits.

| Vertical angle range θL ≤ θ < θH (vertical angle θ above the horizon) | Expected e.i.r.p. (dBm/MHz) |
| --- | --- |
| 0 ≤ θ < 5 | 27 |
| 5 ≤ θ < 10 | 23 |
| 10 ≤ θ < 15 | 19 |
| 15 ≤ θ < 20 | 18 |
| 20 ≤ θ < 30 | 16 |
| 30 ≤ θ < 60 | 15 |
| 60 ≤ θ ≤ 90 | 15 |
| - Note 1: The reference coordinate system for OTA spatial emission requirement is defined in clause 4. 10. - Note 2: The requirement is defined within specific vertical angle range above the horizon and over horizontal angles from −180° to +180 °. Note 3: NR BS shall comply with the specified limits on expected e.i.r.p. spectral density for all mechanical tilts with which it can be deployed |  |

In addition, in last RAN4 meeting, there were some discussions where to capture the core requirement for OTA spatial emission requirements. From our understanding, Unwanted emissions consisting of so-called out-of-band emissions and spurious emissions according to ITU definitions ITU-R SM.329 might be not appropriate place to put this requirement since this clause is targeted for out of band emissions instead of within wanted carriers. In other words, to have new clause e.g. Clause 9.9 OTA spatial emission above the horizon should be one better structure for it.

Proposal 5: have new clause 9.9 for OTA spatial emission above the horizon for band n104.

- Proposal 6: use the legacy measurement grid with updated maximum step size for elevation angle 0 to 20 degree at least for EIRP measurement on upper hemisphere or above the horizon, then use these measured spatial EIRP value to calculate the EEIRP mask.

Δϕref= min⁡(180∘πλDcyl,5∘)

Δθref=⁡min(180∘πλD,5∘)

or

∆θref=min(180°πarcsin⁡(λDz),5°)

Δϕref=min(180°πarcsin⁡(λDy),5°)

- Proposal 7:

EEIRPθL , θH=πMNsinθH−sinθLn=nLnHm=1MEIRPθn,φm cosθn  ,

where

θn	is the discrete elevation sampling angles between the elevation bins

φm is the discrete azimuth sampling angles ranging from −180° to +180 °

nL	is the lowest elevation sampling angles within the θL , θH bounding range

nH	is the highest elevation sampling angles within the θL , θH bounding range

Proposal 8: to consider the manufacture declarations in Table 2.4-1 for different mechanical down-tilt and its the corresponding coverage angular range.

References

- RP-240829,New WID: WI resulting from discussion on BS RF requirement evolution, Approved.

- WRC-23 Provisional Final Acts, https://www.itu.int/dms_pub/itu-r/opb/act/R-ACT-WRC.15-2023-PDF-E.pdf

- R4-2406088,Way Forward for [110bis][311] NR_BS_RF, ZTE, approved.

- R4-2404757,On introduction of protection requirement for FSS UL service operating within 6425 to 7125 MHz

Annex

4.10	The reference coordinate system for OTA spatial emission above horizon

OTA spatial emission requirement over the horizon is defined in the elevation angle above the horizon towards the sky. To be able to meet the OTA spatial emission requirement over the horizon, the reference coordinate system for EEIRP is required.

The coordinate system for OTA spatial emission over horizon is created of a Cartesian coordinate system with rectangular axis (x, y, z) and spherical angles () irrespective of BS mechanical down-tilt as showed in figure 4.xxx-1.

Figure 3: Reference coordinate system for OTA spatial emission above horizon

is the angle in the x/y plane, between the x-axis and the projection of the radiating vector onto the x/y plane and is defined between -180° and +180°, inclusive.  is the angle between the projection of the vector in the x/y plane and the radiating vector and is defined between 0 and +90°, inclusive. Note that  is defined as positive along the up-tilt angle.

9.9	OTA spatial emission above horizon

To ensure protection for the FSS (Earth-to-space), the Expected EIRP of NR BS operating in band n104 shall not exceed the limits in Table 9.9-1.

Table 99: Expected EIRP limits.

| Vertical angle range θL ≤ θ < θH (vertical angle θ above the horizon) | Expected e.i.r.p. (dBm/MHz) |
| --- | --- |
| 0 ≤ θ < 5 | 27 |
| 5 ≤ θ < 10 | 23 |
| 10 ≤ θ < 15 | 19 |
| 15 ≤ θ < 20 | 18 |
| 20 ≤ θ < 30 | 16 |
| 30 ≤ θ < 60 | 15 |
| 60 ≤ θ ≤ 90 | 15 |
| - Note 1: The reference coordinate system for OTA spatial emission requirement is defined in clause 4. 10. - Note 2: The requirement is defined within specific vertical angle range above the horizon and over horizontal angles from −180° to +180 °. Note 3: NR BS shall comply with the specified limits on expected e.i.r.p. spectral density for all mechanical tilts with which it can be deployed |  |