3GPP TSG-RAN WG4 Meeting#111                              R4-2409616

Fukuoka City, Fukuoka , Japan, 20th – 24th May, 2024

Agenda item:	7.16.5.1

Source: 	ZTE Corporation,Sanechips

Title: 	Further discussion on Tx RF requirements for NTN in Ka-band

Document for:	Approval

Introduction

In RAN#92e meeting, the work item [RP-221819] on NR NTN (Non-Terrestrial Networks) enhancements was approved as one of Rel-18 RAN1/2/3 package. In the previous RAN4 meeting, we reached good progress on the VSAT RF requirements, however there are still some remaining issues left for further discussions. In this contribution, we would like to share some further views on these remaining Tx requirements.

Online agreement Issue 1-1: Maximum TRP Online Agreement: - For type 1 and type 4 with mechanical steering antenna (e.g. parabolic antenna) - 35dBm - For type 2,3, 5 with electronic steering antenna (e.g. phase antenna array) - 35dBm, 40dBm or 45dBm for further decision in this meeting. Offline agreement: For maximum TRP: - For type 2,3, 5 with electronic steering antenna (e.g. phase antenna array) - [43dBm] for further ACLR evaluation; if there is coexistence issue identified for 43dBm next meeting, then set 40dBm as maximum TRP power; - The maximum EIRP 76.2 is not changed, the antenna gain for other TRP values could further adjusted; Offline agreement: For the minimum peak EIRP requirement: - To state in RAN4 spec to declare the supported elevation angles to comply the minimum peak EIRP requirement and leave the test for RAN5 discussion. Issue 1-2: Minimum output power WF: Not to define the transmitter Minimum controlled output power requirement; Issue 1-3: Transmit ON-OFF power - Recommended for further discussion: - Option 1: reusing FR2 BS OFF power requirement -36dBm/MHz Online Agreement: - Agree on Option 1. Issue 1-4: Transmit ON-OFF time mask Online Agreement: - Transient period for FR2-NTN is 5us. Issue 1-5: Transmit freq error - Recommended Agreement: - Follow the agreement reached in R4-232197 with fixed doppler shift. - Time varying doppler shift will be discussed in Rel-19 RAN4 NTN WID. Issue 1-6: Transmit modulation quality - Recommended Agreement: - Follow the agreement reached in R4-2317648, R4-2321974 Issue 1-7: SEM Online Agreement: - Agree on proposal 4. - FFS on definition of P_(rated,UE) Offline agreement for NS indication for regional requirements Step 1: to make the regulatory requirement clear and applicable to which bands; Step 2: to further discuss how to arrange the regulatory requirement in the specification next meeting; Issue 1-8: Power control requirement Online Agreement: - No requirement for power control for Ka band for NR-NTN UE Issue 1-9: MPR requirement for type 2 and type 5 to meet OFF-axis EIRP requirement Offline agreement: For type 2 an type 5, the power reduction for OFF-axis EIRP requirement might be needed and the exact value is left for the implementation; Issue 2-1 Minimum EIS requirement Online Agreement: - for type 3 UE, to specify minimum EIS as -115.6dBm for 50MHz, for the other channel bandwidth, the corresponding EIS requirement could be scaled with PRB based compared with 50MHz; - for type 1/2/4/5 UE, to specify minimum EIS as -122dBm for 50MHz, for the other channel bandwidth, the corresponding EIS requirement could be scaled with PRB based compared with 50MHz Issue 2-2: Maximum input power Online Agreement: - For type 3, - [-101]dBm as maximum input power with 64QAM - For type 1/2/4/5: - -109.6dBm with 16QAM or [64QAM]. - FFS for QPSK - The exact MCS or coding rate for FRC of maximum input power need further discussion and confirmation. Issue 2-5: Polarization characteristics Online Agreement: - Either RHCP or LHCP is supported by Ka band VSAT in Rel-18 Open for further discussion Issue 1-11: Antenna performance - Proposal 1: Specify Antenna performance requirements for band n511 and n510 based on FCC 47 CFR 25.209. [Ericsson, R4-2404939] - Proposal 2: Antenna point accuracy should not be introduced into the spec until RAN4 fully discuss such requirements in detail. [Huawei, R4-2405339] Recommended WF: - Further discuss the proposal 1 - To clarify anything is not clear for antenna point accuracy. Issue 1-7: SEM Online Agreement: - Agree on proposal 4. - FFS on definition of P_(rated,UE) - FFS for the boundary for SEM and spurious emission requirements. Issue 1-7: SEM Online Agreement: - Agree on proposal 4. - FFS on definition of P_(rated,UE) - FFS for the boundary for SEM and spurious emission requirements. Issue 2-2: Maximum input power - FFS on Whether the maximum input power should be aligned with ACS interfering signal power level; Issue 2-3: ACS - FFS on the test parameters for ACS requirement Issue 2-4: Antenna off-axis performance - Proposal 1: Specify Rx antenna off-axis performance requirements for band n512 based on ETSI EN 301 360, EN 301 459, EN 303 699, EN 303 978. [Ericsson, Thales, R4-2404940] - Recommended WF: - Need further discussions online.

Discussion

- 2.1. Maximum TRP for type 2, 3 and 5

In last RAN4 meeting, we have some extensive discussions on maximum TRP for type 2, 3 and 5. From our perspective, 43dBm could be used as compromise value between 40dBm and 45dBm considering the power antenna power accuracy around 3dB for 40dBm maximum TRP. In addition, considering the coexistence simulation submitted in the previous RAN4 meeting, the specified ACLR requirement still have some margins to accommodate high output power from our evaluation results.

| ## Scenario [Fixed VSAT] | ## ACLR requirement [Case 1 for Fixed VSAT ACLR] |  |
| --- | --- | --- |
|  | ## Elevation angle =90 | ## Elevation angle =25 |
| ## GEO | ## 6 | ## 8 |
| ## LEO-1200KM | ## 0 | ## 4 |
| ## LEO-600KM | ## 0 | ## 0 |

| ## Scenario [Fixed VSAT] | ## ACLR requirement [Case 3 for Fixed VSAT ACLR] |  |
| --- | --- | --- |
|  | ## Elevation angle =90 | ## Elevation angle =25 |
| ## GEO | ## 0 | ## 0 |
| ## LEO-1200KM | ## 0 | ## 0 |
| ## LEO-600KM | ## 0 | ## [] |

| ## Scenario [Mobile VSAT] | ## ACLR requirement [Case 1 for Mobile VSAT ACLR] |  |
| --- | --- | --- |
|  | ## Elevation angle =90 | ## Elevation angle =25 |
| ## GEO | ## 0 | ## 0 |

| ## Scenario [Mobile VSAT] | ## ACLR requirement [Case 3 for Mobile VSAT ACLR] |  |
| --- | --- | --- |
|  | ## Elevation angle =90 | ## Elevation angle =25 |
| ## GEO | ## 0 | ## 0 |

Proposal 1: use the 43dBm as maximum TRP value for type 2 and 3 and type 5 if there are no coexistence problems identified.

- 2.2. SEM

In last RAN4 meeting, we have some discussion on the Prated,UE and frequency boundary for SEM and spurious emission requirements. First of all, TRPmax refer to the measured TRP power instead of rated TRP power in case of too relaxed emission requirement for VSAT with high rated output power and lower transmission power.

Table 9.5.2.2.1-1: General NR spectrum emission mask for NTN-FR2

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits (dBm) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 2× BW | 0.5 MHz f_offset < 2× BW + 0.5 MHz | max11SE limit, TRPmaxPrated,UE – 10log10(BW) –40×log10 f_offset−0.5BW×2+1dBm | 1 MHz |
| NOTE 1: TRPmax is specified in clause 9.2.1; NOTE 2: Transmission BW is in the unit of MHz; NOTE 3: The 11dBm/1MHz calue corresponds to the spurious emission limit specified in spurious emission clause 9.5.3, and is converted from the SE limit requirement defined on 4 kHz to a value defined over 1 MHz; NOTE 4: PSD attenuation as in ITU-R SM.1541-6 [6], Annex 5 OoB domain emission limits for earth stations. |  |  |  |

Proposal 2: the TRPmax refer to the measured TRP for VSAT instead of rated TRP power.

In addition, there were some comments in last RAN4 meeting on definition of frequency boundary for SEM and SE for VSAT. From our understanding, the existing definition is clear that Out of band emission is defined outside of assigned carrier instead of outside of band with boundary as 2xBW which is inherited from SAN RF definition.

Proposal 3: no need to further discuss the frequency boundary for SEM and SE for VSAT.

Table 9.5.3.1-1: Boundary between NR out of band and spurious emission domain

| Channel bandwidth | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
| OOB boundary FOOB (MHz) | 100 | 200 | 400 | 800 |

Table 9.5.3.1-2: Spurious emissions limits

| Frequency Range | Maximum Level | Measurement bandwidth |
| --- | --- | --- |
| 30 MHz ≤ f ≤ 2nd harmonic of the upper frequency edge of the UL operating band in GHz | -13 dBm | 4 kHz |

Conclusions

In this contribution, we want to share some further views on VSAT RF requirements for NTN in Ka-band.and proposals are made as following:

Proposal 1: use the 43dBm as maximum TRP value for type 2 and 3 and type 5 if there are no coexistence problems identified.

Proposal 2: the TRPmax refer to the measured TRP for VSAT instead of rated TRP power.

Proposal 3: no need to further discuss the frequency boundary for SEM and SE for VSAT.

References

- RP-221819, Core part: NR NTN (Non-Terrestrial Networks) enhancements, Thales, Approved.

- RP-222654, Core part: NR NTN (Non-Terrestrial Networks) enhancements, Thales, Approved.

- TR 38.821

- TR 38.803

- TR 38.828

- https://www.kingsat-tech.com/product/enhanced-vsat-ku-band-3-axis-stabilized-and-4-axis-tracking114.html

- https://www.mitsubishielectric.com/news/2020/0206.html

- R4-2217792, Email discussion summary for [104-bis-e][140] NR_NTN_enh_UERF, ZTE, Noted

- R4-2217743, WF on NTN UE in Ka-band, ZTE, Approved

- http://satstar.net/satellites/space2.html

- https://www.satbeams.com/satellites

- http://satstar.net/beams/inmar5f3_sb11.html

- R4-2216515,NTN enhancements – Regulatory aspects, Ericsson, Noted

- R4-2220573	WF on NTN UE RF requirements, ZTE, approved.

- R4-2220119, Topic summary for [105][139] NR_NTN_enh_UERF, ZTE

- https://www.etsi.org/deliver/etsi_en/301400_301499/301459/02.01.01_60/en_301459v020101p.pdf

- RF Spectrum FCC 47CFR25.138, 47CFR25.202,

- https://www.ecfr.gov/current/title-47/chapter-I/subchapter-B/part-25/subpart-C/section-25.202

- https://www.digisat.org/enterprise-75cm-professional-ka-band-terminal

- https://www.satsig.net/eut2beac.htm

- R4-2303533,WF on NTN UE RF requirements for Ka band, ZTE, Approved.

- R4-2306624, WF on NTN UE RF requirements in Ka-band. ZTE, approved.

- https://www.eutelsat.pl/files/contributed/satellites/pdf/esog120.pdf

- R4-2403649,WF on NR-NTN UE RF requirements, ZTE, Samsung, Approved.

- R4-2401116, big CR for NTN UE, agreed.

- R4-1703046, Transient time – impact ON OFF time mask, Qualcomm, Noted

- R4-1809791, Network Performance Analysis for Power Control Tolerance, Apple.