3GPP TSG-RAN WG4 Meeting #111									                          R4- 2409665

Fukuoka, Japan, May 20 – 24, 2024

Agenda item:			10.8.3

Source:	Ericsson

Title:	Discussion on remaining issues of ATG BS supporting CA

Document for:	Discussion

# Introduction

In 3GPP release 18, RAN4 has completed the standardization work for the air-to-ground (ATG) network WI, the main work focused on the FR1 single carrier scenario. To enhance the throughput to meet growing ATG capacity demand and fully utilize operators’ spectrum, the RAN plenary meeting #103 has approved a new WI on enhancements for the air-to-ground network for NR [1].

# Discussion

In the last meeting, several agreements were made, including TAE requirements and other RF requirements [2].

Issue 1-1: TAE requirements for ATG BS supporting intra-band contiguous CA and inter-band CA

Way Forward

- To reuse the existing NR BS TAE requirements

#### Issue 1-2:  How to handle CA TAE requirements for ATG BS in TS38.104

Way Forward

- Option 1: Remove the sentence “There is no TAE requirement for ATG BS” in 6.5.3.1 and 9.6.3

- Option 2: Remove the sentence “There is no TAE requirement for ATG BS” in 6.5.3.1 and 9.6.3, then add the following in 6.5.3.2 and 9.6.3.2,

  - For ATG BS, for intra-band contiguous carrier aggregation, without MIMO, TAE shall not exceed 260ns.

  - For ATG BS, inter-band carrier aggregation, without MIMO, TAE shall not exceed 3µs.

- Option 3: Find a place to add a general clarification that for ATG, intra-band contiguous CA and inter-band CA are supported but DL MIMO is not supported.

#### Issue 1-3:  RF requirements other than TAE for ATG BS supporting intra-band contiguous CA and inter-band CA

Online agreements:

- To reuse the same requirements as defined for the legacy NR BS RF CA requirements.

The following sessions will discuss the remaining issues, i.e. how to handle CA TAE requirements for ATG BS in TS 38.104.

In Rel-18, the ATG BS does not have a designated core requirement, instead, it follows the existing NR BS requirement, the only exception is the TAE requirement.

It has been agreed in the RAN4 #110bis meeting to reuse the existing NR BS TAE requirements for ATG BS which supports intra-band contiguous and inter-band CA. therefore, it is suggested first to remove the sentence “There is no TAE requirement for ATG BS” in 6.5.3.1 and 9.6.3, and then refer to the existing NR BS TAE requirement for the ATG BS that supports CA operation.

- In Rel-18, the ATG BS does not have a designated core requirement, instead, it follows the existing NR BS requirement, the only exception is the TAE requirement.

- Remove the sentence “There is no TAE requirement for ATG BS” in 6.5.3.1 and 9.6.3, and then refer to the existing NR BS TAE requirement for the ATG BS that supports CA operation.

# Conclusion

In the previous sections, we made the following observations:

Observation 1	In Rel-18, the ATG BS does not have a designated core requirement, instead, it follows the existing NR BS requirement, the only exception is the TAE requirement.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Remove the sentence “There is no TAE requirement for ATG BS” in 6.5.3.1 and 9.6.3, and then refer to the existing NR BS TAE requirement for the ATG BS that supports CA operation.

# References

- RP-240804, New WID on Enhancements for Air-to-ground network for NR.

- R4-2406089, Way Forward for [110bis][312] NR_ATG_enh