3GPP TSG RAN WG4 Meeting #111	R4-2409712

Fukuoka, Japan, May 20 – May 24, 2024

Agenda Item:	7.8.6

Source: 	Ericsson

Title:	Discussion on performance part for NR in less than 5 MHz bandwidth

Document for:	Discussion

# Introduction

In this contribution, we provide initial analysis on performance part requirements for UE supporting NR in less than 5 MHz bandwidth.

# Discussion

## Test configurations

With the reduced CBW, we may need to revisit the following configuration whether we need new values to be calculated or legacy values can be reused.

- OCNG

- RMC

- CBW

- Signal levels

Reference measurement Channels:

RMC for PDSCH

In Rel-15, RAN4 specified reference measurement channel for 24 PRBs for PDSCH. Since transmission BW is 12 or 15 PRB, we need to define a new RMC table for 12/15 PRB.

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | SRNR5.1.1 FDD | SRNR5.1.2 FDD |  |  |  |  |  |
| Channel bandwidth | MHz | Defined in test case | Defined in test case |  |  |  |  |  |
| Number of transmitter antennas |  | 1 | 1 |  |  |  |  |  |
| Allocated resource blocks for PDSCH Note 1 |  | 12 | 15 |  |  |  |  |  |
| Allocated slots per Radio Frame |  | 10 | 10 |  |  |  |  |  |
| Radio frame containing SSB | slots | Note 5 | Note 5 |  |  |  |  |  |
| Radio frame not containing SSB | slots | 10 | 10 |  |  |  |  |  |
| MCS index |  | 4 | 4 |  |  |  |  |  |
| Modulation |  | QPSK | QPSK |  |  |  |  |  |
| Target Coding Rate |  | 1/3 | 1/3 |  |  |  |  |  |
| Number of control symbols |  | 2 | 2 |  |  |  |  |  |
| PDSCH mapping type |  | Type A | Type A |  |  |  |  |  |
| Information Bit Payload |  |  |  |  |  |  |  |  |
| For slots with RMSI Note 2 | bits | TBC | TBC |  |  |  |  |  |
| For slots without RMSI | bits | TBC | TBC |  |  |  |  |  |
| Number of Code Blocks per slot |  | 1 | 1 |  |  |  |  |  |
| Binary Channel Bits Per slot |  |  |  |  |  |  |  |  |
| For slots with RMSI Note 2, Note 4 | bits | TBC | TBC |  |  |  |  |  |
| For slots without RMSI Note 6 | bits | TBC | TBC |  |  |  |  |  |
| Note 1: Allocated outside the SMTC duration in time and in resource blocks which do not overlap with the resource blocks allocated for SS/PBCH block. Note 2: PDSCH is scheduled on the slots with RMSI. Note 3: If necessary the information bit payload size can be adjusted to facilitate the test implementation. The payload sizes are defined in TS 38.213 [3]. Note 4: Derived based on the PDSCH DMRS assumption: dmrs-TypeA-Position=2, dmrs-Type=1, dmrs-AdditonalPositions=2, maxLength=1, Antenna port index: 1000, and Number of PDSCH DMRS CDM group(s) without data: 2. Note 5: PDSCH is not scheduled in slots containing SSB according to the SSB configuration used in the test. SSB configurations are defined in clause A.3.10. Note 6: When DRX is configured, PDCCH can be scheduled both for downlink assignment and/or UL grant only during ([10]ms - drx-InactivityTimer) from timing when drx-onDurationTimer starts, unless otherwise specified in the test case |  |  |  |  |  |  |  |  |

RMC for CORESET for RMSI:

Similar to PDSCH, even for CORESET 0, allocated RB are 24 in NR. We need to define a new RMC table for CORESET for RMSI.

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | CRNR5.1.1 FDD | CRNR5.1.1 FDD |  |  |  |  |  |
| Channel bandwidth | MHz | Defined in test case | Defined in test case |  |  |  |  |  |
| Subcarrier spacing for RMSI CORESET | kHz | 15 | 15 |  |  |  |  |  |
| Allocated resource blocks for RMSI CORESET Note 7 |  | 12 | 15 |  |  |  |  |  |
| Subcarrier spacing for SSB | kHz | 15 | 15 |  |  |  |  |  |
| SSB and RMSI CORESET multiplexing configuration Note 7 |  | Pattern 1 | Pattern 1 |  |  |  |  |  |
| Offset between SSB and RMSI CORESET Note 3, 7 | RB | 0 (Note8) | 0 (Note8) |  |  |  |  |  |
| Configuration of PDCCH monitoring occasions for RMSI CORESET Note 4 |  | Index 4 | Index 4 |  |  |  |  |  |
| Number of transmitter antennas |  | 1 | 1 |  |  |  |  |  |
| Duration of RMSI CORESET Note 7 | symbols | 2 | 2 |  |  |  |  |  |
| DCI Format Note 1 |  | Note 2 | Note 2 |  |  |  |  |  |
| Aggregation level | CCE | 4 | 8 |  |  |  |  |  |
| DMRS precoder granularity |  | 6 | 6 |  |  |  |  |  |
| REG bundle size |  | 6 | 6 |  |  |  |  |  |
| Mapping from REG to CCE |  | Distributed | Distributed |  |  |  |  |  |
| Cell ID |  | Note 5 | Note 5 |  |  |  |  |  |
| Payload (without CRC) | bits | Note 6 | Note 6 |  |  |  |  |  |
| Note 1: DCI formats are defined in TS 38.212. Note 2: DCI format shall depend upon the test configuration. Note 3: The offset is defined with respect to the subcarrier spacing of the CORESET from the smallest RB index of RMSI CORESET to the smallest RB index of the common RB overlapping with the first RB of the SS/PBCH block. Note 4: The configuration of PDCCH monitoring occasions for RMSI CORESET is defined in Table 13-11 in TS 38.213 [3]. Note 5: Cell ID shall depend upon the test configuration. Note 6: Payload size shall depend upon the test configuration. Note 7: The configuration of set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space corresponds to index 0 in Table 13-1 in TS 38.213 [3] Note 8: Other values can be used to align with GSCN [13] as long as SSB does not overlap the RMC. |  |  |  |  |  |  |  |  |

CORESET for RMC scheduling:

Similar to PDSCH, even for CORESET for control, allocated RB are 24 in NR. We may need to define a new RMC table for CORESET for control. If we need to define new table, we can take following table as baseline. Alternatively, if dedicated coreset can refer to coreset ID 0, then current config can work without additional table or new config.

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | CCRNR5.1.1 FDD | CCR.1.2 FDD |  |  |  |  |  |
| Channel bandwidth | MHz | Defined in test case | Defined in test case |  |  |  |  |  |
| Subcarrier spacing | kHz | 15 | 15 |  |  |  |  |  |
| Allocated resource blocks for CORESET Note 3 |  | 15 | 12 |  |  |  |  |  |
| Number of transmitter antennas |  | 1 | 1 |  |  |  |  |  |
| Duration of CORESET | symbols | 3 | 2 |  |  |  |  |  |
| REG bundle size |  | 6 | 6 |  |  |  |  |  |
| DMRS precoder granularity |  | Same as REG bundle size | Same as REG bundle size |  |  |  |  |  |
| CCE to REG mapping |  | Interleaved | Interleaved |  |  |  |  |  |
| Interleave n_shift |  | 0 | 0 |  |  |  |  |  |
| Interleave size |  | 2 | 2 |  |  |  |  |  |
| Beamforming Pre-Coder |  | N/A | N/A |  |  |  |  |  |
| Aggregation level | CCE | 8 | 4 |  |  |  |  |  |
| DCI formats |  | Note 1 | Note 1 |  |  |  |  |  |
| Payload size (without CRC) | bits | Note 2 | Note 2 |  |  |  |  |  |
| Note 1: DCI format shall depend upon the test configuration. Note 2: Payload size shall depend upon the test configuration Note 3: Allocated in the resource blocks where the associated RMC is scheduled. |  |  |  |  |  |  |  |  |

- RAN4 to decide on whether to use existing configuration with some clarification as a note or define new RMC table for PDSCH, RMSI, and UE specific PDCCH. If decided to use new tables, consider tables listed in this paper as baseline.

OCNG:

Though OCNG table can be reused, we need few clarifications like highlighted in yellow in the below table.

| OCNG Parameters | Control Region | Data Region |
| --- | --- | --- |
| Resource allocation | Unused REs (Note 1) | Unused REs (Note 2) |
| Channel | PDCCH | PDSCH |
| Contents | Virtual UE IDs | Uncorrelated pseudo random QPSK modulated data |
| Antenna transmission scheme | Same as used in PDCCH RMC of less than 5 MHz | Same as used in PDSCH RMC of less than 5 MHz |
| Subcarrier spacing | Same as used in PDCCH RMC of less than 5 MHz | Same as used in PDSCH RMC of less than 5 MHz |
| Aggregation level | Same as used in PDCCH RMC of less than 5 MHz | N/A |
| Code rate | Same as used in PDCCH RMC of less than 5 MHz | Same as used in PDSCH RMC of less than 5 MHz |
| Transmit Power | Same as used in PDCCH RMC of less than 5 MHz | Same as used in PDSCH RMC of less than 5 MHz |
| CP length | Same as used in PDCCH RMC of less than 5 MHz | Same as used in PDSCH RMC of less than 5 MHz |
| Note 1: REs not used in the active CORESETs where PDCCH is scheduled for the UE under test. Note 2: REs not allocated to any physical channels, CORESET, SSB or any other reference signal within the channel bandwidth of the cell, confined to BWoccupied where specified in the test case. |  |  |

- RAN4 to define new OCNG table which is similar as legacy table. The table discussed above can be taken as baseline.

## Methods for introducing tests

We think RAN4 can look at optimised way of capturing the tests for NR less than 5 MHz. Since NR less than 5 MHz have limited configurations, we think one configuration from the legacy tests can be referred in the test instead of copying entire parameters table. The parameters which are different for NR less than 5 MHz only to be listed in the test parameters for the test. Further, test procedure is also expected to be same as legacy tests other than the config and the requirements to be met. Ran4 can look at whether a legacy test can be referred instead of repeating all the test procedure and requirements.

- RAN4 to consider at referring legacy test as a reference instead of capturing the same test procedure again.

# Summary and conclusions

- RAN4 to decide on whether to use existing configuration with some clarification as a note or define new RMC table for PDSCH, RMSI, and UE specific PDCCH. If decided to use new tables, consider tables listed in this paper as baseline.

- RAN4 to define new OCNG table which is similar as legacy table. The table discussed above can be taken as baseline.

- RAN4 to consider at refereeing legacy test as a reference instead of capturing the same test procedure again.

# References

- RP-221799 “WID on Mobility enhancements”, MediaTek

- R4-2314273

# Annexe

In previous meeting, following PDCCH parameters are agreed for RLM or BFD OOS and IS.

Table 8.1.2.1-3: PDCCH transmission parameters for out-of-sync evaluation for [less than 5MHz UE]

| Attribute | Value for BLER Configuration #0 |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3MHz (12 PRBs) | 3MHz (15 PRBs) | 5MHz (20 PRBs) |  |
| DCI format | 1-0 |  |  |  |
| Number of control OFDM symbols | [2] | [3] | [3] |  |
| Aggregation level (CCE) | [4] | [8] | [8] |  |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 4dB |  |  |  |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 4dB |  |  |  |
| Bandwidth (PRBs) | 12 | 15 | 20 |  |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |  |  |  |
| DMRS precoder granularity | REG bundle size |  |  |  |
| REG bundle size | 6 |  |  |  |
| CP length | Normal |  |  |  |
| Mapping from REG to CCE | Distributed | Non-Distributed and Distributed |  | Distributed |

Table 8.1.2.1-4: PDCCH transmission parameters for in-sync evaluation for [less than 5MHz UE]

| Attribute | Value for BLER Configuration #0 |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3MHz (12 PRBs) | 3MHz (15 PRBs) | 5MHz (20 PRBs) |  |
| DCI format | 1-0 |  |  |  |
| Number of control OFDM symbols | [2] | [3] | [3] |  |
| Aggregation level (CCE) | [2] | [4] | [4] |  |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 0dB |  |  |  |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 0dB |  |  |  |
| Bandwidth (PRBs) | 12 | 15 | 20 |  |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |  |  |  |
| DMRS precoder granularity | REG bundle size |  |  |  |
| REG bundle size | 6 |  |  |  |
| CP length | Normal |  |  |  |
| Mapping from REG to CCE | Distributed | Non-Distributed and Distributed |  | Distributed |

We have performed following set of simulations based on the above assumptions and the simulation parameters agreed during the core part of the discussion.

Out of sync simulations:

- 12 PRB interleaved with TDL-C and above OOS parameters

- 15 PRB interleaved with TDL-C and above OOS parameters

- 15 PRB non-interleaved with TDL-C and above OOS parameters

- 20 PRB interleaved with TDL-C and above OOS parameters

- 12 PRB interleaved with HST and above OOS parameters

In sync simulations:

- 12 PRB interleaved with TDL-C and above IS parameters

- 15 PRB interleaved with TDL-C and above IS parameters

- 15 PRB non-interleaved with TDL-C and above IS parameters

- 20 PRB interleaved with TDL-C and above IS parameters

- 12 PRB interleaved with HST and above IS parameters

We present results in the following table for relevant simulation assumption.

Simulation Assumption:

PDCCH BW: 12 PRBs, Number of CORESET symbols: 2; Carrier freq.: 900 MHz; CORESET bandwidth 12 PRBs and non-punctured and inter-leaved. Antenna config: 2*2, low.

Simulation Result:

| Scenario | AL | Channel | OOS (with -4 dB adjustment to 10% BLER) | SNR dB for 2% BLER In-Sync | Difference between IS-OOS |
| --- | --- | --- | --- | --- | --- |
| OOS | 8 | TDL-C 300ns 100 Hz | -6.94 |  | 10.04 |
| IS | 4 | TDL-C 300ns 100 Hz |  | 3.1 |  |

Simulation Assumption:

PDCCH BW: 15 PRBs, Number of CORESET symbols: 3; Carrier freq.: 900 MHz; the CORESET is generated from 24-PRB CORESET 0 of which top 9 PRBs are punctured and inter-leaved, Antenna config: 2*2, low

Simulation Result:

| Scenario | AL | Channel | OOS (with -4 dB adjustment to 10% BLER) | SNR dB for 2% BLER In-Sync | Difference between IS-OOS |
| --- | --- | --- | --- | --- | --- |
| OOS | 8 | TDL-C 300ns 100 Hz | -8.66 |  | 7.8 |
| IS | 4 | TDL-C 300ns 100 Hz |  | -0.855 |  |

Simulation Assumption:

PDCCH BW: 15 PRBs, Number of CORESET symbols: 3; Carrier freq.: 900 MHz; the CORESET is generated from 24-PRB CORESET 0 of which top 9 PRBs are punctured and non-inter-leaved, Antenna config: 2*2, low

Simulation Result:

| Scenario | AL | Channel | OOS (with -4 dB adjustment to 10% BLER) | SNR dB for 2% BLER In-Sync | Difference between IS-OOS |
| --- | --- | --- | --- | --- | --- |
| OOS | 8 | TDL-C 300ns 100 Hz | -7,23 |  | 2,86715 |
| IS | 4 | TDL-C 300ns 100 Hz |  | -4,36285 |  |

Simulation Assumption:

PDCCH BW: 20 PRBs, Number of CORESET symbols: 3; Carrier freq.: 900 MHz; the CORESET is generated from 24-PRB CORESET 0 of which top 4 PRBs are punctured and inter-leaved, Antenna config: 2*2, low

Simulation Result:

| Scenario | AL | Channel | OOS (with -4 dB adjustment to 10% BLER) | SNR dB for 2% BLER In-Sync | Difference between IS-OOS |
| --- | --- | --- | --- | --- | --- |
| OOS | 8 | TDL-C 300ns 100 Hz | -8,04762 |  | 3,27125 |
| IS | 4 | TDL-C 300ns 100 Hz |  | -4,77637 |  |

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | CR.1.1 FDD | NEW |  |  |  |  |  |
| Channel bandwidth | MHz | Defined in test case |  |  |  |  |  |  |
| Subcarrier spacing for RMSI CORESET | kHz | 15 |  |  |  |  |  |  |
| Allocated resource blocks for RMSI CORESET Note 7 |  | 24 | 12 is possible |  |  |  |  |  |
| Subcarrier spacing for SSB | kHz | 15 |  |  |  |  |  |  |
| SSB and RMSI CORESET multiplexing configuration Note 7 |  | Pattern 1 | Pattern 1 |  |  |  |  |  |
| Offset between SSB and RMSI CORESET Note 3, 7 | RB | 0 (Note8) |  |  |  |  |  |  |
| Configuration of PDCCH monitoring occasions for RMSI CORESET Note 4 |  | Index 4 |  |  |  |  |  |  |
| Number of transmitter antennas |  | 1 |  |  |  |  |  |  |
| Duration of RMSI CORESET Note 7 | symbols | 2 |  |  |  |  |  |  |
| DCI Format Note 1 |  | Note 2 |  |  |  |  |  |  |
| Aggregation level | CCE | 8 |  |  |  |  |  |  |
| DMRS precoder granularity |  | 6 |  |  |  |  |  |  |
| REG bundle size |  | 6 |  |  |  |  |  |  |
| Mapping from REG to CCE |  | Distributed | Non-distributed or contiguous |  |  |  |  |  |
| Cell ID |  | Note 5 |  |  |  |  |  |  |
| Payload (without CRC) | bits | Note 6 |  |  |  |  |  |  |
| Note 1: DCI formats are defined in TS 38.212. Note 2: DCI format shall depend upon the test configuration. Note 3: The offset is defined with respect to the subcarrier spacing of the CORESET from the smallest RB index of RMSI CORESET to the smallest RB index of the common RB overlapping with the first RB of the SS/PBCH block. Note 4: The configuration of PDCCH monitoring occasions for RMSI CORESET is defined in Table 13-11 in TS 38.213 [3]. Note 5: Cell ID shall depend upon the test configuration. Note 6: Payload size shall depend upon the test configuration. Note 7: The configuration of set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space corresponds to index 0 in Table 13-1 in TS 38.213 [3] Note 8: Other values can be used to align with GSCN [13] as long as SSB does not overlap the RMC. |  |  |  |  |  |  |  |  |