3GPP TSG-RAN WG4 Meeting # 111 	R4-2409748

Fukuoka, Japan, May 20th – 24th, 2024

Agenda Item: 7.5.4

Source: MediaTek Inc.

Title: Discussion on RRM performance requirements for measurements without gaps

Document for: Discussion

# Discussion

RAN4 agreed on the work-split for test cases for MGE-2 part 2 [1]. Yet, during the drafting process of NFG8 test cases of ‘Event triggered reporting [without interruption] intra-frequency measurements without gap or DRX configuration’, we faced some issues. In this contribution document, we provide our views for this NFG8 TC drafting issues.

Based on offline discussion with other companies, NFG8 TC is assumed for the scenario that UE reports support of ‘no gap no interruption’ in a serving cell MO, then the test equipment configures intra-frequency SSB outside the active BWP. Basically, the SSB or the reference signals of the serving cell should be within the active BWP of the serving cell, while the SSB of the neighboring cell is outside the active BWP. Nevertheless, based on legacy test cases, such as A.6.6.1.3 ‘SA event triggered reporting tests with per-UE gaps under non-DRX’, CSI-RS signals were used for RLM measurements. Thus, for the new test cases, RAN4 to follow the legacy TC principles: use CSI-RS but no need to support Option A and no requirements for CSI-RS measurements.

Observation 1: Legacy test cases use CSI-RS for performing RLM measurements for the scenarios when SSB is not within the active BWP.

- RAN4 shall use CSI-RS for RLM measurements in the test cases for NeedForGap of intra-frequency measurements with SSB outside the active BWP but no need to support Option A and no requirements for CSI-RS measurements.

# Summary

In this contribution, we have the following proposal:

Proposal 1: RAN4 shall use CSI-RS for RLM measurements in the test cases for NeedForGap of intra-frequency measurements with SSB outside the active BWP but no need to support Option A and no requirements for CSI-RS measurements.

# References

- R4-2406442, “WF on NR_MG_enh2_part2”, Intel, China’s meeting, April 2024.