3GPP TSG-RAN WG4 Meeting #111                                                                R4-2409763

Fukuoka City, Fukuoka, Japan, 20th – 24th May, 2024

Source:	Samsung

Title:	Discussion on RAN4 aspects for SBFD system parameters

Agenda item:			10.12.1

Document for:	Discussion

# 1. Introduction

Based on Rel-18 study item on evolution of NR duplex operation, the support of subband non-overlapping full duplex has been studied, which is targeted to provide enhanced UL coverage, reduced latency, improved system capacity, and improved configuration flexibility for NR TDD operation [2].

In RAN#102, the work item on evolution of NR duplex operation (SBFD) has been approved, with WID further revised in the follow-up RAN plenary [1]. According to the objectives in WID, RAN1 is tasked to specify the mechanisms to support SBFD, including semi-static indication of time/frequency location, random access in SBFD symbols, and other transmission, reception and measurement behavior and procedures for SBFD aware UE. Furthermore, the enhancement for CLI handing, including gNB-to-gNB and UE-to-UE CLI handling, will also be specified in RAN1.

Accordingly from RAN4 perspective, it is tasked to specify BS RF, necessary RRM and demodulation requirements, and in this contribution, we would like to provide our analysis and viewpoints on RAN4 aspects for SBFD system parameters.

# 2. RAN4 aspects for SBFD system parameters

### 2.1  SBFD frequency-domain configuration

As requested in WID, to support SBFD operation at gNB side within a TDD carrier, the mechanism for semi-static indication of frequency location of SBFD subbands to UE shall be specified. Based on the discussion in RAN4#110bis, the following agreements are achieved:

#### Issue 2-4-1: SBFD frequency-domain configuration in RAN4 specification - Agreement: - It is within RAN4 scope to study/specify the limitation or restriction on the size of subband/guardband, by taking account different feasible BS/UE implementations. - FFS how RAN4 specification captures the subband configurations - FFS the necessity of standardize the guardband; - If needed, FFS the sizes of guardband in RAN4 shall be decided.

Accordingly, we would like to provide our analysis on the RAN4 aspects for SBFD frequency-domain configuration. Firstly, the RAN1 agreements achieved in RAN1#116 and #116bis are provided:

<From RAN1#116> Agreement For RRC connected mode UEs, at least cell-specific configuration on time and frequency(working assumption) location of SBFD subbands is supported within a TDD carrier. - FFS: Additional support of UE-specific configuration on time and/or frequency locations of SBFD subbands Agreement: The maximum number of UL subbands for SBFD operation in an SBFD symbol within a TDD carrier is one. The UL subband can be located at one side of the carrier or can be located at the middle part of the carrier. For semi-static indication of SBFD subband frequency location, down-select from the following options. - Option 1: Frequency locations of UL subband and DL subband(s) are explicitly configured. Guardband(s) if any are implicitly derived as the RBs which are not within UL subband or DL subband(s). - Option 2: Frequency location of UL subband and the number of RBs for guardband(s), if any, are explicitly configured. DL subband(s) are implicitly derived as RBs which are not within UL subband or guardband(s). Agreement The subband frequency-domain resources are same across different SBFD symbols within a TDD carrier. Frequency location of cell specific UL subband, and DL subband(s) if explicitly indicated, are indicated with reference to CRB grid. - RB-level granularity is supported for semi-static indication of SBFD subband frequency location. - Subject to RAN4 guidance on the size of subband/guardband, if any - FFS reference starting RB and reference SCS Agreement For discussion purpose, UL subband frequency resources within active UL BWP are called UL usable PRBs and DL subband(s) frequency resources within active DL BWP are called DL usable PRBs. For determining UL/DL usable PRBs, consider the following options. - Option 1: UL usable PRBs are determined as intersection between cell-specific UL subband and active UL BWP in SBFD symbols. DL usable PRBs are determined as intersection between cell-specific DL subband(s) and active DL BWP in SBFD symbols. - Option 2: UL/DL usable PRBs are explicitly configured within active UL/DL BWP in SBFD symbols. <From RAN1#116bis> Agreement For frequency resource allocation Type 0 for PDSCH or PUSCH in a single slot by DCI based scheduling (without repetition or TBoMS), when an assigned RBG overlaps with the subband boundary, only the PRBs within DL usable PRBs are considered to be valid for PDSCH reception and only the PRBs within UL usable PRBs are considered to be valid for PUSCH transmission. - SBFD aware UE does not expect to be assigned with a RBG for PDSCH which is fully outside DL usable PRBs or a RBG for PUSCH which is fully outside UL usable PRBs. Agreement For cell-specific configuration of frequency locations of SBFD subbands, - Option 1: Cell-specific frequency locations of SBFD subbands are separately configured for each SCS configuration in SCS-SpecificCarrierList. - For each SCS configuration, the reference starting PRB is the PRB determined by the SCS configuration and offsetToCarrier corresponding to this subcarrier spacing.

Limitation or restriction on the size of subband/guardband

From the above RAN1 agreement related to UL/DL subband configuration in frequency domain, we can see RAN1 has agreed to introduce the cell-specific configuration of frequency locations of SBFD subbands, while still FFS on the additional support of UE-specific configuration on frequency locations of SBFD subbands. Furthermore, RB-level granularity is supported for semi-static indication of SBFD subband frequency location.

Observation 1: RAN1 agree to introduce the cell-specific configuration of frequency locations of SBFD subbands, and accordingly RB-level granularity is supported for semi-static indication of SBFD subband frequency location.

For the size of subband and guardband, different feasible BS/UE implementations shall be considered.

Despite being separated in frequency in non-overlapping DL and UL subbands, the simultaneous transmission and reception by the BS TxRU still has an impact on UL reception. The power amplifier (PA) nonlinearity, which is a type of RF impairment in the transmit chains, leads to intermodulation leakage from the DL subband(s) to the UL subband. This leakage can result in receiver blocking at the low noise amplifier (LNA) and/or analog-digital converter (ADC). To address this issue, Self-interference cancellation (SIC) becomes crucial for the BS when the Tx is in the DL subband and Rx is in the UL subband of the same time slot/symbol. If SIC is capable of reducing the Rx power below the received signal level of the UL, then SBFD operation can function effectively at the BS receiver. Besides, the DL signal may also bounce back to the BS from nearby reflectors like metal signs or building walls, leading to clutter echo. This can cause blocking impacts similar to SI.

Based on the Rel-18 study item, SIC in the BS relies on a necessary combination of techniques in antenna, RF, and digital domains, while not all these techniques are mandatory to support a particular BS class in FR1 or FR2-1. Without adequate SIC capability, the interference from the transmitted DL signal would corrupt the received UL signal as illustrated in Fig.1 (a). To solve this problem, various SIC schemes can be used. Using the example of Fig.1 (b), SIC capability can be provided through the antenna or panel design (A), can be applied in RF domain to the RF signal (B) or in digital signal domain (C), or a combination of these.

Fig.1: gNB transceiver architecture with self-interference cancellation capability (Figure 9.2.1.2.1-1, [2])

Especially in the field of RF, direct DL transmission can pose challenges due to channel filtering and potential saturation of the RX receiver. However, passive SIC isolation can help mitigate this issue by incorporating an additional UL subband RF filter. To address interference from the DL and ensure accurate reception of the desired UL signal, an analog filter can be employed prior to the ADC. This filter serves to attenuate DL interference and protect the dynamic range of the ADC. An active approach can also involve an analog interference canceller, although in mMIMO systems, the complexity of the analog canceller increases as the number of Tx and Rx chains grows. All these factors could potentially impact the determination of the size of subband and guardband.

In the digital domain, various techniques can be utilized to reduce the impact on the sensitivity of the BS receiver, aiming to keep it below 1 dB. One such technique is digital interference cancellation (IC), which aims to eliminate interference from the desired UL signal and reduce any remaining self-interference (SI) to an acceptable level. Digital IC involves synthesizing interference from baseband Tx samples with accurate modeling of Tx chain non-linearity or capturing interference from the RF chain on the transmit side and converting it to baseband. This process requires estimating the SI channel taps for each Tx chain to each Rx chain. Additionally, other digital techniques can help mitigate interference. For example, Digital Pre-distortion (DPD) can reduce leakage into the UL subband. Other techniques, such as beam nullification to lessen the impact of clutter reflections or optimizing digital Tx beamforming to decrease SI, can also be employed.

Observation 2: For determine the limitation/restriction of the size of subband and guardband, the BS SBFD implementations including the possibility of adding UL subband RF filter, analog filter prior to the ADC or other RF and digital domain techniques related to subband bandwidth shall be considered.

From that perspective, we propose that RAN4 can consider the following restriction/limitation for the size of subband and guardband, at least as baseline for further RAN4 study.

Proposal 1: The following limitation/restriction of the size of subband and guardband shall be considered as baseline:

- Subband (including both UL/DL subband):

  - Granularity: 1 RB

  - Possible Subband size (expressed in RBs): {Xmin, Xmin+1,..., Xmax}

    - Xmin = 1

    - Xmax = NRB (i.e., Transmission bandwidth configuration)

- Guardband:

  - Granularity: 1 RB

  - Possible guardband size (expressed in RBs): {0, 1, 2, ..., Ymax}

    - Where Ymax is the maximum allowed number of RBs for guardband

      - FFS the value of Ymax

FFS how RAN4 specification captures the subband configurations

From last meeting’s WF [3], the group shall further study how RAN4 specification captures the subband configurations. Specifically, the following text proposal with the specification structure for SBFD-capable BS could be considered as the baseline for RAN4 discussion.

Proposal 2: A new 5.3 clause with suffix B is added to capture the BS channel bandwidth for SBFD (including Transmission bandwidth configuration, guardband and subband configuration for SBFD), with the following text proposal to be considered as baseline skeleton for triggering discussion:

=================== Start of Text Proposal ===================

## 5.3B	BS channel bandwidth for SBFD

### 5.3B.1	General

<To be added, for the definition of channel bandwidth, transmission bandwidth configuration, uplink and downlink subbands and guardband within one NR channel for SBFD operation>

### 5.3B.2	Transmission bandwidth configuration for SBFD

<To be added>

### 5.3B.3	Guardband and Subband configuration for SBFD

<To be added>

=================== End of Text Proposal ===================

# 3. Conclusion

In this contribution, we would like to provide our analysis and viewpoints on RAN4 aspects for SBFD system parameters accordingly with the following observations and proposals obtained:

Observation 1: RAN1 agree to introduce the cell-specific configuration of frequency locations of SBFD subbands, and accordingly RB-level granularity is supported for semi-static indication of SBFD subband frequency location.

Observation 2: For determine the limitation/restriction of the size of subband and guardband, the BS SBFD implementations including the possibility of adding UL subband RF filter, analog filter prior to the ADC or other RF and digital domain techniques related to subband bandwidth shall be considered.

Proposal 1: The following limitation/restriction of the size of subband and guardband shall be considered as baseline:

- Subband (including both UL/DL subband):

  - Granularity: 1 RB

  - Possible Subband size (expressed in RBs): {Xmin, Xmin+1,..., Xmax}

    - Xmin = 1

    - Xmax = NRB (i.e., Transmission bandwidth configuration)

- Guardband:

  - Granularity: 1 RB

  - Possible guardband size (expressed in RBs): {0, 1, 2, ..., Ymax}

    - Where Ymax is the maximum allowed number of RBs for guardband

      - FFS the value of Ymax

Proposal 2: A new 5.3 clause with suffix B is added to capture the BS channel bandwidth for SBFD (including Transmission bandwidth configuration, guardband and subband configuration for SBFD), with the following text proposal to be considered as baseline skeleton for triggering discussion:

=================== Start of Text Proposal ===================

## 5.3B	BS channel bandwidth for SBFD

### 5.3B.1	General

<To be added, for the definition of channel bandwidth, transmission bandwidth configuration, uplink and downlink subbands and guardband within one NR channel for SBFD operation>

### 5.3B.2	Transmission bandwidth configuration for SBFD

<To be added>

### 5.3B.3	Guardband and Subband configuration for SBFD

<To be added>

=================== End of Text Proposal ===================

# 4. Reference

[1] RP-240789 (revision from RP-234035), “WID revision: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, RAN#103.

[2] 3GPP TR 38.858 v18.0.0, “Study on Evolution of NR Duplex Operation (Release 18)”.

[3] R4-2406107, “Way Forward for [110bis][313] NR_duplex_evo”, Samsung, RAN4#110bis.