3GPP TSG-RAN WG4 Meeting #111							R4-2409785

Fukuoka, Japan, May 20-24, 2024

Agenda Item:	10.3.4.1

Source:	Ericsson

Title:	On Co-existence simulation assumptions and methodology for 14800 to 15350 MHz frequency range

Document for:	Approval

# 1	Introduction

ITU-R WP5D sent a LS [1] to 3GPP RAN seeking information on terrestrial component IMT-2030 parameters to be used for sharing and compatibility studies in preparation for WRC-27, for frequency ranges 4400 to 4800 MHz, 7125 to 8400 MHz, and 14800 to 1530 MHz.

In the previous 3GPP RAN4#110-bis meeting, the Work Plan [2] for delivery of information to ITU-R LS for those respective frequency ranges was agreed, following a phase approach to send information in 3 phases as below -

- Track 1: 4400 to 4800 MHz, the estimated date for completion is latest by May 2024 (RAN4#111).

- Track 2: 7125 to 8400 MHz, the estimated date for completion is latest by August 2024 (RAN4#112).

- Track 3: 14800 to 15350 MHz, the estimated date for completion is latest by November 2024 (RAN4#113).

WF [3] captured initial discussions and available options for co-existence simulation assumptions. However, no agreements were made due to limited time slots.  

Two set of parameters requested by ITU-R WP5D are the ACLR and ACS of BS and UE in these frequencies. Currently, different sets of ACLR and ACS are specified in RAN4 specifications [4, 5, 6] for NR BS and UE in FR1 and FR2 based on coexistence studies as captured in TR 38.803 [7]. Therefore, coexistence studies will need to be carried out to provide answers to ITU-R WP5D on these parameters with sound technical justifications from coexistence perspective.

This contribution proposes simulation assumptions to run system level simulations and agree on BS/ UE ACLR/ ACS values for 14800 to 15350 MHz frequency range.

# 2	Discussion

In the previous 3GPP RAN4#110-bis meeting, some initial views for 15 GHz co-existence simulation assumptions were captured in the WF [3]. Most of the issues in the WF have multiple options and here we provide an attempt to further narrow down the assumptions based on our findings.

Issue 3-1 Common understanding on UE antenna array options

Regarding issue 3-1 on the UE (User Equipment) antenna options, two options have been identified in the last meeting:

- Option 1: “UE beamforming” (FR2 like) with 2x2 antennas per panel

- Option 2: FR1 like, with more than 1 TX/RX antenna

  - 2 TX

  - 4 TX

  - 4 RX

  - 6 RX

  - 8 RX

Regarding Option 2, RAN4 should further clarify how the link level performance model usually considered in coexistence evaluation, and discussed e.g., in Section 4.2.7 of TR 38.921, should incorporate the additional gains provided by assuming more TX/RX antennas. Following the AWGN assumptions of the link performance model, a possibility could be to -

- Consider a maximum transmission power for 1 Tx and increase it by 3 dB as the number of TX antennas is doubled.

- Evaluate the DL SINR assuming 1 RX, and then increase it by 3dB as the number of UE RX antennas is doubled.

It is also worth mentioning to further down-size the FR1 like, more than 1 TX/ RX antenna assumptions considering significant simulation efforts.

RAN4 should define common way to model the FR1 like UE antenna array option, including more than 1TX/RX if other companies prefer this option.

In the initial simulation results provided in this contribution, we consider Option 1 based on UE beamforming, with 2x2 antennas per panel at the UE as our preferred choice.

RAN4 to consider UE beamforming antenna configuration of 2 panels, where each panel consists of a 2x2 rectangular array antenna.

Issue 3-2 Initial deployment scenarios

Two reports are considered as reference, TR 38.921 and TR 38.803. We propose due to time constraint to follow a prioritized approach and start focusing on the coverage feasibility of the urban macro, and then on the indoor scenario. Later discuss viability of sub-urban and dense urbans scenarios.

RAN4 to follow a prioritized approach and discuss simulation assumptions for the deployment scenarios starting with Urban Macro, followed by Indoor Scenario and others, considering time constraints and simulation efforts.

In this contribution we follow this approach and we have focused on analysing initial coverage feasibility for the urban macro scenario, looking at wide area case, with ISD (Inter-site distance) 450 m, assuming different TRP values, 46 and 43 dBm. Smaller ISD are not precluded.

Issue 3-3 Initial coordinated operation

We propose to study both the coordinated and uncoordinated operation for outdoor scenarios and in this contribution, we focus on the uncoordinated case, as the more challenging in terms of coexistence evaluation.

RAN4 to study both coordinated and uncoordinated for the outdoor deployment scenarios.

Issue 3-4 Initial views on BS array size

About the BS (Base Station) array size, we propose to start looking at arrays of 1024 antenna elements (AE) and later also 2048 (AE), with different distributions, as discussed in the companion contribution [10].

In this contribution we have focused on 1024 and 2048 AEs as a starting point for the coverage feasibility evaluation.

Issue 3-5 Initial views on BS sub-array architecture

About the sub-array size, different options can be analysed, as discussed in the companion contribution [10]. In this contribution we have started analysing the 4x1 option (Option 1 from Table 2.2.1-1 [10]), and we consider at least also the 6x1 (Option 5 from Table 2.2.1-1 [10]) should be evaluated to balance cost efficiency and grating lobes performance.

It is reasonable to also perform feasibility studies on 6X1 sub-arrays, when considering deployment challenges in terms of costs and performance efficiency.

Other issues concerning co-existence simulations are highlighted below -

TRP

We believe further discussions are needed (see also companion contribution [10]) on defining feasible antenna architectures generating a peak EIRP that guarantees feasible coverage, are manageable in terms of costs, but also ensure a feasible level of TRP and power per branch. Also, when defining the parameters, it is important to consider coexistence issues with incumbents in the band. It is also important to specify clearly to what bandwidth the proposed power refers to, so that ITU-R can in that case scale the power to maintain PSD (Power Spectral Density) and coverage. In this contribution we have considered 43 dBm and 46 dBm for 100 MHz.

A well-balance should be thought of when defining antenna characteristics – EIRP, TRP and others, to ensure coverage feasibility with reasonable cost considerations.

It is also important to specify what BW proposed power refers to, so ITU-R can scale the power to maintain PSD (Power Spectral Density) and coverage accordingly.

RAN4 to consider using 43 dBm/ 100 MHz per polarization for co-existence simulation studies at this stage.

Bandwidth

In this contribution we have focused on 100 MHz, but higher bandwidth of 200 MHz and 400 MHz may also be considered. It is important to also define the common assumption on the UL bandwidth, and how many UEs should be concurrently allocated.

RAN4 to consider 100 MHz as a baseline and further discuss the feasibility of higher channel bandwidth.

RAN4 should clarify the common assumption on the UL bandwidth and number of actives UEs to be considered for concurrent allocation.

Indoor probability

In the following results we have considered 0% indoor probability, following the assumption that in this frequency range indoor coverage will not be a use case.

20% indoor probability assumption is not optimal for outdoor scenarios.

RAN4 should start with 0% indoor probability for co-existence evaluations, considering indoor coverage will not be a use-case for outdoor scenarios.

Further details on the simulation assumptions for the following results are included in the Annex.

In the following figures Figure 2-1 and Figure 2-2, we present some initial results in terms of SINR and User throughput CDFs, from the perspective of network 1 (N1). Plots are shown as a function of two different BS antenna architectures assuming 1024 or 2048 AEs, two different BS power levels, and for UE beamforming option. No Adjacent channel interference is considered for now in the simulations. We observe that with current assumptions, only a small percentage of users is out of coverage in both UL and DL, and Figure 2-3 shows the 5%tile user throughput in both UL and DL, for two different BS antenna architectures and BS power levels.

Figure 2-1 DL and UL SINR in Urban Scenario, assuming two different antenna architectures (1024 and 2048 AE)

Figure 2-2 DL and UL Throughput in Urban Scenario, assuming two different antenna architectures (1024 and 2048 AE)

Figure 3-2 DL and UL 5%-tile User Throughput in Urban Scenario in N1, when considering different antenna architectures

# 3	Co-existence simulation assumptions

The simulation methodologies and assumptions are mainly based on those RAN4 used for NR co-existence study as captured in TR 38.803, the channel model for frequencies from 0.5 to 100 GHz in TR 38.901 [8] and those in RAN4 reply to WP5D for ITU-R sharing and compatibility studies captured in TR 38.921 [9].

## 3.1	Co-existence simulation scenarios

Table 1 below captures the proposed list of scenarios to be simulated

| No. | Usage scenario | Aggressor | Victim | Direction | Simulation frequency | Deployment Scenario | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | eMBB | NR, 100MHz | NR, 100MHz | DL to DL | 15 GHz | Indoor hotspot |  |
| 2 | eMBB | NR, 100MHz | NR, 100MHz | DL to DL | 15 GHz | Urban macro |  |
| 3 | eMBB | NR, 100MHz | NR, 100MHz | UL to UL | 15 GHz | Indoor hotspot |  |
| 4 | eMBB | NR, 100MHz | NR, 100MHz | UL to UL | 15 GHz | Urban macro |  |

Table 1: List of scenarios to be simulated

### 3.2.1	Network layout models

This section captures details on the network layout model for the below proposes scenarios.

#### 3.2.1.1	Urban Macro

Details on urban macro network layout model are listed in Tables 3.2.1.1-1 and 3.2.1.1-2.

Table 3.2.1.1-1: Single operator layout for urban macro

| Parameters |  | Values | Remark |
| --- | --- | --- | --- |
| Network layout |  | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |  |
| Inter-site distance |  | 0.45 km (urban) | Based on cell range: 0.3 km (urban) 0.6 km (suburban) |
| BS antenna height |  | 20 m (urban) |  |
| UE location | Outdoor/indoor | Outdoor and indoor |  |
|  | Indoor UE ratio | 0% | Start with 0% for co-existence |
|  | Low/high penetration loss ratio | 50% low loss, 50% high loss |  |
|  | LOS/NLOS | LOS and NLOS |  |
|  | UE antenna height | Same as 3D-UMa in TR 36.873 |  |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS - UE distance (2D) |  | 35 m |  |
| Channel model |  | UMa |  |
| Shadowing correlation |  | Between cells: 1.0 Between sites: 0.5 |  |

Table 3.2.1.1-2: Multi operators layout for urban macro

| Parameters | Values | Remark |
| --- | --- | --- |
| Multi operators layout | coordinated operation (0% Grid Shift) and un-coordinated operation (100% Grid Shift) |  |

Figure 3.2.1.1-1: Coordinated operation: each network with co-location of sites

Figure 3.2.1.1-2: Uncoordinated operation: second network’s sites are located at the first network’s cell edge

#### 3.2.1.2	Indoor

Details on indoor network layout model are listed in Tables 3.2.1.2-1 and 3.2.1.2-2.

Table 3.2.1.2-1: Single operator layout for indoor

| Parameters |  | Values | Remark |
| --- | --- | --- | --- |
| Network layout |  | 50 m x 120 m, 12 BSs |  |
| Inter-site distance |  | 20m |  |
| BS antenna height |  | 3 m | ceiling |
| UE location | Outdoor/indoor | Indoor |  |
|  | LOS/NLOS | LOS and NLOS |  |
|  | UE antenna height | 1 m |  |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS - UE distance (2D) |  | 0 m |  |
| Channel model |  | Indoor Office |  |
| Shadowing correlation |  | NA |  |

Figure 3.2.1.2-1: Network layout for indoor

Table 3.2.1.2-2: Multi operators layout for indoor

| Parameters | Values | Remark |
| --- | --- | --- |
| Multi operator layout | Coordinated operation (0% Grid Shift) |  |

### 3.2.2	Propagation Model

This section captures details on the propagation model for the above scenarios.

#### 3.2.2.1	Path loss

The pathloss models are summarized in Table 2.2.2.1-1 and the distance definitions are indicated in Figures 3.2.2.1-1 and 3.2.2.1-2. Note that the distribution of the shadow fading is log-normal, and its standard deviation for each scenario is given in Table 3.2.2.1-1.

Figure 3.2.2.1-1: Definition of d2D and d3D for outdoor UTs

Figure 3.2.2.1-2: Definition of d2D-out, d2D-in and d3D-out, d3D-in for indoor UTs

Note that

d3D-out+d3D-in=d2D-out+d2D-in2+hBS-hUT2	(3.2.2-1)

Table 3.2.2.1-1: Pathloss models

| Scenario | LOS/NLOS | Pathloss [dB], fc is in GHz and d is in meters, see note 4 | Shadow fading std [dB] | Applicability range, antenna height default values |
| --- | --- | --- | --- | --- |
| UMa | LOS | PLUMa-LOS=PL110m≤d2D≤dBP'PL2dBP'≤d2D≤5km, see note 1 PL1=28.0+22log10(d3D)+20log10(fc) PL2=28.0+40log10(d3D)+20log10(fc) -9log10((dBP')2+(hBS-hUT)2) | σSF=4 | 1.5m≤hUT≤22.5m hBS=25m |
|  | NLOS | PLUMa-NLOS=max(PLUMa-LOS,PLUMa-NLOS') for 10m≤d2D≤5km | σSF=6 | 1.5m≤hUT≤22.5m hBS=25m Explanations: see note 3 |
|  |  | Optional PL=32.4+20log10fc+30log10d3D | σSF=7.8 |  |
| InH - Office | LOS | PLInH-LOS=32.4+17.3log10(d3D)+20log10(fc) | σSF=3 | 1m≤d3D≤150m |
|  | NLOS | PLInH-NLOS=max(PLInH-LOS,PLInH-NLOS') PLInH-NLOS'=38.3log10d3D+17.30+24.9log10fc | σSF=8.03 | 1m≤d3D≤150m |
|  |  | Optional PLInH-NLOS'=32.4+20log10fc+31.9log10d3D | σSF=8.29 | 1m≤d3D≤150m |
| Note 1: Breakpoint distance d'BP = 4 h'BS h'UT fc/c, where fc is the centre frequency in Hz, c = 3.0108 m/s is the propagation velocity in free space, and h'BS and h'UT are the effective antenna heights at the BS and the UT, respectively. The effective antenna heights h'BS and h'UT are computed as follows: h'BS = hBS – hE, h'UT = hUT – hE, where hBS and hUT are the actual antenna heights, and hE is the effective environment height. For UMi hE = 1.0m. For UMa hE=1m with a probability equal to 1/(1+C(d2D, hUT)) and chosen from a discrete uniform distribution uniform(12,15,…,(hUT-1.5)) otherwise. With C(d2D, hUT) given by Cd2D,hUT=0,hUT<13mhUT-13101.5gd2D,13m≤hUT≤23m, where gd2D=0,d2D≤18m54d2D1003exp-d2D150,18m<d2D. Note that hE depends on d2D and hUT and thus needs to be independently determined for every link between BS sites and UTs. A BS site may be a single BS or multiple co-located BSs. Note 2: The applicable frequency range of the PL formula in this table is 0.5 < fc < fH GHz, where fH = 30 GHz for RMa and fH = 100 GHz for all the other scenarios. It is noted that RMa pathloss model for >7 GHz is validated based on a single measurement campaign conducted at 24 GHz. Note 3: UMa NLOS pathloss is from TR36.873 with simplified format and PLUMa-LOS = Pathloss of UMa LOS outdoor scenario. Note 4: fc denotes the center frequency normalized by 1GHz, all distance related values are normalized by 1m, unless it is stated otherwise. |  |  |  |  |

#### 3.2.2.2	LOS probability

The Line-Of-Sight (LOS) probabilities are given in Table 3.2.2.2-1.

Table 3.2.2.2-1 LOS probability

| Scenario | LOS probability (distance is in meters) |
| --- | --- |
| UMa | PrLOS=1,d2D-out≤18m18d2D-out+exp-d2D-out631-18d2D-out1+C'hUT54d2D-out1003exp-d2D-out150,18m<d2D-outwhere C'(hUT)=0,hUT≤13mhUT-13101.5,13m<hUT≤23m |
| Indoor - Mixed office | PrLOS=1,d2D-in≤1.2mexp-d2D-in-1.24.7,1.2m<d2D-in<6.5mexp-d2D-in-6.532.6⋅0.32,6.5m≤d2D-in |
| Indoor - Open office | PrLOS=1,d2D-in≤5mexp-d2D-in-570.8,5m<d2D-in≤49mexp-d2D-in-49211.7⋅0.54,49m<d2D-in |
| Note: The LOS probability is derived with assuming antenna heights of 3m for indoor, 10m for UMi, and 25m for UMa |  |

#### 3.2.2.3	O-to-I penetration loss (Note – If Indoor probability is agreed for 0%, this section should be removed)

##### 3.2.2.3.1	O-to-I building penetration loss

The pathloss incorporating O2I building penetration loss is modelled as in the following:

PL=PLb+PLtw+PLin+N0,σP2	(3.2.2-2)

where PLb is the basic outdoor path loss given in clause 4.2.2.1, where d3D is replaced by d3D-out+d3D-in . PLtw is the building penetration loss through the external wall, PLin is the inside loss dependent on the depth into the building, and σP is the standard deviation for the penetration loss.

PLtw is characterized as:

PLtw=PLnpi-10log10i=1Npi×10Lmaterial_i-10	(3.2.2-3)

PLnpi is an additional loss is added to the external wall loss to account for non-perpendicular incidence;  Lmaterial_i=amaterial_i+bmaterial_i⋅f, is the penetration loss of material i, example values of which can be found in Table 2.2.2.3-1; pi is proportion of i-th materials, where  i=1Npi=1; and N is the number of materials.

Table 3.2.2.3-1: Material penetration losses

| Material | Penetration loss [dB] |
| --- | --- |
| Standard multi-pane glass | Lglass=2+0.2f |
| IRR glass | LIIRglass=23+0.3f |
| Concrete | Lconcrete=5+4f |
| Wood | Lwood=4.85+0.12f |
| Note: f is in GHz. |  |

Table 3.2.2.3-2 gives PLtw, PLin and σP for two O2I penetration loss models. The O2I penetration is UT-specifically generated and is added to the SF realization in the log domain.

Table 3.2.2.3-2: O2I building penetration loss model

|  | Path loss through external wall: PLtw in [dB] | Indoor loss: PLin in [dB] | Standard deviation: σP in [dB] |
| --- | --- | --- | --- |
| Low-loss model | 5-10log100.3⋅10-Lglass10+0.7⋅10-Lconcrete10 | 0.5 d2D-in | 4.4 |
| High-loss model | 5-10log100.7⋅10-LIIRglass10+0.3⋅10-Lconcrete10 | 0.5 d2D-in | 6.5 |

d2D-in is minimum of two independently generated uniformly distributed variables between 0 and 25 m for Uma and Umi-Street Canyon, and between 0 and 10 m for Rma. d2D-in should be UT-specifically generated.

Both low-loss and high-loss models are applicable to Uma and Umi-Street Canyon.

Only the low-loss model is applicable to Rma.

Only the high-loss model is applicable to InF.

##### 3.2.2.3.2	O-to-I car penetration loss

The pathloss incorporating O2I car penetration loss is modelled as in the following:

PL=PLb+Νμ,σP2	(3.2.2-4)

where PLb is the basic outdoor path loss given in clause 4.2.2.1. μ = 9, and σP = 5. The car penetration loss should be UT-specifically generated. Optionally, for metallized car windows, μ = 20 can be used. The O2I car penetration loss models are applicable for at least 0.6 – 60 GHz.

### 3.2.3	Antenna and Beam forming pattern

The antenna model is discussed in our companion contribution which needs further investigation [10].

### 3.2.4	Transmission power control model

For downlink scenario, no power control scheme is applied.

For uplink scenario, TPC model specified in clause 9.1 TR 36.942 is applied with following parameters.

-	CLx-ile = 88 + 10*log10(100/X) + 11 – Y, where X is UL transmission BW (MHz) and Y is the BS noise figure

-	γ = 1

### 3.2.5	Received power model

The received power in downlink and uplink scenarios is defined as below:

RX_PWR = TX_PWR – Path loss + G_TX + G_RX

where:

-	RX_PWR is the received power

-	TX_PWR is the transmitted power

-	G_TX is the transmitter antenna gain (directional array gain)

-	G_RX is the receiver antenna gain (directional array gain).

### 3.2.6	ACLR and ACS modelling

For DL it seems reasonable from the perspective of simulating worst case scenarios that we assume BS ACLR is modelled as flat in space, and the UE ACS can be modelled flat in space.

If this assumption is for DL, then the similar assumption could be made for the UL because:

-	UE has a much small number of antennas, thus the effect of directivity should be smaller for ACLR (or the adjacent channel interference). It can also be reasonably assumed that the UE ACLR will play a dominant role than the BS ACS in the adjacent channel interference.

-	Again, BS ACS flat in space might mean worse coexistence performance than actual performance because BS has better capability of steering its receive antennas to suppress interference.

1 user scheduling is baseline assumption for coexistence evaluation and the two step ACLR model shown in Table 4.2.6-1 could be used for 3 uplink user scheduling simulation, where a UE occupies a smaller bandwidth than the channel bandwidth for transmission, to avoid overly estimating interference, similar as done in E-UTRA coexistence study (as recorded in TR 36.942).

Table 3.2.6-1: Uplink ACIR value

| Frequency offset between aggressor (91RBs) and victim (91RBs) | ACIR value |
| --- | --- |
| 0 - 90 RBs | 30 + X |
| 91 - 181RBs | 43 + X |
| > 181RBs | 43+ X |

Therefore, it is assumed that both ACLR (or the adjacent channel interference) and ACS are flat in both space and frequency. The ACIR model can be express as:

(assuming ACLR, ACS and ACIR to be linear).

### 3.2.7	Link level performance model

The throughput of a modem with link adaptation can be approximated by an attenuated and truncated form of the Shannon bound. (The Shannon bound represents the maximum theoretical throughput than can be achieved over an AWGN channel for a given SNIR). The following equations approximate the throughput over a channel with a given SNIR, when using link adaptation:

Throughput SNIR, bps/Hz =0                                  for SNIR< SNIRMIN                                        ∝∙SSNIR                     for SNIRMIN≤SNIR<SNIRMAX ∝∙SSNIRMAX               for SNIR ≥ SNIRMAX

Where:

-	S(SNIR)		Shannon bound, S(SNIR) =log2(1+SNIR) bps/Hz

-					Attenuation factor, representing implementation losses

-	SNIRMIN		Minimum SNIR of the code set, dB

-	SNIRMAX		Maximum SNIR of the code set, dB

The parameters α, SNIRMIN and SNIRMAX can be chosen to represent different modem implementations and link conditions. The parameters proposed in Table 3.2.7-1 represent a baseline case, which assumes:

-	1:1 antenna configuration

-	AWGN channel model

-	Link Adaptation (see Table 3.2.7-1 for details of the highest and lowest rate codes)

-	No HARQ

Table 3.2.7-1: Parameters describing baseline Link Level performance for 5G NR

| Parameter | DL | UL | Notes |
| --- | --- | --- | --- |
| α, attenuation | 0.6 | 0.4 | Represents implementation losses |
| SNIRMIN, dB | -10 | -10 | Based on QPSK, 1/8 rate (DL) & 1/5 rate (UL) |
| SNIRMAX, dB | 30 | 22 | Based on 256QAM 0.93(DL) & 64QAM 0.93 (UL) |

Note that the parameters proposed in Table 3.2.7-1 are targeted for eMBB coexistence scenario.

### 3.2.8	Other simulation parameters

Table 3.2.7-1: Other simulation parameters

| Parameters | Urban macro | Indoor |
| --- | --- | --- |
| Carrier frequency | 15 GHz | 15 GHz |
| Channel bandwidth | 100 MHz as starting point | 100 MHz as starting point |
| Scheduled channel bandwidth per UE (DL) | 100 MHz as starting point | 100 MHz as starting point |
| Scheduled channel bandwidth per UE (UL) | Depends on number of UEs | Depends on number of UEs |
| The number of active UE (DL) (Note 1) | 1 | 1 |
| The number of active UE (UL) (Note 1) | TBD | TBD |
| Traffic model | Full buffer | Full buffer |
| DL power control | NO | NO |
| UL power control | YES | YES |
| BS max TX power in dBm | 43 | TBD |
| UE max TX power in dBm | 23 | TBD |
| UE min TX power in dBm | -40 | TBD |
| BS Noise figure in dB | 8 | 8 |
| UE Noise figure in dB | 10 | 10 |
| Handover margin | 3 dB | 3 dB |

## 3.3	Co-existence simulation methodology

Adopt following simulation steps.

1. Aggressor and victim network are generated.

-	UEs are distributed randomly across the network.

2. UE associations: UEs are associated to base station based on coupling loss.

-	Associations are made assuming a single element at both UE and BS.

3.	Once association is done, round robin scheduling is used. BF weights are adjusted to point to the LOS direction between BS-UE. This is done for both victim and aggressor networks.

4.	Throughput is computed in the victim systems without considering ACI as below:

-	ThputNO ACIbpshz=fSINRICI=fSN+IICI, where IICI is the inter-cell interference.

5.	Throughput is computed considering ACI as below:

-	ThputACIbpshz=fSINRICI+ACI=fSN+IICI+IACI, where IACI is the adjacent channel interference.

6.	RF parameters are determined based on the degradation cause by ACI as below:

-	LossACI=1-ThputACIThputSINGLE

Approve the given co-existence simulation assumptions in Section 3.

# Conclusion

In the previous sections we made the following observations:

Observation 1	RAN4 should define common way to model the FR1 like UE antenna array option, including more than 1TX/RX if other companies prefer this option.

Observation 2	It is reasonable to also perform feasibility studies on 6X1 sub-arrays, when considering deployment challenges in terms of costs and performance efficiency.

Observation 3	A well-balance should be thought of when defining antenna characteristics – EIRP, TRP and others, to ensure coverage feasibility with reasonable cost considerations.

Observation 4	It is also important to specify what BW proposed power refers to, so ITU-R can scale the power to maintain PSD (Power Spectral Density) and coverage accordingly.

Observation 5	20% indoor probability assumption is not optimal for outdoor scenarios.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN4 to consider UE beamforming antenna configuration of 2 panels, where each panel consists of a 2x2 rectangular array antenna.

Proposal 2	RAN4 to follow a prioritized approach and discuss simulation assumptions for the deployment scenarios starting with Urban Macro, followed by Indoor Scenario and others, considering time constraints and simulation efforts.

Proposal 3	RAN4 to study both coordinated and uncoordinated for the outdoor deployment scenarios.

Proposal 4	RAN4 to consider using 43 dBm/ 100 MHz per polarization for co-existence simulation studies at this stage.

Proposal 5	RAN4 to consider 100 MHz as a baseline and further discuss the feasibility of higher channel bandwidth.

Proposal 6	RAN4 should clarify the common assumption on the UL bandwidth and number of actives UEs to be considered for concurrent allocation.

Proposal 7	RAN4 should start with 0% indoor probability for co-existence evaluations, considering indoor coverage will not be a use-case for outdoor scenarios.

Proposal 8	Approve the given co-existence simulation assumptions in Section 3.

# References

- R4-2400333, “ITU-R WP5D LS on Parameters of terrestrial component of IMT for sharing and compatibility studies in the frequency bands 4 400-4 800 MHz, 7 125-8 400 MHz, and 14.8-15.35 GHz”.

- R4-2405874, “Work Plan on IMT parameters relevant for sharing and compatibility for 4400 to 4800 MHz, 7125 to 8400 MHz, and 14800 to 1530 MHz frequency ranges”, 3GPP RAN4#110-bis, Ericsson

- R4-2406615, “WF on IMT parameters for other frequency ranges”, 3GPP RAN4#100-bis, CATT

- 3GPP TS 38.101-1 v18.5.0 (2024-03), “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”.

- 3GPP TS 38.101-1 v18.5.0 (2024-03), “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”.

- 3GPP TS 38.104 v18.5.0 (2024-03), “NR; Base Station (BS) radio transmission and reception”.

- 3GPP TR 38.803 v14.3.0 (2022-03), “Study on new radio access technology: Radio Frequency (RF) and co-existence aspects”.

- 3GPP TR 38.901 v18.0.0 (2024-03), “Study on channel model for frequencies from 0.5 to 100 GHz”

- 3GPP TR 38.921 v18.0.1 (2-24-03), “Study on International Mobile Telecommunications (IMT) parameters for 6.425-7.025 GHz, 7.025-7.125 GHz and 10.0-10.5 Ghz (Release 18)”.

- R4-2409786, “On IMT BS-UE parameters for 14800 to 15350 MHz frequency range”, 3GPP RAN4#111, Ericsson

# Annex

In Table 5-1 and Table 5-2, parameters considered for simulation campaign relevant for urban macro and urban hotspot scenarios, FR1 and FR2 are listed.

Table 6-1: Urban Macro scenario

|  |  | Parameters | Scenario |
| --- | --- | --- | --- |
| System parameters |  | Scenario | UMa, Hexagonal layout, 19 BS per operator, 3 sectors per site, with wrapping |
|  |  | Parameters | Scenario |
| System parameters |  | Scenario | UMa, Hexagonal layout, 19 BS per operator, 3 sectors per site, with wrapping |  |
|  |  | ISD | 450 m |  |
|  |  | Carrier Frequency | 15 GHz |  |
|  |  | Duplex Type | TDD (DDDDU) |  |
|  |  | Base Static TDD pattern | 80:20 DL:UL |  |
|  |  | Channel bandwidth | 100 MHz |  |
|  |  | Sub-Carrier spacing | 60 kHz |  |
|  |  | Number of active UEs | 1 active users in UL or DL per cell at a time |  |
|  |  | Channel model | UMa TR 38.921/38.901 |  |
|  |  | UE to BS min 2D distance | 35 m |  |
|  |  | Grid-shift | 100% |  |
| BS |  | (Mg, Ng, M, N, P) | (4,1,4,32,2) (1024 AE) (4,1,8,32,2) (2048 AE) |
|  |  | (Mg, Ng, M, N, P) | (4,1,4,32,2) (1024 AE) (4,1,8,32,2) (2048 AE) |
|  |  | Sub-array configuration | 4x1 |
|  |  | gNB Tx Power | 46 dBm |
|  |  | (dv, dh) | (0.5λ, 0.7λ) |
|  |  | Antenna element gain | 6.4 dBi |
|  |  | Antenna element | TR 38.803 |
|  |  | Noise figure | 8 dB |
|  |  | Link level model | As per TR 38.803/38.921 |
|  |  | BS height | 25 m |
| UE |  | UE antenna |  | 2x2 |
|  | Antenna element gain |  | 3 dBi |
|  | Max UE TX Power |  | 23 dBm per Tx |
|  | UE power control |  | Sec. 9.1 TR36.942 |
|  | SNR target |  | 15 dB |
|  | Noise figure |  | 10 dB |
|  | Link level model |  | As per TR 38.803/38.921 |
|  | UE distribution outdoor/indoor |  | 100:00 uniform distribution |