3GPP TSG-RAN4 Meeting #111   	R4-2409829

# Fukuoka City, Fukuoka, Japan, 20th - 24th May, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | draftCR | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR on FRC for 8Rx UEs TDD 2 layers in CBW 5MHz to 30MHz |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_RF_FR1_enh2-Perf |  |  |  |  |  | Date: |  |  | 2024-05-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | New 8Rx CA demodulation requirements for rank 2 are introduced to 38.101-4. New FRCs need to be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The summary of changes in this CR as below: - update FRC in Clause A.3.2.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No new FRCs for FR1 8Rx CA demodulation requirements will be defined |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# A.3	DL reference measurement channels

## A.3.2	Reference measurement channels for PDSCH performance requirements

### A.3.2.2	TDD

#### A.3.2.2.2	Reference measurement channels for SCS 30 kHz FR1

<Start of changes>

Table A.3.2.2.2-36: PDSCH Reference Channel for TDD CC with UL-DL pattern FR1.30-1 and CA scenario

| Parameter |  | Unit |  | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  |  | R.PDSCH.2-36.1 TDD | R.PDSCH.2-36.2 TDD | R.PDSCH.2-36.3 TDD | R.PDSCH.2-36.4 TDD | R.PDSCH.2-36.5 TDD | R.PDSCH.2-36.6 TDD |
| Channel bandwidth |  | MHz | 5 | 10 | 15 | 20 | 25 | 30 |
| Subcarrier spacing |  | kHz | 30 | 30 | 30 | 30 | 30 | 30 |
| Allocated resource blocks |  | PRBs | 11 | 24 | 38 | 51 | 65 | 78 |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  |  | 4 | 4 | 4 | 4 | 4 | 4 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  |  | 12 | 12 | 12 | 12 | 12 | 12 |
| Allocated slots per 2 frames |  |  | 31 | 31 | 31 | 31 | 31 | 31 |
| MCS table |  |  | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| MCS index |  |  | 19 | 19 | 19 | 19 | 19 | 19 |
| Modulation |  |  | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| Target Coding Rate |  |  | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 |
| Number of MIMO layers |  |  | 2 | 2 | 2 | 2 | 2 | 2 |
| Number of DMRS REs |  |  |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  |  | 6 | 6 | 6 | 6 | 6 | 6 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  |  | 12 | 12 | 12 | 12 | 12 | 12 |
| Overhead for TBS determination |  |  | 0 | 0 | 0 | 0 | 0 | 0 |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} |  | Bits | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | Bits | 2792 | 6144 | 9736 | 13064 | 16392 | 19968 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | Bits | 8712 | 18960 | 30216 | 40976 | 52224 | 62504 |
| Transport block CRC per Slot |  |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} |  | Bits | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | Bits | 16 | 16 | 16 | 24 | 24 | 24 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | Bits | 24 | 24 | 24 | 24 | 24 | 24 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} |  | CBs | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | CBs | 1 | 1 | 2 | 2 | 2 | 3 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,39} |  | CBs | 2 | 3 | 4 | 5 | 7 | 8 |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} |  | Bits | N/A | N/A | N/A | N/A | N/A | N/A |
| For Slots i = 20, 21 |  | Bits | 16632 | 36288 | 57456 | 77112 | 98280 | 117936 |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | Bits | 5544 | 12096 | 19152 | 25704 | 32760 | 39312 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6} for i from {1,…,19,22,…,39} |  | Bits | 17424 | 38016 | 60192 | 80784 | 102960 | 123552 |
| Max. Throughput averaged over 2 frames |  | Mbps | 12.320 | 26.825 | 42.739 | 57.930 | 73.781 | 88.374 |
|  | Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |  |

<End of changes>