3GPP TSG-RAN WG4 Meeting #111	R4-2409839

# Fukuoka, Japan, 20 - 24 May, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | Draft | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on PDSCH requirement with multi-Rx reception for FR2 HST (TS38.101-4, Rel-18) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_HST_FR2_enh-Perf |  |  |  |  |  | Date: |  |  | 2024-05-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduce PDSCH requirement with multi-Rx reception. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For introducing PDSCH requirement with multi-Rx reception, add clause 7.2.2.2.6. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be inconsist between specification and RAN4 agreements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.2.2.6 (New clause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Based on the draft big CR R4-2405985. New clause: 7.2.2.2.6 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of R4-2408960 |  |  |  |  |  |  |  |  |

# <START OF THE CHANGE 1>

##### 7.2.2.2.6	Minimum requirements for multi-Rx simultaneous reception in FR2 HST-DPS

The performance requirements are specified in Table 7.2.2.2.6-3, with the addition of test parameters in Table 7.2.2.2.6-2 and the downlink physical channel setup according to Annex C.5.1.

The test purposes are specified in Table 7.2.2.2.6-1.

Table 7.2.2.2.6-1: Test purposes

| Purpose | Test index |
| --- | --- |
| Verify UE performance for multi-Rx simultaneous reception in FR2 HST-DPS scenario defined in [B.3.4.3] when UE is configured two different values of CORESETPoolIndex in ControlResourceSet and the UE receives multiple PDCCHs scheduling with fully-overlapping PDSCHs | 1-1 |

Table 7.2.2.2.6-2: Test parameters

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  |  | TDD |
| Active DL BWP index |  |  |  | 1 |
| PDCCH configuration | TCI state |  |  | Note 1 |
| PDSCH configuration | Mapping type |  |  | Type A |
|  | k0 |  |  | 0 |
|  | Starting symbol (S) |  |  | 1 |
|  | Length (L) |  |  | Specific to each Reference channel |
|  | PDSCH aggregation factor |  |  | 1 |
|  | PRB bundling type |  |  | Static |
|  | PRB bundling size |  |  | 2 |
|  | Resource allocation type |  |  | Type 0 |
|  | RBG size |  |  | Config2 |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |
|  | TCI state |  |  | Note 1 |
| PDSCH DMRS configuration | DMRS Type |  |  | Type 1 |
|  | Number of additional DMRS |  |  | 2 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |
| CSI-RS for tracking | Resource set #1 | First subcarrier index in the PRB used for CSI-RS (k0) |  | 0 for CSI-RS resource 1,2,3,4 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 |
|  |  |  |  | l0 = 9 for CSI-RS resource 2 and 4 |
|  |  | CSI-RS periodicity | Slots | 80 for CSI-RS resource 1,2,3,4 |
|  |  | CSI-RS offset | Slots | 5 for CSI-RS resource 1 and 2 |
|  |  |  |  | 6 for CSI-RS resource 3 and 4 |
|  |  | QCL info |  | TCI state #4 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB =ceil(BWP size/4)*4 |
|  | Resource set #2 | First subcarrier index in the PRB used for CSI-RS (k0) |  | 0 for CSI-RS resource 5,6,7,8 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 4 for CSI-RS resource 5 and 7 |
|  |  |  |  | l0 = 8 for CSI-RS resource 6 and 8 |
|  |  | CSI-RS periodicity | Slots | 80 for CSI-RS resource 5,6,7,8 |
|  |  | CSI-RS offset | Slots | 5 for CSI-RS resource 5 and 6 |
|  |  |  |  | 6 for CSI-RS resource 7 and 8 |
|  |  | QCL info |  | TCI state #5 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB =ceil(BWP size/4)*4 |
|  | Resource set #3 | First subcarrier index in the PRB used for CSI-RS (k0) |  | 1 for CSI-RS resource 9,10,11,12 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 9 and 11 |
|  |  |  |  | l0 = 9 for CSI-RS resource 10 and 12 |
|  |  | CSI-RS periodicity | Slots | 80 for CSI-RS resource 9,10,11,12 |
|  |  | CSI-RS offset | Slots | 5 for CSI-RS resource 9 and 10 |
|  |  |  |  | 6 for CSI-RS resource 11 and 12 |
|  |  | QCL info |  | TCI state #6 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB =ceil(BWP size/4)*4 |
|  | Resource set #4 | First subcarrier index in the PRB used for CSI-RS (k0) |  | 1 for CSI-RS resource 13,14,15,16 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 4 for CSI-RS resource 13 and 15 |
|  |  |  |  | l0 = 8 for CSI-RS resource 14 and 16 |
|  |  | CSI-RS periodicity | Slots | 80 for CSI-RS resource 13,14,15,16 |
|  |  | CSI-RS offset | Slots | 5 for CSI-RS resource 13 and 14 |
|  |  |  |  | 6 for CSI-RS resource 15 and 16 |
|  |  | QCL info |  | TCI state #7 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB =ceil(BWP size/4)*4 |
|  | Resource set #13 | First subcarrier index in the PRB used for CSI-RS (k0) |  | 2 for CSI-RS resource 17,18,19,20 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 17 and 19 |
|  |  |  |  | l0 = 9 for CSI-RS resource 18 and 20 |
|  |  | CSI-RS periodicity | Slots | 80 for CSI-RS resource 17,18,19,20 |
|  |  | CSI-RS offset | Slots | 5 for CSI-RS resource 17 and 18 |
|  |  |  |  | 6 for CSI-RS resource 19 and 20 |
|  |  | QCL info |  | TCI state #12 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB =ceil(BWP size/4)*4 |
|  | Resource set #14 | First subcarrier index in the PRB used for CSI-RS (k0) |  | 2 for CSI-RS resource 21,22,23,24 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 4 for CSI-RS resource 21 and 23 |
|  |  |  |  | l0 = 8 for CSI-RS resource 22 and 24 |
|  |  | CSI-RS periodicity | Slots | 80 for CSI-RS resource 21,22,23,24 |
|  |  | CSI-RS offset | Slots | 5 for CSI-RS resource 21 and 22 |
|  |  |  |  | 6 for CSI-RS resource 23 and 24 |
|  |  | QCL info |  | TCI state #13 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB =ceil(BWP size/4)*4 |
|  | Resource set #15 | First subcarrier index in the PRB used for CSI-RS (k0) |  | 3 for CSI-RS resource 25,26,27,28 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 25 and 27 |
|  |  |  |  | l0 = 9 for CSI-RS resource 26 and 28 |
|  |  | CSI-RS periodicity | Slots | 80 for CSI-RS resource 25,26,27,28 |
|  |  | CSI-RS offset | Slots | 5 for CSI-RS resource 25 and 26 |
|  |  |  |  | 6 for CSI-RS resource 27 and 28 |
|  |  | QCL info |  | TCI state #14 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB =ceil(BWP size/4)*4 |
|  | Resource set #16 | First subcarrier index in the PRB used for CSI-RS (k0) |  | 3 for CSI-RS resource 29,30,31,32 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 4 for CSI-RS resource 29 and 31 |
|  |  |  |  | l0 = 8 for CSI-RS resource 30 and 32 |
|  |  | CSI-RS periodicity | Slots | 80 for CSI-RS resource 29,30,31,32 |
|  |  | CSI-RS offset | Slots | 5 for CSI-RS resource 29 and 30 |
|  |  |  |  | 6 for CSI-RS resource 31 and 32 |
|  |  | QCL info |  | TCI state #15 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB =ceil(BWP size/4)*4 |
| NZP CSI-RS for CSI acquisition | Resource set #5 | First subcarrier index in the PRB used for CSI-RS (k0) |  | 0 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #0 |
|  | Resource set #6 | First subcarrier index in the PRB used for CSI-RS (k0) |  | 2 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #1 |
|  | Resource set #7 | First subcarrier index in the PRB used for CSI-RS (k0) |  | 4 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #2 |
|  | Resource set #8 | First subcarrier index in the PRB used for CSI-RS (k0) |  | 6 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #3 |
|  | Resource set #17 | First subcarrier index in the PRB used for CSI-RS (k0) |  | 0 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 13 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 1 |
|  |  | QCL info |  | TCI state #8 |
|  | Resource set #18 | First subcarrier index in the PRB used for CSI-RS (k0) |  | 2 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 13 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 1 |
|  |  | QCL info |  | TCI state #9 |
|  | Resource set #19 | First subcarrier index in the PRB used for CSI-RS (k0) |  | 4 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 13 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 1 |
|  |  | QCL info |  | TCI state #10 |
|  | Resource set #20 | First subcarrier index in the PRB used for CSI-RS (k0) |  | 6 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 13 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 1 |
|  |  | QCL info |  | TCI state #11 |
| CSI-RS for beam refinement | Resource set #9 | First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 8 for CSI-RS resource 1 l0 = 9 for CSI-RS resource 2 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #0 |
|  | Resource set #10 | First subcarrier index in the PRB used for CSI-RS |  | k0=1 for CSI-RS resource 3,4 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 8 for CSI-RS resource 3 l0 = 9 for CSI-RS resource 4 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #1 |
|  | Resource set #11 | First subcarrier index in the PRB used for CSI-RS |  | k0=2 for CSI-RS resource 5,6 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 8 for CSI-RS resource 5 l0 = 9 for CSI-RS resource 6 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #2 |
|  | Resource set #12 | First subcarrier index in the PRB used for CSI-RS |  | k0=3 for CSI-RS resource 7,8 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 8 for CSI-RS resource 7 l0 = 9 for CSI-RS resource 8 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #3 |
|  | Resource set #21 | First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 9,10 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 10 for CSI-RS resource 9 l0 = 11 for CSI-RS resource 10 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 1 |
|  |  | QCL info |  | TCI state #8 |
|  | Resource set #22 | First subcarrier index in the PRB used for CSI-RS |  | k0=1 for CSI-RS resource 11,12 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 10 for CSI-RS resource 11 l0 = 11 for CSI-RS resource 12 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 1 |
|  |  | QCL info |  | TCI state #9 |
|  | Resource set #23 | First subcarrier index in the PRB used for CSI-RS |  | k0=2 for CSI-RS resource 13,14 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 10 for CSI-RS resource 13 l0 = 11 for CSI-RS resource 14 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 1 |
|  |  | QCL info |  | TCI state #10 |
|  | Resource set #24 | First subcarrier index in the PRB used for CSI-RS |  | k0=3 for CSI-RS resource 15,16 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 10 for CSI-RS resource 15 l0 = 11 for CSI-RS resource 16 |
|  |  | CSI-RS periodicity | Slots | 160 |
|  |  | CSI-RS offset | Slots | 1 |
|  |  | QCL info |  | TCI state #11 |
| TCI state #0 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking Resource set #1' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking Resource set #1' configuration |
|  |  | QCL Type |  | Type D |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 5 from 'CSI-RS for tracking Resource set #2' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | CSI-RS resource 5 from 'CSI-RS for tracking Resource set #2' configuration |
|  |  | QCL Type |  | Type D |
| TCI state #2 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 9 from 'CSI-RS for tracking Resource set #3' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | CSI-RS resource 9 from 'CSI-RS for tracking Resource set #3' configuration |
|  |  | QCL Type |  | Type D |
| TCI state #3 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 13 from 'CSI-RS for tracking Resource set #4' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | CSI-RS resource 13 from 'CSI-RS for tracking Resource set #4' configuration |
|  |  | QCL Type |  | Type D |
| TCI state #8 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 17 from 'CSI-RS for tracking Resource set #13' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | CSI-RS resource 17 from 'CSI-RS for tracking Resource set #13' configuration |
|  |  | QCL Type |  | Type D |
| TCI state #9 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 21 from 'CSI-RS for tracking Resource set #14' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | CSI-RS resource 21 from 'CSI-RS for tracking Resource set #14' configuration |
|  |  | QCL Type |  | Type D |
| TCI state #10 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 25 from 'CSI-RS for tracking Resource set #15' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | CSI-RS resource 25 from 'CSI-RS for tracking Resource set #15' configuration |
|  |  | QCL Type |  | Type D |
| TCI state #11 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 29 from 'CSI-RS for tracking Resource set #16' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | CSI-RS resource 29 from 'CSI-RS for tracking Resource set #16' configuration |
|  |  | QCL Type |  | Type D |
| TCI state #4 | Type 1 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type D |
| TCI state #5 | Type 1 QCL information | SSB index |  | SSB #1 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | SSB #1 |
|  |  | QCL Type |  | Type D |
| TCI state #6 | Type 1 QCL information | SSB index |  | SSB #2 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | SSB #2 |
|  |  | QCL Type |  | Type D |
| TCI state #7 | Type 1 QCL information | SSB index |  | SSB #3 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | SSB #3 |
|  |  | QCL Type |  | Type D |
| TCI state #12 | Type 1 QCL information | SSB index |  | SSB #4 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | SSB #4 |
|  |  | QCL Type |  | Type D |
| TCI state #13 | Type 1 QCL information | SSB index |  | SSB #5 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | SSB #5 |
|  |  | QCL Type |  | Type D |
| TCI state #14 | Type 1 QCL information | SSB index |  | SSB #6 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | SSB #6 |
|  |  | QCL Type |  | Type D |
| TCI state #15 | Type 1 QCL information | SSB index |  | SSB #7 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | SSB #7 |
|  |  | QCL Type |  | Type D |
| Number of HARQ Processes |  |  |  | 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.3 |
| Note 1: For Test 1-1, SSB # (2k mod 8) , CSI-RS (for tracking) resource set # ((k mod 4)+1), CSI-RS (for CSI acquisition) resource set # ((k mod 4) + 5) and CSI-RS (for beam refinement) resource set # ((k mod 4) + 9) are transmitted by kth RRH; SSB # ((2k mod 8)+1) , CSI-RS (for tracking) resource set # ((k mod 4) + 13), CSI-RS (for CSI acquisition) resource set # ((k mod 4) + 17) and CSI-RS (for beam refinement) resource set # ((k mod 4) + 21) are transmitted by kth RRH. TCI state switching command scheduled by MAC CE with MCS 4 is transmitted in slot #i that satisfy modi,n=0 (i≠0) and modi,n=16457. PDCCH and PDSCH associated with TCI # (k mod 4) is transmitted by kth RRH from slot# 0,k=1k-1*n+1+THARQ+TMAC proc+TfirstSSB_1+TSSB proc+TfirstTRSafterSSB+TTRS proc,k=2,3,4… to slot# k*n,k=1,2,3…, PDCCH and PDSCH associated with TCI # ((k mod 4)+8) is transmitted by kth RRH from slot# 0,k=-1k+1*n+16457+1+THARQ+TMAC proc+TfirstSSB_2+TSSB proc+TfirstTRSafterSSB+TTRS proc,k=0,1,2… to slot# [k+1*n+16457],k=-1,0,1,2…, where k is the RRH number, n = 57600 is the number of slots between two RRHs, THARQ = 4 is the number of slots between PDSCH and corresponding HARQ-ACK information, TMAC proc = 24 is the number of slots for MAC CE processing, TfirstSSB_1= 132 and TfirstSSB_2= 155 is the number of slots to first SSB transmission occasion after MAC CE command is decoded by the UE, TSSB proc= 16 is the number of slots for SSB processing, TfirstTRSafterSSB = 69 is the number of slots to first TRS transmission occasion after first SSB is processed by the UE, TTRSproc= 16 is the number of slots for TRS processing. PDCCH and PDSCH are DTXed in other slots in which throughput statistics are not considered. |  |  |  |  |

Table 7.2.2.2.6-3: Minimum performance for multi-Rx simultaneous reception in FR2 HST-DPS

| Test num | RRH num | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Number of active PDSCH TCI states (Note 3) | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | RRH#i | R.PDSCH.5-18.1 TDD | 200 / 120 | 64QAM, 0.50 | FR2.120-1 | HST-DPS-FR2-BI-B1-MR | 2 | 2x2 | 70 | 15.0 |
|  | RRH#j | R.PDSCH.5-18.2 TDD | 200 / 120 | 16QAM, 0.37 | FR2.120-1 | HST-DPS-FR2-BI-B1-MR | 2 | 2x2 | 70 | 8.2 |
| Note 1 : The RRH#i (i =-1,0,1,2…) and RRH#j (j=1,2,3,…) indicate the RRHs simultaneously transmitting PDSCH to the UE. Note 2: Receive timing difference (RTD) of PDSCH data transmitted from RRH#i (i =-1,0,1,2…) and RRH#j (j=1,2,3,…) at the UE between two Rx chains is 0.88 us (1.5*CP). Note 3: Only 1 available active TCI state per RRH. |  |  |  |  |  |  |  |  |  |  |

# <END OF THE CHANGE 1>