3GPP TSG-RAN WG4 Meeting #111	                        R4-2409856

Fukuoka City, JP, May 20-24, 2024

Source:	China Telecom

Title:		Ad-hoc minutes for NR_cov_enh2_demod WI

Agenda Item:	7.17.3

Document for:	Approval

- Discussion

Issue 1-1: PRACH repetition interval and TDD pattern for Multiple PRACH transmission

Tentative agreement

Consensus: PRACH B4 can be configured in FDD 15kHz SCS

Not consider FR1 15/30 kHz SCS for PRACH repetition requirement definition.

For TDD 120kHz SCS DDDSU

    - For PRACH format A2 and C2:

- Option 1: Define PRACH repetition within 1 slot for fading channel

- Option 2: 10 slots for 120kHz SCS

    - For PRACH format B4:

- DDDSUDDDSUDDDSUDDDSU, i.e.,10 slots

No need to specify PRACH configuration index in the spec.

FFS whether adding AWGN requirements to reduce the impact of fading channel

Issue 1-2: Manufacturer declarations for PRACH repetition test requirements

  - Option 1: Introduce additional manufacturer declarations for supported SCS(s) for PRACH repetition

  - Option 2: No need to introduce additional manufacturer declarations for supported SCS(s) for PRACH repetition

Tentative agreement

option 1

Issue 1-3: SNR requirement value deriving rule for both PUSCH and PUCCH BE demod requirements

- Proposals:

  - Proposal 1 (China Telecom)

    - Reuse the SNR requirement value deriving rule for Rel-15 BS demodulation SNR requirement derivation procedure as agreed in R4-1904713:

Procedure to derive the performance requirements: – Only inputs that consist of a pair of ideal and impaired results can be taken into account. – If the ideal span <= [2]dB: • The AVERAGE impairment results can be used for the performance requirement with [] in the draftCRs/CRs; – Else if the ideal span is larger than [2]dB: • The results farthest from the AVERAGE value is taken out for the AVERAGE and SPAN re-calculation until the ideal span is <=2dB but still with at least 3 companies’ results available: – The ultimate AVERAGE impairment results with corresponding ideal span <=2dB can be used for performance requirement with [] in the draftCRs/CRs. • Otherwise put TBD for the related performance requirements. – If the span of the impairment results after removal the outliers (if any) are larger than 4dB, then the procedure cannot be applied, related performance requirement remain TBD.

- Recommended WF

  - It is recommended to use the above proposal for requirement definition in this meeting.

  - The requirement value will be in [] and companies can still update results in the next meeting as a maintenance part.