3GPP TSG-RAN4 Meeting #111	R4-2409871

# Fukuoka, Japan, 20th – 24th May, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.108 | CR | - | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on performance requirements for PUSCH with DM-RS bundling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_enh-Perf |  |  |  |  |  | Date: |  |  | 2024-05-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In Rel-18 NTN enhancement WI, it was agreed to introduce the PUSCH requirement with DM-RS bundling. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the new section in 8.2.5 for PUSCH requirement with DM-RS bundling |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirement of PUSCH with DM-RS bundling can not be verified well |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.181 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR … CR … |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  | Revision of R4-2409483 |  |  |  |  |  |  |  |  |

<< Unchanged sections omitted >>

<Start of Change >

8.2.5	Requirements for PUSCH with DM-RS bundling

8.2.5.1	General

The performance requirement of PUSCH is determined by a minimum required throughput for a given SNR. The required throughput is expressed as a fraction of maximum throughput for the FRCs listed in annex A. The performance requirements assume HARQ re-transmissions.

Table: 8.2.5.1-1 Test parameters for testing PUSCH with DM-RS bundling

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Channel bandwidth |  | 15kHz SCS: 5MHz 30kHz SCS: 10MHz |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 0, 0, 0 [Note 1] |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | 0 |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | A, B |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
|  | PUSCH aggregation factor | n4 for 15kHz n8 for 30kHz |
| pusch-TimeDomainWindowLength |  | 4 slots for 15kHz 8 slots for 30kHz |
| Frequency domain resource assignment | RB assignment | 6 PRBs in the middle of the test bandwidth |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| Note 1: The effective RV sequence is {0, 2, 3, 1} with slot aggregation. |  |  |

8.2.5.2	Minimum requirements

The throughput shall be equal to or larger than the fraction of maximum throughput for the FRCs stated in tables 8.2.5.2-1 to 8.2.5.2-4 at the given SNR for 1Tx. FRCs are defined in annex A.

Table 8.2.5.2-1: Minimum requirements for PUSCH with DM-RS bundling, Type A, 5 MHz channel bandwidth, 15 kHz SCS in FR1-NTN

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex D) | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLA100-200 Low | 70% | G-FR1-NTN-A3-7 | pos1 | TBD |
|  | 2 | Normal | NTN-TDLA100-200 Low | 70% | G-FR1-NTN-A3-7 | pos1 | TBD |

Table 8.2.5.2-2: Minimum requirements for PUSCH with DM-RS bundling, Type A, 10 MHz channel bandwidth, 30 kHz SCS in FR1-NTN

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex D) | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLA100-200 Low | 70% | G-FR1-NTN-A3-8 | pos1 | TBD |
|  | 2 | Normal | NTN-TDLA100-200 Low | 70% | G-FR1-NTN-A3-8 | pos1 | TBD |

Table 8.2.5.2-3: Minimum requirements for PUSCH with DM-RS bundling, Type B, 5 MHz channel bandwidth, 15 kHz SCS in FR1-NTN

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex D) | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLA100-200 Low | 70% | G-FR1-NTN-A3-7 | pos1 | TBD |
|  | 2 | Normal | NTN-TDLA100-200 Low | 70% | G-FR1-NTN-A3-7 | pos1 | TBD |

Table 8.2.5.2-4: Minimum requirements for PUSCH with DM-RS bundling, Type B, 10 MHz channel bandwidth, 30 kHz SCS in FR1-NTN

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex D) | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | Normal | NTN-TDLA100-200 Low | 70% | G-FR1-NTN-A3-8 | pos1 | TBD |
|  | 2 | Normal | NTN-TDLA100-200 Low | 70% | G-FR1-NTN-A3-8 | pos1 | TBD |

<End of Change >