3GPP TSG-RAN WG4 Meeting #111	R4-2410015

# Fukuoka, Japan, 20 - 24 May, 2024

Title: 	Simulation results on BS demodulation requirements for MIMO evolution

Source: 	Huawei, HiSilicon

Agenda item:	7.19.3.1

Document for:	Discussion

Background

During RAN4#110b meeting, WF [1] on NR_MIMO_evo_DL_UL_demod was approved. In this contribution, we provide our simulation results on UE demodulation requirements for MIMO evolution.

Simulations

TypeII Doppler

Here we provide the performance evaluation using Type I Single Panel and typeII-Doppler-r18 codebook as following Table 2.1-1 and Table 2.1-2.

Table 2.1-1 Performance evaluation for typeII-Doppler-r18 with FDD 15kHz

| Antenna configuration | Gamma @ 90% of maximum throughput |  |
| --- | --- | --- |
|  | 20Hz | 30Hz |
| 2x2 | 29.41 | 7.66 |
| 2x4 | 5.23 | 3.92 |

Table 2.1-2 Performance evaluation for typeII-Doppler-r18 with TDD 30kHz

| Antenna configuration | Gamma @ 90% of maximum throughput |  |
| --- | --- | --- |
|  | 10Hz | 20Hz |
| 2x2 | 20.33 | 22.04 |
| 2x4 | 3.24 | 3.26 |

TypeII CJT

Here we provide the performance evaluation using Type I Single Panel and typeII-CJT-r18 codebook as following Table 2.2-1.

Table 2.2-1 Performance evaluation for typeII-CJT-r18

| Antenna configuration | Gamma @ 90% of maximum throughput |
| --- | --- |
| 2x2 | 2.42 |
| 2x4 | 2.46 |

Rel-18 DMRS

Here we provide the simulation results using legacy DMRS ports and the new DMRS ports as Table 2.3-1.

Table 2.3-1 Simulation results using legacy DMRS ports and the new DMRS ports

| Duplex | SCS | Rank | Antenna configuration | SNR @ 70% of maximum throughput |
| --- | --- | --- | --- | --- |
| FDD | 15kHz | 4 | 4x4 | 12.31 |
| TDD | 30kHz | 4 | 4x4 | 12.53 |

Conclusions

In this contribution, we provide our updated simulation results on UE demodulation requirements for MIMO evolution.

Reference

R4-2406111, WF on [110bis][328] NR_MIMO_evo_DL_UL_demod, RAN4#110b, Samsung