3GPP TSG-RAN4 Meeting # 111	R4-2410021

# Fukuoka City, Fukuoka, Japan, 20th - 24th, May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-5 | CR | Draft CR | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR to 38.101-5: FRC for NTN PDSCH performance requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_enh-Perf |  |  |  |  |  | Date: |  |  | 2024-05-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduce new FRC for FR2 NTN PDSCH performance requirements |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduce new FRC for MCS4 and MCS13 with 200M BW and 120k SCS |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is no proper FRC for the newly introduced FR2 NTN PDSCH performance requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.2.1.2(New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-5 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R4-2408745. |  |  |  |  |  |  |  |  |

----------------------------------------------------- Beginning of Change 1------------------------------------------------------------

Table A.3.2.1.3-3: PDSCH Reference Channel for FDD (QPSK)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  |  |  | R.PDSCH.3-3.1 FDD |  |  |  |  |
| Channel bandwidth |  | MHz |  | 200 |  |  |  |  |
| Subcarrier spacing |  | kHz |  | 120 |  |  |  |  |
| Number of allocated resource blocks |  | PRBs |  | 132 |  |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  | 13 |  |  |  |  |
| Allocated slots per 2 frames |  | Slots |  | 159 |  |  |  |  |
| MCS table |  |  |  | 64QAM |  |  |  |  |
| MCS index |  |  |  | 4 |  |  |  |  |
| Modulation |  |  |  | QPSK |  |  |  |  |
| Target Coding Rate |  |  |  | 0.30 |  |  |  |  |
| Number of MIMO layers |  |  |  | 1 |  |  |  |  |
| Number of DMRS REs |  |  |  | 12 |  |  |  |  |
| Overhead for TBS determination |  |  |  | 0 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |  |
| For Slot i = 0 |  | Bits |  | N/A |  |  |  |  |
| For Slots i = 1,…, 159 |  | Bits |  | 11528 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |  |  |
| For Slot i = 0 |  | Bits |  | N/A |  |  |  |  |
| For Slots i = 1,…, 159 |  | Bits |  | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |
| For Slot i = 0 |  | CBs |  | N/A |  |  |  |  |
| For Slots i = 1,…, 159 |  | CBs |  | 2 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |  |  |
| For Slot i = 0 |  | Bits |  | N/A |  |  |  |  |
| For Slots i = 80, 81 |  | Bits |  | 36432 |  |  |  |  |
| For Slots i =1,…, 79, 82, …, 159 |  | Bits |  | 38016 |  |  |  |  |
| Max. Throughput averaged over 2 frames |  | Mbps |  | 91.648 |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

Table A.3.2.1.3-4: PDSCH Reference Channel for FDD (16QAM)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.3-4.1 FDD |  |  |  |  |
| Channel bandwidth | MHz | 200 |  |  |  |  |
| Subcarrier spacing | kHz | 120 |  |  |  |  |
| Number of allocated resource blocks | PRBs | 132 |  |  |  |  |
| Number of consecutive PDSCH symbols |  | 13 |  |  |  |  |
| Allocated slots per 2 frames | Slots | 159 |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |
| MCS index |  | 13 |  |  |  |  |
| Modulation |  | 16QAM |  |  |  |  |
| Target Coding Rate |  | 0.48 |  |  |  |  |
| Number of MIMO layers |  | 1 |  |  |  |  |
| Number of DMRS REs |  | 12 |  |  |  |  |
| Overhead for TBS determination |  | 0 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A |  |  |  |  |
| For Slots i = 1,…, 159 | Bits | 36896 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A |  |  |  |  |
| For Slots i = 1,…, 159 | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot i = 0 | CBs | N/A |  |  |  |  |
| For Slots i = 1,…, 159 | CBs | 5 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A |  |  |  |  |
| For Slots i = 80, 81 | Bits | 72864 |  |  |  |  |
| For Slots i =1,…, 79, 82, …, 159 | Bits | 76032 |  |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 293.323 |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

--------------------------------------------------------- End of Change 1----------------------------------------------------------------