3GPP TSG-RAN WG4 Meeting #111	R4-2410123

Fukuoka, Japan, 20th May 2024 ‒ 24th May 2024

Agenda item:			7.21.3

Source:	Moderator (Ericsson)

Title:	Topic summary for [111][330] NR_redcap_enh_demod

Document for:	Information

# Introduction

This topic summary lists open issues on Demodulation performance part for Rel-18 WI Enhanced support of reduced capability NR devices.

- 7.21.2.1: UE demodulation performance and CSI requirements [NR_redcap_enh-Perf]

# Topic #1: UE demodulation and CSI reporting requirements

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2407267 | Apple | Simulation results |
| R4-2407268 | Apple | Not found |
| R4-2408774 | Ericsson | Proposal 1: Set SNR=5/6dB and SNR=11/12dB for CQI reporting test in static condition for FDD/HD-FDD/TDD with 1Rx, same as Rel-17 RedCap CQI reporting test in static condition. Proposal 2: Set SNR=8/9dB and SNR=14/15dB for CQI reporting test in static condition for FDD/HD-FDD/TDD with 2Rx, same as Rel-17 RedCap CQI reporting test in static condition. Proposal 3: Set SNR=9/10dB and SNR=15/16dB for CQI reporting test in fading condition for FDD/HD-FDD/TDD with 1Rx, same as Rel-17 RedCap CQI reporting test in fading condition. Proposal 4: Set SNR=6/7dB for CQI reporting test in fading condition for FDD/HD-FDD/TDD with 2Rx, same as Rel-17 RedCap CQI reporting test in fading condition. Proposal 5: Apply the same requirements as Rel-17 RedCap UE CQI reporting test in fading condition. Proposal 6: Apply the same requirements as Rel-17 RedCap UE PMI reporting test for FDD/HD-FDD/TDD with 1Rx/2Rx. |
| R4-2408775 | Ericsson | Simulation results |
| R4-2408776 | Ericsson | Simulation results summary |
| R4-2409014 | Huawei, HiSilicon | Observation 1: For most SNR points of TDD, the throughput gain of follow CQI is quite limited and for some SNR points, the negative gain can be observed. Proposal 1: RAN4 to define CQI requirements under AWGN conditions and PMI requirements by reusing the Rel-17 RedCap test setup and test metric. Proposal 2: RAN4 shall not define CQI requirements under fading conditions. |
| R4-2409015 | Huawei, HiSilicon | Simulation results |
| R4-2409164 | Nokia, Nokia Shanghai Bell | Simulation results |

## Open issues summary

### Sub-topic 1-1 CSI reporting requirements

Background: RAN4#110bis agreed to investigate whether the Rel-17 RedCap CQI/PMI reporting requirements can be applicable for Rel-18 eRedCap with the following configuration:

|  | CQI reporting tests (static/fading) | PMI reporting test |
| --- | --- | --- |
| FDD CBW/SCS=10MHz/15kHz | BWP: 52 RBs CSI-RS: 24RBs (PRB 0 to 23) PDSCH:15 RBs (PRB 0 to 14)) | BWP: 52 RBs CSI-RS: 28RBs (PRB 0 to 27) PDSCH: 25RBs (PRB 0 to 24) |
| TDD CBW/SCS=20MHz/30kHz | BWP: 51 RBs CSI-RS: 24RBs (PRB 0 to 23) PDSCH: 7RBs (PRB 0 to 6) | BWP: 51 RBs CSI-RS: 24RBs (PRB 0 to 23) PDSCH: 12RBs (PRB 0 to 14) |
| Note: BWP size is same as the transmission bandwidth in TS38.101-1 5.3.2. |  |  |

Issue 1-1-1:  CQI reporting test in fading condition

- Observations:

  - Ericsson: Simulation result shows we can keep the same accuracy requirements (i.e., α = 20%).

  - Huawei: For most SNR points of TDD, the throughput gain of follow CQI is quite limited and for some SNR points, the negative gain can be observed.

- Proposals:

  - Option 1 (Ericsson):

    - Set SNR=9/10dB and SNR=15/16dB for CQI reporting test in fading condition for FDD/HD-FDD/TDD with 1Rx, same as Rel-17 RedCap CQI reporting test in fading condition.

    - Set SNR=6/7dB for CQI reporting test in fading condition for FDD/HD-FDD/TDD with 2Rx, same as Rel-17 RedCap CQI reporting test in fading condition.

    - Apply the same requirements as Rel-17 RedCap UE CQI reporting test in fading condition:

      - A CQI index not in the set {median CQI - 1, median CQI, median CQI + 1} shall be reported at least α% of the time, where α = 20%.

      - Throughput ratio of follow CQI and fixed median CQI shall be ≥ γ, where γ = 1.05

      - Average BLER with follow CQI shall be greater than or equal to 0.02.

  - Option 2 (Huawei):

    - RAN4 shall not define CQI requirements under fading conditions.

- Recommended WF

  - Different observations from contributions.

  - Discuss whether to define the CQI reporting requirements in fading condition for Rel-18 eRedCap.

Issue 1-1-2:  CQI reporting test in static condition

- Proposal (Ericsson, Huawei):

  - Define CQI reporting requirements under AWGN conditions by reusing the Rel-17 RedCap test setup and test metric.

    - Set SNR=5/6dB and SNR=11/12dB for CQI reporting test in static condition for FDD/HD-FDD/TDD with 1Rx, same as Rel-17 RedCap CQI reporting test in static condition.

    - Set SNR=8/9dB and SNR=14/15dB for CQI reporting test in static condition for FDD/HD-FDD/TDD with 2Rx, same as Rel-17 RedCap CQI reporting test in static condition.

- Recommended WF

  - Reuse the same test setup and test metrics as Rel-17 RedCap CQI reporting requirements in static condition.

Issue 1-1-3:  PMI reporting tests

- Proposals (Ericsson, Huawei)

  - Define PMI reporting requirements by reusing the Rel-17 RedCap test setup and test metric.

    - Define the test metric as γ=tuetrnd , where tue is 90 % of the maximum throughput obtained at SNRue using the precoders configured according to the UE reports, and trnd is the throughput measured at SNRue with random precoding based on type I Single Panel codebook.

    - Set γ=1.3.

- Recommended WF

  - Reuse the same test setup and test metrics as Rel-17 RedCap PMI reporting requirements in static condition.

# Topic #2: CRs

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2407269 | Apple | Draft CR on PDSCH TDD Requirements for Enhanced Support of RedCap |
| R4-2407270 | Apple | Draft CR on SDR Requirements for Enhanced Support of RedCap |
| R4-2408038 | Qualcomm Incorporated | Draft CR on definition and requirements applicability |
| R4-2408777 | Ericsson | Draft CR on introduction of eRedCap FDD PDSCH demodulation requirements |
| R4-2409016 | Huawei, HiSilicon | Draft CR on introduction of PMI requirements for eRedCap |
| R4-2409163 | Nokia, Nokia Shanghai Bell | Draft CR of CQI reporting requirements for static/fading condition for RedCap enhancements |
| R4-2408778 | Ericsson | Big CR for 38.101-4: Introduction of eRedCap UE demodulation and CSI reporting requirements |

## Open issues summary

### Sub-topic 2-1 review draft CR

Issue 2-1-1: Review draft CRs

PDSCH demodulation requirements to be updated according to the simulation summary R4-2408776.

Companies are encouraged to give comments for draft CRs.

# Recommendation for Tdocs

| T-doc number | Suggested status | Comments |
| --- | --- | --- |
| R4-2407267 | Noted |  |
| R4-2407268 | Withdrawn? | Not found |
| R4-2408774 | Noted |  |
| R4-2408775 | Noted |  |
| R4-2408776 | Noted | Summary of simulation results |
| R4-2409014 | Noted |  |
| R4-2409015 | Noted |  |
| R4-2409164 | Noted |  |
| R4-2407269 | Revised | Draft CR |
| R4-2407270 | Revised | Draft CR |
| R4-2408038 | Revised | Draft CR |
| R4-2408777 | Revised | Draft CR |
| R4-2409016 | Revised | Draft CR |
| R4-2409163 | Revised | Draft CR |
| R4-2408778 | Email approval | Big CR |