3GPP TSG-RAN4 Meeting #111	R4-2410156

# Fukuoka, Japan,  May 20 – 24, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev | 1 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR – Corrections to PRS measurement period with eDRX in RRC_IDLE state |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm, Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 5/13/2024 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Capture new agreement from RAN4#110 (R4-2403475): - For Case 1, PRS measurement start is not limited to PTW when the PRS resource indicated in the assistance data is not within PTW. Harmonize the requirement for the start of the measurement period in RRC_IDLE state across all the NR positioning measurements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct the start of the measurement period for DL RSTD with PRS BW aggregation (cl. 4.5.2.6). - Correct the start of the measurement period for DL RSCPD reported with DL RSTD (cl. 4.5.5.5). - Correct the start of the measurement period for DL RSTD without Rx hopping for RedCap (cl. 4.6.2.5). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PRS measurement period requirements in RRC_IDLE state would be incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5.2.6, 4.5.5.5, 4.6.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Incorporates changes from Big draft CR R4-2405983 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of R4-2407972. |  |  |  |  |  |  |  |  |

----------------------START OF CHANGE----------------------------

#### 4.5.2.6	Measurements Period Requirements with Bandwidth Aggregation

The requirements in this clause apply provided that UE receives requests from LMF to perform PRS measurement on aggregated positioning frequency layers (PFLs) via nr-DL-PRS-JointMeasurementRequestedPFL-List in NR-DL-TDOA-RequestLocationInformation.

After receiving both NR-DL-TDOA-ProvideAssistanceData message and NR-DL-TDOA-RequestLocationInformation message from the LMF via LPP [34], the UE shall be able to measure multiple (up to the UE capability specified in Clause 4.5.2.3) DL RSTD measurements, defined in TS 38.215 [4], during the measurement period TRSTD,Total defined as:

TRSTD,total= TRSTD, aggr+ TRSTD,non-aggr + Tmargin

Where:

-	TRSTD, aggr is the total measurement period for aggregated measurements, and

-	TRSTD,non-aggr is the total measurement period for non-aggregated measurements, and

-	When both TRSTD, aggr  and TRSTD,non-aggr are non-zero, Tmargin=maxTeffect,j, where the maximum Teffect,j is across all the PFL combination(s) and non-aggregated PFL(s) configured for positioning measurements; otherwise Tmargin is equal to zero.

TRSTD,non-aggr is zero if every resource set on every PFL is linked for aggregation to at least one other resource set on another PFL. Otherwise, TRSTD,non-aggr is as defined in clause 4.5.2.5 except that

-	only PFLs containing resource set(s) not linked to any other resource set(s) are considered in L

-	on each PFL i, only resource set(s) not linked to any other resource set(s) are considered in Tavailable_PRS,i, NPRS,islot and Lavailable_PRS,i

-	Nsample= 2 if the UE supports the capability of positioning measurements with reduced number of samples as indicated by supportedDL-PRS-ProcessingSamples-RRC-Inactive specified in TS 37.355 [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples.

TRSTD,aggr is zero if no resource sets on any PFL are linked for aggregation with other resource sets on other PFLs. Otherwise, TRSTD,aggr is defined as

TRSTD,aggr= m=1MTRSTD,aggr,m+ M-1*maxTeffect,aggr,m

where:

-	m is the index of PFL combination,

-	M is total number of PFL combinations,

-	Teffect,aggr,m is the periodicity of the PRS measurement in PFL combination m,

-	TRSTD,aggr,m is the measurement period for PRS RSTD measurement in PFL combination m as specified below.

TRSTD,aggr,m=Kcarrier,aggr*NRx,TEG,aggr,m*NRxBeam,aggr,m*NPRS,aggr,mslotNaggr,m'*LavailablePRS,aggr,mNaggr,m*Nsample-1*Teffect,aggr,m+Tlast,aggr,m

where:

-	Kcarrier,aggr is a scaling factor for PRS measurements in RRC_IDLE. If the UE supports parallelPRS-MeasRRC-Inactive-r17, Kcarrier_PRS= 1. Otherwise,

-	If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, Kcarrier_PRSequals to the sum of Kcarrier in 4.2.2.4 and one positioning layer.

-	If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, Kcarrier_PRS equals to the sum of Nlayer in 4.2.2.7 and one positioning layer.

-	NRx,TEG,m is the Rx TEG specific scaling factor:

-	NRx,TEG,m =1 if the UE is not configured by the LMF to measure a PRS resource with multiple Rx TEGs via measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 [34].

-	NRx,TEG,m is defined as follows if the UE is configured by the LMF with measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 [34] to perform measurement on same DL PRS resource of a TRP using different Rx TEGs in NR-DL-TDOA-RequestLocationInformation [34]:

-	NRx,TEG,m = P, if the UE is not capable of receiving same DL PRS resource simultaneously from multiple Rx TEGs, where P is the number of UE Rx TEGs that the UE is requested by LMF to measure the same DL-PRS Resource of a TRP indicated by measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 in [34], and in case ‘n0’ is indicated, P is the maximum number of Rx TEGs with which UE can support to measure the same PRS resource as reported in NR-UE-TEG-Capability.

-	NRx,TEG,m = PQ , if the UE is capable of receiving the same DL PRS resource simultaneously from multiple Rx TEGs, where Q is the number of UE Rx TEGs for measuring the same DL-PRS Resource simultaneously indicated by measureSameDL-PRS-ResourceWithDifferentRxTEGsSimul-r17 in [34].

-	NRxBeam,aggr,m is a scaling factor for PRS measurements with multiple Rx beams, and is defined as

-	NRxBeam,aggr,m = 1 if PFL combination m is in FR1,

-	NRxBeam,aggr,m is defined as follows if PFL combination m is in FR2

-	NRxBeam,aggr,m equals to the value as UE reported in supportedLowerRxBeamSweepingFactor-FR2 if the capability is reported by the UE for the band containing PFL combination m, and LMF indicates lowerRxBeamSweepingFactor-FR2 in NR-DL-TDOA-RequestLocationInformation,

-	NRxBeam,aggr,m equals to 8 otherwise

-	NPRS,aggr,mslot is the maximum number of DL PRS resources in PFL combination m configured in a slot, and only the PRS resources in resource set(s) linked to other resource set in PFL combination m are counted

-	Naggr,m' is the UE capability on maximum number of DL PRS resources that can be processed in a slot for PFL combination m as indicated by maxNumOfAggregatedDL-PRS-ResourcePerSlot specified in TS 37.355 [34].

-	LavailablePRS,aggr,m is the time duration of available PRS resources in PFL combination m to be measured during TPRS,aggr,m, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of LavailablePRS,aggr,m, only unmuted PRS resources in resource set(s) linked to other resource set in PFL combination m and that are not fully overlapped with other higher-priority DL signals/channels are considered.

-	Naggr,m is the UE capability on duration of DL PRS resources in ms for PFL combination m as indicated by prs-durationOfTwoPRS-BWA-ProcessingSymbolsN or prs-durationOfThreePRS-BWA-ProcessingSymbolsN specified in TS 37.355 [34].

-	Nsample is number of PRS measurement samples,

-	Nsample= 2 if the UE supports the capability of positioning measurements with reduced number of samples as indicated by reducedNumOfSampleInMeasurementWithPRS-BWA-RRC-IdleAndInactive specified in TS 37.355 [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples,

-	Nsample= 4 otherwise.

-	Teffect,aggr,m is the periodicity of the PRS measurement in PFL combination m,

Teffect,aggr,m=Taggr,mTavailable,PRS,aggr,m*Tavailable,PRS,aggr,m

-	Taggr,m is the UE capability on time for processing of DL PRS resources in ms for PFL combination m as indicated by prs-durationOfTwoPRS-BWA-ProcessingSymbolsT or prs-durationOfThreePRS-BWA-ProcessingSymbolsT specified in TS 37.355 [34].

-	Tavailable,PRS,aggr,m=LCMTPRS,aggr,m,TDRX, the least common multiple between TPRS,aggr,m and the DRX cycle length TDRX, defined in TS 38.304 [1], clause 7.1TPRS,aggr,mm.

-	TPRS,aggr,m is the periodicity of DL PRS resource with muting on PFL combination m

-	If more than one PRS periodicities are configured in PFL combination m, the least common multiple of PRS periodicities TperPRS with muting among all DL PRS resource sets that are linked to other resource set in PFL combination m, is used to derive TPRS,aggr,m, and for each applicable PRS resource set,

-	TperPRS with muting=Nmuting*TperPRS, is the PRS periodicity with muting per PRS resource, and

-	TperPRS is the periodicity of PRS resource set given by the higher-layer parameter DL-PRS-Periodicity, and

-	Nmuting is the scaling factor considering PRS resource muting. Nmuting=TmutingPRS*Lmuting, where 	TmutingPRS is the muting repetition factor given by the higher-layer parameter DL-PRS-MutingBitRepetitionFactor, and Lmuting is the size of the bitmap b1.

-	Tlast,aggr,m is the measurement duration for the last PRS sample in PFL combination m, including the sampling time and processing time, Tlast,aggr,m=Taggr,m+Tavailable,PRS,aggr,m.

If the following conditions are met, the time TRSTD,Total starts from the first DL PRS resource(s) instances inside a PTW after both the NR-DL-TDOA-ProvideAssistanceData message and NR-DL-TDOA-RequestLocationInformation message are delivered from LMF to the UE via LPP [34].

-	UE is configured with CN eDRX > 10.24s, and

-	periodic PRS measurement reporting is configured, and

-	CN eDRX cycle is smaller or equal to the PRS measurement reporting periodicity configured via higher layer parameter reportingInterval in TS 37.355 [34], and

-	there is one or more PRS resources occurring in PTW.

Otherwise, the time TRSTD,Total starts from the first DL PRS resource(s) after both the NR-DL-TDOA-ProvideAssistanceData message and NR-DL-TDOA-RequestLocationInformation message are delivered from LMF to the UE via LPP [34].

TRSTD,Total

Note:	No separate requirement on aggregated measurement based on TRSTD,non-aggr or on non-aggregated measurement based on TRSTD,aggr is applied.

If the DRX cycle is reconfigured during the RSTD measurement period, then the measurement period can be longer.

When PRS-RSRP is also reported by UE together with RSTD measurement based on aggregated DL-PRS resources from multiple-PFLs, PRS-RSRP are performed over the measurement period defined in 4.5.2.6.

When PRS-RSRPP is also reported by UE together with RSTD measurement based on aggregated DL-PRS resources from multiple-PFLs, PRS-RSRPP are performed over the measurement period defined in 4.5.2.6.

Longer RSTD measurement period is expected when there are collisions between PRS resources in any of the PFLs configured for aggregation and other higher-priority DL signals/channels.

If Kcarrier_PRS changes for any PFL or any PFL combination during the measurement period, the measurement period could be longer.

The measurement requirements do not apply for a PRS resource, if the PRS resource is across two sampling duration of N within duration Lavailable_PRS,i.

The measurement requirements do not apply for a PRS resource, if time span of the PRS resource instance (including at least the minimum number of repetitions specified in the accuracy requirements) is greater than UE reported capability N.

The requirements in clause 4.5.2.6 do not apply if the PRS configuration given by higher layer paramters NR-DL-PRS-AssistanceData exceeds any of the UE measurement capabilities given by NR-DL-PRS-ResourcesCapability in NR-DL-TDOA-ProvideCapabilities, and it is up to UE implementation which PRS resources are measured, subject to UE measurement capabilities.

If cell re-selection occurs while RSTD measurements are being performed, then the UE shall continue and complete the on-going RSTD measurements after the cell selection is completed. The RSTD measurement period can be longer.

If the RRC state transition occurs from RRC_IDLE to RRC_CONNECTED state during the RSTD measurement period then the UE shall continue the RSTD measurement in the RRC_CONNECTED state. The RSTD measurement period can be longer.

The UE shall meet the RSTD measurement accuracy requirements in clause 10.x.x.x.

----------------------END OF CHANGE----------------------------

----------------------START OF CHANGE----------------------------

#### 4.5.5.5	Measurements Period Requirements

After receiving both NR-DL-TDOA-ProvideAssistanceData message and NR-DL-TDOA-RequestLocationInformation message from the LMF via LPP [34] with nr-DL-PRS-RSCPD-Request and configuring a measurement time window via nr-DL-PRS-MeasurementTimeWindowsConfig, subject to UE capabilities supportOfRSCPD-MeasurementInTimeWindow and supportOfLegacyMeasurementInTimeWindow, the UE shall be able to measure multiple (up to the UE capability specified in Clause 4.5.5.3) DL RSTD and DL RSCPD measurements, defined in TS 38.215 [4], during the time window only.

If the UE is not configured with a measurement time window, the requirements in clause 4.5.2.5 apply. If multiple PFLs are configured in the assistance data, UE is only required to measure DL RSCPD on one of the PFLs.

If a periodic time window is configured, the UE shall be able to measure multiple (up to the UE capability specified in Clause 4.5.5.3) DL RSTD and DL RSCPD measurements, defined in TS 38.215 [4], based on the indicated PRS resource sets occurring inside the time window during the measurement period TRSCPD with RSTD,Total defined as:

TRSCPD with RSTD,Total=i=1LTRSCPD with RSTD,i+ L-1*maxTeffect,i

Where:

-	i is the index of positioning frequency layer,

-	L is total number of positioning frequency layers, and

-	Teffect,i is the periodicity of the PRS RSTD measurement in positioning frequency layer i

TRSCPD with RSTD,i is the measurement period for PRS RSTD measurement in positioning frequency layer i as specified below:

TRSCPD with RSTD,i= Kcarrier_PRS* NRx,TEG,i*NRxBeam,i*NPRS,islotN'*Lavailable_PRS,iN*Nsample-1*Teffect,i+Tlast,i ,

Where:

-	NRxBeam,i is the UE Rx beam sweeping factor:

-	NRxBeam,i = 1 if positioning frequency layer i is in FR1, and if positioning frequency layer i is in FR2

-	NRxBeam,i equals to the value as UE reported in supportedLowerRxBeamSweepingFactor-FR2 if the capability is reported by the UE for the band containing positioning frequency layer i, and LMF indicates lowerRxBeamSweepingFactor-FR2 in NR-DL-TDOA-RequestLocationInformation.

-	NRxBeam,i equals to 8, otherwise.

-	Kcarrier_PRS is a scaling factor for PRS-based NR positioning measurements in RRC_INACTIVE. If the UE supports parallelPRS-MeasRRC-Inactive-r17, Kcarrier_PRS = 1; otherwise,

-	If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, Kcarrier_PRS=Kcarrier+1, where Kcarrier is defined in clause 4.2.2.4

-	If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, Kcarrier_PRS=Nlayers+1, where Nlayers is defined in clause 4.2.2.7.

-	NRx,TEG,i is the Rx TEG specific scaling factor:

-	NRx,TEG,i =1 if the UE is not configured by the LMF to measure a PRS resource with multiple Rx TEGs via measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 [34].

-	NRx,TEG,i is defined as follows if the UE is configured by the LMF with measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 [34] to perform measurement on same DL PRS resource of a TRP using different Rx TEGs in NR-DL-TDOA-RequestLocationInformation [34]:

-	NRx,TEG,i = P, if the UE is not capable of receiving same DL PRS resource simultaneously from multiple Rx TEGs, where P is the number of UE Rx TEGs that the UE is requested by LMF to measure the same DL-PRS Resource of a TRP indicated by measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 in [34], and in case ‘n0’ is indicated, P is the maximum number of Rx TEGs with which UE can support to measure the same PRS resource as reported in NR-UE-TEG-Capability.

-	NRx,TEG,i = PQ , if the UE is capable of receiving the same DL PRS resource simultaneously from multiple Rx TEGs, where Q is the number of UE Rx TEGs for measuring the same DL-PRS Resource simultaneously indicated by measureSameDL-PRS-ResourceWithDifferentRxTEGsSimul-r17 in [34].

-	NPRS,islot is the maximum number of DL PRS resources in positioning frequency layer i configured in a slot.

-	Lavailable_PRS,i is the time duration of available PRS in positioning frequency layer i to be measured TPRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lavailable_PRS,i, only the unmuted PRS resources in the indicated resources sets that are not fully overlapped with other higher-priority DL signals/channels are considered.

-	Nsample is the number of PRS RSTD samples, where

-	Nsample= 1 if the UE supports supportedDL-PRS-ProcessingSamples-RRC-Inactive [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples, and meets the following conditions:

-	PRS bandwidth is within the initial BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

-	Nsample= 2 if the UE supports supportedDL-PRS-ProcessingSamples-RRC-Inactive [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples, and does not meet the following conditions:

-	PRS bandwidth is within the initial BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

-	Nsample= 4 otherwise.

-	Tlast,i is the measurement duration for the last PRS RSTD sample in positioning frequency layer i, including the sampling time and processing time, Tlast,i = Ti + Tavailable_PRS,i ,

-	Teffect,i is the periodicity of the PRS RSTD measurement in positioning frequency layer i defined as:

Teffect,i = TiTavailable_PRS,i*Tavailable_PRS,i

Where:

-	Ti	corresponds to durationOfPRS-ProcessingSymbolsInEveryTms-r17 in TS 37.355 [34],

-	Tavailable_PRS,i=LCMTPRS,i,TDRX, Twindow, the least common multiple between the time window periodicity Twindow, TPRS,i and the DRX cycle length TDRX, defined in TS 38.304 [1], Twindowclause 7.1.

-	TPRS,i is the periodicity of DL PRS resource with muting on positioning frequency layer i, and when calculating TPRS,i, only the PRS resources in the indicated resources sets and overlapped with both the MG and the indicated time window(s) are considered

-	Twindow is the maximum periodicity of the indicated time window(s).

If more than one PRS periodicities are configured in positioning frequency layer i, the least common multiple of PRS periodicities TperPRS with muting among all DL PRS resource sets in the positioning frequency layer is used to derive TPRS,i, where,

-	TperPRS with muting=Nmuting*TperPRS, is the PRS periodicity with muting per PRS resource,

-	TperPRS is the periodicity of PRS resource sets given by the higher-layer parameter DL-PRS-Periodicity.

-	Nmuting is the scaling factor considering PRS resource muting. Nmuting=TmutingPRS*Lmuting, where

-	TmutingPRS is the muting repetition factor given by the higher-layer parameter DL-PRS-MutingBitRepetitionFactor, and Lmuting is the size of the bitmap b1.

-	{N,T} is the UE capability combination per band for RRC_INACTIVE state where N is a duration of DL PRS symbols in ms corresponding to durationOfPRS-ProcessingSymbols-r17 in TS 37.355 [34],  T (ms) corresponds to durationOfPRS-ProcessingSymbolsInEveryTms-r17 in TS 37.355 [34], [ and T-N (>0) is the time required to process duration N of DL PRS symbols already buffered in memory], for a given maximum bandwidth supported by UE corresponding to supportedBandwidthPRS in TS 37.355 [34],

-	N’ is UE capability for number of DL PRS resources that it can process in a slot [in RRC_INACTIVE state as indicated by maxNumOfDL-PRS-ResProcessedPerSlot-RRC-Inactive-r17 specified in TS 37.355 [34].

If the following conditions are met, the time TRSTD,Total starts from the first DL PRS resource(s) instances inside a PTW after both the NR-DL-TDOA-ProvideAssistanceData message and NR-DL-TDOA-RequestLocationInformation message are delivered from LMF to the UE via LPP [34].

-	UE is configured with CN eDRX > 10.24s, and

-	periodic PRS measurement reporting is configured, and

-	CN eDRX cycle is smaller or equal to the PRS measurement reporting periodicity configured via higher layer parameter reportingInterval in TS 37.355 [34], and

-	there is one or more PRS resources occurring in PTW.

Otherwise, the time TRSTD,Total starts from the first DL PRS resource(s) after both the NR-DL-TDOA-ProvideAssistanceData message and NR-DL-TDOA-RequestLocationInformation message are delivered from LMF to the UE via LPP [34].

TRSTD,Total

Note:	No per-positioning frequency layer requirement is applied in scenarios when multiple positioning frequency layers are configured.

If the DRX cycle is reconfigured during the RSTD measurement period, then the measurement period can be longer.

When PRS-RSRP is configured for DL-TDOA, RSTD and PRS-RSRP are performed over the same measurement period.

The measurement requirements do not apply to any PRS resource that always collides with other higher-priority DL signals/channels, as specified in clause 5.6.1.

Longer RSTD measurement period is expected when there are collisions between PRS resources and other higher-priority DL signals/channels.

If Kcarrier_PRS changes for any PFL during the measurement period, the measurement period could be longer.

The measurement requirements do not apply for a PRS resource, if the PRS resource is across two sampling duration of N within duration Lavailable_PRS,i.

The measurement requirements do not apply for a PRS resource, if time span of the PRS resource instance (including at least the minimum number of repetitions specified in the accuracy requirements) is greater than UE reported capability N.

The requirements in clause 4.x1.5 do not apply if the PRS configuration given by higher layer paramters NR-DL-PRS-AssistanceData exceeds any of the UE measurement capabilities given by NR-DL-PRS-ResourcesCapability in NR-DL-TDOA-ProvideCapabilities, and it is up to UE implementation which PRS resources are measured, subject to UE measurement capabilities.

If cell re-selection occurs while RSTD and DL RSCPD measurements are being performed, then the UE shall continue and complete the on-going RSTD and DL RSCPD measurements after the cell selection is completed. The RSTD and DL RSCPD measurement period can be longer.

If the RRC state transition occurs from RRC_IDLE to RRC_CONNECTED state during the measurement period then the UE shall continue the RSTD and DL RSCPD measurement in the RRC_CONNECTED state. The RSTD and DL RSCPD measurement period can be longer.

The UE shall meet the RSTD measurement accuracy requirements in clause 10.1.23.2.

The UE shall meet the DL-RSCPD measurement accuracy requirements in clause 10.x.x.x.

----------------------END OF CHANGE----------------------------

----------------------START OF CHANGE----------------------------

#### 4.6.2.5	Measurement Period Requirements without RX FH

After receiving both NR-DL-TDOA-ProvideAssistanceData message and NR-DL-TDOA-RequestLocationInformation  message from the LMF via LPP [34], the UE shall be able to measure multiple (up to the UE capability specified in Clause 4.6.2.3) DL RSTD measurements without RX FH, defined in TS 38.215 [4], during the measurement period TRSTD,Total defined in 4.5.2.5.

If the following conditions are met, the time TRSTD,Total starts from the first DL PRS resource(s) instances inside a PTW after both the NR-DL-TDOA-ProvideAssistanceData message and NR-DL-TDOA-RequestLocationInformation message are delivered from LMF to the UE via LPP [34].

-	UE is configured with CN eDRX > 10.24s, and

-	periodic PRS measurement reporting is configured, and

-	CN eDRX cycle is smaller or equal to the PRS measurement reporting periodicity configured via higher layer parameter reportingInterval in TS 37.355 [34], and

-	there is one or more PRS resources occurring in PTW.

Otherwise, the time TRSTD,Total starts from the first DL PRS resource(s) after both the NR-DL-TDOA-ProvideAssistanceData message and NR-DL-TDOA-RequestLocationInformation message are delivered from LMF to the UE via LPP [34].

TRSTD,Total

Note:	No per-positioning frequency layer requirement is applied in scenarios when multiple positioning frequency layers are configured.

If the DRX cycle is reconfigured during the RSTD measurement period, then the measurement period can be longer.

When PRS-RSRP is configured for DL-TDOA, RSTD and PRS-RSRP are performed over the same measurement period.

When PRS-RSRPP is configured for DL-TDOA, RSTD and PRS-RSRPP are performed over the same measurement period.

The measurement requirements do not apply to any PRS resource that always collides with other higher-priority DL signals/channels, as specified in clause 4.6.2.

Longer RSTD measurement period is expected when there are collisions between PRS resources and other higher-priority DL signals/channels.

If Kcarrier_PRS changes for any PFL during the measurement period, the measurement period could be longer.

The measurement requirements do not apply for a PRS resource, if the PRS resource is across two sampling duration of N within duration Lavailable_PRS,i.

The measurement requirements do not apply for a PRS resource, if time span of the PRS resource instance (including at least the minimum number of repetitions specified in the accuracy requirements) is greater than UE reported capability N.

The requirements in clause 4.6.2.5 do not apply if the PRS configuration given by higher layer paramters NR-DL-PRS-AssistanceData exceeds any of the UE measurement capabilities given by NR-DL-PRS-ResourcesCapability in NR-DL-TDOA-ProvideCapabilities, and it is up to UE implementation which PRS resources are measured, subject to UE measurement capabilities.

If cell re-selection occurs while RSTD measurements are being performed, then the UE shall continue and complete the on-going RSTD measurements after the cell selection is completed. The RSTD measurement period can be longer.

If the RRC state transition occurs from RRC_IDLE to RRC_CONNECTED state during the RSTD measurement period then the UE shall continue the RSTD measurement in the RRC_CONNECTED state. The RSTD measurement period can be longer.

The UE shall meet the RSTD measurement accuracy requirements in clause 10.1.23.2.

----------------------END OF CHANGE----------------------------