3GPP TSG-RAN4 Meeting #111	R4-2410157

# Fukuoka, Japan, 20 – 24 May, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | 1 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR on RRM requirements for LPHAP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-05-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RAN4 has defined when PRS measurement start is limited to PTW or not, and it depends on a number of conditions. Those conditions are captured in spec in different wording in different clauses, and some are not accurate or complete. For example, the condition eDRX >10.24s is missing in several clauses, and the condition that there is one or more PRS resources occurring in PTW is missing in all clauses. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Align the wording for the conditions when PRS measurement start is limited to PTW acorss all clauses. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements on conditions when PRS measurement start is limited to PTW are not clear nor aligned in different clauses. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5.2.5, 4.5.2.6, 4.6.3.5, 5.6.2.5, 5.6.3.5, 5.6.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The draftCR is based on 38.133 18.5.0 and Big draftCR R4-2405983 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

#### 4.5.3.5	Measurement Period Requirements

When the physical layer receives NR-DL-AoD-ProvideAssistanceData message and NR-DL-AoD-RequestLocationInformation message from LMF via LPP [34], the UE shall be able to measure multiple (up to the UE capability specified in Clause 4.5.3.3) PRS-RSRP measurements, defined in TS 38.215 [4], from configured PRS resources for configured TRPs on configured positioning frequency layers, within TPRS-RSRP,total ms.

TPRS-RSRP,Total=i=1LTPRS-RSRP,i+ L-1*maxTeffect,i

Where:

-	i is the index of positioning frequency layer,

-	L is total number of positioning frequency layers,

-	Teffect,i is the periodicity of the PRS-RSRP measurement in positioning frequency layer i.

TPRS-RSRP,i=Kcarrier_PRSi*NRxBeam,i*NPRS,islotN'Lavailable_PRS,iN*Nsample-1*Teffect,i+Tlast

Where:

-	Kcarrier_PRS is a scaling factor for PRS-based NR positioning measurements in RRC_IDLE. If the UE supports parallelPRS-MeasRRC-Inactive-r17, Kcarrier_PRS= 1. Otherwise,

-	If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, Kcarrier_PRSequals to the sum of Kcarrier in 4.2.2.4 and one positioning layer.

-	If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, Kcarrier_PRS equals to the sum of Nlayer in 4.2.2.7 and one positioning layer.

-	NRxBeam,i is the scaling factor for Rx beam sweeping:

- NRxBeam,i=1 if positioning frequency layer i is in FR1, and

- If positioning frequency layer i is in FR2

-	NRxBeam,i equals to the value as UE reported in supportedLowerRxBeamSweepingFactor-FR2 if the capability is reported by the UE for the band containing positioning frequency layer i, and LMF indicates lowerRxBeamSweepingFactor-FR2 in NR-DL-AoD-RequestLocationInformation.

-		NRxBeam,i equals to 8, otherwise.

-	Lavailable_PRS,i is the time duration of available PRS to be measured in the positioning frequency layer i to be measured during TPRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lavailable_PRS,i, only unmuted PRS resources that are not fully overlapped with other higher-priority DL signals/channels are considered.

-	NPRS,islot is the maximum number of DL PRS resources of positioning frequency layer i configured in a slot,

-	{N,T} is UE capability combination per band where N is a duration of DL PRS symbols in ms corresponding to durationOfPRS-ProcessingSymbols-r17 in TS 37.355 [34] processed every T ms corresponding to durationOfPRS-ProcessingSymbolsInEveryTms-r17 in TS 37.355 [34] for a given maximum bandwidth supported by UE corresponding to supportedBandwidthPRS in TS 37.355 [34],

-	N’ is UE capability for number of DL PRS resources that it can process in a slot as indicated by maxNumOfDL-PRS-ResProcessedPerSlot-RRC-Inactive-r17 in clause 6.4.3 of TS 37.355 [34],

-	Nsample is the number of PRS-RSRP measurement samples and

-	Nsample= 1, if UE supports supportedDL-PRS-ProcessingSamples-RRC-Inactive [34], and the LMF indicates the UE to perform positioning measurements with reduced number of samples by reducedDL-PRS-ProcessingSamples [34], and the following conditions are met:

-	PRS bandwidth is within the initial BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

-	Nsample= 2, if UE supports supportedDL-PRS-ProcessingSamples-RRC-Inactive [34], and the LMF indicates the UE to perform positioning measurements with reduced number of samples by reducedDL-PRS-ProcessingSamples [34], and the following conditions are not met:

-	PRS bandwidth is within the initial BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

-	Nsample= 4 otherwise

Tlast,i = Ti +Tavailable_PRS,i is the measurement duration for the last PRS-RSRP sample, including the sampling time and processing time,

-	Teffect,i= TiTavailable_PRS,i*Tavailable_PRS,i is the periodicity of PRS-RSRP measurement in positioning frequency layer i,

-	Ti	corresponds to durationOfPRS-ProcessingSymbolsInEveryTms-r17 in TS 37.355 [34],

-	Tavailable_PRS,i=LCMTPRS,i,TDRX is the least common multiple between TPRS,i and TDRX.

-	TPRS,i is the maximum PRS resource periodicity among all PRS resources in positioning frequency layer i,

-	TDRX is the DRX cycle length if UE is not configured with eDRX_IDLE cycle, and TDRX is defined as T in TS 38.304[1] if UE is configured with eDRX_IDLE cycle.

If positioning frequency layer i has more than one DL PRS resource set with different PRS periodicities with muting,  TperPRS with muting=Nmuting*TperPRS, the least common multiple of  TperPRS with muting among the DL PRS resource sets is used to derive TPRS,i, where:

-	TperPRS is the periodicity of PRS resource sets given by the higher-layer parameter DL-PRS-Periodicity.

-	Nmuting is the scaling factor considering PRS resource muting. Nmuting=TmutingPRS*Lmuting, where TmutingPRS is the muting repetition factor given by the higher-layer parameter DL-PRS-MutingBitRepetitionFactor, and Lmuting is the size of the bitmap b1.

If the following conditions are met, the time TPRS-RSRP,Total starts from the first DL PRS resource(s) instances inside a PTW after both the NR-DL-AoD-ProvideAssistanceData message and NR-DL-AoD-RequestLocationInformation message are delivered from LMF to the UE via LPP [34].

-	UE is configured with CN eDRX > 10.24s, and

-	periodic PRS measurement reporting is configured, and

-	CN eDRX cycle is smaller or equal to the PRS measurement reporting periodicity configured via higher layer parameter reportingInterval in TS 37.355 [34], and

-	there is one or more PRS resources occurring in PTW.

Otherwise, the time TPRS-RSRP,Total starts from the first DL PRS resource(s) after both the NR-DL-AoD-ProvideAssistanceData message and NR-DL-AoD-RequestLocationInformation message are delivered from LMF to the UE via LPP [34].

TPRS-RSRP,total

TPRS-RSRP,total

Note:	No per-positioning frequency layer requirement is applied in scenarios when multiple positioning frequency layers are configured.

When the PRS-RSRP measurement is configured together with RSTD measurement then the PRS-RSRP measurement shall meet the RSTD measurement requirements defined in clause 4.5.2.

The measurement requirements do not apply for a PRS resource:

-	if the PRS resource is across two sampling duration of N within duration Lavailable_PRS,i or

-	if time span of the PRS resource instance (including at least the minimum number of repetitions specified in the accuracy requirements) is greater than UE reported capability N.

Longer PRS-RSRP measurement period is expected when there is collision/overlap between other DL signals/channels and PRS resources in RRC_IDLE state.

The requirements in clause 4.5.3 do not apply if the PRS configuration given by higher layer paramters NR-DL-PRS-AssistanceData exceeds any of the UE measurement capabilities given by NR-DL-PRS-ResourcesCapability in NR-DL-AoD-ProvideCapabilities, and it is up to UE implementation which PRS resources are measured, subject to UE measurement capabilities.

If the DRX cycle is reconfigured during the PRS-RSRP measurement period then the PRS-RSRP measurement period can be longer.

If cell reselection occurs while PRS-RSRPP measurement is being performed, then the UE shall continue and complete the on-going PRS-RSRP measurement after the cell selection is completed. The PRS-RSRP measurement period can be longer.

If the UE’s RRC state changes from the RRC_IDLE to RRC_CONNECTED during the PRS-RSRP measurement period, then the UE shall continue the PRS-RSRP measurement in the RRC_CONNECTED state. The PRS-RSRP measurement period can be longer.

The UE shall meet the PRS-RSRP measurement accuracy requirements in clause 10.1.24.2.

<End of Change 1>

<Start of Change 2>

#### 4.6.3.5	Measurement Period Requirements without RX FH

When the physical layer receives NR-DL-AoD-ProvideAssistanceData message and NR-DL-AoD-RequestLocationInformation message from LMF via LPP [34], the UE shall be able to measure multiple (up to the UE capability specified in Clause 4.6.3.3) PRS-RSRP measurements, defined in TS 38.215 [4], from configured PRS resources for configured TRPs on configured positioning frequency layers, within TPRS-RSRP,total ms.

TPRS-RSRP, total=i=1LTPRS-RSRP,i+L-1*maxTeffect,i

Where:

-	i is the index of positioning frequency layer,

-	L is total number of positioning frequency layers,

-	Teffect,i is the periodicity of the PRS-RSRP measurement in positioning frequency layer i.

TPRS-RSRP,i=Kcarrier_PRS_RedCapi*NRxBeam,i*NPRS,islotN'Lavailable_PRS,iN*Nsample-1*Teffect,i+Tlast

Where:

-	Kcarrier_PRS_RedCapi is a scaling factor for PRS-based NR positioning measurements in RRC_IDLE. If the UE is capable of performing RRM measurement and PRS measurement in parallel to each other, Kcarrier_PRS_RedCapi= 1. Otherwise,

-	If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, Kcarrier_PRS_RedCap=Kcarrier_RedCap+1, where Kcarrier_RedCap is defined in 4.2B.2.4.

-	If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, Kcarrier_PRS_RedCap=Nlayers+1, where Nlayer is defined in 4.2.2.7.

-	NRxBeam,i is the scaling factor for Rx beam sweeping:

- NRxBeam,i=1 if positioning frequency layer i is in FR1 or UE has only 1 Rx branch, and if positioning frequency layer i is in FR2 or UE has 2 Rx branches

-	NRxBeam,i equals to the value as UE reported in supportedLowerRxBeamSweepingFactor-FR2 if the capability is reported by the UE for the band containing positioning frequency layer i, and LMF indicates lowerRxBeamSweepingFactor-FR2 in NR-DL-AoD-RequestLocationInformation.

-		NRxBeam,i equals to 8, otherwise.

-	Lavailable_PRS,i is the time duration of available PRS to be measured in the positioning frequency layer i to be measured during TPRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lavailable_PRS,i, only unmuted PRS resources that are not fully overlapped with other higher-priority DL signals/channels are considered.

-	NPRS,islot is the maximum number of DL PRS resources of positioning frequency layer i configured in a slot,

-	{N,T} is UE capability combination per band where N is a duration of DL PRS symbols in ms corresponding to durationOfPRS-ProcessingSymbols-r17 in TS 37.355 [34] processed every T ms corresponding to durationOfPRS-ProcessingSymbolsInEveryTms-r17in TS 37.355 [34] for a given maximum bandwidth supported by UE corresponding to supportedBandwidthPRS in TS 37.355 [34],

-	N’ is UE capability for number of DL PRS resources that it can process in a slot as indicated by maxNumOfDL-PRS-ResProcessedPerSlot-RRC-Inactive-r17 in clause 6.4.3 of TS 37.355 [34],

-	Nsample is the number of PRS-RSRP measurement samples and

-	Nsample= 1, if UE supports supportedDL-PRS-ProcessingSamples-RRC-Inactive [34], and the LMF indicates the UE to perform positioning measurements with reduced number of samples by reducedDL-PRS-ProcessingSamples [34], and the following conditions are met:

-	PRS bandwidth is within the initial BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

-	Nsample= 2, if UE supports supportedDL-PRS-ProcessingSamples-RRC-Inactive [34], and the LMF indicates the UE to perform positioning measurements with reduced number of samples by reducedDL-PRS-ProcessingSamples [34], and the following conditions are not met:

-	PRS bandwidth is within the initial BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

-	Nsample= 4 otherwise

Tlast,i = Ti +Tavailable_PRS,i is the measurement duration for the last PRS-RSRP sample, including the sampling time and processing time,

-	Teffect,i= TiTavailable_PRS,i*Tavailable_PRS,i is the periodicity of PRS-RSRP measurement in positioning frequency layer i,

-	Ti	corresponds to durationOfPRS-ProcessingSymbolsInEveryTms-r17 in TS 37.355 [34],

-	Tavailable_PRS,i=LCMTPRS,i,TDRX, the least common multiple between TPRS,i and TDRX.

-	TPRS,i is the maximum PRS resource periodicity among all PRS resources in positioning frequency layer i,

-	TDRX is the DRX cycle length if UE is not configured with eDRX. Otherwise, TDRX is defined as T in TS 38.304 [1].

If positioning frequency layer i has more than one DL PRS resource set with different PRS periodicities with muting,  TperPRS with muting=Nmuting*TperPRS, the least common multiple of  TperPRS with muting among the DL PRS resource sets is used to derive TPRS,i, where:

-	TperPRS is the periodicity of PRS resource sets given by the higher-layer parameter DL-PRS-Periodicity.

-	Nmuting is the scaling factor considering PRS resource muting. Nmuting=TmutingPRS*Lmuting, where TmutingPRS is the muting repetition factor given by the higher-layer parameter DL-PRS-MutingBitRepetitionFactor, and Lmuting is the size of the bitmap b1.

If the following conditions are met, the time TPRS-RSRP,Total starts from the first DL PRS resource(s) instances inside a PTW after both the NR-DL-AoD-ProvideAssistanceData message and NR-DL-AoD-RequestLocationInformation message are delivered from LMF to the UE via LPP [34].

-	UE is configured with CN eDRX > 10.24s, and

-	periodic PRS measurement reporting is configured, and

-	CN eDRX cycle is smaller or equal to the PRS measurement reporting periodicity configured via higher layer parameter reportingInterval in TS 37.355 [34], and

-	there is one or more PRS resources occurring in PTW.

Otherwise, the time TPRS-RSRP,Total starts from the first DL PRS resource(s) after both the NR-DL-AoD-ProvideAssistanceData message and NR-DL-AoD-RequestLocationInformation message are delivered from LMF to the UE via LPP [34].

TPRS-RSRP,total

Note:	No per-positioning frequency layer requirement is applied in scenarios when multiple positioning frequency layers are configured.

When the PRS-RSRP measurement is configured together with RSTD measurement then the PRS-RSRP measurement shall meet the RSTD measurement requirements defined in clause 4.6.2.

The measurement requirements do not apply for a PRS resource:

-	if the PRS resource is across two sampling duration of N within duration Lavailable_PRS,i or

-	if time span of the PRS resource instance (including at least the minimum number of repetitions specified in the accuracy requirements) is greater than UE reported capability N.

Longer PRS-RSRP measurement period is expected when there is collision/overlap between other DL signals/channels and PRS resources in RRC_IDLE state.

The requirements in clause 4.6.3 do not apply if the PRS configuration given by higher layer parameters NR-DL-PRS-AssistanceData exceeds any of the UE measurement capabilities given by NR-DL-PRS-ResourcesCapability in NR-DL-AoD-ProvideCapabilities, and it is up to UE implementation which PRS resources are measured, subject to UE measurement capabilities.

If the DRX cycle is reconfigured during the PRS-RSRP measurement period then the PRS-RSRP measurement period can be longer.

If cell reselection occurs while PRS-RSRP measurement is being performed, then the UE shall continue and complete the on-going PRS-RSRP measurement after the cell selection is completed. The PRS-RSRP measurement period can be longer.

If the UE’s RRC state changes from the RRC_IDLE to RRC_CONNECTED during the PRS-RSRP measurement period, then the UE shall continue the PRS-RSRP measurement in the RRC_CONNECTED state. The PRS-RSRP measurement period can be longer.

<End of Change 2>

<Start of Change 3>

#### 5.6.3.5	Measurement Period Requirements

When the physical layer receives NR-DL-AoD-ProvideAssistanceData message and NR-DL-AoD-RequestLocationInformation message from LMF via LPP [34], the UE shall be able to measure multiple (up to the UE capability specified in Clause 5.6.3.3) PRS-RSRP measurements, defined in TS 38.215 [4], from configured PRS resources for configured TRPs on configured positioning frequency layers, within TPRS-RSRP,total ms.

TPRS-RSRP, total=i=1LTPRS-RSRP,i+L-1×maxTeffect,i

Where:

-	i is the index of positioning frequency layer,

-	L is total number of positioning frequency layers,

-	Teffect,i is the periodicity of the PRS-RSRP measurement in positioning frequency layer i.

TPRS-RSRP,i=Kcarrier_PRSi*NRxBeam,i*NPRS,islotN'Lavailable_PRS,iN*Nsample-1*Teffect,i+Tlast

Where:

-	Kcarrier_PRS is a scaling factor for PRS-based NR positioning measurements in RRC_INACTIVE. If the UE supports parallelPRS-MeasRRC-Inactive-r17, Kcarrier_PRS= 1. Otherwise,

-	If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, Kcarrier_PRSequals to the sum of Kcarrier in 4.2.2.4 and one positioning layer.

-	If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, Kcarrier_PRS equals to the sum of Nlayer in 4.2.2.7 and one positioning layer.

-	NRxBeam,i is the scaling factor for Rx beam sweeping:

- NRxBeam,i=1 if positioning frequency layer i is in FR1, and if positioning frequency layer i is in FR2

-	NRxBeam,i equals to the value as UE reported in supportedLowerRxBeamSweepingFactor-FR2 if the capability is reported by the UE for the band containing positioning frequency layer i, and LMF indicates lowerRxBeamSweepingFactor-FR2 in NR-DL-AoD-RequestLocationInformation.

-		NRxBeam,i equals to 8, otherwise.

-	Lavailable_PRS,i is the time duration of available PRS to be measured in the positioning frequency layer i to be measured during TPRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lavailable_PRS,i, only unmuted PRS resources that are not fully overlapped with other higher-priority DL signals/channels are considered.

-	NPRS,islot is the maximum number of DL PRS resources of positioning frequency layer i configured in a slot,

-	{N,T} is UE capability combination per band where N is a duration of DL PRS symbols in ms corresponding to durationOfPRS-ProcessingSymbols-r17 in TS 37.355 [34] processed every T ms corresponding to durationOfPRS-ProcessingSymbolsInEveryTms-r17in TS 37.355 [34] for a given maximum bandwidth supported by UE corresponding to supportedBandwidthPRS in TS 37.355 [34],

-	N’ is UE capability for number of DL PRS resources that it can process in a slot as indicated by maxNumOfDL-PRS-ResProcessedPerSlot-RRC-Inactive-r17 in clause 6.4.3 of TS 37.355 [34],

-	Nsample is the number of PRS-RSRP measurement samples and

-	Nsample= 1, if UE supports supportedDL-PRS-ProcessingSamples-RRC-Inactive [34], and the LMF indicates the UE to perform positioning measurements with reduced number of samples by reducedDL-PRS-ProcessingSamples [34], and the following conditions are met:

-	PRS bandwidth is within the initial BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

-	Nsample= 2, if UE supports supportedDL-PRS-ProcessingSamples-RRC-Inactive [34], and the LMF indicates the UE to perform positioning measurements with reduced number of samples by reducedDL-PRS-ProcessingSamples [34], and the following conditions are not met:

-	PRS bandwidth is within the initial BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

-	Nsample= 4 otherwise

Tlast,i = Ti +Tavailable_PRS,i is the measurement duration for the last PRS-RSRP sample, including the sampling time and processing time,

-	Teffect,i= TiTavailable_PRS,i*Tavailable_PRS,i is the periodicity of PRS-RSRP measurement in positioning frequency layer i,

-	Ti	corresponds to durationOfPRS-ProcessingSymbolsInEveryTms-r17 in TS 37.355 [34],

-	Tavailable_PRS,i=LCMTPRS,i,TDRX is the least common multiple between TPRS,i and TDRX,

-	TPRS,i is the maximum PRS resource periodicity among all PRS resources in positioning frequency layer i,

-	TDRX is defined as following:

-	TDRX is DRX cycle length when no extended DRX (eDRX) cycle is configured

-	TDRX is defined as T in clause 7.1 TS 38.304 when RAN eDRX <= 10.24s and CN eDRX <= 10.24s

-	TDRX is the maximum of the T inside and outside of the CN PTW, where T inside and outside of the CN PTW are defined in clause 7.1 TS 38.304, when RAN eDRX <= 10.24s and CN eDRX > 10.24s

-	TDRX is the maximum of the DRX cycles within the CN PTW and the RAN PTW when RAN eDRX > 10.24s

If positioning frequency layer i has more than one DL PRS resource set with different PRS periodicities with muting,  TperPRS with muting=Nmuting*TperPRS, the least common multiple of  TperPRS with muting among the DL PRS resource sets is used to derive TPRS,i, where:

-	TperPRS is the periodicity of PRS resource sets given by the higher-layer parameter DL-PRS-Periodicity.

-	Nmuting is the scaling factor considering PRS resource muting. Nmuting=TmutingPRS*Lmuting, where TmutingPRS is the muting repetition factor given by the higher-layer parameter DL-PRS-MutingBitRepetitionFactor, and Lmuting is the size of the bitmap b1.

If the following conditions are met, the time TPRS-RSRP,Total starts from the first DL PRS resource(s) instances inside a PTW after both the NR-DL-AoD-ProvideAssistanceData message and NR-DL-AoD-RequestLocationInformation message are delivered from LMF to the UE via LPP [34].

-	UE is configured with RAN eDRX > 10.24s, and

-	periodic PRS measurement reporting is configured, and

-	RAN eDRX cycle is smaller or equal to the PRS measurement reporting periodicity configured via higher layer parameter reportingInterval in TS 37.355 [34], and

-	there is one or more PRS resources occurring in PTW.

Otherwise, the time TPRS-RSRP,Total starts from the first DL PRS resource(s) after both the NR-DL-AoD-ProvideAssistanceData message and NR-DL-AoD-RequestLocationInformation message are delivered from LMF to the UE via LPP [34].

TPRS-RSRP,total

TRSTD,TotalTRSTD,Total

Note 1:	No per-positioning frequency layer requirement is applied in scenarios when multiple positioning frequency layers are configured.

When the PRS-RSRP measurement is configured together with RSTD measurement then the PRS-RSRP measurement shall meet the RSTD measurement requirements defined in clause 5.6.2.

When the PRS-RSRP measurement is configured together with UE Rx-Tx time difference measurement then the PRS-RSRP measurement shall meet the UE Rx-Tx time difference measurement requirements defined in clause 5.6.4.

The measurement requirements do not apply for a PRS resource:

-	if the PRS resource is across two sampling duration of N within duration Lavailable_PRS,i or

-	if time span of the PRS resource instance (including at least the minimum number of repetitions specified in the accuracy requirements) is greater than UE reported capability N.

Longer PRS-RSRP measurement period is expected when there is collision/overlap between other DL signals/channels and PRS resources in RRC_INACTIVE state.

The requirements in clause 5.6.3 do not apply if the PRS configuration given by higher layer paramters NR-DL-PRS-AssistanceData exceeds any of the UE measurement capabilities given by NR-DL-PRS-ResourcesCapability in NR-DL-AoD-ProvideCapabilities, and it is up to UE implementation which PRS resources are measured, subject to UE measurement capabilities.

If the DRX cycle is reconfigured during the PRS-RSRP measurement period then the PRS-RSRP measurement period can be longer.

If cell reselection occurs while PRS-RSRP measurement is being performed, then the UE shall continue and complete the on-going PRS-RSRP measurement after the cell selection is completed. The PRS-RSRP measurement period can be longer.

If the UE’s RRC state changes from the RRC_INACTIVE to RRC_CONNECTED during the PRS-RSRP measurement period, then the UE shall continue the PRS-RSRP measurement in the RRC_CONNECTED state. The PRS-RSRP measurement period can be longer.

<End of Change 3>

<Start of Change 4>

#### 5.6.4.5	Measurement Period Requirements

When physical layer receives last of NR-Multi-RTT-ProvideAssistanceData message and NR-Multi-RTT-RequestLocationInformation message from LMF via LPP [34], UE shall be able to measure multiple (up to the UE capability specified in clause 5.6.4.3) UE Rx-Tx time difference measurements as defined in TS 38.215 [4] in configured positioning frequency layers within the measurement period TUERxTx,Total ms.

TUERxTx, Total=i=1LTUERxTx,i+L-1*maxTeffect,i

Where:

-	i is the index of positioning frequency layer,

-	TUERxTx,i is the measurement period for UE Rx-Tx time difference measurements in positioning frequency layer i as further defined in this clause,

-	L is total number of positioning frequency layers,

-	Teffect,i is the periodicity of the UE Rx-Tx time difference measurement in positioning frequency layer i as defined further in this clause.

TUERxTx,i=Kcarrier_PRS*NRxTx,TEG,i*NRxBeam,i*NPRS,islotN'Lavailable_PRS,iN*Nsample-1*Teffect,i+Tlast,i

Where:

-	KcarrierPRS =1 if the UE is capable of parallelPRS-MeasRRC-Inactive-r17 defined in [34].

-	Kcarrier_PRS=Nlayer+1 if the UE is not capable of parallelPRS-MeasRRC-Inactive-r17 defined in [34] and if Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ; where Nlayeris defined in clause 4.2.2.7.

-	Kcarrier_PRS=Kcarrier+1 if the UE is not capable of parallelPRS-MeasRRC-Inactive-r17 defined in [34] and if Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ; where Kcarrieris defined in clause 4.2.2.4.

-	NRxBeam,i is the scaling factor for UE Rx beam sweeping:

-	NRxBeam,i=1 if positioning frequency layer i is in FR1, and if positioning frequency layer i is in FR2.

-		NRxBeam,i equals to the value as UE reported in supportedLowerRxBeamSweepingFactor-FR2 if the capability is reported by the UE for the band containing positioning frequency layer i, and LMF indicates lowerRxBeamSweepingFactor-FR2 in NR-Multi-RTT -RequestLocationInformation.

-		NRxBeam,i equals to 8, otherwise.

NRxTx,TEG,i is the Rx TEG specific scaling factor:

-	NRxTx,TEG,i = 1 if UE is not configured by LMF with measureSameDL-PRS-ResourceWithDifferentRxTxTEGs-r17 or measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 [34].

-	NRxTx,TEG,i = measureSameDL-PRS-ResourceWithDifferentRxTxTEGs-r17 or measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17 if UE is configured by LMF to measurement same DL PRS with multiple UE RxTx TEGs or multiple UE Rx TEGs [34], and in case ‘n0’ is indicated, NTEG,i is the maximum number of Rx TEGs with which UE can support to measure the same PRS resource as reported in NR-UE-TEG-Capability.

-	Lavailable_PRS,i is the time duration of available PRS resources in the positioning frequency layer i, to be measured during TPRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lavailable_PRS,i, only unmuted PRS resources that are not fully overlapped with other higher-priority DL signals/channels are considered.

-	NPRS,islot is the maximum number of DL PRS resources of positioning frequency layer i configured in a slot,

-	{N,T} is UE capability combination per band where N is a duration of DL PRS symbols in ms corresponding to durationOfPRS-ProcessingSymbols-r17 in TS 37.355 [34] processed every T ms corresponding to durationOfPRS-ProcessingSymbolsInEveryTms-r17 in TS 37.355 [34] for a given maximum bandwidth supported by UE corresponding to supportedBandwidthPRS in clause 4.2.7.2 of TS 37.355 [34],

-	N’ is UE capability for number of DL PRS resources that it can process in a slot corresponding to maxNumOfDL-PRS-ResProcessedPerSlot-RRC-Inactive-r17 as specified in clause 6.4.3 of TS 37.355 [34],

-	Nsample is the number of UE Rx-Tx time difference measurement samples:

-	Nsample= 4 if the UE is not capable of supportedDL-PRS-ProcessingSamples-RRC-Inactive defined in [34] or not configured to perform positioning measurements with reduced number of samples by requestedDL-PRS-ProcessingSamples [34].

-	Nsample= 1 if the UE is capable of supportedDL-PRS-ProcessingSamples-RRC-Inactive defined in [34] and LMF requests the UE to perform positioning measurements with reduced number of samples by requestedDL-PRS-ProcessingSamples [34] and the following conditions are met:

-	PRS bandwidth is within the initial BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

-	Nsample=  2 if the UE is capable of supportedDL-PRS-ProcessingSamples-RRC-Inactive defined in [34] and the LMF requests the UE to perform positioning measurements with reduced number of samples by requestedDL-PRS-ProcessingSamples [34] but the following conditions are not met:

-	PRS bandwidth is within the initial BWP and

-	Magnitude of difference between the serving cell’s SS-RSRP and the neighbor cell’s PRS-RSRP is within 6 dB.

-	Tlast,i is the measurement duration for the last UE Rx-Tx time difference measurement sample in the positioning layer i, including the sampling time and processing time, Tlast,i = Ti + Tavailable_PRS,i ,

-	Teffect,i is periodicity of UE Rx-Tx time difference measurement in positioning frequency layer i:

Teffect,i= TiTavailable_PRS,i*Tavailable_PRS,i

Where:

-	Ti corresponds to durationOfPRS-ProcessingSymbolsInEveryTms-r17 in TS 37.355 [34],

-	Tavailable_PRS,i= LCMTPRS,i,TDRX, the least common multiple between TPRS,i and TDRX.

-	When UE is configured with RAN eDRX ≤ 10.24s:

-	TDRX is defined as T in TS 38.304 [1] when CN eDRX ≤ 10.24s.

-	TDRX is the maximum of the T inside and outside of the CN PTW when CN eDRX > 10.24s, where T 			inside and outside of the CN PTW are defined in TS 38.304 [1].

-	When UE is configured with RAN eDRX > 10.24s:

-	TDRX is the maximum of the DRX cycles within the CN PTW and the RAN PTW.

-	Otherwise, TDRX is the DRX cycle of the UE in the serving cell.

-	TPRS,i is the PRS resource periodicity in positioning frequency layer i. If the positioning frequency layer i has more than one DL PRS resource sets with different PRS periodicities with muting,  TperPRS with muting=Nmuting*TperPRS, the least common multiple of TperPRS with muting among DL PRS resource sets is used to derive TPRS,i, where:

-	TperPRS is the periodicity of PRS resource sets given by the higher-layer parameter DL-PRS-Periodicity.

-	Nmuting is the scaling factor considering PRS resource muting. Nmuting=TmutingPRS*Lmuting, where TmutingPRS is the muting repetition factor given by the higher-layer parameter DL-PRS-MutingBitRepetitionFactor, and Lmuting is the size of the bitmap b1

If the following conditions are met, the time TUERxTx, Total starts from the first DL PRS resource(s) instances inside a PTW after both the NR-Multi-RTT-ProvideAssistanceData message and NR-Multi-RTT-RequestLocationInformation message are delivered from LMF to the UE via LPP [34].

-	UE is configured with RAN eDRX > 10.24s, and

-	periodic PRS measurement reporting is configured, and

-	RAN eDRX cycle is smaller or equal to the PRS measurement reporting periodicity configured via higher layer parameter reportingInterval in TS 37.355 [34], and

-	there is one or more PRS resources occurring in PTW.

Otherwise, the time TUERxTx, Total starts from the first DL PRS resource(s) after both the NR-Multi-RTT-ProvideAssistanceData message and NR-Multi-RTT-RequestLocationInformation message are delivered from LMF to the UE via LPP [34].

TUERxTx,Total

Note 1: No per-positioning frequency layer requirement is applied in scenarios when multiple positioning frequency layers are configured.

If the RRC state transition occurs from RRC_INACTIVE to RRC_CONNECTED state during the UE Rx-Tx time difference measurement period then the UE shall restart the UE Rx-Tx time difference measurement after it obtains SRS configuration and Timing Advance command from the serving cell.

If cell reselection occurs during the UE Rx-Tx time difference measurement period then the UE shall restart the UE Rx-Tx time difference measurement after it obtains SRS configuration and Timing Advance command from the new serving cell.

If cell reselection occurs, and UE reselects to a cell out of the positioning validity area or if UE performs autonomous TA adjustment at reselection, then UE shall restart the measurement.

The measurement requirements do not apply for a PRS resource:

-	if the PRS resource is across two sampling duration of N within duration Lavailable_PRS,i or

-	if time span of the PRS resource instance (including at least the minimum number of repetitions specified in the accuracy requirements) is greater than UE reported capability N.

If the DRX cycle is reconfigured during the UE Rx-Tx time difference measurement period then the UE Rx-Tx time difference measurement period can be longer.

If during UE Rx-Tx time difference measurement period PRS resources overlap with other DL signals/channels then the UE Rx-Tx time difference measurement period can be longer.

When PRS-RSRP is configured for multi-RTT, the UE Rx-Tx time difference measurements and PRS-RSRP measurements are performed over the same measurement period.

When PRS-RSRPP is configured for multi-RTT, the UE Rx-Tx time difference measurements and PRS-RSRPP measurements are performed over the same measurement period.

The requirements in clause 5.6.4 do not apply if the PRS configuration given by higher layer paramters NR-DL-PRS-AssistanceData exceeds any of the UE measurement capabilities given by NR-DL-PRS-ResourcesCapability in NR-Multi-RTT-ProvideCapabilities, and it is up to UE implementation which PRS resources are measured, subject to UE measurement capabilities.

If UE uplink transmission timing changes due to the network-configured Timing Advance command during the UE Rx-Tx measurement period, then the UE Rx-Tx time difference measurement period is restarted after uplink transmission timing changes, and the UE Rx-Tx time difference measurement period requirements in this clause shall not apply.

If UE uplink transmission timing changes due to the change in the NTA_offset defined in Table 7.1.2-2 during the UE Rx-Tx measurement period, then the UE Rx-Tx time difference measurement period is restarted after uplink transmission timing changes, and the UE Rx-Tx time difference measurement period requirements in this clause shall not apply.

The UE shall meet the UE Rx-Tx time difference measurement accuracy requirements in clause 10.1.25.2.

<End of Change 4>