3GPP TSG-RAN WG4 Meeting #111 	R4-2410305

# Fukuoka City, Fukuoka , Japan, 20th – 24th May, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev |  | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR on PDCCH ordered RACH delay requirements in R18 LTM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo, CATT, Apple, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-05-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 17) Rel-17 (Release 18) Rel-18 (Release 19) Rel-19 (Release 20) |  |
| Reason for change: |  | 1. The common understanding agreed in R4-2406502 needs to be captured. Otherwise at least the UE Tx beam might not be determined in FR2 2. Merge the changes for 6.2.2C on the [TBD] value from R4-2407350 and R4-2408685 3. Wording refinement is also done based on R4-2407483, with further revision to further optimize the wording. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Revise above parts as discussed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements are not correct |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2C |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R4-2406513 |  |  |  |  |  |  |  |  |

<Start of Change #1>

6.2.2C	PDCCH ordered Random Access for LTM

### 6.2.2C.1	Introduction

The requirements in this clause apply for a UE capable of [FG 45-5: RACH-based early TA acquisition] configured to trigger RACH procedure toward target LTM candidate cell before receiving cell switch command MAC-CE.

The requirements in this clause apply if PDCCH order-based RACH on candidate cell is triggered based on L1-RSRP report or L3-RSRP beam-level report in FR1, or based on L1-RSRP report in FR2.

### 6.2.2C.2	PDCCH ordered Random Access delay

UE shall complete the RACH transmission as defined in TS38.213 [3] clause 8.1, in which TSSB and ∆RF/BB preparation are defined as the following:

-	TSSB is the time for T/F tracking

-	TSSB = 0 if the following conditions are met.

-	L1-RSRP measurement is configured and

-	The target LTM candidate cell is on an FR1 carrier whose TCI state associated with SSB indicated in the PDCCH-order is activated, and the time gap between the MAC-CE activating the target TCI state and the PDCCH is larger than TCI state activation delay defined in clause 8.x.3, and

-   L1-RSRP measurement period of candidate cell is not larger than 160ms, or

-	The time between the MAC-CE activating the target TCI state and the PDCCH is not larger than 160ms.

-	TSSB = Tfirst-SSB_RACH + TSSB-proc, otherwise

, where

- Tfirst-SSB_RACH is the time to the first SSB occasion, after [1slot from] the end of the slot of the PDCCH, when SSB is within active BWP

[TSSB

- Tfirst-SSB_RACHTSSB is the time to the first SSB occasion, overlapped with MGL after [2]ms and [1slot from] the end of the slot of the PDCCH when SSB is outside active BWP

FFS: this is only applicable to UE supporting inter-frequency L1 measurement with MG.

- TSSB-proc = 2 ms, which is the time for SSB processing;

-	∆RF/BB preparation is the time for RF and baseband preparation. The following apply:

-	If PRACH bandwidth is outside active UL BWP of the SpCell but within one of configured UL BWPs of other active serving cell, the ∆RF/BB preparation is the DCI based BWP switching delay specified in clause 8.6.

-	If PRACH bandwidth is not within any of the configured UL BWPs of any active serving cell, ∆RF/BB preparation equals to capability value reported in [FG 39-5: RF/BB preparation time for PDCCH-order RACH].

-	Otherwise, ∆RF/BB preparation = 0.

<End of Change #1>