3GPP TSG-RAN WG4 Meeting # 111                                                        	R4-2410567

Fukuoka, Japan, 20th May 2024 - 24th May 2024

Agenda item:			10.13.4

Source:	CMCC

Title:	WF on co-existence evaluation for ambient IoT

Document for:	Approval

Introduction

This way forward captures the agreements for co-existence evaluation for Rel-19 ambient IOT study item.

The summary in RAN4#111 is R4-2408945. The way forward agreed in RAN4#110bis is in R4-2406714.

Deployment scenarios and spectrum usage

- Topic 2-1: Deployment scenario

Issue 2-1-1: deployment scenarios for D1T1

Agreement in RAN4#110bis: Issue 2-1-1: deployment scenarios for D1T1 Option 1-1: Legacy NR gNB are outdoor macro gNB while AIoT reader/CW/devices are all indoors. Legacy NR UE is only allowed outdoors. Option 1-2: Legacy NR gNB are outdoor macro gNB while AIoT reader/CW/devices are all indoors. Legacy NR UE is indoor accessing to outdoor NR marco gNB Option 2-1: Legacy NR gNB are co-located with AIoT reader and CW. All of NR and AIoT BS/UE/Reader/Device/CW are indoors. AIoT reader /CW and Legacy gNB share same hardware Option 2-2: Legacy NR gNB are co-located with AIoT reader and CW. All of NR and AIoT BS/UE/Reader/Device/CW are indoors. AIoT reader /CW and Legacy NR gNB do not share same hardware. (less limitation on the power boosting) Agreement: - RAN4 to first evaluate co-existence for deployment scenario of option 1-1 and 1-2, and further study option 2-1 and 2-2. Issue 2-3-2: Priorities of spectrum deployment mode for co-existence evaluation Agreement: - Prioritize the following spectrum deployment mode for RAN4 co-existence evaluation - A-IoT is located within a NR transmission bandwidth configuration - A-IoT which is operating indoor shares in-band spectrum with outdoor macro BS

Agreement in RAN4#111:

- Consider only adjacent RB/channel co-existence evaluation for in-band deployment scenario for NR and AIOT

- Encourage companies to provide the simulation results for option 1-1 and 1-2

  - FFS on co-site scenario (option 2-1 and 2-2)

Issue 2-1-2: deployment scenarios for D2T2

Agreement in RAN4#110bis: Option 1-1: Legacy NR gNB are outdoor macro gNB, AIoT intermediate UE/CW/devices are all indoors. Legacy NR UE is only allowed outdoor. Option 1-2: Legacy NR gNB are outdoor macro gNB, AIoT intermediate UE/CW/devices are all indoors. Legacy NR UE is indoor. Agreement: - For D2T2 co-existence evaluation, Legacy NR gNB are outdoor macro gNB, AIoT intermediate UE/CW/devices are all indoors. - Consider option 1-1 and option 1-2 as the starting point

- Topic 2-2: Spectrum usage

Issue 2-2-1: Spectrum usage for R2D in D1T1

Agreement in RAN4#110bis: - FFS on whether to prioritize FDD DL spectrum for R2D for D1T1 for co-existence evaluation.

Agreement in RAN4#111:

- Use FDD DL as starting point for co-existence evaluation for R2D in D1T1

  - FFS on FDD UL spectrum.

Issue 2-2-2: Spectrum usage for CW transmission in D1T1 for the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering

Agreement in RAN4#110bis: For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, and for topology 1, the following cases for CW transmission are studied. · Case 1-1: CW is transmitted from inside the topology, transmitted in DL spectrum · Case 1-2: CW is transmitted from inside the topology, transmitted in UL spectrum · Case 1-4: CW is transmitted from outside the topology, transmitted in UL spectrum Agreement: - For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, consider the following for co-existence evaluation - CW transmits in either UL or DL spectrum - FFS on inside topology and outside topology.

Agreement in RAN4#111:

- Use inside topology as starting point for co-existence evaluation (case 1-1, case 1-2) for calibration.

- Further discuss the difference of outside topology (case 1-4) from co-existence study perspective.

Issue 2-2-4: Spectrum usage for R2D in D2T2

Agreement in RAN4#110bis: - Use FDD UL spectrum for R2D in D2T2.

Issue 2-2-5: Spectrum usage for CW transmission in D2T2 for the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering

Agreement in RAN4#110bis: For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, and for topology 2, the following cases for CW transmission are studied. · Case 2-2: CW is transmitted from inside the topology (i.e., intermediate UE), transmitted in UL spectrum · Case 2-3: CW is transmitted from outside the topology, transmitted in DL spectrum · Case 2-4: CW is transmitted from outside the topology, transmitted in UL spectrum Agreement: - For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering - Use UL spectrum as the starting point for co-existence evaluation. - It won’t preclude the use of DL for backscattering transmission. - FFS on the minimum distance between the intermediate UE and A-IoT device

Agreement:

- Use case 2-2 as starting point for co-existence evaluation for calibration.

- FFS on case 2-3

- Further discuss the difference of outside topology (case2-4) from co-existence study perspective.

Evaluation methodology and cases

- Topic 3-1: Evaluation methodology

Issue 2-4-1: Evaluation methodology

Agreement in RAN4#110bis: - Use the Monte-Carlo method as baseline for co-existence evaluation, i.e. Section 5.3 in TR38.803 - Depending on the discussion on deployment scenarios, for some cases, calculation for the worst interference link may be enough. - FFS on whether RAN4 needs to perform link level simulation

Agreement in RAN4#111:

- Use the Monte-Carlo method as baseline for co-existence evaluation, i.e. Section 5.3 in TR38.803

- FFS on whether RAN4 needs to perform link level simulation

Issue 2-4-2: Performance metric for AIOT

Agreement in RAN4#110bis: - For NR system, use 5% throughput loss as performance metric as legacy. - For AIOT system, including reader, device, intermediate UE, further discuss the performance metric: - Option 1: [10%] BLER, [Rx power] - Option 2: SINR degradation - Other options are precluded

Agreement in RAN4#111:

- Use SINR for calibration purpose

- FFS on performance metric for co-existence evaluation and requirements definition.

Issue 2-4-2: SINR definition for D2R

Agreement in RAN4#111:

Do not consider CW interference for calibration purpose for D1T1-A2 and D2T2-A2

FFS on how to consider CW cancellation capability in formal simulation

Issue 2-4-2: SINR definition for R2D

Agreement in RAN4#111:

SINR for R2D for calibration purposes

- signal power of device to the noise and interference within 10MHz

  - Assume interference NR BW is 10MHz

- FFS on BB LPF

- Topic 3-2: Evaluation cases

Issue 3-2-1: device type

Agreement in RAN4#110bis: - Prioritize device 1 and 2a without a frequency shifter for coexistence evaluation.

Issue 3-2-4: Evaluation cases for D1T1 for device 1 and 2a between NR and AIOT

Agreement in RAN4#111:

Use the following cases for calibration purposes.

| Deployment scenario and topology | spectrum | aggressor | victim | Note |
| --- | --- | --- | --- | --- |
| · Case 1-1: CW is transmitted from inside the topology, transmitted in DL spectrum Case 1-2: CW is transmitted from inside the topology, transmitted in UL spectrum | R2D: DL CW2D and D2R: UL | device | NR UL | D2R |
|  |  | NR UL | reader | D2R |
|  |  | reader | NR DL | R2D |
|  |  | NR DL | device | R2D |
|  | R2D: DL CW2D and D2R: DL | device | NR DL | D2R |
|  |  | NR DL | reader | D2R |

Issue 3-2-5: Evaluation cases for D2T2 for device 1 and 2a between NR and AIOT

Agreement in RAN4#111:

Use the following cases for calibration purposes.

| Deployment scenario and topology | spectrum | aggressor | victim | Note |
| --- | --- | --- | --- | --- |
| \ Case 2-2: CW is transmitted from inside the topology (i.e., intermediate UE), transmitted in UL spectrum | R2D: UL CW2D and D2R: UL | device | NR UL | D2R |
|  |  | NR UL | reader | D2R |
|  |  | reader | NR UL | R2D |
|  |  | NR UL | device | R2D |

Issue 3-2-7: Multi-operator scenario

Agreement in RAN4#111:

FFS on co-existence between AIOT system and adjacent operator NR system

Evaluation parameters

- Topic 4-1: Adjacent RB Tx and Rx charateristics

Issue 4-1-1:  A-IOT reader

Agreement in RAN4#111:

For calibration purpose, use 0RB guard band between AIOT and NR for in-band spectrum deployment mode

|  |  | In-band |  |
| --- | --- | --- | --- |
|  |  | Tx | Rx |
| NR UE/A-IOT Intermediate UE | For calibration purpose | Legacy UE IBE | ACS |
| NR BS | For calibration purpose | ACLR of legacy gNB | ACS of legacy gNB |
| A-IOT BS | For calibration purpose | ACLR of legacy gNB (i.e. 45) | ACS of legacy gNB |

Issue 4-1-2: Tx for device 1 and 2a

Agreement in RAN4#111:

For device 1 and 2a, 25dBc is used for calibration purposes.

Issue 4-1-3: Rx for device 1 and 2a

Agreement in RAN4#111:

Assume no frequency selectivity for co-existence evaluation for calibration purposes for device 1 and 2a.

- Topic 4-2: General parameters and layout

Issue 4-2-1: General parameters

Agreement in RAN4#111:

Use following parameters for calibration purposes.

| General Parameter | D1T1&D2T2 Values for calibration purposes |
| --- | --- |
| Carrier frequency | 900MHz |
| BW for NR | 10MHz with 15KHz SCS |
| BW for AIOT system | 180KHz |
| Waveform (CW) | CW: Unmodulated single tone |
| Waveform (R2D) | OOK waveform generated by OFDM modulator |
| A-IoT DL power control | No |
| A-IoT UL power control | No |
| Traffic model | Full buffer |
| Frequency reuse | 1 |

Issue 4-2-2: Layout for D1T1

Agreement in RAN4#111:

Use following parameters for calibration purposes, i.e. scenario 1-1 and 1-2.

For Indoor NR, parameters are FFS, i.e. scenario 2-1 and 2-2.

| Parameter | Assumptions for D1T1 |
| --- | --- |
| Scenario | InF-DH |
| Hall size | 120x60 m |
| Room height | 10 m |
| Sectorization | None |
| Pathloss model | NLOS and LOS in TR38.901 |
| BS deployment / Intermediate UE dropping | For D1T1-A2: 18 BSs on a square lattice with spacing D, located D/2 from the walls. - L=120m x W=60m; D=20m - BS height = 8 m For D1T1-A1: - FFS on layout: one node as transmission and CW, the neighbour node as reception |
| Device distribution | Device Height= 1.5 m AIoT devices drop uniformly distributed over the horizontal area Number of A-IoTs = Total area × activated density (1.5 A-IOT devices/m²) 1 active AIOT device under one reader at one drop MCL between device and reader is 45dB for calibration. |
| NR BS deployment (outdoor), i.e. scenario 1-1 and 1-2 | Hexagonal grid, 19 macro sites, 3 sectors per site with wrap around, 1 AIOT indoor scenario per sector the minimum 2D distance between macro BS and indoor factory centre is set as 100m. |
| NR BS Inter-site distance | 750 |
| Minimum NR BS – NR UE distance (2D) | 35 m |
| NR UE (D1T1) dropping | For scenario option 1-1, uniformly distributed outdoor. For scenario option 1-2, uniformly distributed, 80% indoor, 20% outdoor UE number: - DL active UE: 1 UE per cell - UL active UE: 3UE per cell |
| O2I penetration loss | High penetration loss as in TR 38.901 for calibration |

Issue 4-2-3: Layout for D2T2

Agreement in RAN4#111:

Use InH-office as baseline for D2T2 co-existence evaluation.

Use following parameters for calibration purposes, i.e. scenario 1-1 and 1-2.

| Parameter | Assumptions for D2T2 |
| --- | --- |
| Scenario | InH-office |
| Hall size | 120 x50 m |
| Room height | 3m |
| Sectorization | None |
| Pathloss model | LOS and NLOS in TR38.901 |
| BS deployment / Intermediate UE dropping | - L=120m x W=50m; - Intermediate UE height = 1.5 m For D2T2-A2: The intermediate UEs selected from the fixed positions. Number of intermediate UE for calibration: - Option 1: 2 UE at one drop - Option 2: 12 UE at one drop For D2T2-A1: FFS on layout |
| Device distribution | Device Height= 1.5 m AIoT devices drop uniformly distributed over the horizontal area Number of A-IoTs = Total area × activated density (1.5 A-IOT devices/m²) 1 active AIOT device under one reader at one drop Minimum distance between reader and device is 1m |
| NR BS deployment | Hexagonal grid, 19 macro sites, 3 sectors per site with wrap around, 1 AIOT indoor scenario per sector the minimum 2D distance between macro BS and indoor factory centre is set as 100m. |
| NR UE dropping | For scenario option 1-1, uniformly distributed outdoor. For scenario option 1-2, uniformly distributed, 80% indoor, 20% outdoor UE number: - DL active UE: 1 UE per cell - UL active UE: 3 UE per cell |
| O2I penetration loss | High penetration loss as in TR 38.901 |

- Topic 4-3: Paramters for AIOT BS/intermedaite UE and device

Issue 4-3-1: AIOT micro-BS parameters for D1T1

Agreement in RAN4#111:

Use following parameters for calibration purposes.

| A-IoT micro BS parameters | Values for calibration purposes |
| --- | --- |
| A-IoT micro-BS total Tx power | 33dBm |
| A-IoT micro-BS receiver Noise Figure（dB） | 10 |
| A-IoT micro-BS antenna gain (dBi) | 6 dBi |
| Antenna pattern | Refer to TR25.942 |

Issue 4-3-2: Intermediate UE parameters for D2T2

Agreement in RAN4#111:

Use following parameters for calibration purposes.

| intermediate UE parameters | Values for calibration purposes |
| --- | --- |
| intermediate UE total Tx power（dBm） | 23dBm |
| gain of antenna intermediate UE (dBi) | 0 |
| intermediate UE receiver Noise Figure（dB） | 9 |
| Antenna configuration | Omni direction antenna |

Issue 4-3-3: CW parameters

Agreement in RAN4#111:

Do not use CW2D for calibration purposes.

FFS on:

- CW interference to NR BS/UE/AIOT

- CW remaining interference after CW cancellation.

  - [CW IBE]

Issue 4-3-4: AIOT device parameters

Agreement in RAN4#111:

Use Device 1 with following parameters for calibration purposes.

| A-IoT device parameters | Device 1 Values for calibration purposes |
| --- | --- |
| A-IoT device effective antenna gain per Tx or Rx branch (dBi) | 0 |
| A-IoT device reflection （backscatter）loss (dB) Note: due to, e.g., impedance mismatch | OOK: -6 dB |
| A-IoT device power gain of reflection amplifier (dB) | N/A |
| A-IoT Device receiver sensitivity (dBm) Use this value to determine whether device can camp on the cell. | -36 |
| A-IoT device noise figure (dB) | 24 |
| Guard band | 0PRB |

- Topic 4-4: Paramters for legacy NR

Issue 4-4-1: NR macro BS parameters

Agreement in RAN4#111:

Use following parameters for calibration purposes.

| NR macro-BS Parameter | Values for calibration purposes |
| --- | --- |
| Macro-BS Tx power (dBm) | 46 |
| BS antenna gain (dBi) and antenna pattern | Refer to TR36.942 |
| Height of macro NR BS (m) | 25 |
| NR Macro-BS Noise Figure(dB) | 5 |
| Network location | outdoor |

Issue 4-4-2: NR UE parameters

Agreement in RAN4#111:

Use following parameters for calibration purposes.

| NR UE Parameter | Values for calibration purposes |
| --- | --- |
| UE TX power in dBm | -40 to 23 |
| NR UE Antenna gain (dBi) | 0 |
| Height of UE antenna (m) | 1.5 |
| NR UE ACLR（dB） | 30 |
| NR UE Noise Figure（dB） | 9 |
| Antenna configuration | Omni direction antenna |