3GPP TSG-RAN WG4 Meeting#111                                                         R4-2410573

Fukuoka City, Fukuoka , Japan, 20th – 24th May, 2024

Agenda item:			7.16.9

Source:	Moderator (ZTE)

Title:	WF on Rel-18 NR-NTN UE RF requirements

Document for:	Approval

WF

Issue 1-0: Maximum EIRP

Agreement:

- Note: Maximum EIRP is defined using 13 PRBs allocation with 120KHz SCS.

Issue 1-1: Maximum TRP

Agreement:

- 43dBm for type 2 and type 5, and max EIRP and off-axis EIRP requirements CANNOT be relaxed

- 43dBm for type 3 if max EIRP requirements CANNOT be relaxed

Issue 1-2: Power backoff for type 2 and type 5 with phase array to meet OFF-axis requirement for GSO orbit

Agreement:

- Add the NOTE in the VSAT class table

- Note X: 	UE may need power reduction for meeting OFF-axis EIRP requirement defined in clause 9.2.2. Value is implementation dependent.

Issue 1-3:  TRPmax

Agreement:

- To update TRPmax in the emission requirements with TRPrated which  is the declared rated output power lower than TRPmax specified in clause 9.2.1.

Issue 1-4:  Number of VSAT simultaneously transmitting, N

Agreement:

- Clarify that the number of VSAT simultaneously transmitting N shall be declared by the VSAT manufacturer.

Issue 1-5: the clarification of  “uncoordinated” from the additional Off-axis EIRP density requirements for protection of fixed services (sub-clause 9.2.2.3.3)

Agreement:

- remove the uncoordinated in the specification

Issue 1-6:  NS value for FR2 NTN bands

Agreement:

- For  band n512,  to use the NS approach to define the regional requirements;

- For band n511 and n510,  not to define the NS values;

Issue 1-7: Beam steering range and test direction to fulfill minimum EIRP requirement.

Agreement:

- [For measurement metric for min peak EIRP , the min peak EIRP should be verified between the declared supported lowest transmitter elevation angles (as shown in Figure 1-3). ]

- Figure 1-3 example measurement grid for min peak EIRP of the supported elevation angle

Issue 1-8: On-axis cross polarization isolation requirements VS Pointing accuracy requirements in clause 9.6.1

Agreement:

- For the type 3 VSAT,  linear polarization requirement is not applicable

- For the type 1/2/4/5,  both linear and circular polarization requirement are applicable.

Issue 1-9: Off-Axis EIRP

Agreement:

- ”Off-axis EIRP should” be ”Off-axis EIRP emission density limit within the operating band”

Issue 1-10: Fixed VSAT Off-axis EIRP requirements for n512

- Proposal 1 (R4-2409758):

  - For co-polarized transmissions, the requirements specified in table 9.2.2.3.1-1 apply to Fixed VSAT type 1 or 2 when transmitting towards GSO. [THALES, Ericsson]

  - For cross-polarized transmissions, the requirements specified in table 9.2.2.3.1-2 apply to Fixed VSAT type 1 or 2 when transmitting towards GSO. [THALES, Ericsson]

Agreement:

- Proposal 1 is agreeable.

Issue 2-1  Minimum EIS requirement

Agreement:

- 10.3.2 Minimum requirement The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as [specified in Annexes A.2.3.2 and A.3.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD for the DL-signal as described in Annex A.5.2.1) with peak reference sensitivity specified in Table 10.3.2-1. And EISREFSENS_50M declared by the vendor is an integer value in the range specified in Table 10.3.2-2 for different types of NTN VSAT]. The requirement is verified with the test metric of EIS (Link=RX beam peak direction, Meas=Link Angle). Table 10.3.2-1: OTA reference sensitivity requirement for NTN VSAT NTN VSAT channel bandwidth (MHz); UL/DL RB allocation; OTA reference sensitivity level, EISREFSENS (dBm). 50, 100, 200, 400; Full RB allocation NRB as specified in clause 5.3.2; EISREFSENS_50MHz + 10log10(NRB x SCS x 12 / factor) (NOTE 1). NOTE 1: The “factor” represents the normalized factor to scale EIS for different (Channel bandwidth, SCS) configurations. The value of factor is 66 RBs x 60 kHz SCS x 12, i.e. 47520 kHz.. Table 10.3.2-2: The range of EISREFSENS_50MHz declared by vendor per NTN VSAT Operating band; NTN VSAT class; NTN VSAT type; The range of EISREFSENS_50MHz (dBm). n512, n511; Fixed VSAT; 1, 2; ≤ -122. 3; ≤ -115.6. n512, n511, n510; Mobile VSAT; 4, 5; ≤ -122.

Issue 2-2: Maximum input power

Agreement:

  - For type 1/2/3,

- -101dBm as maximum input power with 64QAM

  - For type 4/5:

- -109.6dBm with 16QAM and [64QAM ].

  - For MCS for 64QAM

- Please check the MCS in R4-2409618.

Issue 2-3: ACS

Agreement:

For type 1/2/4/5/3,  to define the following requirements;

| Rx Parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Power in Transmission Bandwidth Configuration | dBm | EISREFSENS_50M + 6 dB + 10log10(NRB x SCS x 12 / factor) |  |  |  |
| PInterferer for band n512, n511, n510 | dBm | (EISREFSENS_50M + 28.7.)+ 10log10(NRB x SCS x 12 / factor) |  |  |  |
| BWInterferer | MHz | 50 | 100 | 200 | 400 |
| FInterferer (offset) | MHz | 50 / -50 NOTE 3 | 100 / -100 NOTE 3 | 200 / -200 NOTE 3 | 400 / -400 NOTE 3 |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annex A.3.2 with one sided dynamic OCNG Pattern as described in Annex A.3.2 and set-up according to Annex C. NOTE 2: EISREFSENS_50M declared by the vendor is an integer value in the range specified in Table 10.3.2-2 for different types of NTN VSAT. NOTE 3: The absolute value of the interferer offset FInterferer (offset) shall be further adjusted to (CEIL(¦FInterferer(offset)¦/SCS) + 0.5)*SCS MHz with SCS the sub-carrier spacing of the wanted signal in MHz. Wanted and interferer signal have same SCS. [NOTE 4: The transmitter shall be set to same as the PUMAX,f,c as defined in clause 6.2.4, with uplink configuration specified in Clause 10.3.] NOTE 5: The “factor” represents the normalized factor to scale wanted signal and interference level for different (Channel bandwidth, SCS) configurations. The value of factor is 66 RBs x 60 kHz SCS x 12, i.e. 47520 kHz. |  |  |  |  |  |

The wanted signal and interference signal is supposed to come from the same directions

Issue 2-4: Beam steering range and test direction to fulfill minimum EIS requirement

Agreement:

- NTN VSAT need to declare its supported lowest elevation angle from receiver perspective regardless of mechanical steering antenna or electronic steering antenna or hybrid

- For measurement metric for min EIS , the EIS should be verified between the declared supported receiver lowest elevation angles (as shown in Figure 1-3).

- Figure 1-3 example measurement grid for min EIS of the supported elevation angle

  - Offline agreement:

  - The minimum elevation angle for both transmitter and receiver is up to the declaration by vendor..

Issue 2-5: Receiver antenna off-axis performance

- Proposal 1: to consider receiver antenna off-axis performance as in R4-2408702

Agreement:

- Proposal 1 is agreeable.