# 3GPP TSG-RAN WG4 Meeting #111                                   R4-2410587

# Fukuoka, Japan, 20 – 24 May, 2024

Title: 	WF on NR_SL_ intraB_CA_ITS_part2

Agenda Item:	10.9.4

Source: 	Huawei, LGE, Meta Ireland

Document for:	Approval

Issue 1: Tx UE RF requirements for PC2 SL intra-band contiguous CA

Issue 1-1: MOP

Online Agreement:

- For PC2 intra-band contiguous SL CA, the following two aspects should be specified in Rel-19

  - Minimum requirements for Power Class 2 should be applicable for SL intra-band contiguous CA combination configuration, and update the maximum output power for sidelink CA

- Define SL intra-band contiguous CA PC2 MOP as total transmitted power (Per UE) for PC2 SL CA UE

Issue 1-2: MPR

Online Agreement:

- MPR/A-MPR requirements for intra-band contiguous SL CA with PC2 should be specified in Rel-19 referring to the methodology of that with PC3 in Rel-18 including, PSSCH/PSCCH, PSFCH and S-SSB

  - For PSSCH/PSCCH: specify MPR for power class 2 contiguous CA with contiguous/non-contiguous RB allocation, including inner and outer.

  - For PSFCH: specify MPR for PC2 contiguous CA based on different range of R, R is the ratio of the gap bandwidth between the two PSFCH transmitted on the two intra-band carrier by the total bandwidth of the two carrier

  - For S-SSB: specify MPR for PC2 contiguous CA by single S-SSB and two S-SSB, separately.

Issue 1-3: Simulation Assumption for MPR

<Way forward>: Keep 25dBc carrier leakage as agreed WF(R4-2406611) for PC2 intra-band SL contiguous CA.

Issue 1-4: UE RF architecture for MPR evaluation

Online Agreement:

- The PC2 UE RF architecture of NR intra-band contiguous CA for MPR can be found as below for information (1 PA, dual Tx(Tx diversity), and dualPA)

  - High priority for MPR evaluations

    - 1 PA : 1x26dBm + 1LO

    - Dual Tx : 2x23dBm + 1LO

  - Second priority for MPR evaluations

    - dualPA : 2x23dBm + 2LO

Issue 1-6: ACLR

Online Agreement:

- Reuse 31dB ACLR for intra-band contiguous PC2 CA

Issue 1-7: Whether to introduce ΔPPowerClass,SL CA

Online Agreement:

- Not to introduce ΔPPowerClass,SL CA due to no SAR related issues for sidelink CA on the ITS band

Issue 1-8-1: A-MPR for EU regulation

Online Agreement:

- No A-MPR requirement based on EU regulation is needed for sidelink intra-band contiguous CA.

Issue 1-8-2: A-MPR for US regulation

Online Agreement:

- Study and specify the related A-MPR requirements with NS_52 after final FCC announcement for the additional emission limits in US

Issue 1-9: Other Tx requirements than MOP, MPR and A-MPR

<Way forward>: Reuse the requirements of PC3 SL intra-band contiguous CA

- Minimum output / transmit OFF power /transmit ON/OFF time mask

- Power control

- Transmit signal quality

- SEM/

- General spurious emissions/UE-to-UE coexistence SE

- Transmit intermodulation

Issue 2: Rx requirements for SL intra-band contiguous CA

Sub-topic 2-1 : Rx UE RF requirements for PC2 SL intra-band contiguous CA

Issue 2-1: Rx UE RF requirements

Online Agreement:

- The Rx requirements including REFSENS, ACS, maximum input power and blockings for intra-band contiguous SL CA with PC3 in current Spec shall apply to the intra-band contiguous SL CA with PC2 directly

Issue 3: System Parameter

Online Agreement:

- For PC2 intra-band contiguous CA, reuse the same channel bandwidth of PC3 intra-band contiguous CA.