3GPP TSG-RAN WG4 Meeting #111	R4-2410602

Fukuoka, Japan, May 20th – 24th, 2024

Agenda item:			10.10.3

Source:	Intel Corporation

Title:	WF on UE RF requirements for Rel-19 NR channel BW less than 5MHz for FR1 Phase 2

Document for:	Approval

# Topic #1: UE RF requirements for inter-band NR CA/DC with less than 5MHz CBW

### Issue 1-1: Support of 5MHz CBW in band n100 for CA_n100-n101 and and NR DC_n100-n101

- Agreement (online session)

  - In RAN4 common understanding, the support of 5MHz CBW on band n100 and 5MHz/10MHz on n101 in the band combination with band n100 and n101 can be included in Rel-19 WID.

### Issue 1-2: ΔTIB,c  for CA_n100-n101 and and NR DC_n100-n101

- Agreement (online session)

  - ΔTIB,c = 0.3 dB for CA_n100-n101 and NR DC_n100-n101

| Inter-band CA combination | ΔTIB,c for NR bands (dB) |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration |  |
| CA_n100-n101 | 0.3 | 0.3 |

### Issue 1-3: MSD requirements

- Agreement (online session)

  - Do not specify MSD requirements for CA_n100-n101 and NR DC_n100-n101

  - MSD requirements for other band combinations for BW less than 5 MHz can be discussed in the basket WI(s)

### Issue 1-4: Spurious emissions for UE co-existence for CA_n100-n101 and and NR DC_n100-n101

- Agreement (online session)

  - Spurious emissions for UE co-existence for CA_n100-n101

| NR CA combination | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected Band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n100-n101 | Frequency range | 758 | - | 788 | -50 | 1 |  |

### Issue 1-5: Power class for UL inter-band CA for CA_n100-n101

- Whether to define PC1 for UL inter-band CA for CA_n100-n101 can be discussed in RANP.

### Issue 1-6: UE capability and network signalling

- FFS whether any updates to Rel-18 UE capability signalling are needed to enable less than 5 MHz CBW operation for CA/DC.

- Further potential enhancements can be further discussed for SCell with less than 5MHz.

  - FFS whether UEs needs to be informed that SCell has 12 PRB or 20 PRB transmission bandwidth for 3MHz or 5MHz CBW, respectively

### Issue 1-7: Sync raster applicability for Scell with < 5MHz CBW for CA_n100-n101

- FFS whether any restrictions on ARFCN for SSB locations for SCell SSB need to be defined

  - Option 1: Require less than 5 MHz SCell to be associated with the new sync raster points for less than 5MHz, similar as PCell.

  - Other options are not precluded

### Issue 1-8: CR handling

- Rapporteur (Intel) will prepare a single draft CR for RAN4 #112 covering the agreements in RAN4 #110bis and #111 and share before the RAN4 #112 with interested companies.