3GPP TSG-RAN WG4 Meeting #111	R4-2410611

Fukuoka, Japan, May 20 - 24, 2024

Title: 	WF on NR_cov_enh2_R18

Agenda Item:	7.17.3

Source: 	Huawei, HiSilicon

Document for:	Approval

1. On Rel-18 power boosting applicability

1.1 How to handle MSD for Rel-18 power boosting extension

Agreement:

- No new MSD test cases are introduced to reflect Rel-18 power boosting.

  - TS 38.101-1 shall explicitly state that no MSD requirements unique to power boosting operation are defined. How to capture is FFS.

- Case ‘2’ in feature list for Rel-18 power boosting feature is supported in Rel-18. FFS CR wording subject to parallel Rel-17 power class maintenance discussion.

1.2. Rel-18 power boosting applicability for inter-band UL CA/DC

Agreement:

- Case ‘3’ in feature list for Rel-18 power boosting feature is supported in Rel-18.

1.3. Rel-18 power boosting applicability for FR1+FR2 CA/DC

Agreement:

- Case ‘4’ in feature list for Rel-18 power boosting feature is supported in Rel-18.

1.4. Sanity check on existing RAN2 signalling aspect

Agreement:

- RAN4 to verify that can per FS signalling of associated capabilities allow UE to indicate support of case A (DL CA) but not case B (case descried in issue 1.2) or C (case descried in issue 1.3)?

- Check that when parent BC does not indicate the support of Rel-18 power boosting capability, can UE indicate support of Rel-18 power boosting capability for the same band in fallback BC?

1.5. Background only: on ‘cases’ referenced above

Case 1: FR1 single band with single uplink CC configured in the band where power boosting capability is indicated in this band. Case 2: FR1 DL CA with a single uplink CC configured in a band where power boosting capability is indicated. The power boosting feature can be configured in this FR1 NR band. Case 3: FR1 CA/DC with inter-band UL CA/DC, where a single CC is configured in the uplink bands where power boosting capability is indicated. The power boosting feature can be configured only in one of the bands where capability is indicated. Case 4: FR1+FR2 UL CA, FR1+FR2 DC, where a single CC is configured in the uplink bands where power boosting capability is indicated. The power boosting feature can be configured in the FR1 NR band