3GPP TSG-RAN WG4 Meeting # 111	R4-2410641

# Fukuoka, Japan, May 20 – May 24, 2024

Source:	Huawei, HiSilicon

Title:	Discussion and TP for TR 38.718-02-01 to introduce CA_n3A-n39A

Agenda item:	6.1.1.2

Document for:	Approval

1 Background

This contribution provides the technical text proposal on the NR CA band combination CA_n3A-n39A, which has been captured in the latest basket WI [1].

2 Discussion

In last meeting, CA_n3-n39 contribution was noted as companies need more time to check the cross band isolation from band n3 UL to band n39 DL due to fly back effect inside n39. In this paper, we just analyse the MSD issue due to cross band isolation. The RF reference architecture is shown below for CA_n3-n39.

Figure 1 RF reference architecture for CA_n3-n39

It’s noted that the following filter performance is assumed.

Table 1 The performance for some key components

| Parameters | Value |
| --- | --- |
| Antenna isolation | 10dB |
| Tx attenuation at 1880~1920MHz for band n3 duplexer | 30dB |
| Rx attenuation at 1710~1785MHz for band n3+n39 Rx filter | 40dB |

The MSD calculation for CA_n3-n39 is shown below.

Table 2 The MSD calculation for CA_n3-n39 due to cross band isolation

|  | parameters | n39 main path | n39 diversity path |
| --- | --- | --- | --- |
| transmit power for n3, dBm |  | 23 | 23 |
| Tx band n3 BW(MHz, Lcrb@15kHz) | 30 |  |  |
| RFFE loss, dB | 4 |  |  |
| Diplexer isolation at n3 uplink freq，dB | 0 |  |  |
| antenna isolation, dB | 10 |  |  |
| n39 receival singal at ANT port, dBm |  | 13 | 13 |
| n39 filter rejection at 1710~1785MHz, dB | 40 |  |  |
| signal After n39 filter, dBm |  | -31 | -31 |
| Typical receiver IIP2, dB | 50 |  |  |
| TX IM2 noise level refer to RX LNA input, dBm |  | -112 | -112 |
| front-end loss | 4 |  |  |
| TX IM2 noise level at ANT port, dBm |  | -108 | -108 |
| noise figure dB | 9 |  |  |
| Thermal noise at RX ant port(dBm/Hz) | -165 |  |  |
| Rx band n39 BW(MHz, NRB) | 4.5 |  |  |
| Thermal noise, dBm |  | -98.47 | -98.47 |
| n3 PA noise PSD at PA output port, dBm/Hz | -130 |  |  |
| n3 PA noise PSD at PA output port, dBm/RxBW |  | -63.47 |  |
| n3 Tx filter rejection at 1880~1920MHz, dB | 30 |  |  |
| n3 PA noise power at Rx n104 ant port at 6425~7125MHz, dBm |  | -103.47 | -103.47 |
| Total noise level at ANT port |  | -96.92 | -96.92 |
| SNR requirement for QPSK | -1 |  |  |
| REFSENSE (referred to antenna)(20MHz BW) |  | -97.92 | -97.92 |
| Implementation Margin, dB | 2.5 |  |  |
| combined REFSENS(5MHz BW), dBm | -98.43 |  |  |
| MSD | 1.57 |  |  |

Thus, the following MSD test configuration can be considered.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n3 | n39 | 1770 | 30 | 15 | 160 (RBstart=0) | 1882.5 | 5 | 1.5 dB | >ACLR2 |

3 Text Proposal

---Start of changes TR 38.718-02-01 ---

5.x	CA_n3-n39

5.x.1	Common for 1 band UL and 2 bands UL CA

5.x.1.1	Operating bands for CA

Table 5.x.1.1-1:  CA band combination of band n3+n39

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n3 | 1710 | – | 1785 | 1805 | – | 1880 | FDD |
| n39 | 1880 | – | 1920 | 1880 | – | 1920 | TDD |

5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination of band n3+n39

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n39A | n3 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n39 | 5, 10, 15, 20, 25, 30, 35, 40 |  |

5.x.1.3	UE co-existence studies

Table 5.x.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n3-n39.

Table 5.x.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n3 | 1710 | 1785 | 1805 | 1880 | 3420 | 3570 | 5130 | 5355 | 6840 | 7140 | 8550 | 8925 |
| n39 | 1880 | 1920 | 1880 | 1920 | 3760 | 3840 | 5640 | 5760 | 7520 | 7680 | 9400 | 9600 |

Table 5.x.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n3 | 1710 | 1785 | 1805 | 1880 | 3610 | 3760 | 5415 | 5640 | 7220 | 7520 | 9025 | 9400 |
| n39 | 1880 | 1920 | 1880 | 1920 | 3760 | 3840 | 5640 | 5760 | 7520 | 7680 | 9400 | 9600 |

5.x.1.4	∆TIB,c and ∆RIB,c values

For CA_n3-n39, the TIB,c and RIB,c values are given in the tables below, which refer to CA_n3-n34/ CA_n3-n38/ CA_n3-n41.

Table 5.x.1.4-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| CA_n3-n39 | 0.5 | 0.5 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.x.1.4-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| CA_n3-n39 | - | - |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.x.1.5	REFSENS requirements

There is no harmonic / harmonic mixing issue for CA_n3-n39. For cross band isolation, since the band n3 Tx filter can provide 30~40dB attenuation at 1880~1920MHz and 10 dB above antenna isolation can be observed, the following MSD test configuration can be considered with the presented reference RF architecture.

figure 5.x.1.5-1 reference RF architecture for CA_n3-n39

Table 5.x.1.5-1: MSD test configuration due to cross band isolation for CA_n3-n39

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n3 | n39 | 1770 | 30 | 15 | 50 (RBstart=110) | 1882.5 | 5 | 2.1 dB | >ACLR2 |

5.x.1.6	OOB blocking exception requirements

There is no need to specify OOB blocking exception for CA_n3-n39, but some clarification about how to apply FDL_low and FDL_high. Referring to CA_20A-28A, the following note can be introduced into table 7.6.3-2 of TS 38.101-1.

“For a UE supporting CA_n3A-n39A and higher order band combinations in which CA_n3A-n39A is a subset, the requirements for Band n3 and Band n39 apply with FDL_low given by the lower limit of the restricted operating frequency range in Band n3 and FDL_high by Band n39.”

---End of changes---

Reference

[1]	RP-232921, “Revised WID Rel-18 NR Inter-band Carrier Aggregation/Dual Connectivity  for 2 bands DL with x bands UL (x=1,2)”, ZTE Corporation