# 3GPP TSG-RAN WG4 Meeting # 111		                           							R4-2410644

Fukuoka, Japan, May 20 – May 24, 2024

Source:	Huawei, HiSilicon

Title:	TP for TR 38.718-03-01 to introduce CA_n3A-n39A-n79A

Agenda item:	6.11.2

Document for:	Approval

1 Background

This contribution provides text proposal on the NR CA band combination CA_n3A-n39A-n79A as defined in Revised WID: Rel-18 NR Inter-band Carrier Aggregation/Dual Connectivity for 3 bands DL with x bands UL (x=1,2) [1]. It’s noted that the fallback combo CA_n3A-n39A has been submitted in this meeting together and other fallback CA_n3A-n79A/ CA_n39A-n79A have been captured into the spec.

2 Text Proposal

---Start of changes---

5.X	CA_n3-n39-n79

5.X.1	Common for 1 band UL and 2 bands UL CA

5.X.1.1	Operating bands for CA

Table 5.X.1.1-1: 3DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n3-n39-n79 | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n39 | 1880 MHz | – | 1920 MHz | 1880 MHz | – | 1920 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

5.X.1.2	Channel bandwidths per operating band for CA

Table 5.X.1.2-1: Supported channel bandwidths

| NR CA configuration | Uplink CA configuration or single uplink carrier6 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n39A-n79A | - | n3 | 5. 10, 15, 20, 25, 30 | 0 |
|  |  | n39 | 5, 10, 15, 20, 25, 30, 35, 40 |  |
|  |  | n79 | 10, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |

5.X.1.3	∆TIB,c and ∆RIB,c values

For CA_n3-n39-n79, the TIB,c and RIB,c values are given in the tables below.

Table 5.X.1.3-1: ΔTIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n3-n39-n79 | 0.5 | 0.5 | 0.8 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n5 the band order from left to right is n1, n3 and n5. |  |  |  |

Table 5.X.1.3-2: ΔRIB,c due to NR CA (three bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration** |  |  |
| CA_n3-n39-n79 | - | - | 0.5 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3-n8 the band order from left to right is n1, n3 and n8. |  |  |  |

5.X.1.4	MSD requirement

Since CA_n3-n79, CA_n3-n39 and CA_n39-n79 are the fallback combination of CA_n3-n39-n79, and the studies for the corresponding MSD have been covered by these fallback combinations.

---End of changes---

Reference

[1]	RP-240166, “Revised WID: Rel-18 NR Inter-band Carrier Aggregation/Dual Connectivity for 3 bands DL with x bands UL (x=1,2)”, ZTE Corporation