3GPP TSG-RAN WG4 Meeting # 111									                 R4-2410687

Fukuoka, Japan, May 20 – May 24, 2024

Agenda item:			10.8.4

Source:	CMCC

Title:	WF on Rel-19 ATG UE requirements

Document for:	Approval

Introduction

This document captures the agreements on Rel-19 ATG UE RF requirements in RAN4#110bis meeting. The way forward in RAN4#110bis meeting is R4-2406594.

R19 UE RF requirements

intra-band contiguous CA

Issue 2-1-1: BCS for DL CA_n79C

Agreement:

- Reuse the existing BCS table for ATG

- Clarify that uplink CA is not supported for ATG UE elsewhere for Rel-19

- FFS on whether BCS#4 and #5 are not supported for ATG UE for Rel-19

Issue 2-1-2: whether/how to reflect operation band information in spec

Agreement:

- The information that only DL CA is supported in Rel-19 for ATG can be reflected in the clause of operating band for ATG

Issue 2-2: ATG Rx requirements applicability

Agreement:

- Following requirements needs to be updated for ATG CA compared with TN CA.

  - Max input level

  - ACS testing case 2

- Following requirements can reuse legacy TN CA requirements

  - Blocking including in-band and out of band blocking, narrow-band blocking

  - REFSENSE

  - Spurious response

  - Intermodulation

- Following requirement is not applicable for ATG CA

  - No CA Rx Spurious emission

  - No CA narrow-band blocking

- Following single carrier requirement of ATG is enough and no need to define ATG CA specific requirements

  - diversity characteristics requirement

Issue 2-3: max input level

Agreement:

Table 3: Maximum input level for ATG with CA_n79C

| Rx Parameter | Units | ATG UE Types |  |
| --- | --- | --- | --- |
|  |  | Omni-directional antenna | Antenna array |
| Power in largest transmission bandwidth configuration CC, Plargest BW | dBm | -422 | -302 |
|  |  | -443 | -323 |
| Power in each other CC | dBm | Plargest BW +10*log{(NRB,c*SCSc)/(NRB,largest BW*SCSlargest BW)} |  |
| The applicable NR CA Bandwidth Class | MHz | C |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 and 7.3F.2-3 for shared spectrum channel access operation with PCMAX_L,f,c as defined in clause 6.2.4. NOTE 2: Reference measurement channel is A.3.2.3 or A.3.3.3 for 64 QAM. NOTE 3: Reference measurement channel is A.3.2.4 or A.3.3.4 for 256 QAM. |  |  |  |

Issue 2-4: ACS testing parameters for case 2

Agreement:

- the principle of TN contiguous CA could be reused

inter-band CA

Issue 3-1: clarify the antenna type for each band for inter-band CA

Agreement:

- No limitation on antenna types for ATG CA

- FFS on whether to assume omni-antenna type can be assumed for both band n3 and n39 in DL CA_n3-n39

- FFS on whether new capability is needed for ATG CA

Issue 3-2-1: whether to allow n39 UL for CA_n3A-n39A

Agreement:

- FFS on whether to allow n39 UL for CA_n3A-n39A

Issue 3-3: BCS for DL CA_n3_n39

Agreement:

- FFS on BCS for DL CA_n3_n39

Issue 3-4-1: ATG Rx requirements applicability

Agreement:

- Following requirements needs further analysis:

  - delta RIB,

  - MSD,

  - Diversity requirements

    - FFS whether current ATG single carrier requirement is enough

- legacy inter-band CA requirements applies for following requirements while each CC shall meet the ATG UE requirement for single CC

  - REFSENSE

  - Max input level

  - ACS

  - in-band blocking

  - out of band blocking

    - FFS whether no OOB exception for CA_n3A-n39A

  - spurious response

  - wide band intermodulation

  - spurious emissions

- Following requirements are not applicable

  - Narrow band blocking

UL MIMO

Issue 4-1-1: ULFPTx mode for UL-MIMO

Agreement:

- FFS on whether ULFPTx mode for UL-MIMO is supported for ATG UE

Issue 4-2-1: how to modify the NR UL MIMO requirement with ATG capability antennaArrayType-r18

Agreement:

- if the legacy requirement for UL MIMO is defined as sum of measurements of all UE transmit antenna connectors, then for ATG UE with capability antennaArrayType-r18, the requirement should be defined as sum of all TAB connector including both polarization.

- If the legacy requirement for UL MIMO is defined as per antenna port or per layer, FFS

Issue 4-2-2: maximum output power

Agreement:

- Reuse the rated maximum output power reported by the ATG UE capability maxOutputPowerATG-r18, and consider the existing NR UL-MIMO requirement with some adaption in requirement description specific to ATG UE.

  - Detailed modification of NR UL MIMO requirement with ATG capability antennaArrayType is based on conclusion of issue 4-2-1

Issue 4-2-3: configured transmitted power

Agreement

- Configured transmitted power for UL MIMO is for each UE

- Reuse ATG single carrier tolerance requirement

- modify the NR UL MIMO requirement with ATG capability antennaArrayType-r18, detailed based on issue 4-2-1

Issue 4-2-4: output power dynamic

Agreement:

- Legacy NR UL MIMO requirements is baseline

  - modify the NR UL MIMO requirement with ATG capability antennaArrayType-r18, detailed based on issue 4-2-1

Issue 4-2-5: transmit signal quality

Agreement:

- Legacy NR UL MIMO requirements is baseline

  - modify the NR UL MIMO requirement with ATG capability antennaArrayType-r18, detailed based on issue 4-2-1

Issue 4-2-6: whether to apply coherent UL MIMO requirement for ATG UE

Agreement:

- FFS on whether to apply coherent UL MIMO requirement for ATG UE

Issue 4-2-7: Output RF spectrum emissions

Agreement:

- Legacy NR UL MIMO requirements is baseline

- modify the NR UL MIMO requirement with ATG capability antennaArrayType-r18, detailed based on issue 4-2-1

Issue 4-3: receiver requirements

Agreement:

- FFS on whether to have Rx requirements for UL-MIMO for ATG UE