3GPP TSG-RAN WG4 Meeting #111	R4-2410691

Fukuoka, Japan, 20 – 24 May 2024

Source:	Ericsson, T-Mobile USA

Title:	TP for 38.718-03-01 to add UL CA_n41C-n71 and CA_n41C-n77A for CA_n41C-n71A-n77A

Agenda item:			6.10.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for 38.718-03-01 to add UL CA_n41C-n71 and CA_n41C-n77A for CA_n41C-n71A-n77A.

# 2. Text Proposal

# ---Start of changes---

## 5.x	CA_n41-n71-n77

#### 5.x.1.2	Channel bandwidths per operating band for CA

| Table 5.x.1.2-1: Supported bandwidths per CA band combination of band n41+n71+n77NR CA configuration |  | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- |
| CA_n41C-n71A-n77A | n417,9 n777,9 CA_n41A-n71A7 CA_n41A-n77A7 CA_n41C7 CA_n71A-n77A7 CA_n41C-n71A CA_n41C-n77A |  | n41 | CA_n41C_BCS0 | 0 |
|  |  |  | n71 | 5, 10, 15, 20 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n41 | CA_n41C BCS 4 and 5 | 4 and 5 |
|  |  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |

### 5.x.2	Specific for 2 bands UL CA

#### 5.x.2.1	UE co-existence studies

Table 5.x.2.1-1 lists Band n41A + Band n71C 2UL bands CA 1st order triple beat (IMD3) related to 2UL band 3CC (one band support intra-band ULCA) for the UE-to-UE coexistence analysis into the third receive band of Band n77, where Band n71C is the uplink band supporting two uplink carriers and Band n41 is the single uplink carrier.

Table 5.x.2.1-1: Band n41 and Band n71 triple beat IMD products

| CC location | fU1L | fU2L | fU3L | fU1H |  | CBW |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency | 2496 | 2506 | 2686 | 2690 |  | 10 |
| CC location | fSCCL | fSCCH | fU2H | fU3H |  | Min ch. separation |
| Frequency | 663 | 698 | 2680 | 2500 |  | 0 |
| 1st order TB | IfU3L -fU1L- fSCCL¦ | IfU2L -fU1L + fSCCL¦ | IfU2L -fU1L- fSCCH¦ | IfU3L -fU1L + fSCCH¦ |  | Max ch. separation |
| Ranges | 473 | 673 | 688 | 888 |  | 190 |
| 1st order TB | IfU2L+fU1L-fSCCH¦ | IfU1H+fU2H-fSCCL¦ | IfU2L +fU1L+fSCCL¦ | IfU1H +fU2H+fSCCH¦ |  |  |
| Ranges | 4304 | 4707 | 5665 | 6068 |  |  |

Based on Table 5.x.2.1-1, 1st order triple beat IMD is not falling inside band n77 and no need to define MSD value.

.

Table 5.x.2.1-2 lists Band n77A + Band n71C 2UL bands CA 1st order triple beat (IMD3) related to 2UL band 3CC (one band support intra-band ULCA) for the UE-to-UE coexistence analysis into the third receive band of Band n41, where Band n71C is the uplink band supporting two uplink carriers and Band n77 is the single uplink carrier.

Table 5.x.2.1-2: Band n41 and Band n77 triple beat IMD products

| CC location | fU1L | fU2L | fU3L | fU1H |  | CBW |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency | 2496 | 2506 | 2686 | 2690 |  | 10 |
| CC location | fSCCL | fSCCH | fU2H | fU3H |  | Min ch. separation |
| Frequency | 3300 | 4200 | 2680 | 2500 |  | 0 |
| 1st order TB | IfU3L -fU1L- fSCCL¦ | IfU2L -fU1L + fSCCL¦ | IfU2L -fU1L- fSCCH¦ | IfU3L -fU1L + fSCCH¦ |  | Max ch. separation |
| Ranges | 3110 | 3310 | 4190 | 4390 |  | 190 |
| 1st order TB | IfU2L+fU1L-fSCCH¦ | IfU1H+fU2H-fSCCL¦ | IfU2L +fU1L+fSCCL¦ | IfU1H +fU2H+fSCCH¦ |  |  |
| Ranges | 802 | 2070 | 8302 | 9570 |  |  |

Based on Table 5.x.2.1-2, 1st order triple beat IMD is not falling inside band n71, and no need to define MSD value.

#### 5.x.2.2	REFSENS requirements

Based on the triple beat analysis of the added ULCA in Table 5.x.2.1-1 and Table 5.x.2.1-2, there is no triple beat impact and there is no need to define additional REFSENS requirements.

---End of changes---

# Reference

[1] 		RP-24xxxx, Revised WID:Rel-18 NR Inter-band Carrier Aggregation/Dual Connectivity for 2 bands DL with x bands UL (x=1,2), ZTE