3GPP TSG-RAN WG4 Meeting #111                                                                                        R4-2407513

Fukuoka, JP, May. 20 – May. 24, 2024

Source: 	CATT

Title: 	Discussion on RF requirement for NTN RedCap UE

Agenda item:	10.15.2.1

Document for:	Approval

- Introduction

In RAN4 #110-bis meeting, A WF [1] on NR NTN UE RF was agreed. However, there are still some remaining issues about RF requirement for NTN RedCap UE need to be discussed in the next meeting.

In this contribution, we would like to share our views on some issues for RF requirement.

- Discussion

The agreement on the approach of reuse of TN requirements and specification structure was achieved during the previous meetings, as shown below:

## Sub-topic 2-3: Re-use of TN requirements and specification structure Agreement: Add RedCap and eRedCap NTN requirements under the same new suffix into TS 38.101-5. Consider re-use of TN specification structure, i.e. which clauses are under suffix I in TS 38.101-1, as starting point.

For TS 38.101-5, there is no suffix table in current specification, as shown below:

## 4.3 Specification suffix information Specification suffix information is not defined for the time being in Release 17.

Therefore, we believed that the clause 4.3, specification suffix information should be updated.

Considering that no suffix has been defined in TS 38.101-5 yet, RAN4 may consider defining the definition table of suffix starting from suffix A, as shown below:

Table 4.3-1: Definition of suffixes

| Clause suffix | Variant |
| --- | --- |
| None | Single Carrier |
| A | RedCap |

Proposal 1: RAN4 may consider the definition of suffixes for NTN RedCap / eRedCap UE shown in Table 4.3-1 as starting point.

For the specification structure, the agreements made in RAN4-110bis meetings encourages to re-use the TN specification structure, for TS 38.101-5, the RedCap related requirements could be supplemented under the Clause 5.3 (Channel bandwidth), Clause 6.2 (Tx power) and Clause 7.3 (REFSENSE).

Proposal 2: RAN4 may consider supplementing the RedCap requirements for NTN RedCap / eRedCap UE under the Clause 5.3 (Channel bandwidth), Clause 6.2 (Tx power) and Clause 7.3 (REFSENSE) as starting point.

- Conclusion

This contribution provides our consideration on RF requirement for NTN RedCap and eRedCap UE. The following proposals are concluded as follows:

Proposal 1: RAN4 may consider the definition of suffixes for NTN RedCap / eRedCap UE shown in Table 4.3-1 as starting point.

Proposal 2: RAN4 may consider supplementing the RedCap requirements for NTN RedCap / eRedCap UE under the Clause 5.3 (Channel bandwidth), Clause 6.2 (Tx power) and Clause 7.3 (REFSENSE) as starting point.

- References

- R4-2406610, WF on NR_NTN_Ph3_UERF, Qualcomm, RAN4 #110-bis