3GPP TSG-RAN WG4 Meeting # 111                                                               R4-2407523

# Fukuoka City, Fukuoka, Japan, 20th – 24th May, 2024

Title: 	                 Discussion on RF requirements of Ambient IoT device

Source: 	CATT

Agenda item:	10.13.3.2

Document for:	Discussion

- Introduction

According to the SID [1], RF requirements for Ambient IoT need to be analyzed, in this contribution, we analyze the potential RF requirements of A-IoT device.

- Potential RF requirements of A-IoT device

In the SID [1], the RF requirements objectives for Ambient IoT can be seen below:

- RAN4-led:

  - Coexistence study of Ambient IoT and NR/LTE.

  - RF requirements study for Ambient IoT:

    - Ambient IoT BS transmission and reception

    - Ambient IoT Device, as per the General Scope, transmission and reception

    - Intermediate node (UE), as per the General Scope, transmission and reception

A-IoT device includes D2R backscattering signal transmitter, CW2D receiver and R2D interrogation signal receiver, CW is transmitted to device 1 or device 2a.

In this contribution, we only analyze potential requirements of device 1 and device 2a, requirements of device 2b will be analyzed later. We analyze A-IoT requirements based on RFID standard [2] and regulation[3], NB-IoT UE standards[4], as well as A-IoT topology, scenario deployment, device architecture, etc.[5].

2.1 A-IoT device potential Tx requirements

Table 1: A-IoT device potential Tx requirements

| No. | ISO-IEC 18000-6C Tag RF Tx requirements | RFID EN 302 208 Tag transmitter conformance requirements | NB-IoT UE RF Tx requirements | A-IOT Device potential Tx requirements |
| --- | --- | --- | --- | --- |
| 1 | Max EIRP | Radiated power (e.r.p.) | UE maximum output power | Radiated power dynamic range |
| 2 |  |  | Minimum output power |  |
| 3 |  |  | Transmit OFF power | According to how the D2R signal is generated, not necessary to define these requirements |
| 4 |  |  | ON/OFF time mask |  |
| 5 |  |  | Power Control |  |
| 6 |  |  | Frequency error |  |
| 7 |  |  | EVM |  |
| 8 |  |  | Carrier leakage |  |
| 9 |  |  | In-band emissions |  |
| 10 | Occupied bandwidth |  | Occupied bandwidth |  |
| 11 | Spectrum masks | Unwanted emissions: Spectrum mask | Spectrum emission mask | RAN4 needs further analysis |
| 12 |  |  | ACLR | Not necessary |
| 13 | Spurious emissions |  | Spurious emissions | Spurious emissions |
| 14 |  |  | Transmit intermodulation | Not necessary |
| 15 |  |  |  | Device backscatter loss is needed to be analysed. |

A-IoT device potential Tx requirements are shown in table 1. According to how the D2R signal is generated, not necessary to define No.3~No.10, No.12 and No.14 requirements, but device backscatter loss is needed to be analysed.

Observation 1: Potential Tx requirements of device 1 or device 2a are shown in Table 1.

2.2 A-IoT device CW2D potential Rx requirements

In order to calculate power level of D2R signal, input signal power range requirement of CW2D Rx is necessary.

Proposal 1: Input signal power range requirement of CW2D Rx is necessary.

2.3 A-IoT device R2D potential Rx requirements

Table 2: A-IoT device R2D potential Rx requirements

| No. | NB-IoT UE RF Rx requirements | A-IOT Device R2D potential Rx requirements |
| --- | --- | --- |
| 1 | Reference sensitivity power level | Input signal power dynamic range |
| 2 | Maximum input level |  |
| 3 | Adjacent Channel Selectivity | Adjacent Channel Selectivity |
| 4 | In-band blocking | In-band blocking |
| 5 | Out-of-band blocking | Out-of-band blocking |
| 6 | Spurious response | Not necessary |
| 7 | Intermodulation characteristics | Not necessary |
| 8 | Spurious emissions | Not necessary |

A-IoT device R2D potential Rx requirements are shown in table 2. ACS and In-band blocking requirements are important for A-IoT systems to be implementable and coexist with other systems, out-of-band blocking need to be filtered out to ensure receiver performance, so they are needed to be defined. Considering the receiver architectures and costs of device 1 and device 2a, not necessary to define No.6~No.8 requirements.

Observation 2: Potential R2D Rx requirements of device 1 or device 2a are shown in Table 2.

2.4 RF filter

RF BPF is operating frequency selective filter, which can supply suppressions to frequency bands out of the operating frequency band. Firstly, it ensures that the devices of the receiving path will not be blocked or damaged due to too large input power of interference signals. In addition, it is needed to provide certain suppressions to the frequency bands which will finally enter the wanted band through envelope detector. RF BPF is needed to guarantee the receiver performance in the presence of in-band and out-of-band interference signals.

In A-IoT device receiver path, RF BPF can suppress the interference signals before the RF envelope detector, it can select detection frequency band and improve the detection accuracy. Interference signals may include NR, LTE BSs and UEs transmit signals, etc. The performance requirement of the RF BPF can be derived by analysing the reception capability deterioration limits in the presence of interference signals, the receiver architecture, wanted signal power range, interference signal power and frequency, bandwidth, waveform, etc..

In theory, the transition band of RF LC filter is relatively wide, it is challenging to design narrow-bandwidth RF bandpass filters with LC, for example, BPF with a centre frequency of 900MHz and a bandwidth of 5MHz is difficult to design with LC. Even for the 20MHz pass band case, the transient band will be multiple times of the 20MHz if a good rejection performance is needed.

Observation 3: It’s very challenging to design good performance narrow-bandwidth RF bandpass filters with LC.

Proposal 2: The feasibility and the performance for the RF BPF filter before the RF envelope detector should be studied in RAN4.

Some assumptions are needed for the further study of the BPF filter.

Proposal 3: The following assumptions should be aligned for the RF BPF filter study:

Operating frequency, cutoff frequency, performance assumption, etc.

- Summary

This contribution provides our preliminary analysis for RF requirements of A-IoT device. We have the following observations and proposals.

Observation 1: Potential Tx requirements of device 1 or device 2a are shown in Table 1.

Proposal 1: Input signal power range requirement of CW2D Rx is necessary.

Observation 2: Potential R2D Rx requirements of device 1 or device 2a are shown in Table 2.

Observation 3: It’s very challenging to design good performance narrow-bandwidth RF bandpass filters with LC.

Proposal 2: The feasibility and the performance for the RF BPF filter before the RF envelope detector should be studied in RAN4.

Proposal 3: The following assumptions should be aligned for the RF BPF filter study:

Operating frequency, cutoff frequency, performance assumption, etc.

Reference

[1] RP-234058, “Study on solutions for Ambient IoT (Internet of Things) in NR”, RAN#102

[2] ISO/IEC 18000-6C

[3] Radio Frequency Identification Equipment operating in the band 865 MHz to 868 MHz with power levels up to 2 W and in the band 915 MHz to 921 MHz with power levels up to 4 W; Harmonised Standard for access to radio spectrum, ETSI EN 302 208

[4] TS 36.101, User Equipment (UE) radio transmission and reception

[5] R1-2403782, “LS on Ambient-IoT evaluation scenarios and assumptions”, RAN1#116bis