3GPP TSG-RAN WG4 Meeting # 111                                                               R4-2407526

# Fukuoka City, Fukuoka, Japan, 20th – 24th May, 2024

Title: 	Discussion on modification of existing requirements for SBFD operation

Source: 	CATT

Agenda item:	10.12.2.1

Document for:	Discussion

- Background

Summary [2] of RAN4#110bis concluded that further studies are needed for some TX and RX RF requirements. This contribution provides our further analysis of the requirements.

- Impact on existing BS RF Requirement

  - TX RF Requirement

2.1.1 BS output power

The PSD scaling assumption for co-existence simulations is considered from the perspective of coverage. PSD of normal and SBFD slots/symbols are same; the power of the SBFD slot/symbols should be converted according to the ratio of the DL RB number of SBFD to the RB number of normal slot. The assumption is still applicable to the standard.

Proposal 1: The PSD scaling assumption for co-existence simulations is still applicable for the SBFD output power requirement.

2.1.2 TX intermodulation requirement

Traditionally for the transmitter intermodulation requirement is to address the coexistence between the transmitter antenna from one BS and the transmitter antenna from another BS in case the antennas are co-located with assumption that the worst-case coupling loss between them is 30dB. The requirement assumes that the co-located BSs transmit the same power, and the transmitted signals are adjacent to each other in the frequency domain. For SBFD gNB, if co-located BS Tx power is 46 dBm and the coupling loss is 30 dB, then SBFD UL band received power from other co-located BS is 16 dBm because there’s no RF filter rejection for this signal. It will block SBFD Rx path. So the legacy Tx IMD requirement can’t be reused for SBFD capable BS. Whether the coupling loss assumption can be larger than 30dB need further discussion.

Manufacturer can declare coupling loss required to support transmitter intermodulation co-located requirement, and the existing requirements and declared coupling losses are used for testing.

SBFD is full- duplex, so if the Tx intermodulation requirement is applicable in SBFD slots/symbols, the receiver must be active.

Observation 1: For transmitter intermodulation requirement, co-location coupling loss assumption can’t use 30 dB for SBFD capable gNB.

Proposal 2: Transmitter intermodulation co-located requirement is defined based on the CL declaration by the manufacturers, the existing requirements and declared coupling losses are used for the test.

Proposal 3: If the TX intermodulation requirement is applicable in SBFD slots/symbols, the receiver should be active.

  - RX RF Requirement

2.2.1 OTA sensitivity

OTA test environment only consider SI is relatively easy to achieve, comparing the accuracy of test results considering only SI and self-interference, inter-site interference, and inter-sector interference at the same time, the former is better. According to the SI feasibility analysis results of each company in [1], 1.0dB is a more appropriate value.

Proposal 4: 1.0 dB degradation is agreed for OTA sensitivity requirement.

2.2.2 RX intermodulation requirement

There’s one issue that if additional requirement based on a single input signal is needed and if Rx blocking requirement can cover this requirement. For the single input interference scenario, there’s possibility that the IM product of interference signal and Tx sub-band signal falls in UL subband for some UL/DL subband configurations.

If the SBFD configuration is DUD, there could be two IMD interfering scenarios. One is two interfering signals input; the other is only single interfering signal input.

(1) Two interfering signals input

(2) One interfering signal input

Figure 1:  SBFD DUD configuration

As shown in Figure 1, f1, f2 and f4 are interfering signals, f3 is the Tx subband signal received by Rx path. For the two interference signals scenario, the whole IM product for f1 and f2 falls in the UL subband, but only part of the IM product of f2 and f3 fall in the UL subband if the f1 and f3 BW is the same as UL subband. For single interference signal scenario, the IM product of f3 and f4 can be overlapped with the whole UL subband. It’s difficult to say the two interfering signal scenario can cover the one interfering signal scenario if f3 signal level is big. According the SI feasibility analysis provided by companies, usually IBB blocking signal level is assumed for the Tx subband signal level received in Rx path. So the f3 signal level may be bigger than IMD interference signals, in that case one interfering signal scenario may be needed.

If the SBFD configuration is DU or UD, there may be also two IMD interfering scenarios. One is two interfering signals input, the other is only single interfering signal input, as shown in Figure 2, the IM analysis and results are similar with scenario which SBFD configuration is DUD.

(1) Two interfering signals input

(2) One interfering signal input

Figure 2: SBFD DU configuration

Observation 2: For receiver IMD requirement, it’s difficult to say the two interfering signal scenario can cover the single interfering signal scenario for both DUD and DU subband configuration.

For the issue of if single interfering signal scenario is covered by IBB requirement, it depends on the frequency position of the single interfering signal. There’s also a difference between the IBB and single interfering signal that IBB signal is modulated signal but single interfering signal for Rx IMD could be CW. So we have the following observation.

Observation 3: For receiver IMD requirement, single interfering signal scenario may not be equal to IBB scenario.

According to analysis above, IM product of single interfering signal scenario maybe part overlap with that of two interfering signal scenario, whether need to add requirement for single interfering signal scenario require further analysis.

Proposal 5: Whether IMD requirement for single interfering signal scenario is needed FFS.

- Summary

In this contribution, we have the following proposals and observations.

Proposal 1: The PSD scaling assumption for co-existence simulations is still applicable for the SBFD output power requirement.

Observation 1: For transmitter intermodulation requirement, co-location coupling loss assumption can’t use 30 dB for SBFD capable gNB.

Proposal 2: Transmitter intermodulation co-located requirement is defined based on the CL declaration by the manufacturers, the existing requirements and declared coupling losses are used for the test.

Proposal 3: If the TX intermodulation requirement is applicable in SBFD slots/symbols, the receiver should be active.

Proposal 4: 1.0 dB degradation is agreed for OTA sensitivity requirement.

Observation 2: For receiver IMD requirement, it’s difficult to say the two interfering signal scenario can cover the single interfering signal scenario for both DUD and DU subband configuration.

Observation 3: For receiver IMD requirement, single interfering signal scenario may not be equal to IBB scenario.

Proposal 5: Whether IMD requirement for single interfering signal scenario is needed FFS.

- Reference

[1] 3GPP TR 38.858, Study on Evolution of NR Duplex Operation

[2] R4-2405833, “Topic summary for [110bis][313] NR_duplex_evo”, RAN4#110bis[3]