3GPP TSG-RAN WG4 Meeting#111		R4-2407716

Fukuoka City, Fukuoka, Japan, May 20th -24th, 2024

Agenda item:	10.13.2.2

Source: 	Spreadtrum Communications

Title: 	Discussion on co-existence evaluation for Ambient IoT and NR/LTE

Document for:	Discussion

Introduction

In RAN4#110-bis meeting, a WF [1] was approved, in which there were remaining open issues as listed. This contribution continues to be discussed as follow.

- Topic 2-4: Evaluation methodology Issue 2-4-1: Evaluation methodology Agreement: - Use the Monte-Carlo method as baseline for co-existence evaluation, i.e. Section 5.3 in TR38.803 - Depending on the discussion on deployment scenarios, for some cases, calculation for the worst interference link may be enough. - FFS on whether RAN4 needs to perform link level simulation Issue 2-4-2: Performance metric for AIOT Agreement: - For NR system, use 5% throughput loss as performance metric as legacy. - For AIOT system, including reader, device, intermediate UE, further discuss the performance metric: - Option 1: [10%] BLER, [Rx power] - Option 2: SINR degradation - Other options are precluded - Topic 2-5: Evaluation cases Issue 2-5-1: device type Agreement: - Prioritize device 1 and 2a without a frequency shifter for coexistence evaluation. Issue 2-5-2: Evaluation cases for D1T1 for device 1 and 2a between NR and AIOT Agreement: - FFS on evaluation cases for D1T1 for device 1 and 2a between NR and AIOT. Corresponding evaluation cases are listed for information. Note that some duplicated cases are omitted in the table.

Issue 2-5-3: Evaluation cases for D2T2 for device 1 and 2a between NR and AIOT Agreement: - FFS on evaluation cases for D2T2 for device 1 and 2a between NR and AIOT. Corresponding evaluation cases are listed for information. Note that some duplicated cases are omitted in the table. Issue 2-5-4: Evaluation cases for device 2b between AIOT and NR Agreement: - FFS for evaluation cases for device 2b Issue 2-5-5: Interference between AIOT systems Agreement: - RAN4 to first evaluate interference between AIOT and NR. - FFS on interference between AIOT and AIOT - Topic 2-6: Evaluation parameters There are no agreements on detailed parameters in RAN4#110bis meeting. Deatiled parameters for information can be found in summary document R4-2405289 and will be further discussed in next meeting. Issue 2-6-2 and 2-6-3: Layout for D1T1 and D2T2 Agreement: Use RAN1 agreements in RAN1#116bis meeting as baseline (copied as below). - FFS on whether any updates are needed for RAN4 co-existence evaluation. - FFS on other parameters. RAN1 agreements: For D1T1, - InF-DH NLOS model defined in TR38.901 is used for D2R and R2D links as pathloss model in coverage evaluation. For D2T2, - InF-DL and InH-Office model defined in TR38.901is used as pathloss model in coverage evaluation, - NLOS for D2R and R2D links if InF-DL is used - LOS for D2R and R2D links if InH-Office is used The following layout is used for evaluation purpose, - FFS: CW distribution for D1T1-B and D2T2-B

Discussion

Evaluation methodology

- Performance metric for AIoT

In RAN4#110-bis meeting, Monte-Carlo method was used as a baseline of co-existence evaluation, that is, Section 5.3 in TR 38.803 was agreed as evaluation methodology. This method adopts the way by comparing the throughput loss before and after interference. In our understanding, it is reasonable to use BLER as a performance metric. As is known to all, the performance of the A-IoT system is worse and its coverage is much smaller than the legacy NR system. For NR system, use 5% throughput loss as a performance metric for legacy. Therefore, it is reasonable to use 10% throughput loss [BLER] for A-IoT system, including reader, device and intermediate UE.

Proposal 1: For A-IoT system, use 10% throughput loss as a performance metric.

Evaluation cases

- Device type

In the last meeting, an agreement was reached that prioritized device 1 and 2a without a frequency shifter for co-existence evaluation. In our understanding, a clarification about a frequency shifter is large frequency shift or small frequency shift or both is needed. Large frequency shift means that shifting backscattered signal from one frequency (e.g., FDD-DL frequency) to another frequency (e.g., FDD-UL frequency). There can be impact on the out-of-band emission performance of D2R transmission due to the leaked image signal and harmonic signal during the large frequency shifting. Small frequency shift (e.g., tens of kHz [up to a few MHz] for shifting backscattered signal to avoid CW interference, which can generate harmonic signals to interfere with other devices. Whether it is a large frequency shift or a small frequency shift, it will cause some interference within the A-IoT system. In our understanding, to simplify co-existence evaluation, neither large frequency shift nor small frequency shift should be considered.

Proposal 2: For co-existence evaluation, device 1 and 2a with small or large frequency shift capability are deprioritized.

- Evaluation cases for D1T1 for device 1 and 2a between NR and AIoT

Based on the analysis of spectrum usage [2], adopt Table 1 as the proposed scenarios for in-band co-existence evaluation.

Table 1: A-IoT co-existence scenarios with in-band NR deployment for D1T1

| Deployment scenario and topology | spectrum | aggressor | victim |
| --- | --- | --- | --- |
| · Case 1-1: CW is transmitted from inside the topology, transmitted in DL spectrum | R2D: DL CW2D and D2R: DL | CW and/or device | NR DL |
|  |  | NR DL | device and/or reader |
| Self-interference cancelation is needed for reader · Case 1-1: CW is transmitted from inside the topology, transmitted in DL spectrum | R2D: DL CW2D and D2R: DL | CW and/or device | NR DL |
|  |  | NR DL | device and/or reader |
| Self-interference cancellation is needed for reader · Case 1-4: CW is transmitted from outside the topology, transmitted in UL spectrum | R2D: DL CW2D and D2R: UL | CW and/or device | NR UL |
|  |  | NR UL | device and/or reader |
|  |  | reader | NR DL |
|  |  | NR DL | device |

Proposal 3: Adopt Table 1 as the in-band co-existence evaluation scenarios for D1T1.

- Evaluation cases for D2T2 for device 1 and 2a between NR and AIoT

Based on the analysis of spectrum usage [2], adopt Table 2 as the proposed scenarios for in-band co-existence evaluation.

Table 2: A-IoT co-existence scenarios with in-band NR deployment for D2T2

| Deployment scenario and topology | spectrum | aggressor | victim |
| --- | --- | --- | --- |
| Case 2-2: CW is transmitted from inside the topology (i.e., intermediate UE), transmitted in UL spectrum | R2D: UL CW2D and D2R: UL | CW and/or device | NR UL |
|  |  | NR UL | Device and/or reader |
|  |  | reader | NR UL |
|  |  | NR UL | device |
| \ Case 2-2: CW is transmitted from inside the topology (i.e., intermediate UE), transmitted in UL spectrum Self-interference cancelation is needed for reader | R2D: UL CW2D and D2R: UL | CW and/or device | NR UL |
|  |  | NR UL | Device and/or reader |
|  |  | reader | NR UL |
|  |  | NR UL | device |
| Case 2-3: CW is transmitted from outside the topology, transmitted in DL spectrum Self-interference cancelation is needed for reader | R2D: UL CW2D and D2R: DL | CW and/or device | NR DL |
|  |  | NR DL | device and/or reader |

Proposal 4: Adopt Table 2 as the in-band co-existence evaluation scenarios for D2T2.

Evaluation parameters

- Network layout model for D1T1

For D1T1, the indoor network layout model in TR 38.803 and InF-DH defined in TR 38.901are adopted as a starting point.

For D1T1-A, macro gNB are all in outdoors, and all A-IoT reader (gNB)/device are all indoors, NR UE is indoor or outdoor. Thus, the topology includes two BS deployments, namely outdoor (UMa: Table 4)+ indoor(indoor Factory: Table 3), the Figure 1 below shows the overall deployment block diagram. The red box represents the factory building, and its central indoor factory is 100 meters away from the base station (the distance from the restricted area of the base station). This setting refers to SBFD simulation, and defines that the minimum distance between the indoor office center and UMA is set as 100 meters, which indicates that it is the closest to the base station and has the worst interference.

.

Figure 1: overall deployment block diagram.

Proposal 5: For co-existence evaluation, the minimum 2D distance between macro BS and indoor factory centre is set as 100m.

Table 3: Single operator layout for indoor factory

| Parameters |  | Values | Remark |
| --- | --- | --- | --- |
| Scenario |  | InF-DH |  |
| Network layout |  | 60m x 120m, 18BSs |  |
| Inter-site distance |  | 20m |  |
| BS antenna height |  | 8 m | ceiling |
| A-IoT device location | Outdoor/indoor | Indoor |  |
|  | LOS/NLOS | LOS and NLOS |  |
|  | A-IoT device antenna height | 1.5 m |  |
| A-IoT device distribution (horizontal) |  | uniform dropping for indoor with minimum 2D distance of 1 m |  |
| Minimum BS – A-IoT device distance (2D) |  | 0 m |  |
| Channel model |  | Indoor Factory |  |
| BS deployment |  | 18 BSs on a square lattice with spacing D, located D/2 from the walls. - for the small hall (L=120m x W=60m): D=20m |  |

Table 4: Macro layout for outdoor

| Parameters |  | Values | Remark |
| --- | --- | --- | --- |
| Network layout |  | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |  |
| Inter-site distance |  | 200m (baseline) 300m (optional) |  |
| BS antenna height |  | 25 m |  |
| NR UE location | Outdoor/indoor | Outdoor |  |
|  | LOS/NLOS | LOS/NLOS |  |
|  | NR UE antenna height | 1.5m |  |
| NR UE distribution (horizontal) |  | Uniform dropping with minimum 2D distance of 1 m |  |
| Minimum BS – NR UE distance (2D) |  | 35 m |  |
| Channel model |  | UMa |  |
| BS deployment |  |  |  |

- Propagation model

Path loss

RAN1 has reached an agreement about path loss model. For D1T1, InF-DH is adopted.

Agreement For D1T1, - InF-DH NLOS model defined in TR38.901 is used for D2R and R2D links as pathloss model in coverage evaluation. For D2T2, - InF-DL and InH-Office model defined in TR38.901is used as pathloss model in coverage evaluation, - NLOS for D2R and R2D links if InF-DL is used - LOS for D2R and R2D links if InH-Office is used

More details can be shown below.

Table 5: Path loss model for D1T1-A/B (5G NR BS is outdoor and NR UE is indoor)

|  | D1T1-A1 | D1T1-A2 | D1T1-B |
| --- | --- | --- | --- |
| R2D | InF-DH | InF-DH | InF-DH |
| CW2D | InF-DH | InF-DH |  |
| NR UE to D | InF-DH | InF-DH | InF-DH |
| D2R | InF-DH | InF-DH | InF-DH |
| D to NR UE | InF-DH | InF-DH | InF-DH |
| NR UE to Reader | InF-DH | InF-DH | InF-DH |
| Reader to NR UE | InF-DH | InF-DH | InF-DH |
| D to gNB | Including three parts: - O-to-I penetration loss model: Low-loss model - Outdoor part(UMa) - Indoor part(Indoor loss PLin, D is the building to device) | Including three parts: - O-to-I penetration loss model: Low-loss model - Outdoor part(UMa) - Indoor part(Indoor loss PLin, D is the building to device) | Including three parts: - O-to-I penetration loss model: Low-loss model - Outdoor part(UMa) - Indoor part(Indoor loss PLin, D is the building to device) |
| gNB to D | Including three parts: - O-to-I penetration loss model: Low-loss model - Outdoor part(UMa) - Indoor part(Indoor loss PLin, D is the building to device) | Including three parts: - O-to-I penetration loss model: Low-loss model - Outdoor part(UMa) - Indoor part(Indoor loss PLin, D is the building to device) | Including three parts: - O-to-I penetration loss model: Low-loss model - Outdoor part(UMa) - Indoor part(Indoor loss PLin, D is the building to device) |

- Antenna pattern

BS antenna configuration

2 or 4 antenna elements, with (M,N,P,Mg,Ng)=(1,1,2,1,1) or (2,1,2,1,1)

Intermediate UE/A-IoT device antenna configuration

1 element (vertically polarized), Isotropic antenna gain pattern

- General parameters

Based on the RAN1’s A-IoT Link budget and some proposals about CW. We need to modify some parameters for summary [2]. About carrier frequency, 900MHz is mandatory, 2GHz is optional. In addition, both unmodulated single-tone and multiple unmodulated single-tone CW waveforms for backscattering are considered. Single-tone CW waveform is low complexity and may be impacted by deep fading. Multi-tone is beneficial to combat the frequency selective fading. Single tone is more suitable from the point of view of co-existence simulation.

Table 6: general parameters for co-existence evaluation

| General Parameter | D1T1&D2T2 |
| --- | --- |
| Carrier frequency | 900 MHz or 2GHz |
| Channel BW for NR | 10MHz or 20MHz with 15KHz SCS |
| Channel BW for AIOT | R2D: 180kHz with 15KHz SCS D2R: 15KHz or 180KHz |
| Waveform | R2D: OFDM based waveform D2R: single-tone |
| The number of active A-IoT (DL) | 1 |
| The number of active A-IoT (UL) | 1 |
| A-IoT DL power control | No |
| A-IoT UL power control | No |
| Sample frequency for DL | 1.92MHz |
| Sample frequency for UL | 1.92MHz |
| Traffic model | Full buffer |

- NR macro BS parameters

Table 7: NR macro BS parameters for co-existence evaluation

| Parameters | Topo1/Topo2 |
| --- | --- |
| BS antenna gain (dBi) | 8 dBi |
| BS max TX power in dBm | 46 dBm |
| BS noise figure | 5 dB |

- NR UE parameters

Table 8: NR UE parameters for co-existence evaluation

| Parameters | Topo1/Topo2 |
| --- | --- |
| tx power in dBm | -40 to 23dBm or 26dBm |
| Antenna gain | 0 dBi |
| noise figure | 9 dB |
| ACLR | 30 dBc |
| Antenna configuration | Omi direction antenna |

- A-IoT BS parameters

Table 9: A-IoT BS parameters for co-existence evaluation

| A-IoT micro BS parameters | Recommended value |
| --- | --- |
| A-IoT micro-BS total Tx power | 33dBm |
| A-IoT micro-BS receiver Noise Figure（dB） | 5 |
| A-IoT micro-BS antenna gain including feeder loss (dBi) | 5 |
| A-IoT micro-BS ACLR (dB) | ACLR: 45 |

- Intermediate UE parameters

Table 10: A-IoT BS parameters for co-existence evaluation

| intermediate UE parameters | Recommended value |
| --- | --- |
| intermediate UE total Tx power（dBm） | 23dBm baseline 26dBm |
| gain of antenna intermediate UE including feeder loss (dBi) | 0 |
| intermediate UE receiver Noise Figure（dB） | 7 |

- CW parameters

| intermediate UE parameters | D1T1 | D2T2 |
| --- | --- | --- |
| Max Tx power（dBm） | Same as A-IoT BS | Same as intermediate UE |

Table 11: CW parameters for co-existence evaluation

- A-IoT device parameters

Table 12: A-IoT device parameters for co-existence evaluation

| A-IoT device parameters | Device 1 | Device 2a | Device 2b |
| --- | --- | --- | --- |
| A-IoT device Tx power (dBm) | － | － |  |
| A-IoT device effective antenna gain(dBi) | 0 | 0 | 0 |
| A-IoT device reflection （backscatter）loss (dB) Note: due to, e.g., impedance mismatch | -6 dB and 0 dB for OOK / FSK and BPSK | -6 dB and 0 dB for OOK / FSK and BPSK | N/A |
| A-IoT device power gain of reflection amplifier (dB) | N/A | [10 / 20] | N/A |
| A-IoT Device receiver sensitivity (dBm) | -36 | -46 | [FFS] |
| A-IoT device noise figure (dB) | 24 | 15 | 9 |
| BB filter type | 3/5 Butterworth filter | 3/5 Butterworth filter | 3/5 Butterworth filter |
| Cutoff frequency | Half transmission bandwidth | Half transmission bandwidth | Half transmission bandwidth |
| On-object gain penalties (dB) [3] | 0.9~10.4 | 0.9~10.4 | 0.9~10.4 |

Initial simulations results

- UL A-IoT vs UL NR UE

In this part, we provide the initial simulation results about UL co-existence according to the information provided from the last meeting. We evaluated the A-IoT UL (NR UE UL aggressor vs A-IoT UL victim) and NR UE UL (A-IoT UL aggressor vs NR UE UL victim performance in D1T1-A2, NR UE is indoor, macro gNB for NR is outdoor for in-band deployment. Simulation parameters of UL co-existence is shown in Table 14.

Table 13: simulation parameters of UL co-existence

| 5G NR UE parameters |  |
| --- | --- |
| Parameter | Value |
| SCS for NR | 15kHz |
| Tx power (dBm) | 23 dBm |
| Tx antenna gain | 0 dBi |
| Transmission bandwidth of 5G NR | 10MHz (50RB) |
| Height of NR UE (m) | 1.5 m |
| NR UE to device path loss model | NLOS, InF-DH |
| Sample rate | 15.36MHz |
| Channel model | TDL-A |
| A-IoT BS parameters |  |
| Parameter | Value |
| Tx power of CW | 33 dBm |
| Tx antenna gain | 8 dBi |
| Height of gNB | 8 m |
| Transmission bandwidth of A-IoT | 180 KHz (1RB) |
| The distance between BS and Device | ＜15 m |
| A-IoT BS to Device path loss model | NLOS, InF-DH |
| Coding of A-IoT | Manchester (16bits) |
| Modulation | OOK |
| A-IoT BS NF | 5dB |
| A-IoT Device parameters |  |
| Parameter | Value |
| Height of Device | 1.5 m |
| Return loss | 6dB |
| Date rate | 5kpbs |
| Receiver antenna gain | 0 dBi |
| The distance between A-IoT BS and device | 10m |

In the process of simulation, we gradually moved the NR UE away from the A-IoT BS, increasing the distance by 50 meters each time, from 5 meters until the distance between the NR UE and the BS reached 35 meters. We get the results for different distances (NR UE to A-IoT BS) with different BLER. The results are shown in Figure 2 ,Figure 3 and Figure 4.

- A-IoT device UL performance (NR UE UL as aggressor)

Figure 2: sample rate=240 kHz                  Figure 3: sample rate=960 kHz                    Figure 4: sample rate=1.92 MHz

According to the above results, we can see that as the distance between NR UE and A-IoT BS is closer, the BLER is higher. When the distance between NR UE and A-IoT BS is 5 m, in order to meet 10% BLER, two guard RBs are needed. In addition, we can also see that the demodulation performance of A-IoT BS is different with different sample rates. The demodulation performance is gradually improved with the increase of the sample rate. However, it is observed that there is no difference between the sample rates is 960 kHz and 1.92 MHz. As we all know, high sample rate means high power consumption and complexity. Therefore, 1.92MHz of sample rate for UL co-existence is considered.

Observation 1: The sample rate of A-IoT BS has an influence on demodulation performance. 1.92MHz of sample rate for UL co-existence could be considered.

Observation 2: Guard RB is needed for UL in-band co-existence with NR when NR UE UL as aggressor vs A-IoT device victim.

- NR UE UL performance (A-IoT device as aggressor)

Similarly, we gradually moved the NR UE away from BS, increasing the distance by 50 meters each time, from 100 meters until the distance between the NR UE and BS reached 45 meters. We get the results for different distances (NR UE to A-IoT BS) with different BLER. At the same time, we have simulated different distance between wall and NR macro BS. The results are shown in Figure 5 and Figure 6.

Figure 5: the distance between wall and NR macro BS is 35m         Figure 6: the distance between wall and NR macro BS is 70m

According to the above results, we can see that BELR is above 90%, which means that NR UE is indoor access to NR macro BS is outdoor hardly cannot work. It is mainly due to the noise power, which inundates the signal and leads to the inability of the NR UE to work. The influence of A-IoT system on NR UE is almost negligible. There is no improvement even if guard RB is increased.

Observation 3: For in-band deployment, NR UE hardly cannot work when the NR UE is indoor and NR macro BS is outdoor when A-IoT device aggressor vs NR UE UL victim.

- DL A-IoT vs DL NR UE

In this part, we provide the initial simulation results about DL co-existence according to the information provided from the last meeting. We evaluated the A-IoT DL (NR UE DL aggressor vs A-IoT DL victim) and NR UE DL (A-IoT UL aggressor vs NR UE UL victim) performance in D1T1-A2, NR UE is indoor and macro gNB for NR is outdoor for in-band deployment.

Table 14: simulation parameters of DL co-existence

| 5G NR macro BS parameters |  |
| --- | --- |
| Parameter | Value |
| SCS for NR | 15kHz |
| Tx power (dBm) | 46 |
| Tx antenna gain | 8 dBi |
| Transmission bandwidth of 5G NR | 10MHz (50RB) |
| Height of gNB(m) | 25m |
| The distance between macro BS and Wall | 100 m |
| The distance between wall and device | ≤ 120 m |
| NR BS to Wall path loss model | NLSO, UMa |
| Wall to Device path loss model | NLOS, InF-DH |
| A-IoT BS parameters |  |
| Parameter | Value |
| Tx power of R2D | 33 dBm |
| Tx antenna gain | 8 dBi |
| Height of gNB | 8 m |
| Transmission bandwidth of A-IoT | 180 KHz (1RB) |
| The distance between BS and Device | ＜15 m |
| A-IoT BS to Device path loss model | NLOS, InF-DH |
| Coding of A-IoT | Manchester (16bits) |
| Modulation | OOK-1 |
| A-IoT Device parameters |  |
| Parameter | Value |
| Height of Device | 1.5 m |
| Sample rate | 1.92 MHz |
| Butterworth filter order | 3/5 |

- NR UE DL aggressor vs A-IoT DL victim

In the process of simulation, we gradually moved the device away from the wall, increasing the distance from 0 meters until the distance between the NR UE and the BS reached 100 meters. At the same time, we have set 3 types of distance between A-IoT BS and device, respectively 1meters, 8 meters and 15 meters. Simulation results are shown in the figures as follows. In Figure 7 and Figure 8, the result shows BLER is 0% that there is no need Guard RB for this scenario whatever Butterworth LPF with 3rd or 5th order.

Figure 7: BLER with 3th order Butterworth LPF                                 Figure 8: BLER with 5th order Butterworth LPF

In addition, we have simulated different distance between indoor factory centre and NR macro BS, respectively 35 meters and 70 meters. Simulation results are shown in the figures as follows. In Figure 2, the result shows BLER is 0% that there is no difference between 35 meters and 70 meters.

Figure 9: BLER with Tx Wall-Distance=70m                                                 Figure 10: BLER with Tx Wall-Distance=35m

Observation 4: For in-band deployment, guard RB is not required for the case that NR macro BS is outdoor and NR UE is indoor when NR UE DL aggressor vs A-IoT DL victim.

- A-IoT DL aggressor vs NR UE DL victim

In the process of simulation, we gradually moved NR UE away from the wall, increasing the distance from 0 meters until the distance between the Wall and NR UE reached 100 meters. At the same time, we have set 3 types of distance between A-IoT BS and device, respectively 1meters, 8 meters and 15 meters. Simulation results are shown in the figures as follows. In Figure 11 and Figure 12, the result shows BER is above 10% when the distance is above 10m between wall and NR UE. It seems that this scenario, that NR UE is indoor access to NR macro BS is outdoor, maybe is not suitable due to the signal for NR UE being too weak. Even though that there is an improvement to set guard RB, but the improvement is not enough for the NR UE of indoor access to NR macro BS of outdoor.

Figure 11: BER with guard RB=0                                                    Figure 12: BER with guard RB=1

Figure 13: BER with the distance between Wall to NR macro BS is 35m.

Observation 5: For in-band deployment, NR UE hardly cannot work when the NR UE is indoor and NR macro BS is outdoor when A-IoT DL aggressor vs NR UE DL victim.

Multiple A-IoT operators co-existence

To improve A-IoT system capacity and spectrum efficiency, FDMA should be studied for R2D transmission, especially for multiple A-IoT operators’ coexistence scenario. FDMA divides different frequency resource units in the frequency domain which are used for data transmission by different devices. Ideally, the frequency resource units should not overlap with each other, and a guard band may be needed between adjacent frequency resource units.

However, due to relatively large RF filter (Matching Network) bandwidth in front of RF envelop detection, e.g., 10MHz, reader may have to reserve at least 10MHz system bandwidth for A-IoT R2D transmission to avoid any interference falling into the RF envelop detector, i.e., Bsys,R2D = 10MHz. Specifically, within the wide system bandwidth, only one R2D signal can be transmitted at one instance; otherwise, if multiple R2D signals are simultaneously transmitted by FDMA within the wide system bandwidth, all others R2D signals will be absorbed by the RF envelop detector, which will greatly affect the performance. Considering that the spectrum allocated for A-IoT systems may be limited, if the total spectrum is less than twice of system bandwidth Bsys,R2D, FDMA may not be feasible without any improvements.

To increase system deployment flexibility and save the spectrum allocated for A-IoT system, FDMA divides different frequency resource units for different operators in the frequency domain which are used for different devices deployed in different operators. That is to say, the frequency resource can divide into two R2D signal channel, as shown in Figure 14. Two R2D signal channels are allocated at the edge of the A-IoT system bandwidth and for different operators, at the same time defining the system bandwidth Gap in the overlapped frequency resource, guaranteeing that there is no signals transmitted in the system bandwidth Gap, FDMA can still be achieved without introducing any interference between these two R2D channels.

Comparing with only one R2D signal channel within one system bandwidth, the total bandwidth to achieve FDMA is (2×Bsys,R2D – Gap), which obviously saves the spectrum resources and improve spectrum efficiency. Therefore, FDMA should be studied for R2D transmission for multiple A-IoT operators’ coexistence scenario.

Figure 14: one possible FDM candidate for multiple A-IoT operators scenario

Proposal 6: Multiple A-IoT operators co-existence scenario should be investigated in Rel-19 A-IoT.

Conclusions

In this paper, we provide our views on co-existence between A-IoT and NR/UE, we have made the following observations and proposals.

Proposal 1: For A-IoT system, use 10% throughput loss as a performance metric.

Proposal 2: For co-existence evaluation, device 1 and 2a with small or large frequency shift capability are deprioritized.

Proposal 3: Adopt Table 1 as the in-band co-existence evaluation scenarios for D1T1.

Proposal 4: Adopt Table 2 as the in-band co-existence evaluation scenarios for D2T2.

Proposal 5: For co-existence evaluation, the minimum 2D distance between macro BS and indoor factory centre is set as 100m.

Observation 1: The sample rate of A-IoT BS has an influence on demodulation performance. 1.92MHz of sample rate for UL co-existence could be considered.

Observation 2: Guard RB is needed for UL in-band co-existence with NR when NR UE UL as aggressor vs A-IoT device victim.

Observation 3: For in-band deployment, NR UE hardly cannot work when the NR UE is indoor and NR macro BS is outdoor when A-IoT device aggressor vs NR UE UL victim.

Observation 4: For in-band deployment, guard RB is not required for the case that NR macro BS is outdoor and NR UE is indoor when NR UE DL aggressor vs A-IoT DL victim.

Observation 5: For in-band deployment, NR UE hardly cannot work when the NR UE is indoor and NR macro BS is outdoor when A-IoT DL aggressor vs NR UE DL victim.

Proposal 6: Multiple A-IoT operators co-existence scenario should be investigated in Rel-19 A-IoT.

Reference

- RP-234058, New SID: Study on solutions for Ambient IoT (Internet of Things)in NR, Huawei, INC.TSG-RAN 102

- R4-2405289, Topic summary for [110bis][136] FS_Ambient IoT solution

- F. Amato, C. W. Peterson, B. P. Degnan and G. D. Durgin, "A 45 μW bias power, 34 dB gain reflection amplifier exploiting the tunneling effect for RFID applications," 2015 IEEE International Conference on RFID (RFID), San Diego, CA, USA, 2015, pp. 137-144, doi: 10.1109/RFID.2015.7113084.

- Joshua David Griffin, A Dissertation Presented to The Academic Faculty. Georgia Institute of Technology May 2009