3GPP TSG-RAN WG4 Meeting #111	R4-2408604

Fukuoka, Japan, May 20-24, 2024

Source: 	Ericsson

Title:  	Testability and interoperability issues for beam management

Agenda Item:	10.11.2

Document for:	Discussion

# Introduction

In last meeting 3GPP TSG RAN WG4 Meeting #110bis, RAN4 discussed RRM requirements for AI/ML based beam management and the discussion summary is listed in [1]. Following the discussions, we herein in this contribution present our views and proposals on the remaining open issues.

# Discussion

## Sub-topic 2-1 Metrics/KPIs for Beam management

Issue 2-1: Metrics/KPIs for Beam Management requirements/tests

Agreement: Companies are invited to provide inputs/proposals to refine the definition of RSRP accuracy Hold on the discussions for concrete test metrics until RAN1 had conclusions on the schemes.

During the SI phase, it was agreed in RAN4 that the existing requirements and tests can be used as a “baseline” for characterizing AI performance, different metrics/KPIs have been discussed and captured in the TR 38.843:

For metrics for beam management requirements/tests, the following test metrics are identified and could be considered - Option 1: RSRP accuracy - Option 2: Beam prediction accuracy - Top-1 (%) : the percentage of "the Top-1 strongest beam is Top-1 predicted beam" - Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams" - Top-1/K (%) : the percentage of "the Top-1 predicted beam is one of the Top-K strongest beams" - Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB, - Related measurement accuracy can be considered to determine x - Option 4: combinations of above options RAN4 should continue to discuss what metrics are more appropriate and how they impact defining the requirements and testing. There is also a proposal to update Option 3 to also include +x dB.

In the last meeting, moderator suggested to wait for RAN1’s agreement on AI/ML model output as prerequisite to study test metrics. Then, we copy the agreement in RAN1 #116 meeting as follows:

Agreement For UE-sided model, at least for BM-Case1, for content in the report of inference results, support - Opt 1: Beam information on predicted Top K beam(s) among a set of beams - Opt 2: Beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams - At least K=1 and more, FFS on max value

To our understanding, RAN4 test metrics shall support both options.

Proposal 1: RAN4 shall deal with the below two kind of test metrics with separated definitions, and beam information is to be interpreted by RAN1.

- Option 1: Beam information on predicted Top K beam(s) among a set of beams.

- Option 2: Beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams.

Furthermore, moderator suggested refining the definition of RSRP accuracy in the following RAN4 meetings. Since the value of K has not been decided yet, we present a variety of definitions for RSRP accuracy based on two hypotheses: The first is K equal to 1, while the second is K being greater than 1.

Firstly, for K=1, we listed 4 potential definitions of the RSRP accuracy.

- Option1: L1-RSRP accuracy is the difference between the ideal L1-RSRP of the predicted Top-1 beam and the ideal L1-RSRP of the Top-1 genie-aided beam.

This is inspired by the beam prediction accuracy related KPIs defined in Clause 6.3.1 in TR 38.843 (see below). Specifically, it is the ideal RSRP offset between the predicted beam and the genie beam, it doesn’t care if it is the RSRP from the identical beam or different beams. In this case, the RSRP values are not subject to any errors since they are both ideal values. Furthermore, the definition is similar to the prediction accuracy on beam ID for beam information prediction.

Beam prediction accuracy related KPIs, including: - Top-1 genie-aided Tx beam considers the following definitions: - Option A (baseline), the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx and Rx beams - Option B (optional), the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx beams with specific Rx beam(s) - Specific Rx beam(s) are to be reported. Note: specific Rx beams are a subset of all Rx beams. - Top-1 genie-aided Tx-Rx beam pair considers the following definitions: - Option A: The Tx-Rx beam pair that results in the largest L1-RSRP over all Tx and Rx beams - Other options not precluded and can be reported - Average L1-RSRP difference of Top-1 predicted beam: - The difference between the ideal L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of the Top-1 genie-aided beam

- Option2: L1-RSRP accuracy is the difference between the predicted L1-RSRP of the predicted Top-1 beam and the measured L1-RSRP of the same beam, where the measured L1-RSRP are obtained in advance in the same test environment.

At the least one of companies brought up the view in the last meeting. The advantage of this proposal is easy to pass the test under certain conditions, where both prediction and measurement can be implemented. Specifically, it takes measurement accuracy into account and treats it as benchmark, since measurement accuracy is inherited in the prediction procedure including training and inference.

- Option3: L1-RSRP accuracy is the difference between the predicted L1-RSRP of the predicted Top-1 beam and the ideal L1-RSRP of the same beam.

At the least one of companies brought up the view in the last meeting. Specifically, it treats the ideal L1-RSRP as the benchmark, while the predicted L1-RSRP may include some measurement errors due to the measurement inaccuracy.

- Option4: L1-RSRP accuracy is the difference between the measured L1-RSRP of the predicted Top-1 beam and the L1-RSRP of Top-1 genie-aided beam.

This idea aims to check the measurement accuracy and the ideal RSRP offset between the predicted beam and the genie beam.

Proposal 2: Regarding Top-1 predicted beam, RSRP accuracy could be determined by any of the following:

- Option1: RSRP accuracy is the difference between the ideal L1-RSRP of the predicted Top-1 beam and the ideal L1-RSRP of the Top-1 genie-aided beam.

- Option2: RSRP accuracy is the difference between the predicted Top-1 L1-RSRP and the measured L1-RSRP of the same beam.

- Option3: RSRP accuracy is the difference between the predicted L1-RSRP of the predicted Top-1 beam and the ideal L1-RSRP of the same beam.

- Option4: RSRP accuracy is the difference between the measured L1-RSRP of the predicted Top-1 beam and the L1-RSRP of the Top-1 genie beam.

Among the options, we intend towards Option 3.

Secondly, for K>1, we listed 2 potential definitions for the RSRP accuracy for the case of Top-K predicted beams could be one of the following:

- Option1: RSRP accuracy is a set of values including the RSRP accuracy of each beam in the Top-K predicted beams, where the definition of RSRP accuracy for each beam can reuse the options defined for RSRP accuracy of the Top-1 predicted beam.

- Option2: RSRP accuracy is a range of RSRP values (such as a  upper bound and lower bound) for all of the Top-K beams. In a similar way as the options of Top-1 predicted beam, the range of RSRP accuracy may be categorized as follows:

  - Option 2.1: Difference between average/maximum/minimum value of the ideal L1-RSRPs of the predicted Top-K beams and average/maximum/minimum value of the ideal L1-RSRP of the Top-K genie-aided beams.

  - Option 2.2: Difference between average/maximum/minimum value of the predicted L1-RSRP of the predicted Top-K beams and average/maximum/minimum value of the measured L1-RSRP of the same beam.

  - Option 2.3: Difference between average/maximum/minimum value of the predicted L1-RSRP of the predicted Top-K beams and average/maximum/minimum value of the ideal L1-RSRP of the same beam.

  - Option 2.4: Difference between average/maximum/minimum value of the predicted L1-RSRP of the predicted Top-K beams and average/maximum/minimum value of the ideal L1-RSRP of the same beam.

In the applicability perspective, the determination of multiple beams is insensitive to RSRP of each beam in some use cases and ensuring a suitable RSRP range is suffice for the prediction purpose of AI/ML models.  However, we intend towards a precise approach to verify each beam from conformance test perspective.

Proposal 3: Regarding Top-K predicted beams, RSRP accuracy may be at least one of the following options:

- Option1: RSRP accuracy is a set of values includes the RSRP accuracy of each beam, and the accuracy of each beam can reuse the definition for RSRP accuracy in K=1 case, then the test is passed if all of the predicted Top-K beams reached the prediction accuracy.

- Option2: RSRP accuracy is a RSRP range given the upper bound and lower bound, then the test is passed if all of the predicted Top-K beams are within the range.

Between the options, we intend towards Option 1.

In above paragraphs, the RSRP accuracy is defined as a difference between predicted beam’s RSRP and a benchmark, wherein the benchmark hasn’t been decided, as a result, the RSRP accuracy herein is referred to as the absolute RSRP accuracy, which is consistent with the definition of legacy requirements on the absolute RSRP accuracy. In addition, the relative RSRP accuracy shall be considered as well. The legacy relative RSRP accuracy definition in Clause ﻿10.1.20.1.2 in TS38.133 is copied as follows:

﻿The relative accuracy of SSB based L1-RSRP is defined as the L1-RSRP measured from one SSB compared to the largest measured value of L1-RSRP among all SSBs of the serving cell.

Proposal 4:  Above RSRP accuracy under discussion is the absolute RSRP accuracy. After the absolute RSRP accuracy is established, relative RSRP accuracy, as described in legacy, shall be determined.

Recap an agreement in RAN1 #116bis meeting as follows.

Agreement For report content of inference results for UE-sided model for BM-Case 1, for the RSRP of predicted Top K beam(s) in the report of inference results, when applicable, further study the following options: - Option A: Predicted RSRP. - Option B: Predicted RSRP, if the beam is not configured for corresponding measurement, and measured L1-RSRP if the beam is configured for corresponding measurement. - Where the predicted RSRP is based on AI/ML output. - Note: Support both Option A and Option B is not precluded. Working Assumption For report content of inference results for UE-sided model for BM-Case 2, the RSRP of predicted beam(s) in the report of inference results, is the predicted RSRP, where the predicted RSRP is based on AI/ML output.

The interpretation is that if a beam is configured for measurement in Set B, then the AI/ML model may produce the measured RSRP or predicted RSRP. Such that we think it might influence the above discussions and proposals.

## Sub-topic 2-2 Measurement accuracy

Issue 2-2: Measurement accuracy

Issue 2-2: Measurement accuracy - Proposals - Option 1: RAN4 should study the impact of measurement accuracy on prediction accuracy - RAN4 should discuss a study framework - Option 2: measured L1-RSRP should be used in the tests, an error model is needed when deriving the requirements - Option 3: requirements should be derived based on ideal measurement results - Option 4: Others

Firstly, the definition and source of measurement error has yet to reach a consensus. In last meeting, different views were presented on the definition of measurement error, which are categorized as follows:

- Alt1: Measurement error is the difference between the measured RSRP of the Set A/B and the ideal RSRP of the Set A/B in both training and inference phase. Therefore, the measurement error occurs at both the input of the training and inference phase of AI/ML model.

- Alt2: Measurement error is the difference between the measured RSRP of the Set A/B and the ideal RSRP of the Set A/B in inference phase. Herein the measurement error occurs at the input of inference phase of AI/ML model.

- Alt3: Measurement error is the difference between the measured RSRP of the Set B and the ideal RSRP of the Set B after prediction. Herein the measurement error occurs at the output of AI/ML model inference phase.

Alt 1 is consistent with our interpretation of the TR38.843 performance metric as quoted below. In our understanding, the measurement error happens at the input of the training and the inference phase of AI/ML model, is the prerequisite of the validity of AI/ML model.

Performance with measurement error For BM-Case1 DL Tx beam prediction (unless otherwise stated), when Set B is a subset (1/4 unless otherwise stated) of Set A, without differentiating BB errors and RF errors modelled as truncated Gaussian distribution (unless otherwise stated), ...... Note that: - In the above results, measurement errors are considered in both training (input data and label) and inference phase (except the ground-truth) unless otherwise stated. - Beams could be measured regardless of their SNR. - Measured in a single-time instance (within a channel-coherence time interval). - No quantization for the L1-RSRP measurements. - No constraint on UCI payload overhead for full report of the L1-RSRP measurements of Set B for NW-side models are assumed.

Proposal 5: Measurement error is the difference between the measured RSRP of the Set A/B beam and the ideal RSRP of the Set A/B beam in both training and inference phase. In other words, measurement error occurs at both the input of training and inference phase of AI/ML model.

At least one company in the last meeting proposed that the prediction accuracy maybe not be worse than the current accuracy in case that L1-RSRP is one of AI/ML model outputs. We understand there are various views since the RSRP metric isn’t decided and the content in TR38.843 only covers the case of that measurement error impacting the beam ID by prediction. Therefore, there isn’t comprehensive analysis on the issue. Even though, it is always true that the predicted RSRP is worse than the measured RSRP, which also is proved by the simulation which we have presented in last meeting as copied as follows.

This clause takes spatial beam prediction as reference. The measurement error impact to prediction is mainly manifested by introducing variants on training data to the AI/ML model, where training data are based on real measurements in field. The measurement accuracy is defined in Clause 10.1.20 of TS 38.133 for L1-RSRP accuracy requirements for FR2. Figure 2 shows the performance comparison of non-AI and AI method with and without measurement error. Where, baseline 1 and baseline 2 are defined as follows: • Baseline 1: Select the best beam within Set A of beams based on the measurement of all RS resources or all possible beams of beam Set A (exhaustive beam sweeping) • Baseline 2: Select the best beam within Set B of beams based on the measurement of all RS resources or all possible beams of beam Set B For the results with measurement error, the beam is selected based on the measured RSRP value including the error, however the real RSRP of the selected beam is used for calculating the RSRP difference metric. In both AI and non-AI scenario, Set B consists of 8 CSI-RS beams out of all 32 Tx beams (1/4 of all Tx beams), and set A contains of all 32 Tx beams. Also, for this evaluation we only perform beam selection algorithm among the Tx beam and it is assumed that the Rx beam with highest RSRP is used in all cases. Details of the parameter settings can be found in R1-2306930. Figure 2 Performance comparison of non-AI baselines and AI method with and without measurement error The results show that the measurement error impact on RSRP prediction accuracy of AI/ML model (the difference between the black solid curve and the green solid curve is about 7dB at 90%) is considerably larger than the legacy method 1, i.e. the exhaustive search of all beams in set A (the difference between the red dashed curve and the y-axis is about 3dB at 90%), or the legacy method 2, i.e. selecting the best beam among all measured beams in set B (the difference between the red solid curve and the blue solid curve is about 1 dB at 90%).

Proposal 6: Measurement error impact to prediction shall be verified, since the predicted RSRP is always worse than the measured RSRP if measurement error is introduced in training/inference phase of AI/ML model.

Proposal 7: If tightening measurement error under current side condition is difficult from UE implementation perspective, RAN4 can add another side condition for tightened measurement error for enabling AI-ML model.

Proposal 8: Measurement error impact to prediction shall be verified (e.g. by simulation) to guarantee an acceptable prediction performance.  Furthermore, measurement error impact to the predicted beam ID and the predicted RSRP should be studied separately.

At the least one company in the last meeting proposed that the measurement error is 6.5dB for FR2, in which RX beam gain error plays a dominating factor and difficult to tighten this part. However, Rx beam gain error is relevant to another issue on Rx beam which is captured by TR38.843 with below content.

For the purpose of DL Tx beam prediction evaluations, consider the following options for Rx beam as AI/ML model input for training and/or inference if applicable: - Option 1: Measurements of the "best" Rx beam with exhaustive beam sweeping for each model input sample. - Companies expected to report how to select the "best" Rx beam(s). - Option 2: Measurements of specific Rx beam(s). - Companies expected to report how to select specific Rx beam(s). - Option 3: Measurements of random Rx beam(s) per model input sample. - Option 4: Measurements of quasi-optimal Rx beam (i.e., not all the measurements as inputs of AI/ML are from the "best" Rx beam) with less measurement/RS overhead compared to exhaustive Rx beam sweeping. - Identify the quasi-optimal Rx beams to be utilized for measuring Set B/Set C based on the previous measurements. Companies can report the time information and beam type (e.g., whether the same Tx beam(s) in Set B) of the reference signal to use. Companies expected to report the measurement/RS overhead together with the beam prediction accuracy, as well as, how to find the quasi-optimal Rx beam with "previous measurement". - Other options are not precluded and can be reported by companies.

For example, if a particular Rx beam is chosen when applying AI/ML model, then RX beam gain error doesn’t contribute error to AI/ML model.

Proposal 9: In measurement error, RX beam gain impact may refer to the outcome of the RAN1 study on selecting Rx beam for DL Tx beam prediction evaluations. If a particular beam is chosen when applying AI/ML model, then Rx beam gain error doesn’t contribute error to AI/ML model.

## Sub-topic 2-3 Consistency between set A and set B

Issue 2-3: set A and set B consistency

- Issues raised: - Option 1: Consistency(spatial) between set A and set B should be ensured in the test(and in the real deployment - Option 2: Consistency between set A and set B should be taken as a side condition for the requirements - FFS how to define the level of consistency - Option 3: RAN4 should consider the size and composition of set B and set A for the prediction accuracy - Option 4: other issues/proposals related to consistency

Not considering AI/ML model performance in-field, in the scope of conformance test, we understand consistency between set A and set B shall comprise: Same NW antenna/beam configurations for set A and set B, and its configurations don’t change during training and inference; same channel (model or condition) for set A and set B, and its configurations don’t change during training and inference. However, if channel is non-static (time-varying), can it be treated to as consistency?

Proposal 10: From RAN4 test perspective, consistency between set A and set B is reflected by below aspects:

- Same NW antenna/beam configurations for set A and set B, and that its configurations don’t change during training and inference.

- Same channel model for set A and set B, and that its configurations don’t change during training and inference. (Question: if consistency is valid for a non-static(time-varying) channel for set A and set B?)

After agreed, consistency should be taken as a side condition for the requirements.

## Sub-topic 2-4 Channel models in tests

Issue 2-4: Channel models

- Proposals - Option 1: CDL channels are needed (with fading) - Option 2: Only use AWGN channels - Option 4: multipath fading is needed, FFS how this can be emulated in the chamber

In last meeting, more than one company proposed that at least multipath fading is needed since AWGN will not provide any spatial spread. The use of multipath fading channel enables sufficient randomness and variation in time and spatial domain. According to the discussion, we don’t observe that multipath fading has essential difference from CDL channel. We think CDL channel in discussion may be a particular example of multipath fading with additional varying parameters. Apart from that, some discussions took place regarding the difference between time-varying channel and time-static channel. We understand time-varying channel may reflect some characteristics of AI/ML model which can’t be observed through time-static channel test, by which some advanced AI/ML models may achieve better performance. However, since the purpose of test is guaranteeing the minimal requirements, we think time-static channel is enough for the purpose.

Proposal 11:  RAN4 to use ‘time static’ multipath fading channel model for AI/ML beam prediction tests. RAN4 to further discuss whether ‘time static or time dynamic’ CDL can be used a specific channel model.

## Sub-topic 2-5 Test environment limitation/requirements

Issue 2-5: Test environment

- Proposals - Option 1: Test environment needs to emulate at least X beams for set B. X=8, 16 - Option 2: Test environment must ensure spatial consistency between beams in set A and set B - Option 3: number of angles of arrivals (or angle of arrival range/spread) - Option 4: number of AoDs - Option 5: UE rotation - Option 6: others

Proposal 12: We can wait for agreements on other issues before agreeing on the options on test environment limitation/requirements which have been listed in the last meeting. Moreover, we may need some simulations to evaluate the requirements.

## Sub-topic 2-6 Training data

Issue 2-6:	Datasets for training

- Proposals - Option 1: Training Data set to be specified in RAN4(directly or through some algorithm ) - Option 2: Training data set to be left to implementation (companies can generate it based on knowledge of the test environment - Option 3: others

To our understanding, training data set herein is a set of information that includes the input (Set B) and output (Set A) of AI/ML model in training phase. In conformance test, by such training data set, a UE doesn’t need to do off-line training in OTA chamber before inference test.

Observation 1:  Training data set herein is the set of information that includes the input (Set B) and output (Set A) of AI/ML model in training phase. Once training data is loaded, a UE is assumed to ‘training ready’.

Proposal 13: No strong view between Option 1 and Option 2, Option 2 at the least is acceptable from training perspective. But regarding Option 2, the question is: how to repeat the test and get same output in case that training data set is updated due to any reason.

There still is an open question on training data set. Surely, the training data set is loaded into UE memory directly, but whether the training data set doesn’t cover the hardware impairment before AI/ML model at UE, or the training data set by UE vendor implementation covers the hardware impairment before AI/ML model at UE? We do recommend clarifying the question.

Proposal 14: RAN4 to clarify whether training data set covers or doesn’t cover the hardware (analog and digital part) impairment prior to the input of AI/ML model in baseband part at UE.

# Conclusion

Observation 1:  Training data set herein is the set of information that includes the input (Set B) and output (Set A) of AI/ML model in training phase. Once training data is loaded, a UE is assumed to ‘training ready’.

Proposal 1: RAN4 shall deal with the below two kind of test metrics with separated definitions, and beam information is to be interpreted by RAN1.

- Option 1: Beam information on predicted Top K beam(s) among a set of beams.

- Option 2: Beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams.

Proposal 2: Regarding Top-1 predicted beam, RSRP accuracy could be determined by any of the following:

- Option1: RSRP accuracy is the difference between the ideal L1-RSRP of the predicted Top-1 beam and the ideal L1-RSRP of the Top-1 genie-aided beam.

- Option2: RSRP accuracy is the difference between the predicted Top-1 L1-RSRP and the measured L1-RSRP of the same beam.

- Option3: RSRP accuracy is the difference between the predicted L1-RSRP of the predicted Top-1 beam and the ideal L1-RSRP of the same beam.

- Option4: RSRP accuracy is the difference between the measured L1-RSRP of the predicted Top-1 beam and the L1-RSRP of the Top-1 genie beam.

Among the options, we intend towards Option 3.

Proposal 3: Regarding Top-K predicted beams, RSRP accuracy may be at least one of the following options:

- Option1: RSRP accuracy is a set of values includes the RSRP accuracy of each beam, and the accuracy of each beam can reuse the definition for RSRP accuracy in K=1 case, then the test is passed if all of the predicted Top-K beams reached the prediction accuracy.

- Option2: RSRP accuracy is a RSRP range given the upper bound and lower bound, then the test is passed if all of the predicted Top-K beams are within the range.

Between the options, we intend towards Option 1.

Proposal 4:  Above RSRP accuracy under discussion is the absolute RSRP accuracy. After the absolute RSRP accuracy is established, relative RSRP accuracy, as described in legacy, shall be determined.

Proposal 5: Measurement error is the difference between the measured RSRP of the Set A/B beam and the ideal RSRP of the Set A/B beam in both training and inference phase. In other words, measurement error occurs at both the input of training and inference phase of AI/ML model.

Proposal 6: Measurement error impact to prediction shall be verified, since the predicted RSRP is always worse than the measured RSRP if measurement error is introduced in training/inference phase of AI/ML model.

Proposal 7: If tightening measurement error under current side condition is difficult from UE implementation perspective, RAN4 can add another side condition for tightened measurement error for enabling AI-ML model.

Proposal 8: Measurement error impact to prediction shall be verified (e.g. by simulation) to guarantee an acceptable prediction performance.  Furthermore, measurement error impact to the predicted beam ID and the predicted RSRP should be studied separately.

Proposal 9: In measurement error, RX beam gain impact may refer to the outcome of the RAN1 study on selecting Rx beam for DL Tx beam prediction evaluations. If a particular beam is chosen when applying AI/ML model, then Rx beam gain error doesn’t contribute error to AI/ML model.

Proposal 10: From RAN4 test perspective, consistency between set A and set B is reflected by below aspects:

- Same NW antenna/beam configurations for set A and set B, and that its configurations don’t change during training and inference.

- Same channel model for set A and set B, and that its configurations don’t change during training and inference. (Question: if consistency is valid for a non-static(time-varying) channel for set A and set B?)

After agreed, consistency should be taken as a side condition for the requirements.

Proposal 11:  RAN4 to use ‘time static’ multipath fading channel model for AI/ML beam prediction tests. RAN4 to further discuss whether ‘time static or time dynamic’ CDL can be used a specific channel model.

Proposal 12: We can wait for agreements on other issues before agreeing on the options on test environment limitation/requirements which have been listed in the last meeting. Moreover, we may need some simulations to evaluate the requirements.

Proposal 13: No strong view between Option 1 and Option 2, Option 2 at the least is acceptable from training perspective. But regarding Option 2, the question is: how to repeat the test and get same output in case that training data set is updated due to any reason.

Proposal 14: RAN4 to clarify whether training data set covers or doesn’t cover the hardware (analog and digital part) impairment prior to the input of AI/ML model in baseband part at UE.

# References

- https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/TSGR4_110bis/Inbox/Drafts/%5B110bis%5D%5B100%5D%20Main%20Session/02.Tuesday/09.%5B135%5D_R4_2405288_AIML_Summary.docx