3GPP TSG-RAN WG4 Meeting #112											                R4-2411094

Maastricht, Netherland, August 19th ‒ 23rd, 2024

Agenda item:			8.22.3

Source:	CATT

Title:	Further discussion on BS RF requirements for LP-WUS

Document for:	Discussion

# Introduction

For BS RF requirements for LP-WUS, RAN4#111 has agreed a way forward, where several open issues are identifed in addition to some consensus [1]. In this contribution, we continue discussion on these open issues and share our views respectively.

# Discussion

2.1 Manufacture declaration on LP-WUS power boosting

WF FFS on the following options for next meeting: - Option 1: Set the LP-WUS power boosting a complete manufacture declaration feature, including whether supporting LP-WUS power boosting and the supported boosting level. - Option 2: Minimum power boosting level in core specification together with manufacturer declaration in the conformance test specification.

The primary difference between Option 1 and Option 2 is that Option 1 implicitly treats power boosting for LP-WUS as an optional feature, while Option 2 as a mandatory feature if LP-WUS is supported. Moreover, Option 2 provides a way for an infra vendor to declare its superiority over the minimum requirements specified in specs. In order to exploit most of LP-WUS in reality, we think Option 2 is a reasonable to go.

Proposal 1: For manufacture declaration on LP-WUS power boosting, RAN4 to specify minimum power boosting level in core specs together with manufacturer declaration in the conformance test specification, i.e., Option 2, for the sake of making most of LP-WUS in reality.

2.2 Concept of LP-WUS dynamic range/power boosting

WF FFS on the following options for next meeting: - Option 1: Stick with the one in RAN4#110bis approved WF R4-2406140, which is: - The LP-WUS RB power dynamic range (or LP-WUS power boosting) is the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration and the average power over all REs (from both LP-WUS and the NR carrier containing the LP-WUS REs). - Option 2: Define EPRE ratio between LP-WUS and NR signals instead of power dynamic range.

As the proponent for Option 2, our previous paper explains clearly why we could not just reuse the definition for NB-IoT power dynamic range [2]. The relationship between EPRE ratio and power dynamic range is illustrated by the following equation where δLP is the power dynamic range and α is the EPRE ratio:

δLP=NRB∙αNLP_RB∙α+(NRB-NLP_RB)                                (1)

The two tables below indicate how a fixed EPRE is associated with power dynamic ranges, and how a fixed power dynamic range is associated with different EPRE values respectively for different channel bandwidth assuming WUS occupies 5MHz.

Table – 1 Power dynamic range for LP-WUS

| EPRE ratio (dB) | CBW 10MHz | CBW 20MHz | CBW 30MHz | CBW 40MHz | CBW 50MHz |
| --- | --- | --- | --- | --- | --- |
| +3dB | 1.3 dB | 2.1 dB | 2.4 dB | 2.5 dB | 2.6 dB |
| +6dB | 2.1 dB | 3.7 dB | 4.4 dB | 4.7 dB | 4.9 dB |
| +9dB | 2.5 dB | 4.4 dB | 5.3 dB | 5.8 dB | 6.1 dB |

Table – 2 EPRE ratio for LP-WUS

| Power dynamic range (dB) | CBW 10MHz | CBW 20MHz | CBW 30MHz | CBW 40MHz | CBW 50MHz |
| --- | --- | --- | --- | --- | --- |
| +3dB | 14.3 | 4.6 | 3.9 | 3.6 | 3.5 |
| +6dB | N/A | 17.3 | 9.5 | 8.2 | 7.6 |
| +9dB | N/A | N/A | 19.1 | 12.4 | 10.9 |

It could be observed that for some channel bandwidths, power dynamic range values may not be achievable at all. Actually EPRE ratio is also used in RAN1 discussion. Therefore, we propose to introduce EPRE ratio between LP-WUS and NR signals instead of power dynamic range.

Proposal 2: Define EPRE ratio between LP-WUS and NR signals instead of power dynamic range.

A possible definition could be seen as following:

The LP-WUS power boosting is the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration and the average power NR REs (the NR carrier excluding the LP-WUS REs).

Proposal 3: Introduce the definition of LP-WUS power boosting as: The LP-WUS power boosting is the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration and the average power NR REs (the NR carrier excluding the LP-WUS REs).

2.3 Whether to preclude small CBW for consideration of LP-WUS power boosting

WF FFS on the following options for next meeting: - Option 1: Consider a power degradation limit, e.g., 2dB, for validating a configuration for LP-WUS representing by (EPRE ratio, channel bandwidth). - Option 2: Only consider LP-WUS power boosting for CBWs larger than 20MHz. - Option 3: Focus on CBWs >= 10MHz and different power boosting values can be considered for different CBW. - Option 4: Not preclude small CBW for LP-WUS power boosting.

Fig. 1, Power degradation threshold for validating EPRE ratio and channel bandwidth for power boosting

The power degradation ∆ could be calculated as:

∆=1+(α-1)NLP_RBNRB                                       (2)

When performing power boosting on LP-WUS signal, it should not have significant impact on the existing NR operation in the same carrier from operators’ perspective. A suitable controlling index is power degradation as shown in Fig. 1. For some channel bandwidth and EPRE ratio, it could introduce a large power degradation which may not be acceptable, e.g., a 6 dB EPRE power boosting for a 10MHz channel bandwidth may introduce a 4dB power degradation on NR signal. To strictly control the impact on NR operation from power boosting on LP-WUS, power degradation for power boosting should be specified as one of the core requirements for LP-WUS.

Proposal 4: RAN4 to introduce the maximum allowed power degradation as one of the core requirements for LP-WUS power boosting, i.e., Option 1, for the sake of guaranteeing NR network performance.

2.4 On applicable BS type for LP-WUS

WF FFS on the following proposals for next meeting: - Proposal 1: Not to set restriction on applicable BS types to support LP-WUS. - Proposal 2: To decide on the applicable BS types after the applicable frequency range and bands for LP-WUS have been decided. - Proposal 3: Consider BS type 1-C as applicable type to further discuss of LP-WUS power boosting. FFS other BS types. - Proposal 4: Depend on power boosting level for certain BS type. - If power boosting is limited to 3 dB, all BS type 1-C, 1-H and 1-O can be considered.

Considering deployment flexibility, it’s better to enable the potential benefits of applying LP-WUS in different scenarios, hence there should be no restriction on applicable BS types.

Proposal 5: RAN4 not to set a restriction on applicable BS types for LP-WUS.

2.5 On minimum value for LP-WUS power boosting

WF FFS on the following options for next meeting: - Option 1: Use [3dB] as starting point - It should be considered in conjunction with the supported CBWs. - Option 2: To consider the power degradation of RBs other than LP-WUS signal within the carrier after the number of LP-WUS RBs have been decided in RAN1.

First of all, power boosting is represented by EPRE ratio as proposed above. A 3dB minimum value could be a reasonable starting point together with the supported channel bandwidths, and of course, it can be revisited if any issue is identified later.

Proposal 6: RAN4 to consider 3dB as a starting point as the minimum requirements for different channel bandwidths for different channel bandwidths.

2.6 Whether a cap for LP-WUS power boosting should be considered

WF FFS on the following options for next meeting: - Option 1: Limit to 3dB for BS type 1-C, 1-H and 1-O. - Option 2: FFS on the value in conjunction with the supported CBWs. - Option 3: To consider the power degradation of RBs other than LP-WUS signal within the carrier after the number of LP-WUS RBs have been decided in RAN1. - Option 4: Not consider to cap the LP-WUS power boosting.

As discussed in section 2.3, if introducing the maximum allowed power degradation as one of the core requirements, then the cap for LP-WUS power boosting could be determined accordingly, and the cap may vary with different channel bandwidths.

Observation: The cap for LP-WUS power boosting may vary with different channel bandwidths if introducing the maximum allowed power degradation.

Proposal 7: Do not introduce a universal cap for LP-WUS power boosting when specifying the maximum allowed power degradation.

# Conclusion

In this contribution, we have the following observation and proposals for BS requirements for LP-WUS:

Proposal 1: For manufacture declaration on LP-WUS power boosting, RAN4 to specify minimum power boosting level in core specs together with manufacturer declaration in the conformance test specification, i.e., Option 2, for the sake of making most of LP-WUS in reality.

Proposal 2: Define EPRE ratio between LP-WUS and NR signals instead of power dynamic range.

Proposal 3: Introduce the definition of LP-WUS power boosting as: The LP-WUS power boosting is the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration and the average power NR REs (the NR carrier excluding the LP-WUS REs).

Proposal 4: RAN4 to introduce the maximum allowed power degradation as one of the core requirements for LP-WUS power boosting, i.e., Option 1, for the sake of guaranteeing NR network performance.

Proposal 5: RAN4 not to set a restriction on applicable BS types for LP-WUS.

Proposal 6: RAN4 to consider 3dB as a starting point as the minimum requirements for different channel bandwidths for different channel bandwidths.

Observation: The cap for LP-WUS power boosting may vary with different channel bandwidths if introducing the maximum allowed power degradation.

Proposal 7: Do not introduce a universal cap for LP-WUS power boosting when specifying the maximum allowed power degradation.

# Reference

[1] R4-2409956, Way Forward for [111][314] NR_LPWUS, Huawei, HiSilicon

[2] R4-2407547, On BS RF requirements for LP-WUS/LP-WUR, CATT