# 3GPP TSG-RAN WG4 Meeting #112	R4-2412123

# Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda Item:	5.14.1

Source:	China Telecom

Title:	Discussion on RRM core requirements maintenance for SCell activation delay reduction

Document for:	Discussion

- Introduction

In RAN4 #111 meeting, RRM core requirements maintenance for SCell activation delay reduction were discussed and the WF [1] was approved.

In this paper, we present our views on RRM core requirements maintenance for SCell activation delay reduction issues captured in the WF [1].

- Discussion

Status in the WF [1]:

Issue 1-4: Other issues related with FR1 SCell activation enhancement - FFS following proposals: - Proposal 1: - RAN4 to discuss if measurement period shall be considered as a condition to differentiate the requirements for FR1 SCell activation enhancement with L3 report. - Like the legacy FR1 known SCell activation, SCell activation delay requirement with L3 report shall be differentiated according to measurement period below or above 2400ms, and decide if AGC refinement or T/F tracking is needed. - Proposal 2: - RAN4 also consider applying enhancements of L3 reporting during SCell activation in FR1 to the case when only one SSB is transmitted in ssb-PositionInBurst. - RAN4 further extend the requirement applicability of 8.3.17 and 8.3.18 to the scenarios in FR1 where only one SSB is considered. If only one SSB is considered, Tuncertainty_MAC, Tuncertainty_SP, Tuncertainty_RRC and TRRC_delay are counted as zero, and the 3ms MACE CE decoding delay for TCI state activation is removed, i.e. the overall delay Tactivation_time is 7ms + TL3,report+ THARQ + TFineTiming + 2ms.

In our understanding, UE with valid L3 reporting means that UE already has AGC and coarse T/F information, then TCI activation and fine timing tracking can be performed with the valid L3-RSRP reporting or valid L1-RSRP reporting. Therefore, it may not be needed to differentiate the requirements for FR1 SCell activation enhancement with L3 report according to measurement period.

Observation 1: UE with valid L3 reporting means that UE already has AGC and coarse T/F information, then TCI activation and fine timing tracking can be performed with the valid L3-RSRP reporting or valid L1-RSRP reporting.

Proposal 1: It may not be needed to differentiate the requirements for FR1 SCell activation enhancement with L3 report according to measurement period.

- Conclusion

The contribution presents our views on RRM core requirements maintenance for SCell activation delay reduction, with the following observation and proposal:

Observation 1: UE with valid L3 reporting means that UE already has AGC and coarse T/F information, then TCI activation and fine timing tracking can be performed with the valid L3-RSRP reporting or valid L1-RSRP reporting.

Proposal 1: It may not be needed to differentiate the requirements for FR1 SCell activation enhancement with L3 report according to measurement period.

- References

- R4-2410259, WF for [111][205] NR_RRM_enh3_part1, Apple, RAN WG4 Meeting #111, May 2024.