3GPP TSG-RAN WG4 Meeting #112                                        R4-2412576

Maastricht, Netherlands, August 19 - 23, 2024

Source: 	Huawei, HiSilicon

Title: 	On general aspects for SBFD

Agenda Item:	8.19.1

Document for:	Approval

Introduction

During RAN4#111, general aspects (including RAN4 aspects for SBFD system parameters) for SBFD have been discussed. Following conclusions are reproduced from [1].

- Issue 1-1-1: SBFD as band specific or general feature to all TDD bands - Agreement: - SBFD is a feature with requirements of which can potentially be applied to all TDD band, under the following considerations: - Option 1: Declaration based method - The band supported for SBFD shall be manufacturer declaration based - The channel bandwidth supported for SBFD shall be manufacturer declaration based - The supported channel bandwidth can be impacted by the subband/guard band size discussion - Option 2: General principle but with consideration(s) for a specific band - General principle: The channel bandwidth shall be larger than X MHz - FFS the value of X - X can be different for high and middle TDD bands - For a specific band, the following further restriction(s) can be considered: - Certain restriction can be provided by operator(s) for certain band - Other restrictions are not precluded. - Issue 1-1-3: Applicability of SBFD and non-SBFD requirements - Agreement: - The conformance testing is needed for both SBFD and non-SBFD slots/symbols even though RF requirement might be same. - Issue 1-1-6: CLI handling impact on RF requirement - Agreement: - The CLI handling impact on RF requirement: - Option 1: RAN4 define RF requirements for the frequency outside of wanted carrier based on no CLI handling as baseline. - Option 2: FFS CLI handling impact on RF requirements - Issue 1-2-2: How to handle guard band and subband configurations in specification - Agreement: - Add clarification that only DUD and DU patterns are specified for the sub-band configuration. - FFS which channel bandwidth(s) or all channel bandwidths shall be defined for RF requirements - For a certain channel bandwidth which RAN4 agree to introduce RF requirements: - FFS RAN4 only define the UL/DL subbands configuration(s) for RF requirements - FFS which UL/DL subbands configuration(s) will be defined in RAN4 - FFS Guard band size is declaration based and can be different for different BS classes - FFS the limitation on the maximum guard band - FFS possible range for UL/DL subband sizes - From RAN4 perspective, FFS restriction or no restriction to RAN1 definition for UL/DL subband sizes within the transmission configuration for this channel bandwidth, except: - 1RB granularity (already introduced in RAN1) - Issue 1-4-1: BS RF Specification structure for SBFD requirements - Agreement: - How to introduce BS RF new requirements for SBFD-capable BS: - FFS firstly the feasibility of creating new and standalone sub-clauses (e.g., with a suffix) in TS 38.104 for SBFD-specific existing or new gNB RF requirements - FFS detailed how to implement such approach in the later phase of WI. - FFS secondly the new specification for SBFD-capable BS

In this contribution, we would like to share our views regarding this topic.

Discussion

Before focusing on the specific RF requirements for SBFD, it is important to go through general aspects including target supporting band, BS type, BS channel bandwidth and guard band. In addition, we would like to share our preference on specification structure regarding how to capture the SBFD feature.

The example bands for SBFD

For the target SBFD operating band for FR1, we agree with the WF that SBFD can be applied to all TDD bands, but we understand the difficulty on adding new modules or incorporating new architecture with legacy equipment especially operating in low bands. So our suggestion is to pick U6G, 4.9GHz and 3.5GHz as example frequency bands for RF requirement discussion.

Observation: While potentially SBFD can be applied to all TDD bands, example bands for SBFD RF requirement discussion can be selected as U6G, 4.9GHz and 3.5GHz.

The guard band and subband configuration for SBFD

On the subband configuration, we think it would be sufficient to specify following two patterns given that those two were assumed through the whole SI phase.

Proposal 1: Specify following subband configurations for SBFD.

- Pattern #1: ‘DU’, i.e. 80MHz (or 60MHz) for the DL subband and 20MHz (or 40MHz) for the UL subband

- Pattern #2: ‘DUD’, i.e. 40MHz for the two DL subbands and 20MHz for the UL subband

In the Rel-18 SI for duplex, extensive analysis and discussion regarding the interference caused by simultaneous Tx/Rx at gNB side have been conducted. To guarantee the feasibility of SBFD, multiple techniques in RF domain have been proposed. One straightforward solution is to have dedicated subband filter in order to provide sufficient interference suppression between subbands. As a result, the size of guard band should be carefully considered with expected interference suppression level, transition band of the subband filter and the RB overheads. Note that different vendor may have different kinds of trade-off, we prefer to leave the size of guard band for manufacturer declaration but not to have limitation on minimum/maximum value.

Proposal 2: The size of guard band for SBFD should be manufacturer declaration based:

- It can be different for different BS classes

- Not to have limitation on the minimum/maximum value of the size of guard band

The supported BS CBW for SBFD

For single carrier operation, we see no necessity to preclude the current specified maximum BS CBW i.e. 100MHz for supporting SBFD, while we think RAN4 can check whether to preclude the CBW less than 60MHz. For CA operation, we prefer to limit the SBFD operation on only one carrier for all the listed example on CA configuration in our proposal.

Proposal 3: For single carrier operation, the supported BS CBW for SBFD should be no less than 60MHz and applying 100MHz as maximum.

Proposal 4: For CA operation, the SBFD can be supported on only one of the carriers, and the supported CBW for this carrier should be no less than 60MHz and applying 100MHz as maximum.

- For instance, 100MHz CC for SBFD + N*100MHz CC, where N could be e.g. 1, 2, and 3

BS RF Specification structure for SBFD requirements

According to the WF, RAN4 will further study how to capture the SBFD feature in the TS 38.104 firstly. Thus in our view, new clauses for each requirement for SBFD can be added, while details like whether new suffix for SBFD is necessary or not can be determined when the discussion on RF requirements can be stable.

Proposal 5: New clauses for each requirement for SBFD can be added in TS 38.104, details like whether new suffix for SBFD is needed or not can be further determined when the discussion on RF requirements can be stable.

CLI handling impact on RF requirement

From our understanding, CLI handling solution, whether hardware based or not, should not be involved into RF minimum requirement derivation. Instead it can be considered for performance enhancement. Thus we tend to agree with Option 1 as in the WF.

Proposal 6: RAN4 define RF requirements for the frequency outside of wanted carrier based on no CLI handling as baseline.

Conclusion

In this contribution we discussed on the general aspects for SBFD. We have the following observation and proposals:

Observation: While potentially SBFD can be applied to all TDD bands, example bands for SBFD RF requirement discussion can be selected as U6G, 4.9GHz and 3.5GHz.

Proposal 1: Specify following subband configurations for SBFD.

- Pattern #1: ‘DU’, i.e. 80MHz (or 60MHz) for the DL subband and 20MHz (or 40MHz) for the UL subband

- Pattern #2: ‘DUD’, i.e. 40MHz for the two DL subbands and 20MHz for the UL subband

Proposal 2: The size of guard band for SBFD should be manufacturer declaration based:

- It can be different for different BS classes

- Not to have limitation on the minimum/maximum value of the size of guard band

Proposal 3: For single carrier operation, the supported BS CBW for SBFD should be no less than 60MHz and applying 100MHz as maximum.

Proposal 4: For CA operation, the SBFD can be supported on only one of the carriers, and the supported CBW for this carrier should be no less than 60MHz and applying 100MHz as maximum.

- For instance, 100MHz CC for SBFD + N*100MHz CC, where N could be e.g. 1, 2, and 3

Proposal 5: New clauses for each requirement for SBFD can be added in TS 38.104, details like whether new suffix for SBFD is needed or not can be further determined when the discussion on RF requirements can be stable.

Proposal 6: RAN4 define RF requirements for the frequency outside of wanted carrier based on no CLI handling as baseline.

- References

- R4-2409958, “Way Forward for [111][313] NR_duplex_evo”, Samsung, 3GPP TSG-RAN4 Meeting #111, Fukuoka, Japan, May 20-24, 2024.