| 3GPP TR 38.719-02-01V0.1.0 (2024-08) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Rel-19 NR inter-band CA/DC configurations including inter band CA for 2 different bands DL with up to 2 different bands UL (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	6

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	8

4	TR Maintenance	8

5	Both bands within FR1 Carrier Aggregation: Specific Band Combination Part	8

5.x	CA_nX-nY	8

5.1	 CA_n3-n104	13

5.2	 CA_n8-n104	17

5.3	 CA_n2-n5	20

5.4	 CA_n2-n48	23

5.5	 CA_n2-n66	24

5.6	 CA_n5-n48	26

5.7	 CA_n48-n66	29

5.8	 CA_n26-n78	30

5.9	 CA_n1-n71	32

5.10	CA_n3-n71	36

5.11	CA_n20-n77	39

6	Dual Connectivity: Specific Band Combination Part	45

6.x	DC_nX-nY	45

Annex A:Band group definition	46

Annex B: Valid UL configurations	46

Annex C:Void	46

Annex D:Void	46

Annex E (informative): Change history	46

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

Introduction

The present document is a technical report for NR inter-band CA and DC for 3 bands DL with 2 bands UL under Rel-19 time frame. The document covers the RF requirements for each band combination specific issues (i.e. one sub-clause defined per band combination), including:

1:  Common issues for both 1 band UL and 2 bands UL NR CA, including the impact of UL/DL harmonic/ harmonic mixing associated with REFSEN,  ∆TIB, c and ∆RIB, c, and OOB blocking exception, etc.

2:  2 bands UL NR CA specific issues, including MSD caused by IMD issue, etc.

It shall be noted that no new issue for inter-band NR DC combination, the corresponding 2 bands UL NR CA specific issues shall be re-used.

1	Scope

The present document is a technical report for power class 3 NR inter-band CA and DC for 2 bands DL with up to 2 bands UL under Rel-19 time frame. The purpose is to gather the relevant background information and studies in order to address NR inter-band CA and DC for 2 bands DL with up to 2 bands UL for the Rel-19 band combinations.

This TR contains the RF requirements of band specific combination part. The actual requirements are added to the corresponding technical specifications.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[3]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone".

[4]	3GPP TS 38.101-3: "NR; User Equipment (UE) radio transmission and reception; Part 3:  Range 1 and Range 2 Interworking operation with other radios".

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Carrier aggregation: Aggregation of two or more component carriers in order to support wider transmission bandwidths.

Inter-band carrier aggregation: Carrier aggregation of component carriers in different operating bands.

NOTE:	Carriers aggregated in each band can be contiguous or non-contiguous.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

ΔRIB,c	Allowed reference sensitivity relaxation due to support for inter-band CA operation, for serving cell c

ΔTIB,c	Allowed maximum configured output power relaxation due to support for inter-band CA operation, inter-band NR-DC operation and due to support for SUL operations, for serving cell c

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

CA	Carrier Aggregation

DC	Dual Connectivity

DL	DownLink

FDD	Frequency Division Duplex

IMD	Inter-modulation

MSD	Maximum Sensitivity Deduction

TDD	Time Division Duplex

UL	UpLink

4	TR Maintenance

A single company is responsible for introducing all approved TPs in the current TR, i.e. TR editor. However, it is the responsibility of the contact person of each band combination to ensure that the TPs related to the band combination have been implemented.

5	Both bands within FR1 Carrier Aggregation: Specific Band Combination Part

5.x	CA_nX-nY

5.x.1	Common for 1 band UL and 2 bands UL CA

5.x.1.1	Operating bands for CA

Table 5.x.1.1-1:  CA band combination of band nX+nY

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| nX | xxxx MHz – xxxx MHz | xxxx MHz – xxxx MHz | XXX |
| nY | xxxx MHz – xxxx MHz | xxxx MHz – xxxx MHz | XXX |
| NOTE X: Editor's note: Whether or not supporting simultaneous Rx/Tx capability should be identified for TDD-TDD or FDD-TDD band combination |  |  |  |

If the band combination is TDD/TDD, is SimRx/Tx supported (YES/NO/N-A)?

Editor's note: If yes, please add the NOTE in the above table.

5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination of band nX+nY

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_nXA-nYA | - * or CA_nXA-nYA | nX | Channel BWs | X |
|  |  | nY | Channel BWs |  |
| CA_nX(A/B/C(2A)-nY(A/B/C(2A) | CA_nXB/C/(2A) or CA_nXA/B/C-nYA | nX | Channel BW or CA BCS | X |
|  |  | nY | Channel BW or CA BCS |  |

Editor's note*: ‘-’ is for 1UL

Editor's note: The table format shall be the same with Table 5.5A.3.1-1 in the latest TS 38.101-1. The table format above is from the latest Rel.18 38.101-1, note that the table format might be changed in Rel.19.

Editor's note: The valid UL configurations shall refer to the Annex B.

5.x.1.3	UE co-existence studies for 1 band UL

Editor’s Note: The tables in this section are provided to identify potential issues to be analysed based on interference frequency range calculations, whether to specify the MSD related to collisions with the victim receiver frequency range should be based on the detailed REFSENS analysis.

Table 5.x.1.3-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_ nX-nY.

Table 5.x.1.3-1 UL/DL harmonics/harmonic mixing analysis

| UL/DL harmonics |  | nX | UL12 | UL2 | UL33 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | fLow |  |  |  |  |  |  |
| nY | fLow | fHigh |  |  |  |  |  |  |
| DL1 |  |  | N/A |  |  |  |  | UL harmonic |
| DL23 |  |  |  | N/A |  | N/A | N/A | Harmonic mixing |
| DL34 |  |  |  |  | N/A |  | N/A |  |
| DL4 |  |  |  | N/A | N/A | N/A | N/A |  |
| DL54 |  |  |  |  | N/A | N/A | N/A |  |
| Analysis |  |  | Editor’s Note: The analysis conclusion for the UL harmonic/harmonic mixing shall be included here, even if no issues are identified. |  |  |  |  |  |
| UL/DL harmonics |  | nY | UL14 | UL2 | UL33 | UL4 | UL5 | MSD type |
|  |  | fLow |  |  |  |  |  |  |
| nX | fLow | fHigh |  |  |  |  |  |  |
| DL1 |  |  | N/A |  |  |  |  | UL harmonic |
| DL23 |  |  |  | N/A |  | N/A | N/A | Harmonic mixing |
| DL34 |  |  |  |  | N/A |  | N/A |  |
| DL4 |  |  |  | N/A | N/A | N/A | N/A |  |
| DL54 |  |  |  |  | N/A | N/A | N/A |  |
| Analysis |  |  | Editor’s Note: The analysis conclusion for the UL harmonic/harmonic mixing shall be included here, even if no issues are identified. |  |  |  |  |  |
| Note 1: ULx means UL xth harmonic frequency, and DLy means DL yth harmonic frequency range Note 2: When a collision is detected with an overlap >0Hz between the ULx with DLy frequency ranges, the ULx/DLy cell is marked “D” for direct hit. When the gap between ULx and DLy frequency range is from 0Hz to x*MinULCBW, the ULx/DLy cell is marked “N” for Near miss. Note 3: UL3/DL2 harmonic mixing direct hit case for PC3/5 only apply for DL>3GHz Note 4: For harmonic mixing, near-miss cases only apply for UL1 and odd DLy orders. |  |  |  |  |  |  |  |  |

Table 5.x.1.3-2 summarizes frequency ranges where cross band isolation may occur for CA_nX-nY.

Table 5.x.1.3-2: Cross-band isolation analysis

| Bands | nX |  | nY |  |
| --- | --- | --- | --- | --- |
| Frequency limit | flow | fhigh | flow | fhigh |
| fUL (MHz) |  |  |  |  |
| fDL (MHz) |  |  |  |  |
| UL CBW (MHz)2 | Minimum CBW | Maximum CBW | Minimum CBW | Maximum CBW |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) |  |  |  |  |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) |  |  |  |  |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) |  |  |  |  |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) |  |  |  |  |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) |  |  |  |  |
| Analysis | Editor’s Note: The analysis conclusion for the cross- band isolation shall be included here, even if no issues are identified. |  | Editor’s Note: The analysis conclusion for the cross- band isolation shall be included here, even if no issues are identified. |  |
| NOTE 1: Even if there is no overlap up to ACLR5, MSD beyond the ACLR5 range should be evaluated further if: - The UL aggressor band and DL aggressor band are part of the same or adjacent band group as described in table A.1 - If the DL band is above the UL band, it’s lower frequency edge must be below the UL lowest 2nd harmonic frequency - As an indicative threshold, if >45dB UL rejection at the DL band frequency can be guaranteed, assuming a -130dBm/Hz TX noise floor level, the transmitter noise floor related MSD should be negligible NOTE 2: The maximum UL channel bandwidth of the BCS (noted maxULCBW) is used to calculate the band ACLR ranges while the minimum DL channel bandwidth of the BCS (noted minDLCBW) is used for the DL band victim channel bandwidth. |  |  |  |  |

Table 5.X.1.3-3 summarizes frequency ranges where IMD products caused by one UL band with 2CC intra-band UL CA may occur for CA_ nX-nY

Table 5.x.1.3-3: Intra-band ULCA IMD overlap with the other DL band analysis.

| All in MHz | flow | fhigh | BB IMD range3 |  |  |
| --- | --- | --- | --- | --- | --- |
| nX fUL |  |  | Order | flow | fhigh |
| nY fDL |  |  |  |  |  |
| 2CCBW1 | Minimum | Maximum | IMD2 (1-1) | Min2CCBW | Max2CCBW |
| Close to UL IMD range2 |  |  | IMD4 (2-2) | 2*Min2CCBW | 2*Max2CCBW |
| Order | flow | fhigh |  |  |  |
| IMD3 (2-1) | fULlow-Max2CCBW | fULhigh+Max2CCBW | IMD6 (3-3) | 3*Min2CCBW | 3*Max2CCBW |
| IMD5 (3-2) | fULlow-2*Max2CCLBW | fULhigh+2*Max2CCBW | Close to H2 IMD range4 |  |  |
|  |  |  | Order | flow | fhigh |
| IMD7 (4-3) | fULlow-3*Max2CCBW | fULhigh+3*Max2CCBW | IMD4 (3-1) | 2*fULlow-Max2CCBW | 2*fULhigh+Max2CCBW |
| IMD9 (5-4) | fULlow-4*Max2CCBW | fULhigh+4*Max2CCBW | IMD6 (4-2) | 2*fULlow-2*Max2CCBW | 2*fULhigh+2*Max2CCBW |
| IMD11 (6-5) | fULlow-5*Max2CCBW | fULhigh+5*Max2CCBW | Close to H3 IMD range4 |  |  |
|  |  |  | Order | flow | fhigh |
| IMD13 (7-6) | fULlow-6*Max2CCBW | fULhigh+6*Max2CCBW | IMD5 (4-1) | 3*fULlow-Max2CCBW | 3*fULhigh+Max2CCBW |
| Analysis | Editor’s Note: The analysis conclusion for the UL CA IMD shall be included here, even if no issues are identified. |  |  |  |  |
| NOTE 1: 2CCBW is the instantaneous transmit bandwidth of the two intra-band UL CCs: - The minimum 2CCBW for contiguous / non-contiguous intra-band ULCA is 0 / minimum UL channel bandwidth - The maximum 2CCBW for contiguous / non-contiguous ULCA is Min(maximum aggregated bandwidth / maximum separation bandwidth(600MHz),fULhigh-fULlow) NOTE 2: The close to UL IMD range is the most critical when the victim DL band in proximity to the UL band: - For contiguous/non-contiguous intra-band ULCA within a TDD band, IMD order up to 9/7 should be considered and MPR assumed - For intra-band ULCA within a FDD band, IMD order up to 13 should be considered for bands in the same band group and MPR is not assumed. If justified by poor filtering performance, higher order IMD may need to be specified. NOTE 3: The BB IMD range should only be considered if the DL band is below the UL band and for non-contiguous ULCA within a TDD band >3GHz (assuming CA with 450MHz bands is not considered) - IMD2 is not considered assuming CA with 450MHz bands is not considered - IMD4 is considered for FDD or SimRx/Tx TDD bands <1GHz - IMD6 is considered for FDD or SimRx/Tx TDD bands <1.68GHz NOTE 4: The harmonic 2 and 3 IMD ranges should only be considered if the DL band is above the UL band |  |  |  |  |  |

5.x.1.4	∆TIB,c and ∆RIB,c values

Editor’s note: for the table of ∆TIB,c and ∆RIB,c values, please use the same table format as in the latest TS 38.101-1, the table below is from the latest Rel.18 38.101-1, note that the table format might be changed in Rel.19.

For CA_nX-nY, the TIB,c and RIB,c values are given in the tables below.

Table 5.x.1.4-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| CA_nX-nY |  |  |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.x.1.4-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| CA_nX-nY |  |  |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.x.1.5	REFSENS requirements

Editor's note 1: Text may be added if harmonics/harmonic mixing/cross band isolation/Intra-band ULCA IMD issues are identified in subclause 5.x.1.3.

Editor's note 2: Proponent are encouraged to provide the detailed technical analyzed of the MSD requirements. For example: RF architecrture, RF components parameter, and etc. In the case where the proponent is missing some elements to calculate the REFSENS exception cases, the TP can be submitted to the “not for block approval” AI for expert support.

Editor's note 3: The table format shall be the same with the corresponding tables in TS38.101-1, i.e.: Table 7.3A.4-1, Table 7.3A.4-4, Table 7.3A.6-1 for harmonics, harmonic mixing and cross band isolation, respectively.

5.x.1.6	OOB blocking exception requirements

Table 5.x.1.6-1: CA band combination with exceptions allowed

CA band combination

5.x.2	Specific for 2 bands UL CA

Editor's note: Text will be added if 2 bands UL CA are supported, otherwise all the clauses shall be void.

5.x.2.1	Maximum output power for inter-band CA

Table 5.x.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Power Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_nXA-nYA | 23 | +x/-y |
| CA_nXA/B/C-nYA | 23 | +x/-y |

5.x.2.2	UE co-existence studies for 2 bands UL

Editor’s note: The tables in this section are provided to identify potential issues to be analysed based on interference frequency range calculations, whether to specify the MSD related to collisions with the victim receiver frequency range should be based on the detailed REFSENS analysis.

Table 5.x.2.2-1 lists Band nX + Band nY 2 bands UL CA(2CC) 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.x.2.2-1: Band nX and Band nY for 2CC UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | – |  | – |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | – |  | – |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | – |  | – |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | – |  | – |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | – |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | – |  | – |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | – |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | – |  | – |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | – |  | – |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | – |  | – |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | – |  | – |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on Table 5.x.2.2-1, nth order IMD may also fall into Rx frequencies of bands nX or band nY.

Table 5.x.2.2-2 lists Band nX + Band nY 2 bands UL CA(3CC) triple beat IMD analysis for Band nX- Band nY

Table 5.x.2.2-2: Band nX and Band nY for 3CC UL IMD products

| Band / CA1 | nX |  | CA_nYB/C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | fx_low | fx_high | fy_low | fy_high |
| fUL | – |  | – |  |
| fDL | – |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | – |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | – |  | – |  |
| Analysis | Editor’s Note: The analysis conclusion for the 3UL CC IMD shall be included here, even if no issues are identified. |  |  |  |
| NOTE 1: If the two bands are not part of the same or adjacent band groups as defined in table A.1, the analysis can be ignored. NOTE 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

Table 5.x.2.2-3 lists the protected bands required for the 2UL bands CA configuration.

Editor's note: If no need to add band combination to the protected band list, then Table 5.x.2.2-3 can be omitted.

Table 5.x.2.2-3: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_nX-nY | Frequency range |  | - |  |  |  | x |
| NOTE x: ..... Editor's note: The NOTE order must keep consistent with the Table 6.5A.3.2.3-1 in TS38.101-1. |  |  |  |  |  |  |  |

5.x.2.3	REFSENS requirements

Editor's note 1: Text will be added if  IMD due to 2 bands UL with 2 UL carriers or triple beat due to 2 bands UL with 3 UL carriers issues are identified in table 5.x.2.2-1 and table 5.x.2.2-2, respectively.

Editor's note 2: Proponent are encouraged to provide the detailed technical analysis of the MSD requirements. For example: RF architecture, RF components parameter, and etc. In the case where the proponent is missing some elements to calculate the REFSENS exception cases, the TP can be submitted to the “not for block approval” AI for expert support.

  - 5.1		CA_n3-n104

    - 5.1.1	Common for 1 band UL and 2 bands UL CA

      - 5.1.1.1	Operating bands for CA

Table 5.1.1.1-1:  CA band combination of band n3+n104

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low | - | FUL_high | FDL_low | - | FDL_high |  |
| n3 | 1710 MHz | - | 1785 MHz | 1805 MHz | - | 1880 MHz | FDD |
| n104 | 6425 MHz | - | 7125 MHz | 6425 MHz | - | 7125 MHz | TDD |

      - 5.1.1.2	Channel bandwidths per operating band for CA

Table 5.1.1.2-1: Supported bandwidths per CA band combination of band n3+n104

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n104A | CA_n3A-n104A | n3 | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 0 |
|  |  | n104 | 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |

      - 5.1.1.3	UE co-existence studies for 1 band UL

Table 5.1.1.3-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n3-n104.

Table 5.1.1.3-1 UL/DL harmonics/harmonic mixing analysis

| UL/DL |  | n3 | UL12 | UL2 | UL33 | UL4 | UL5 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| harmonics |  | fLow | 1710 | 3420 | 5130 | 6840 | 8550 | MSD type |
| n104 | fLow | fHigh | 1785 | 3570 | 5355 | 7140 | 8925 |  |
| DL1 | 6425 | 7125 | N/A | - | - | D | - | UL harmonic |
| DL23 | 12850 | 14250 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 19275 | 21375 | - | - | N/A | - | N/A |  |
| DL4 | 25700 | 28500 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 32125 | 35625 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | Direct hit n3 UL4 / n104 DL1, and no near miss n3 ULx / n104 DLy (5MHz) |  |  |  |  |  |
| UL/DL |  | n104 | UL14 | UL2 | UL33 | UL4 | UL5 | MSD type |
| harmonics |  | fLow | 6425 | 12850 | 19275 | 25700 | 32125 |  |
| n3 | fLow | fHigh | 7125 | 14250 | 21375 | 28500 | 35625 |  |
| DL1 | 1805 | 1880 | N/A | - | - | - | - | UL harmonic |
| DL23 | 3610 | 3760 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 5415 | 5640 | - | - | N/A | - | N/A |  |
| DL4 | 7220 | 7520 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 9025 | 9400 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | Neither direct hit n104 ULx / n1 DLy nor Near miss n104 ULx / n3 DLy (20MHz) |  |  |  |  |  |
| Note 1: ULx means UL xth harmonic frequency, and DLy means DL yth harmonic frequency range Note 2: When a collision is detected with an overlap >0Hz between the ULx with DLy frequency ranges, the ULx/DLy cell is marked “D” for direct hit. When the gap between ULx and DLy frequency range is from 0Hz to x*MinULCBW, the ULx/DLy cell is marked “N” for Near miss. Note 3: UL3/DL2 harmonic mixing direct hit case for PC3/5 only apply for DL>3GHz Note 4: For harmonic mixing, near-miss cases only apply for UL1 and odd DLy orders. |  |  |  |  |  |  |  |  |

Table 5.1.1.3-2 summarizes frequency ranges where cross band isolation may occur for CA_n3-n104.

Table 5.1.1.3-2: Cross-band isolation analysis

| Bands | n3 |  | n104 |  |
| --- | --- | --- | --- | --- |
| Frequency limit | flow | fhigh | flow | fhigh |
| fUL (MHz) | 1710 | 1785 | 6425 | 7125 |
| fDL (MHz) | 1805 | 1880 | 6425 | 7125 |
| UL CBW (MHz)2 | Minimum CBW | Maximum CBW | Minimum CBW | Maximum CBW |
|  | 5 | 50 | 20 | 100 |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) | 1660 | 1835 | 6325 | 7225 |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) | 1610 | 1885 | 6225 | 7325 |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) | 1560 | 1935 | 6125 | 7425 |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) | 1510 | 1985 | 6025 | 7525 |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) | 1460 | 2035 | 5925 | 7625 |
| Analysis | There are no cross-band isolation problem for n3 UL to n104 DL up to ACLR 5 according to the calculation. Moreover, n3 and n104 are not the same or adjacent band group as described in table A.1, so there is no need to check >ACLR5 MSD. |  | There are no cross-band isolation problem for n104 UL to n3 DL up to ACLR 5 according to the calculation. Moreover, n3 and n104 are not the same or adjacent band group as described in table A.1, so there is no need to check >ACLR5 MSD. |  |
| Note 1: Even if there is no overlap up to ACLR5, MSD beyond the ACLR5 range should be evaluated further if: -The UL aggressor band and DL aggressor band are part of the same or adjacent band group as described in table A.1. -If the DL band is above the UL band, it’s lower frequency edge must be below the UL lowest 2nd harmonic frequency -As an indicative threshold, if >45dB UL rejection at the DL band frequency can be guaranteed, assuming a -130dBm/Hz TX noise floor level, the transmitter noise floor related MSD should be negligible Note 2: The maximum UL channel bandwidth of the BCS (noted maxULCBW) is used to calculate the band ACLR ranges while the minimum DL channel bandwidth of the BCS (noted minDLCBW) is used for the DL band victim channel bandwidth. |  |  |  |  |

      - 5.1.1.4	∆TIB,c and ∆RIB,c values

For CA_n3-n104, it is proposed to use n3-n102  TIB,c and RIB,c requirements, which are given in the tables below.

Table 5.1.1.4-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| CA_n3-n104 | 0.3 | 0.8 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.1.1.4-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| CA_n3-n104 | - | 0.5 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

      - 5.1.1.5	REFSENS requirements

Considering the n3 is mid band and n104 is ultra-band, here we use the full separate antenna RF architecture assumption for the MSD derivation. The MSD for H4 caused by n3 UL falls into n104 DL are proposed in the table 5.1.1.5-1:

Table 5.1.1.5-1: Reference sensitivity exceptions and uplink/downlink configurations due to UL harmonic from a PC3 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n3 | n104 | 5 | 15 | 25 (RBstart=0) | 20 | 11.7 | NOTE 4 | UL4/DL1 direct-hit |
| n3 | n104 | 50 | 15 | 100 (RBstart=0) | 100 | 6.1 | NOTE 4 | UL4/DL1 direct-hit |
| NOTE 4: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |

      - 5.1.1.6	OOB blocking exception requirements

No OOB blocking issues for CA_n3-n104.

    - 5.1.2	Specific for 2 bands UL CA

      - 5.1.2.1	Maximum output power for inter-band CA

Table 5.1.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Power Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n3A-n104A | 23 | +2/-3 |

      - 5.1.2.2	UE co-existence studies for 2 bands UL

Table 5.1.2.2-1 lists Band n3 + Band n104 2 bands UL CA(2CC) 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.1.2.2-1: Band n3 and Band n104 for 2CC UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low |  | fy_high |
| --- | --- | --- | --- | --- | --- |
| UL frequency | 1710 | 1785 | 6425 |  | 7125 |
| 2nd order IMD products | ¦fy_low-fx_high¦ | ¦fy_high-fx_low¦ | ¦fy_low+fx_low¦ |  | ¦fy_high+fx_high¦ |
| IMD frequency limits (MHz) | 4640 -- 5415 |  |  | 8135 -- 8910 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ |  | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3705 -- 2855 |  |  | 11065 -- 12540 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ |  | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 9845 -- 10695 |  |  | 14560 -- 16035 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ |  | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1995 -- 1070 |  |  | 17490 -- 19665 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | 10830 -- 9280 |  |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high+1*y_high¦ | ¦3*fy_low + 1*fx_low¦ |  | ¦3*fy_high+1*fx_high¦ |
| IMD frequency limits (MHz) | 11555 -- 12480 |  |  | 20985 -- 23160 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2*y_high¦ |  |  |  |
| IMD frequency limits (MHz) | 16270 -- 17820 |  |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ |  | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 26790 -- 23915 |  |  | 715 -- 285 |  |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ |  | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 17955 -- 15705 |  |  | 7495 -- 9120 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ |  | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 27410 -- 30285 |  |  | 13265 -- 14265 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + *fy_high¦ | ¦2*fy_low + 3*fx_low¦ |  | ¦2*fy_high + *fx_high¦ |
| IMD frequency limits (MHz) | 22695 -- 24945 |  |  | 17980 -- 19605 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |  |

Based on Table 5.1.2.2-1, 4th order IMD may also fall into Rx frequencies of bands n3.

For CA_n3-n104, although PHS frequency range 1884.5-1915.7MHz is as pretected band of band n3, due to this combination will not be deployed in the PHS region, there is no need to add PHS frequency range as protected band for CA_n3-n104. Thus no protected bands are needed.

      - 5.1.2.3	REFSENS requirements

Based on the co-existence studies there is a need to define IMD4 MSD values, which is proposed in table 5.1.2.3-1:

Table 5.1.2.3-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n104 | n3 | 1750 | 5 | 25 | 1845 | 9.0 | FDD | IMD4 |
|  | n104 | 7095 | 20 | 50 | 7095 | N/A | TDD | N/A |

  - 5.2		CA_n8-n104

    - 5.2.1	Common for 1 band UL and 2 bands UL CA

      - 5.2.1.1	Operating bands for CA

Table 5.2.1.1-1:  CA band combination of band n8+n104

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low | - | FUL_high | FDL_low | - | FDL_high |  |
| n8 | 880 MHz | - | 915 MHz | 925 MHz | - | 960 MHz | FDD |
| n104 | 6425 MHz | - | 7125 MHz | 6425 MHz | - | 7125 MHz | TDD |

      - 5.2.1.2	Channel bandwidths per operating band for CA

Table 5.2.1.2-1: Supported bandwidths per CA band combination of band n8+n104

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n8A-n104A | CA_n8A-n104A | n8 | 5, 10, 15, 20, 25, 30, 35 | 0 |
|  |  | n104 | 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |

      - 5.2.1.3	UE co-existence studies for 1 band UL

Table 5.2.1.3-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n8-n104.

Table 5.2.1.3-1 UL/DL harmonics/harmonic mixing analysis

| UL/DL |  | n8 | UL12 | UL2 | UL33 | UL4 | UL5 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| harmonics |  | fLow | 880 | 1760 | 2640 | 3520 | 4400 | MSD type |
| n104 | fLow | fHigh | 915 | 1830 | 2745 | 3660 | 4575 |  |
| DL1 | 6425 | 7125 | N/A | - | - | - | - | UL harmonic |
| DL23 | 12850 | 14250 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 19275 | 21375 | - | - | N/A | - | N/A |  |
| DL4 | 25700 | 28500 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 32125 | 35625 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | Neither direct hit nor near miss n8 ULx / n104 DLy (5MHz) |  |  |  |  |  |
| UL/DL |  | n104 | UL14 | UL2 | UL33 | UL4 | UL5 | MSD type |
| harmonics |  | fLow | 6425 | 12850 | 19275 | 25700 | 32125 |  |
| n8 | fLow | fHigh | 7125 | 14250 | 21375 | 28500 | 35625 |  |
| DL1 | 925 | 960 | N/A | - | - | - | - | UL harmonic |
| DL23 | 1850 | 1920 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 2775 | 2880 | - | - | N/A | - | N/A |  |
| DL4 | 3700 | 3840 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 4625 | 4800 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | Neither direct hit nor near miss n104 ULx / n8 DLy (20MHz) |  |  |  |  |  |
| Note 1: ULx means UL xth harmonic frequency, and DLy means DL yth harmonic frequency range Note 2: When a collision is detected with an overlap >0Hz between the ULx with DLy frequency ranges, the ULx/DLy cell is marked “D” for direct hit. When the gap between ULx and DLy frequency range is from 0Hz to x*MinULCBW, the ULx/DLy cell is marked “N” for Near miss. Note 3: UL3/DL2 harmonic mixing direct hit case for PC3/5 only apply for DL>3GHz Note 4: For harmonic mixing, near-miss cases only apply for UL1 and odd DLy orders. |  |  |  |  |  |  |  |  |

Table 5.2.1.3-2 summarizes frequency ranges where cross band isolation may occur for CA_n8-n104.

Table 5.2.1.3-2: Cross-band isolation analysis

| Bands | n8 |  | n104 |  |
| --- | --- | --- | --- | --- |
| Frequency limit | flow | fhigh | flow | fhigh |
| fUL (MHz) | 880 | 915 | 6425 | 7125 |
| fDL (MHz) | 925 | 960 | 6425 | 7125 |
| UL CBW (MHz)2 | Minimum CBW | Maximum CBW | Minimum CBW | Maximum CBW |
|  | 5 | 35 | 20 | 100 |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) | 845 | 950 | 6325 | 7225 |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) | 810 | 985 | 6225 | 7325 |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) | 775 | 1020 | 6125 | 7425 |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) | 740 | 1055 | 6025 | 7525 |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) | 705 | 1090 | 5925 | 7625 |
| Analysis | There are no cross-band isolation problem for n8 UL to n104 DL up to ACLR 5 according to the calculation. Moreover, n8 and n104 are not the same or adjacent band group as described in table A.1, so there is no need to check >ACLR5 MSD. |  | There are no cross-band isolation problem for n104 UL to n8 DL up to ACLR 5 according to the calculation. Moreover, n8 and n104 are not the same or adjacent band group as described in table A.1, so there is no need to check >ACLR5 MSD. |  |
| Note 1: Even if there is no overlap up to ACLR5, MSD beyond the ACLR5 range should be evaluated further if: -The UL aggressor band and DL aggressor band are part of the same or adjacent band group as described in table A.1. -If the DL band is above the UL band, it’s lower frequency edge must be below the UL lowest 2nd harmonic frequency -As an indicative threshold, if >45dB UL rejection at the DL band frequency can be guaranteed, assuming a -130dBm/Hz TX noise floor level, the transmitter noise floor related MSD should be negligible Note 2: The maximum UL channel bandwidth of the BCS (noted maxULCBW) is used to calculate the band ACLR ranges while the minimum DL channel bandwidth of the BCS (noted minDLCBW) is used for the DL band victim channel bandwidth. |  |  |  |  |

      - 5.2.1.4	∆TIB,c and ∆RIB,c values

For CA_n8-n104, the TIB,c and RIB,c requirements are given in the tables below.

Table 5.2.1.4-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| CA_n8-n104 | 0.3 | 0.8 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n8 the band order from left to right is n1 and n8. |  |  |

Table 5.2.1.4-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| CA_n8-n104 | - | 0.5 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

      - 5.2.1.5	REFSENS requirements

There are no harmonic/harmonic mixing/cross band isolation co-existence problem according to UE co-existence study in section 5.2.1.3, so there is no need to defined additional REFSEN requirements(MSD).

      - 5.2.1.6	OOB blocking exception requirements

Since band n8 is a low band and n104 is a high and wide band, the OOBB exception is needed.

Table 5.2.1.6-1: CA band combination with exceptions allowed

CA band combination

CA_n8-n104

    - 5.2.2	Specific for 2 bands UL CA

      - 5.2.2.1	Maximum output power for inter-band CA

Table 5.2.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Power Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n8A-n104A | 23 | +2/-3 |

      - 5.2.2.2	UE co-existence studies for 2 bands UL

Table 5.2.2.2-1 lists Band n8 + Band n104 2 bands UL CA(2CC) 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.2.2.2-1: Band n8 and Band n104 for 2CC UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency | 880 | 915 | 6425 | 7125 |
| 2nd order IMD products | ¦fy_low-fx_high¦ | ¦fy_high-fx_low¦ | ¦fy_low+fx_low¦ | ¦fy_high+fx_high¦ |
| IMD frequency limits (MHz) | 5510 -- 6220 |  | 7305 -- 8015 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 5340 -- 4595 |  | 11935 -- 13320 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8185 -- 8930 |  | 13730 -- 15115 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high– 1*fy_low¦ | ¦3*fy_low–1*fx_high¦ | ¦3*fy_high– 1*fx_low¦ |
| IMD frequency limits (MHz) | 4460 -- 3680 |  | 18360 -- 20420 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high– 2*fy_low¦ |  |  |
| IMD frequency limits (MHz) | 12440 -- 11020 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1*y_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9065 -- 9845 |  | 20155 -- 22215 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high+2*fy_high¦ |  |  |
| IMD frequency limits (MHz) | 14610 -- 16030 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 27520 -- 24785 |  | 2765 -- 3580 |  |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high–3*fy_low¦ | ¦2*fy_low–3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 19540 -- 17445 |  | 10105 -- 11560 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 26580 -- 29315 |  | 9945 -- 10760 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high+ *fy_high¦ | ¦2*fy_low+3*fx_low¦ | ¦2*fy_high+3*fx_high¦ |
| IMD frequency limits (MHz) | 21035 -- 23130 |  | 15490 -- 16945 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on Table 5.2.2.2-1, No IMD fall into Rx frequencies of bands n8.

For CA_n8-n104, although PHS frequency range 1884.5-1915.7MHz is as pretected band of band n8, due to this combination will not be deployed in the PHS region, there is no need to add PHS frequency range as protected band for CA_n8-n104. Thus no protected bands are needed.

      - 5.2.2.3	REFSENS requirements

There are no IMD co-existence problem according to UE co-existence study in section 5.2.2.2, so there is no need to defined additional REFSEN requirements(MSD).

- 5.3		CA_n2-n5

- 5.3.1	Common for 1 band UL and 2 bands UL CA

- 5.3.1.1	Operating bands for CA

Table 5.3.1.1-1:  CA band combination of band n2+n5

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
| n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |

- 5.3.1.2	Channel bandwidths per operating band for CA

Table 5.3.1.2-1: Supported bandwidths per CA band combination of band n2+n5

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n2A-n5A | CA_n2A-n5A | n2 | n2 channel bandwidths in Table 5.3.5-1 of 38.101-1 | 4 and 5 |
|  |  | n5 | n5 channel bandwidths in Table 5.3.5-1 of 38.101-1 |  |

- 5.3.1.3	UE co-existence studies for 1 band UL

#### 5.3.1.3.1	Co-existence studies for 2UL band with 1CC per band

Table 5.3.1.3.1-1 summarizes frequency ranges where harmonics and/or harmonic mixing occur for CA_n2A-n5A.

Table 5.3.1.3.1-1: UL/DL harmonics/harmonic mixing analysis

| UL/DL |  | n5 | UL1 | UL2 | UL3 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| harmonics |  | fLow | 824 | 1648 | 2472 | 3296 | 4120 |  |
| n2 | fLow | fHigh | 849 | 1698 | 2547 | 3396 | 4245 |  |
| DL1 | 1930 | 1990 | N/A |  |  |  |  | UL Harmonic |
| DL2 | 3860 | 3980 |  | N/A |  | N/A | N/A | Harmonic Mixing |
| DL3 | 5790 | 5970 |  |  | N/A |  | N/A |  |
| DL4 | 7720 | 7960 |  | N/A | N/A | N/A | N/A |  |
| DL5 | 9650 | 9950 |  |  | N/A | N/A | N/A |  |
| Analysis |  |  | There is no collision detected with both harmonic and harmonic mixing. |  |  |  |  |  |
| UL/DL |  | n2 | UL13 | UL2 | UL3 | UL4 | UL5 | MSD type |
| harmonics |  | fLow | 1850 | 3700 | 5550 | 7400 | 9250 |  |
| n5 | fLow | fHigh | 1910 | 3820 | 5730 | 7640 | 9550 |  |
| DL1 | 869 | 894 | N/A |  |  |  |  | UL Harmonic |
| DL2 | 1738 | 1788 |  | N/A |  | N/A | N/A | Harmonic Mixing |
| DL3 | 2607 | 2682 |  |  | N/A |  | N/A |  |
| DL4 | 3476 | 3576 |  | N/A | N/A | N/A | N/A |  |
| DL5 | 4345 | 4470 |  |  | N/A | N/A | N/A |  |
| Analysis |  |  | There is no collision detected with both harmonic and harmonic mixing. |  |  |  |  |  |

Based on the above table:

- There is no harmonic or harmonic mixing collision detected for this band combination.

Table 5.3.1.3.1-2 summarizes frequency ranges where cross-band isolation may occur for CA_n2A-n5A.

Table 5.3.1.3.1-2: Cross-band isolation analysis

| Bands | n2 |  | n5 |  |
| --- | --- | --- | --- | --- |
| Frequency limit | fx_low / min | fx_high / max | fy_low / min | fy_high / max |
| fUL (MHz) | 1850 | 1910 | 824 | 849 |
| fDL (MHz) | 1930 | 1990 | 869 | 894 |
| CBW (MHz) | 5 | 40 | 5 | 20 |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) | 1810 | 1950 | 804 | 869 |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) | 1770 | 1990 | 784 | 889 |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) | 1730 | 2030 | 764 | 909 |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) | 1690 | 2070 | 744 | 929 |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) | 1650 | 2110 | 724 | 949 |
| Analysis | There is no overlap of the band n2 ACLR range with the band n5 DL up to order 5. The sufficient rejection of the transmitter noise floor should be at band n5 so no cross-band MSD is needed. |  | There is no overlap of the band n5 ACLR range with the band n2 DL up to order 5. The sufficient rejection of the transmitter noise floor should be at band n2 so no cross-band MSD is needed. |  |

Based on the above table:

There is no uplink band overlapping with the downlink in ACLR range. The existing diplexer isolation should provide sufficient rejection.

- 5.4		CA_n2-n48

- 5.4.1	Common for 1 band UL and 2 bands UL CA

- 5.4.1.1	Operating bands for CA

Table 5.4.1.1-1:  CA band combination of band n2+n48

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
| n48 | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz | TDD |

- 5.4.1.2	Channel bandwidths per operating band for CA

Table 5.4.1.2-1: Supported bandwidths per CA band combination of band n2+n48

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n2A-n48A | CA_n2A-n48A | n2 | n5 channel bandwidths in Table 5.3.5-1 of 38.101-1 | 4 and 5 |
|  |  | n48 | n48 channel bandwidths in Table 5.3.5-1 of 38.101-1 |  |

- 5.4.1.3	UE co-existence studies for 1 band UL

#### 5.4.1.3.1	Co-existence studies for 2UL band with 1CC per band

Table 5.4.1.3.1-1 summarizes frequency ranges where harmonics and/or harmonic mixing occur for CA_n2-n48.

Table 5.4.1.3.1-1: UL harmonics and harmonic mixing analysis

| UL/DL |  | n48 | UL1 | UL2 | UL3 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| harmonics |  | fLow | 3550 | 7100 | 10650 | 14200 | 17750 |  |
| n2 | fLow | fHigh | 3700 | 7400 | 11100 | 14800 | 18500 |  |
| DL1 | 1930 | 1990 | N/A |  |  |  |  | UL Harmonic |
| DL2 | 3860 | 3980 |  | N/A |  | N/A | N/A | Harmonic Mixing |
| DL3 | 5790 | 5970 |  |  | N/A |  | N/A |  |
| DL4 | 7720 | 7960 |  | N/A | N/A | N/A | N/A |  |
| DL5 | 9650 | 9950 |  |  | N/A | N/A | N/A |  |
| Analysis |  |  | There is no collision detected with both harmonic and harmonic mixing. |  |  |  |  |  |
| UL/DL |  | n2 | UL13 | UL2 | UL3 | UL4 | UL5 | MSD type |
| harmonics |  | fLow | 1850 | 3700 | 5550 | 7400 | 9250 |  |
| n48 | fLow | fHigh | 1910 | 3820 | 5730 | 7640 | 9550 |  |
| DL1 | 3550 | 3700 | N/A | N |  |  |  | UL Harmonic |
| DL2 | 7100 | 7400 |  | N/A |  | N/A | N/A | Harmonic Mixing |
| DL3 | 10650 | 11100 |  |  | N/A |  | N/A |  |
| DL4 | 14200 | 14800 |  | N/A | N/A | N/A | N/A |  |
| DL5 | 17750 | 18500 |  |  | N/A | N/A | N/A |  |
| Analysis |  |  | There is a near miss collision between the band n2 2nd harmonic to the band n48 DL. |  |  |  |  |  |

Based on the above table:

For the n2 uplink 2nd harmonic falling to the band n48, the MSD defined in Table 7.3A.4-1 of 38.101-1 can be applied. There is no newer requirement needed.

Table 5.4.1.3.1-2 summarizes frequency ranges where cross-band isolation issues may occur for both UL bands into the other band DL.

Table 5.4.1.3.1-2: Cross-band isolation analysis

| Bands | n2 |  | n48 |  |
| --- | --- | --- | --- | --- |
| Frequency limit | fx_low / min | fx_high / max | fy_low / min | fy_high / max |
| fUL (MHz) | 1850 | 1910 | 3550 | 3700 |
| fDL (MHz) | 1930 | 1990 | 3550 | 3700 |
| CBW (MHz) | 5 | 40 | 5 | 100 |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) | 1810 | 1950 | 3450 | 3800 |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) | 1770 | 1990 | 3350 | 3900 |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) | 1730 | 2030 | 3250 | 4000 |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) | 1690 | 2070 | 3150 | 4100 |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) | 1650 | 2110 | 3050 | 4200 |
| Analysis | There is no overlap of the band n2 ACLR range with the band n48 DL up to order 5. The sufficient rejection of the transmitter noise floor should be at band n48 so no cross-band MSD is needed. |  | There is no overlap of the band n48 ACLR range with the band n2 DL up to order 5. The sufficient rejection of the transmitter noise floor should be at band n2 so no cross-band MSD is needed. |  |

Based on the above table:

- There is no uplink band overlapping with the downlink in ACLR range. The existing diplexer isolation should provide sufficient rejection.

- 5.5		CA_n2-n66

- 5.5.1	Common for 1 band UL and 2 bands UL CA

- 5.5.1.1	Operating bands for CA

Table 5.5.1.1-1:  CA band combination of band n2+n66

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
| n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

- 5.5.1.2	Channel bandwidths per operating band for CA

Table 5.5.1.2-1: Supported bandwidths per CA band combination of band n2+n66

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n2A-n66A | CA_n2A-n66A | n2 | See n2 channel bandwidths in Table 5.3.5-1 of 38.101-1 | 4 and 5 |
|  |  | n66 | See n66 channel bandwidths in Table 5.3.5-1 of 38.101-1 |  |

- 5.5.1.3	UE co-existence studies for 1 band UL

#### 5.5.1.3.1	Co-existence studies for 2UL band with 1CC per band

Table 5.5.1.3.1-1 summarizes frequency ranges where harmonics and/or harmonic mixing occur for CA_n2-n66.

Table 5.5.1.3.1-1: UL harmonics and harmonic mixing analysis

| UL/DL |  | n66 | UL1 | UL2 | UL3 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| harmonics |  | fLow | 1710 | 3420 | 5130 | 6840 | 8550 |  |
| n2 | fLow | fHigh | 1780 | 3560 | 5340 | 7120 | 8900 |  |
| DL1 | 1930 | 1990 | N/A |  |  |  |  | UL Harmonic |
| DL2 | 3860 | 3980 |  | N/A |  | N/A | N/A | Harmonic Mixing |
| DL3 | 5790 | 5970 |  |  | N/A |  | N/A |  |
| DL4 | 7720 | 7960 |  | N/A | N/A | N/A | N/A |  |
| DL5 | 9650 | 9950 |  |  | N/A | N/A | N/A |  |
| Analysis |  |  | There is no collision detected with both harmonic and harmonic mixing. |  |  |  |  |  |
| UL/DL |  | n2 | UL13 | UL2 | UL3 | UL4 | UL5 | MSD type |
| harmonics |  | fLow | 1850 | 3700 | 5550 | 7400 | 9250 |  |
| n66 | fLow | fHigh | 1910 | 3820 | 5730 | 7640 | 9550 |  |
| DL1 | 2110 | 2200 | N/A |  |  |  |  | UL Harmonic |
| DL2 | 4220 | 4400 |  | N/A |  | N/A | N/A | Harmonic Mixing |
| DL3 | 6330 | 6600 |  |  | N/A |  | N/A |  |
| DL4 | 8440 | 8800 |  | N/A | N/A | N/A | N/A |  |
| DL5 | 10550 | 11000 |  |  | N/A | N/A | N/A |  |
| Analysis |  |  | There is no collision detected with both harmonic and harmonic mixing. |  |  |  |  |  |

Based on the above table:

- There is no harmonic or harmonic mixing collision detected for this band combination.

Table 5.5.1.3.1-2 summarizes frequency ranges where cross-band isolation issues may occur for both UL bands into the other band DL.

Table 5.5.1.3.1-2: Cross-band isolation analysis

| Bands | n2 |  | n66 |  |
| --- | --- | --- | --- | --- |
| Frequency limit | fx_low / min | fx_high / max | fy_low / min | fy_high / max |
| fUL (MHz) | 1850 | 1910 | 1710 | 1780 |
| fDL (MHz) | 1930 | 1990 | 2110 | 2200 |
| CBW (MHz) | 5 | 40 | 5 | 45 |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) | 1810 | 1950 | 1665 | 1825 |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) | 1770 | 1990 | 1620 | 1870 |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) | 1730 | 2030 | 1575 | 1915 |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) | 1690 | 2070 | 1530 | 1960 |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) | 1650 | 2110 | 1485 | 2005 |
| Analysis | There is an overlap of the band n2 ACLR5 range with the band n66 DL. Therefore, the cross-band isolation MSD should be studied. |  | There are an overlaps of the band n66 ACLR 4 and 5 ranges with the band n2 DL. Therefore, the cross-band isolation MSD should be studied. |  |

Based on the table above:

- For the cross-band overlapping from uplink n66 to n2 downlink, the same degradation proposal approved in CR [4] will be applied as shown below. No additional MSD is needed.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n66 | n2 | 1757.5 | 45 | 15 | 240 (RBstart=2) | 1932.5 | 5 | 1.2 | >ACLR2 |

- For the cross-band overlapping from uplink n2 to n66 downlink, the RBs in uplink n2 are never fully loaded in REFSENS configuration. There is no issue and there is no need to specify MSD for the PC3.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n2 | n66 | 1910 | 40 | 15 | 40 (RBstart=176) | 2112.5 | 5 | [0] | >ACLR2 |

- 5.6		CA_n5-n48

- 5.6.1	Common for 1 band UL and 2 bands UL CA

- 5.6.1.1	Operating bands for CA

Table 5.6.1.1-1:  CA band combination of band n5+n48

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
| n48 | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz | TDD |

- 5.6.1.2	Channel bandwidths per operating band for CA

Table 5.6.1.2-1: Supported bandwidths per CA band combination of band n5+n48

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n5A-n48A | CA_n5A-n48A | n5 | n5 channel bandwidths in Table 5.3.5-1 of 38.101-1 | 4 and 5 |
|  |  | n48 | n48 channel bandwidths in Table 5.3.5-1 of 38.101-1 |  |
| NOTE 3: This UE channel bandwidth is applicable only to downlink. NOTE 6: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as a downlink SCell part of CA configuration. |  |  |  |  |

- 5.6.1.3	UE co-existence studies for 1 band UL

#### 5.6.1.3.1	Co-existence studies for 2UL band with 1CC per band

Table 5.6.1.3.1-1 summarizes frequency ranges where harmonics and/or harmonic mixing occur for CA_n5-n48.

Table 5.6.1.3.1-1: UL harmonics and harmonic mixing analysis

| UL/DL |  | n48 | UL1 | UL2 | UL3 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| harmonics |  | fLow | 3550 | 7100 | 10650 | 14200 | 17750 |  |
| n5 | fLow | fHigh | 3700 | 7400 | 11100 | 14800 | 18500 |  |
| DL1 | 869 | 894 | N/A |  |  |  |  | UL Harmonic |
| DL2 | 1738 | 1788 |  | N/A |  | N/A | N/A | Harmonic Mixing |
| DL3 | 2607 | 2682 |  |  | N/A |  | N/A |  |
| DL4 | 3476 | 3576 | D | N/A | N/A | N/A | N/A |  |
| DL5 | 4345 | 4470 |  |  | N/A | N/A | N/A |  |
| Analysis |  |  | There is collision detected from downlink band n5 4th harmonic mixing with n48 uplink. |  |  |  |  |  |
| UL/DL |  | n5 | UL13 | UL2 | UL3 | UL4 | UL5 | MSD type |
| harmonics |  | fLow | 824 | 1648 | 2472 | 3296 | 4120 |  |
| n48 | fLow | fHigh | 849 | 1698 | 2547 | 3396 | 4245 |  |
| DL1 | 3550 | 3700 | N/A |  |  |  |  | UL Harmonic |
| DL2 | 7100 | 7400 |  | N/A |  | N/A | N/A | Harmonic Mixing |
| DL3 | 10650 | 11100 |  |  | N/A |  | N/A |  |
| DL4 | 14200 | 14800 |  | N/A | N/A | N/A | N/A |  |
| DL5 | 17750 | 18500 |  |  | N/A | N/A | N/A |  |
| Analysis |  |  | There is no collision detected with both harmonic and harmonic mixing. |  |  |  |  |  |

Based on the above table:

- For the 4th DL harmonic-mixing falling on the downlink, the BSD for CA_n5-n48 for PC3 should be same as the defined CA_n26-n48. This is because the new higher channel bandwidth introduced for band n48 BCS 4 and 5 is the same as the BCS 0 and 1.

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n48 | n5 | 10 | 15 | 25 (RBstart=0) | 5 | 5.7 | NOTE 8 | UL1/DL4 |

Table 5.6.1.3.1-2 summarizes frequency ranges where cross-band isolation issues may occur for both UL bands into the other band DL.

Table 5.6.1.3.1-2: Cross-band isolation analysis

| Bands | n5 |  | n48 |  |
| --- | --- | --- | --- | --- |
| Frequency limit | fx_low / min | fx_high / max | fy_low / min | fy_high / max |
| fUL (MHz) | 824 | 849 | 3550 | 3700 |
| fDL (MHz) | 869 | 894 | 3550 | 3700 |
| CBW (MHz) | 5 | 25 | 5 | 100 |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) | 799 | 874 | 3450 | 3800 |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) | 774 | 899 | 3350 | 3900 |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) | 749 | 924 | 3250 | 4000 |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) | 724 | 949 | 3150 | 4100 |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) | 699 | 974 | 3050 | 4200 |
| Analysis | There is no overlap of the band n5 ACLR range with the band n48 DL up to order 5. The sufficient rejection of the transmitter noise floor should be at band n48 so no cross-band MSD is needed. |  | There is no overlap of the band n48 ACLR range with the band n5 DL up to order 5. The sufficient rejection of the transmitter noise floor should be at band n5 so no cross-band MSD is needed. |  |

Based on the above table:

- There is no uplink band overlapping with the downlink in ACLR range. The existing diplexer isolation should provide sufficient rejection.

- 5.7		CA_n48-n66

- 5.7.1	Common for 1 band UL and 2 bands UL CA

- 5.7.1.1	Operating bands for CA

Table 5.7.1.1-1:  CA band combination of band n48+n66

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n48 | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz | TDD |
| n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

- 5.7.1.2	Channel bandwidths per operating band for CA

Table 5.7.1.2-1: Supported bandwidths per CA band combination of band n48+n66

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n48A-n66A | CA_n48A-n66A | n48 | See n48 channel bandwidths in Table 5.3.5-1 of 38.101-1 | 4 and 5 |
|  |  | n66 | See n66 channel bandwidths in Table 5.3.5-1 of 38.101-1 |  |
| NOTE 6: For this bandwidth, the minimum requirements are restricted to operation when carrier is configured as a downlink SCell part of CA configuration. |  |  |  |  |

- 5.7.1.3	UE co-existence studies for 1 band UL

#### 5.7.1.3.1	Co-existence studies for 2UL band with 1CC per band

Table 5.7.1.3.1-1 summarizes frequency ranges where harmonics and/or harmonic mixing occur for CA_n48-n66.

Table 5.7.1.3.1-1: UL harmonics and harmonic mixing analysis

| UL/DL |  | n66 | UL1 | UL2 | UL3 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| harmonics |  | fLow | 1710 | 3420 | 5130 | 6840 | 8550 |  |
| n48 | fLow | fHigh | 1780 | 3560 | 5340 | 7120 | 8900 |  |
| DL1 | 3550 | 3700 | N/A | D |  |  |  | UL Harmonic |
| DL2 | 7100 | 7400 |  | N/A |  | N/A | N/A | Harmonic Mixing |
| DL3 | 10650 | 11100 |  |  | N/A |  | N/A |  |
| DL4 | 14200 | 14800 |  | N/A | N/A | N/A | N/A |  |
| DL5 | 17750 | 18500 |  |  | N/A | N/A | N/A |  |
| Analysis |  |  | There is a direct hit from the band n66 2nd harmonic to the band n48 |  |  |  |  |  |
| UL/DL |  | n48 | UL13 | UL2 | UL3 | UL4 | UL5 | MSD type |
| harmonics |  | fLow | 3550 | 7100 | 10650 | 14200 | 17750 |  |
| n66 | fLow | fHigh | 3700 | 7400 | 11100 | 14800 | 18500 |  |
| DL1 | 2110 | 2200 | N/A |  |  |  |  | UL Harmonic |
| DL2 | 4220 | 4400 |  | N/A |  | N/A | N/A | Harmonic Mixing |
| DL3 | 6330 | 6600 |  |  | N/A |  | N/A |  |
| DL4 | 8440 | 8800 |  | N/A | N/A |  | N/A |  |
| DL5 | 10550 | 11000 |  |  | N/A | N/A | N/A |  |
| Analysis |  |  | There is no collision detected with both harmonic and harmonic mixing |  |  |  |  |  |

Based on the above table:

- For the uplink n66 2nd harmonic falling to the band n77, the MSD value has been defined in Table 7.3A.4-1 of 38.101-1. In fact, the band n66 channel bandwidth for BCS 4/5 is the same as BCS 2. Therefore, there is no newer requirement needed.

Table 5.7.1.3.1-2 summarizes frequency ranges where cross-band isolation issues may occur for both UL bands into the other band DL.

Table 5.7.1.3.1-2: Cross-band isolation analysis

| Bands | n48 |  | n66 |  |
| --- | --- | --- | --- | --- |
| Frequency limit | fx_low / min | fx_high / max | fy_low / min | fy_high / max |
| fUL (MHz) | 3550 | 3700 | 1710 | 1780 |
| fDL (MHz) | 3550 | 3700 | 2110 | 2200 |
| CBW (MHz) | 5 | 100 | 5 | 45 |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) | 3450 | 3800 | 1665 | 1825 |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) | 3350 | 3900 | 1620 | 1870 |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) | 3250 | 4000 | 1575 | 1915 |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) | 3150 | 4100 | 1530 | 1960 |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) | 3050 | 4200 | 1485 | 2005 |
| Analysis | There is no overlap of the band n48 ACLR range with the band n66 DL up to order 5. The sufficient rejection of the transmitter noise floor should be at band n66 so no cross-band MSD is needed. |  | There is no overlap of the band n66 ACLR range with the band n48 DL up to order 5. The sufficient rejection of the transmitter noise floor should be at band n48 so no cross-band MSD is needed. |  |

Based on the above table:

- There is no uplink band overlapping with the downlink in ACLR range. The existing diplexer isolation should provide sufficient rejection.

5.8		CA_n26-n78

5.8.1		Specific for 1 bands UL CA

5.8.1.1 Operating bands for CA

CA_n26-n78 has already been specified in TS 38.101-1 and this section does not need to be revisited.

5.8.1.2	Channel bandwidths per operating band for CA

UL CA_n78(2A) need to be added to DL CA_n26A-n78(2A) and DL CA_n26(2A)-n78(2A) in the existing configuration table of 38.101-1. See Table 5.8.1.2-2 below.

Table 5.8.1.2-2: Supported bandwidths per CA band combination of band n26+n78

| CA_n26A-n78(2A) | CA_n26A-n78A | n26 | 5, 10, 15, 20, 25, 30 | 0 |
| --- | --- | --- | --- | --- |
|  |  | n78 | CA_n78(2A)_BCS0 |  |
|  | CA_n78(2A) | n26 | n26 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |
| CA_n26(2A)-n78(2A) | CA_n26(2A) CA_n78(2A) CA_n26A-n78A | n26 | CA_n26(2A)_BCS0 | 0 |
|  |  | n78 | CA_n78(2A)_BCS0 |  |

5.8.1.3	Co-existence studies

Table 5.8.2.2-3 gives a non-contiguous uplink IMD interference analysis

Table 5.8.2.2-3: Non-contiguous uplink IMD analysis

| UE UL carriers | fx_low = 3300 | fx_high = 3320 | fy_low = 3780 | fy_high = 3800 |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 460 – 500 |  | 7080 – 7120 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2800 – 2860 |  | 4240 – 4300 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 10380 – 10440 |  | 10860 – 10920 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 6100 – 6180 |  | 8020 – 8100 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 920 – 1000 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 13680 – 13760 |  | 14640 – 14720 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 14160 – 14240 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 11800 – 11900 |  | 9400 – 9500 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4700 – 4800 |  | 2300 – 2400 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 18420 – 18520 |  | 16980 – 17080 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 17940 – 18040 |  | 17460 – 17560 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on the table 5.8.2.2-3, there are no IMD products falling inside n26 DL.

5.8.1.4	∆TIB and ∆RIB values

CA_n26-n78 has already been specified and this section does not need to be revisited.

5.8.1.5	REFSENs requirements

Based on the co-existence studies there is no need to define MSD values.

5.9		CA_n1-n71

5.9.1	Common for 1 band UL and 2 bands UL CA

5.9.1.1	Operating bands for CA

Table 5.9.1.1-1:  CA band combination of band n1+n71

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
| n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |

5.9.1.2	Channel bandwidths per operating band for CA

Table 5.9.1.2-1: Supported bandwidths per CA band combination of band n1+n71

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n71A | CA_n1A-n71A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |

5.9.1.3	UE co-existence studies

Table 5.9.1.3-1	 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_ n1-n71.

Table 5.9.1.3-1 UL/DL harmonics/harmonic mixing analysis

| UL/DL |  | n1 | UL12 | UL2 | UL33 | UL4 | UL5 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| harmonics |  | 1920 | 1920 | 3840 | 5760 | 7680 | 9600 | MSD type |
| n71 | 663 | 698 | 1980 | 3960 | 5940 | 7920 | 9900 |  |
| DL1 | 617 | 652 | N/A | Clear | Clear | Clear | Clear | UL harmonic |
| DL23 | 1234 | 1304 | Clear | N/A | Clear | N/A | N/A | Harmonic mixing |
| DL34 | 1851 | 1956 | D | Clear | N/A | Clear | N/A |  |
| DL4 | 2468 | 2608 | Clear | N/A | N/A | N/A | N/A |  |
| DL54 | 3085 | 3260 | Clear | Clear | N/A | N/A | N/A |  |
| Analysis |  |  | There is UL1/DL3 harmonic mixing into n71 |  |  |  |  |  |
| UL/DL |  | n71 | UL14 | UL2 | UL33 | UL4 | UL5 | MSD type |
| harmonics |  | 663 | 663 | 1326 | 1989 | 2652 | 3315 |  |
| n1 | 1920 | 1980 | 698 | 1396 | 2094 | 2792 | 3490 |  |
| DL1 | 2110 | 2170 | N/A | Clear | Clear | Clear | Clear | UL harmonic |
| DL23 | 4220 | 4340 | Clear | N/A | Clear | N/A | N/A | Harmonic mixing |
| DL34 | 6330 | 6510 | Clear | Clear | N/A | Clear | N/A |  |
| DL4 | 8440 | 8680 | Clear | N/A | N/A | N/A | N/A |  |
| DL54 | 10550 | 10850 | Clear | Clear | N/A | N/A | N/A |  |
| Analysis |  |  | No issues |  |  |  |  |  |
| Note 1: ULx means UL xth harmonic frequency, and DLy means DL yth harmonic frequency range Note 2: When a collision is detected with an overlap >0Hz between the ULx with DLy frequency ranges, the ULx/DLy cell is marked “D” for direct hit. When the gap between ULx and DLy frequency range is from 0Hz to x*MinULCBW, the ULx/DLy cell is marked “N” for Near miss. Note 3: UL3/DL2 harmonic mixing direct hit case for PC3/5 only apply for DL>3GHz Note 4: For harmonic mixing, near-miss cases only apply for UL1 and odd DLy orders. |  |  |  |  |  |  |  |  |

Table 5.9.1.3-2 summarizes frequency ranges where cross band isolation may occur for CA_n1-n71.

| Bands | n1 |  | n71 |  |
| --- | --- | --- | --- | --- |
| Frequency limit | flow | fhigh | flow | fhigh |
| fUL (MHz) | 1920 | 1980 | 663 | 698 |
| fDL (MHz) | 2110 | 2170 | 617 | 652 |
| UL CBW (MHz)2 | Minimum CBW | Maximum CBW | Minimum CBW | Maximum CBW |
|  | 5 | 50 | 5 | 30 |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) | 1870 | 2030 | 633 | 728 |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) | 1820 | 2080 | 603 | 758 |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) | 1770 | 2130 | 573 | 788 |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) | 1720 | 2180 | 543 | 818 |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) | 1670 | 2230 | 513 | 848 |
| Analysis | No overlap |  | No overlap |  |
| NOTE 1: Even if there is no overlap up to ACLR5, MSD beyond the ACLR5 range should be evaluated further if: - The UL aggressor band and DL aggressor band are part of the same or adjacent band group as described in table A.1 - If the DL band is above the UL band, it’s lower frequency edge must be below the UL lowest 2nd harmonic frequency - As an indicative threshold, if >45dB UL rejection at the DL band frequency can be guaranteed, assuming a -130dBm/Hz TX noise floor level, the transmitter noise floor related MSD should be negligible NOTE 2: The maximum UL channel bandwidth of the BCS (noted maxULCBW) is used to calculate the band ACLR ranges while the minimum DL channel bandwidth of the BCS (noted minDLCBW) is used for the DL band victim channel bandwidth. |  |  |  |  |

5.9.1.4	∆TIB,c and ∆RIB,c values

For CA_n1-n71, the TIB,c and RIB,c values are given in the tables below re-used from CA_n1-n105.

Table 5.9.1.4-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| CA_n1-n71 | 0.3 | 0.6 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.9.1.4-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| CA_n1-n71 | - | 0.3 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.9.1.5	REFSENS requirements

Based on the co-existence studies n1 uplink and n71 third harmonic mixes into the n71 downlink. The value is based on DC_1A_n71A, which has similar harmonic issue.

Table 5.9.1.5-2: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC3 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n1 | n713 | 5 | 15 | 25 (RBstart=0) | 5 | 26.8 | NOTE 4 | UL1/DL3 |
| NOTE 3: These requirements apply when there is at least one individual RE within the downlink transmission bandwidth of the victim (lower) band for which the 3rd harmonic is within the uplink transmission bandwidth or the uplink adjacent channel's transmission bandwidth of an aggressor (higher) band. NOTE 4: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |  |

5.9.1.6	OOB blocking exception requirements

No additional OOB blocking exceptions are required for this CA band combination.

5.9.2	Specific for 2 bands UL CA

5.9.2.1	Maximum output power for inter-band CA

Table 5.9.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n1A-n71A | 23 | +2/-3 |

5.9.2.2	UE co-existence studies

Table 5.9.2.2-1 lists Band n1 + Band n71 2 bands UL CA(2CC) 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.9.2.2-1: Band n1 and Band n71 for 2CC UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1317 - 1222 |  | 2583 - 2678 |  |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3142 - 3297 |  | 654 - 524 |  |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4503 - 4658 |  | 3246 - 3376 |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 1890 - 2010 |  | 613 - 748 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 5062 - 5277 |  | 9 - 174 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2444 - 2634 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6423 - 6638 |  | 3909 - 4074 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 5236 - 5286 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 872 - 672 |  | 7257 - 6982 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1746 - 1971 |  | 4614 - 4364 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4572 - 4772 |  | 8343 - 8618 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5829 - 6054 |  | 7086 - 7336 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on Table 5.9.2.2-1, 3rd order IMD may also fall into Rx frequencies of bands n1 or band n71. The analysis shows IMD3 will fall inside n71 DL.

As agreed, there’s no regulatory requirements that need to be protected in the geographically area that this combination is used.

5.9.2.3	REFSENS requirements

Table 5.9.2.3-1 lists the MSD required due to the 3rd IMD product for the dual uplink configuration. It is based on the similar case of DC_1A_n71A.

Table 5.9.2.3-1: MSD due to IMD issue

| Operating band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n71 | n1 | 1958 | 5 | 25 | 2148 | N/A | FDD | N/A |
|  | n71 | 668 | 5 | 25 | 622 | 15.1 | FDD | IMD3 |

## 5.10	CA_n3-n71

### 5.10.1	Common for 1 band UL and 2 bands UL CA

#### 5.10.1.1	Operating bands for CA

Table 5.10.1.1-1:  CA band combination of band n3+n71

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
| n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |

#### 5.10.1.2	Channel bandwidths per operating band for CA

Table 5.10.1.2-1: Supported bandwidths per CA band combination of band n3+n71

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n71A | CA_n3A-n71A | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |

#### 5.10.1.3	UE co-existence studies

Table 5.10.1.3-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_ n3-n71.

Table 5.10.1.3-1 UL/DL harmonics/harmonic mixing analysis

| UL/DL |  | n1 | UL12 | UL2 | UL33 | UL4 | UL5 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| harmonics |  | 1710 | 1710 | 3420 | 5130 | 6840 | 8550 | MSD type |
| n71 | 663 | 698 | 1785 | 3570 | 5355 | 7140 | 8925 |  |
| DL1 | 617 | 652 | N/A | Clear | Clear | Clear | Clear | UL harmonic |
| DL23 | 1234 | 1304 | Clear | N/A | Clear | N/A | N/A | Harmonic mixing |
| DL34 | 1851 | 1956 | Clear | Clear | N/A | Clear | N/A |  |
| DL4 | 2468 | 2608 | Clear | N/A | N/A | N/A | N/A |  |
| DL54 | 3085 | 3260 | Clear | Clear | N/A | N/A | N/A |  |
| Analysis |  |  | No issues |  |  |  |  |  |
| UL/DL |  | n71 | UL14 | UL2 | UL33 | UL4 | UL5 | MSD type |
| harmonics |  | 663 | 663 | 1326 | 1989 | 2652 | 3315 |  |
| n1 | 1710 | 1785 | 698 | 1396 | 2094 | 2792 | 3490 |  |
| DL1 | 1805 | 1880 | N/A | Clear | Clear | Clear | Clear | UL harmonic |
| DL23 | 3610 | 3760 | Clear | N/A | Clear | N/A | N/A | Harmonic mixing |
| DL34 | 5415 | 5640 | Clear | Clear | N/A | Clear | N/A |  |
| DL4 | 7220 | 7520 | Clear | N/A | N/A | N/A | N/A |  |
| DL54 | 9025 | 9400 | Clear | Clear | N/A | N/A | N/A |  |
| Analysis |  |  | No issues |  |  |  |  |  |
| Note 1: ULx means UL xth harmonic frequency, and DLy means DL yth harmonic frequency range Note 2: When a collision is detected with an overlap >0Hz between the ULx with DLy frequency ranges, the ULx/DLy cell is marked “D” for direct hit. When the gap between ULx and DLy frequency range is from 0Hz to x*MinULCBW, the ULx/DLy cell is marked “N” for Near miss. Note 3: UL3/DL2 harmonic mixing direct hit case for PC3/5 only apply for DL>3GHz Note 4: For harmonic mixing, near-miss cases only apply for UL1 and odd DLy orders. |  |  |  |  |  |  |  |  |

Table 5.10.1.3-2 summarizes frequency ranges where cross band isolation may occur for CA_n3-n71.

| Bands | n3 |  | n71 |  |
| --- | --- | --- | --- | --- |
| Frequency limit | flow | fhigh | flow | fhigh |
| fUL (MHz) | 1710 | 1785 | 663 | 698 |
| fDL (MHz) | 1805 | 1880 | 617 | 652 |
| UL CBW (MHz)2 | Minimum CBW | Maximum CBW | Minimum CBW | Maximum CBW |
|  | 5 | 50 | 5 | 30 |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) | 1660 | 1835 | 633 | 728 |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) | 1610 | 1885 | 603 | 758 |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) | 1560 | 1935 | 573 | 788 |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) | 1510 | 1985 | 543 | 818 |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) | 1460 | 2035 | 513 | 848 |
| Analysis | No overlap |  | No overlap |  |
| NOTE 1: Even if there is no overlap up to ACLR5, MSD beyond the ACLR5 range should be evaluated further if: - The UL aggressor band and DL aggressor band are part of the same or adjacent band group as described in table A.1 - If the DL band is above the UL band, it’s lower frequency edge must be below the UL lowest 2nd harmonic frequency - As an indicative threshold, if >45dB UL rejection at the DL band frequency can be guaranteed, assuming a -130dBm/Hz TX noise floor level, the transmitter noise floor related MSD should be negligible NOTE 2: The maximum UL channel bandwidth of the BCS (noted maxULCBW) is used to calculate the band ACLR ranges while the minimum DL channel bandwidth of the BCS (noted minDLCBW) is used for the DL band victim channel bandwidth. |  |  |  |  |

5.10.1.4	∆TIB,c and ∆RIB,c values

For CA_n3-n71, the TIB,c and RIB,c values are given in the tables below re-used from DC_3_n71.

Table 5.10.1.4-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| CA_n3-n71 | 0.3 | 0.3 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.10.1.4-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| CA_n3-n71 | - | - |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

#### 5.10.1.5	REFSENS requirements

Based on the co-existence there is no requirements to REFSENS.

#### 5.10.1.6	OOB blocking exception requirements

No additional OOB blocking exceptions are required for this CA band combination.

### 5.10.2	Specific for 2 bands UL CA

#### 5.10.2.1	Maximum output power for inter-band CA

Table 5.10.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n3A-n71A | 23 | +2/-3 |

#### 5.10.2.2	UE co-existence studies

Table 5.10.2.2-1 lists Band n3 + Band n71 2UL bands CA 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.10.2.2-1: Band n3 and Band n71 for 2CC UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1122 - 1012 |  | 2373 - 2483 |  |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2722 - 2907 |  | 459 - 314 |  |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4083 - 4268 |  | 3036 - 3181 |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 1680 - 1815 |  | 613 - 748 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4432 - 4692 |  | 204 - 384 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 2024 - 2244 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5793 - 6053 |  | 3699 - 3879 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 4816 - 4896 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1082 - 867 |  | 6477 - 6142 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1326 - 1581 |  | 4029 - 3734 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4362 - 4577 |  | 7503 - 7838 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5409 - 5664 |  | 6456 - 6751 |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

The analysis shows there’s no IMD issues.

As agreed, there’s no regulatory requirements that need to be protected in the geographically area that this combination is used.

#### 5.10.2.3	REFSENS requirements

There is no additional REFSENS requirements.

- 5.11	CA_n20-n77

- 5.11.1	Common for 1 band UL and 2 bands UL CA

- 5.11.1.1	Operating bands for CA

Table 5.11.1.1-1:  CA band combination of band n20+n77

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
| n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

- 5.11.1.2	Channel bandwidths per operating band for CA

Table 5.11.1.2-1: Supported bandwidths per CA band combination of band n20+n77

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n20A-n77A | CA_n20A-n77A | n20 | 5, 10, 15, 20 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n20A-n77(2A) | CA_n20A-n77A | n20 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |

- 5.11.1.3	UE co-existence studies

Table 5.11.1.3-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n20-n77.

Table 5.11.1.3-1 UL/DL harmonics/harmonic mixing analysis

| UL/DL |  | n20 | UL12 | UL2 | UL33 | UL4 | UL5 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| harmonics |  | 832 | 832 | 1664 | 2496 | 3328 | 4160 | MSD type |
| n77 | 3300 | 4200 | 862 | 1724 | 2586 | 3448 | 4310 |  |
| DL1 | 3300 | 4200 | N/A | Clear | Clear | D | D | UL harmonic |
| DL23 | 6600 | 8400 | Clear | N/A | Clear | N/A | N/A | Harmonic mixing |
| DL34 | 9900 | 12600 | Clear | Clear | N/A | Clear | N/A |  |
| DL4 | 13200 | 16800 | Clear | N/A | N/A | N/A | N/A |  |
| DL54 | 16500 | 21000 | Clear | Clear | N/A | N/A | N/A |  |
| Analysis |  |  | UL4 and UL5 falls inside n77 DL |  |  |  |  |  |
| UL/DL |  | n77 | UL14 | UL2 | UL33 | UL4 | UL5 | MSD type |
| harmonics |  | 3300 | 3300 | 6600 | 9900 | 13200 | 16500 |  |
| n20 | 832 | 862 | 4200 | 8400 | 12600 | 16800 | 21000 |  |
| DL1 | 791 | 821 | N/A | Clear | Clear | Clear | Clear | UL harmonic |
| DL23 | 1582 | 1642 | Clear | N/A | Clear | N/A | N/A | Harmonic mixing |
| DL34 | 2373 | 2463 | Clear | Clear | N/A | Clear | N/A |  |
| DL4 | 3164 | 3284 | Clear | N/A | N/A | N/A | N/A |  |
| DL54 | 3955 | 4105 | D | Clear | N/A | N/A | N/A |  |
| Analysis |  |  | DL5 falls inside n77 DL |  |  |  |  |  |
| Note 1: ULx means UL xth harmonic frequency, and DLy means DL yth harmonic frequency range Note 2: When a collision is detected with an overlap >0Hz between the ULx with DLy frequency ranges, the ULx/DLy cell is marked “D” for direct hit. When the gap between ULx and DLy frequency range is from 0Hz to x*MinULCBW, the ULx/DLy cell is marked “N” for Near miss. Note 3: UL3/DL2 harmonic mixing direct hit case for PC3/5 only apply for DL>3GHz Note 4: For harmonic mixing, near-miss cases only apply for UL1 and odd DLy orders. |  |  |  |  |  |  |  |  |

Table 5.11.1.3-2 summarizes frequency ranges where cross band isolation may occur for CA_n20-n77.

Table 5.11.1.3-2: Cross-band isolation analysis

| Bands | n20 |  | n77 |  |
| --- | --- | --- | --- | --- |
| Frequency limit | flow | fhigh | flow | fhigh |
| fUL (MHz) | 832 | 862 | 3300 | 4200 |
| fDL (MHz) | 791 | 821 | 3300 | 4200 |
| UL CBW (MHz)2 | Minimum CBW | Maximum CBW | Minimum CBW | Maximum CBW |
|  | 5 | 20 | 10 | 100 |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) | 812 | 882 | 3200 | 4300 |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) | 792 | 902 | 3100 | 4400 |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) | 772 | 922 | 3000 | 4500 |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) | 752 | 942 | 2900 | 4600 |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) | 732 | 962 | 2800 | 4700 |
| Analysis | No overlap |  | No overlap |  |
| NOTE 1: Even if there is no overlap up to ACLR5, MSD beyond the ACLR5 range should be evaluated further if: - The UL aggressor band and DL aggressor band are part of the same or adjacent band group as described in table A.1 - If the DL band is above the UL band, it’s lower frequency edge must be below the UL lowest 2nd harmonic frequency - As an indicative threshold, if >45dB UL rejection at the DL band frequency can be guaranteed, assuming a -130dBm/Hz TX noise floor level, the transmitter noise floor related MSD should be negligible NOTE 2: The maximum UL channel bandwidth of the BCS (noted maxULCBW) is used to calculate the band ACLR ranges while the minimum DL channel bandwidth of the BCS (noted minDLCBW) is used for the DL band victim channel bandwidth. |  |  |  |  |

5.11.1.4	∆TIB,c and ∆RIB,c values

For CA_n20-n41, the TIB,c and RIB,c values are given in the tables below re-used from DC_20_n77.

Table 5.11.1.4-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| CA_n20-n77 | 0.6 | 0.8 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.11.1.4-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| CA_n20-n77 | - | 0.5 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

- 5.11.1.5	REFSENS requirements

Based on the co-existence studies n20 fourth and fifth harmonic uplink falls into the n77 downlink. The UL harmonic MSD value is based on CA_n20A_n78A, from current meeting document R4-2413019 CR to TS 38.101-1 Rel-18 NR CA Uplink Harmonic clean-up PC3.

Table 5.11.1.5-1: Reference sensitivity exceptions and uplink/downlink configurations due to UL harmonic from a PC3 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n20 | n77 | 5 | 15 | 6 | 10 | 10.8 | NOTE 4 | UL4/DL1 direct-hit |
| n20 | n77 | 5 | 15 | 6 | 100 | 3.1 | NOTE 4 | UL4/DL1 direct-hit |
| NOTE 4: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |

Based on the co-existence studies n77 falls into the fifth harmonic n20 downlink. The harmonic mixing MSD value is based on CA_n28A_n77A, from current meeting document R4-2412621 CR for EN-DC Harmonic Mixing clean-up PC3.

Table 5.11.1.5-2: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC3 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n77 | 20 | 10 | 15 | 25 | 5 | 31 | NOTE 2 | UL1/DL5 |
| n77 | 20 | 10 | 15 | 25 | 20 | 23.5 | NOTE 2 | UL1/DL5 |
| NOTE 2: The requirements should be verified for DL NR-ARFCN of the Victim (lower) band (superscript LB) such that fDLLB=fULHB/0.150.1 and FUL_lowHB+BWCℎannelHB/2≤fULHB≤FUL_ℎigℎHB−BWCℎannelHB/2 with fULHB the UL carrier frequency and BWCℎannelHB the channel bandwidth configured in the higher band, both in MHz. |  |  |  |  |  |  |  |  |

- 5.11.1.6	OOB blocking exception requirements

No additional OOB blocking exceptions are required for this CA band combination.

- 5.11.2	Specific for 2 bands UL CA

- 5.11.2.1	Maximum output power for inter-band CA

Table 5.11.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n20A-n77A | 23 | +2/-3 |

- 5.11.2.2	UE co-existence studies

Table 5.11.2.2-1 lists Band n20 + Band n77 2UL bands CA 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis, where only n20 can fall victim, since it is FDD+TDD.

Table 5.11.2.2-1: Band n20 and Band n41 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |  |
| --- | --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 2438 - 3368 |  | 4132 - 5062 |  |  |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 2536 - 1576 |  | 5738 - 7568 |  |  |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 4964 - 5924 |  | 7432 - 9262 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |  |
| IMD frequency limits (MHz) | 732 - 762 |  | 3280 - 4180 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |  |
| IMD frequency limits (MHz) | 1704 - 714 |  | 9038 - 11768 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | 6736 - 4876 |  |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |  |
| IMD frequency limits (MHz) | 5796 - 6786 |  | 10732 - 13462 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |  |
| IMD frequency limits (MHz) | 8264 - 10124 |  |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |  |
| IMD frequency limits (MHz) | 15968 - 12338 |  | 148 - 872 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |  |
| IMD frequency limits (MHz) | 10936 - 8176 |  | 4014 - 5904 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |  |
| IMD frequency limits (MHz) | 14032 - 17662 |  | 6628 - 7648 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |  |
| IMD frequency limits (MHz) | 11564 - 14324 |  | 9096 - 10986 |  |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. |  |  |  |  |

The analysis shows IMD4 and IMD5 will fall inside n20 DL.

Based on the table 5.11.2.2-1, the 4th order and 5th order intermodulation product fall inside the RX band of n20.

The two bands are not part of the same or adjacent band groups, so the Triple beat analysis is omitted.

As agreed, there’s no regulatory requirements that need to be protected in the geographically area that this combination is used.

- 5.11.2.3	REFSENS requirements

Table 5.11.2.3-1 lists the MSD required due to the 4rd and 5th IMD product for the dual uplink configuration. It is based on the similar case of DC_20A_n77A.

Table 5.11.2.3-1: MSD due to IMD issue

| Operating band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n20-n77 | n20 | 850 | 5 | 25 | 809 | 11 | FDD | IMD4 |
|  | n77 | 3359 | 10 | 50 | 3359 | N/A | TDD | N/A |
|  | n20 | 840 | 5 | 25 | 790.5 | 6.5 | FDD | IMD5 |
|  | n77 | 4159 | 10 | 50 | 4159 | N/A | TDD | N/A |

6	Dual Connectivity: Specific Band Combination Part

6.x	DC_nX-nY

Editor's note: The texts for NR DC can only be added together with the texts for the corresponding inter-band 2 bands UL NR CA above, which means TP to TR to include pure NR DC configuration is not allowed.

6.x.1	Configurations for DC_nX-nY

Table 6.x.1-1: Inter-band NR DC configurations

| NR DC configuration | Uplink NR DC configuration |
| --- | --- |
| DC_nXA-nYA | DC_nXA-nYA |
| DC_nX(A/B/C(2A)-nY(A/B/C(2A) | DC_nXA/B/C-nYA |

6.x.2	Maximum output power for NR-DC

Table 6.x.2-1: UE Power Class for uplink inter-band DC

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_nXA-nYA | 23 | +x/-y |
| DC_nXA/B/C-nYA | 23 | +x/-y |

Annex A:		Band group definition

For the following MSD issues detection, a criterion applies in terms of band group as defined in Table A.1:

-   For 2DL 1UL/1CC cross-band isolation, and for the Tx noise floor MSD to be considered, the two bands should be part of the same or adjacent band group.

-   For 2DL 2UL/3CC triple beat MSD to be considered, the two bands should be part of the same or adjacent band group.

Table A.1: Band group definition for same or adjacent band-group criterion

| FR1 band group range |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name | FR1-a | FR1-b | FR1-c | FR1-d | FR1-e |
| Range (MHz) | 600-1000 | 1400-2200 | 2300-2700 | 3300-5000 | 5150-7125 |
| Duplex mode | Mostly FDD | Mostly FDD | FDD and TDD | TDD only | TDD only |

Annex B:		Valid UL configurations

For CA_nX-nY two band DL inter band combinations, the following UL configurations are applicable:

-  One band UL with one CC: nXA, nYA

-  One band UL with two CC: CA_nXB, CA_nXC, CA_nX(2A), CA_nYB, CA_nYC, CA_nY(2A)

-  two band UL with one CC per band: CA_nXA-nYA

-  two band UL with two CC in one band: CA_nXB-nYA, CA_nXC-nYA, CA_nXA-nYB, CA_nXA-nYC

The following three UL cluster cases are not supported: CA_nX(2A)-nYA, CA_nXA-nY(2A)

Combinations with four UL CCs are not supported.

Annex C:	Void

Annex D:	Void

Annex E (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2024-08 | RAN4 #112 | R4-2411938 |  |  |  | TR skeleton | v0.0.0 |
| 2024-08 | RAN4 #112 | R4-2411939 |  |  |  | Some updates in the tables 5.x.1.3-1/2/3 and Table 5.x.2.2-1/2 for the skeletion. Including the following approved TP in RAN4 #112 meeting: R4-2414326 TP for TR 38.719-02-01: PC3 NR BCS4/5 inter-band CA R4-2414328 TP for TR38.719-02-01_CA_n3A-n104A R4-2414329 TP for TR38.719-02-01_CA_n8A-n104A R4-2414330 TP to TR 38.719-02-01 adding CA_n78(2A) UL to CA_n26A-n78(2A) and CA_n26(2A)-n78(2A) DL R4-2414336 TP to TR 38.719-02-01 Addition of CA_n1A-n71A R4-2414337 TP to TR 38.719-02-01 Addition of CA_n3A-n71A R4-2414338 TP to TR 38.719-02-01 Addition of CA_n20A_n77A | v0.1.0 |