3GPP TSG-RAN WG4 Meeting #112	R4-2413130

# Maastricht, Netherlands, 19 – 23 August 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-5 | CR | 0121 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_NTN_solutions-Core) CR to TS 38.101-5: variable duplex distance |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions-Core |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RAN2 specifications allow configuration UL and DL center frequency independently. Normally RAN4 defines minimum requirements with a single default Tx-Rx separation. Satellite industry has a strong need to use different Tx-Rx spacing in different deployments. Therefore, this CR adds a requirement to validate that different Tx-Rx spacing is supported. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of new Tx-Rx separation for n255 and n256 for 20 MHz and 5 MHz channel bandwidth. Addition of reference sensitivity for these Tx-Rx separations. Removal of square brackets from n255 reference sensitivity |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | While RAN2 specifications allow configuring UL and DL center frequency independently, there will no requirement to verify this functionality for NR NTN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.4, 7.1, 7.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-5 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change 1>

### 5.4.4	TX–RX frequency separation

The default TX channel (carrier centre frequency) to RX channel (carrier centre frequency) separation for operating bands is specified in Table 5.4.4-1.

Table 5.4.4-1: UE TX-RX frequency separation (FR1-NTN)

| NTN Satellite Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| n256 | 190 MHz1 165 to 215 MHz2 |
| n255 | -101.5 MHz1 -72.5 to -130.5 MHz2 |
| n254 | 862 – 885 MHz |
| NOTE 1: Default Tx-Rx separation NOTE 2: The verification of flexible Tx-Rx frequency separation within this range is limited to reference sensitivity. Further details are specified in clause 7.3.2 |  |

# 6	Conducted transmitter characteristics

<End of change 1>

<Start of change 1>

## 7.1	General

Unless otherwise stated the receiver characteristics are specified at the antenna connector(s) of the UE. For UE(s) with an integral antenna only, a reference antenna(s) with a gain of 0 dBi is assumed for each antenna port(s). UE with an integral antenna(s) may be taken into account by converting these power levels into field strength requirements, assuming a 0 dBi gain antenna. For UEs with more than one receiver antenna connector, identical interfering signals shall be applied to each receiver antenna port if more than one of these is used (diversity).

The levels of the test signal applied to each of the antenna connectors shall be as defined in the respective clauses below.

With the exception of clause 7.3, the requirements shall be verified with the network signalling value NS_01 configured in Table 6.2.3.1-1.

All requirements in this section are applicable to devices supporting GSO and/or NGSO satellites.

All the parameters in clause 7 are defined using the UL reference measurement channels specified in 3GPP TS 38.101-1 [5] Annex A.2.2, the DL reference measurement channels specified in 3GPP TS 38.101-1 [5] Annex A.3.2 and using the set-up specified in 3GPP TS 38.101-1 [5] Annex C.3.1.

All Rx requirements are verified with default Tx-Rx separation specified in Table 5.4.4-1. Additional Tx-Rx frequency separations specified in clause 7.3.2 do not apply to other requirements.

## 7.2	Diversity characteristics

<End of change 1>

<Start of change 2>

### 7.3.2	Reference sensitivity power level

The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annex A3.2.2 of 3GPP TS 38.101-1 [5], with parameters specified in Table 7.3.2-1.

Table 7.3.2-1: Two antenna port reference sensitivity QPSK PREFSENS for FDD bands

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
|  | 15 | -99.5 | -96.3 | -94.5 | -93.3 |  |  |  |  |  |  |
| n256 | 30 |  | -96.6 | -94.6 | -93.5 |  |  |  |  |  |  |
|  | 60 |  | -97.0 | -94.9 | -93.7 |  |  |  |  |  |  |
|  | 15 | -100.0 | -96.8 | -95.0 | -93.8 |  |  |  |  |  |  |
| n255 | 30 |  | -97.1 | -95.1 | -94.0 |  |  |  |  |  |  |
|  | 60 |  | -97.5 | -95.4 | -94.2 |  |  |  |  |  |  |
|  | 15 | -99.5 | -96.3 | -94.5 |  |  |  |  |  |  |  |
| n254 | 30 |  | -96.6 | -94.6 |  |  |  |  |  |  |  |
|  | 60 |  | -97.0 | -94.9 |  |  |  |  |  |  |  |
| NOTE：The transmitter shall be set to PUMAX as defined in clause 6.2.4 of 3GPP TS 38.101-1 [5]. |  |  |  |  |  |  |  |  |  |  |  |

The reference receiver sensitivity (REFSENS) requirement specified in Table 7.3.2-1 shall be met with uplink transmission bandwidth less than or equal to that specified in Table 7.3.2-2 and with default Tx-Rx carrier center frequency separation except for cases specified in Table 7.3.2-3.

Table 7.3.2-2: Uplink configuration for reference sensitivity

| Operating band / SCS (kHz) / Channel bandwidth (MHz) / Duplex mode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS | 5 | 10 | 15 | 20 | Duplex Mode |
|  | 15 | 25 | 50 | 75 | 100 |  |
| n256 | 30 |  | 24 | 36 | 50 | FDD |
|  | 60 |  | 10 | 18 | 24 |  |
|  | 15 | 25 | 50 | 75 | 752 503 |  |
| n255 | 30 |  | 24 | 36 | 362 243 | FDD |
|  | 60 |  | 10 | 18 | 182 103 |  |
|  | 15 | 25 | 50 | 75 |  |  |
| n254 | 30 |  | 24 | 36 |  | FDD |
|  | 60 |  | 10 | 18 |  |  |
| NOTE 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth in Table 5.3.2-1. NOTE 2: Applicable for Tx-Rx frequency separation of -101.5 MHz and -115.5 MHz. NOTE 3: Applicable for Tx-Rx frequency separation of -87.5 MHz. |  |  |  |  |  |  |

Table 7.3.2-3: TX – RX carrier centre frequency separation for REFSENS verification

| Operating Band | Channel bandwidth | TX – RX carrier centre frequency separation for REFSENS verification |
| --- | --- | --- |
| n256 | 5 MHz | 165 MHz, 215 MHz |
|  | 20 MHz | 180 MHz, 200 MHz |
| n255 | 5 MHz | -72.5 MHz, -130.5 MHz |
|  | 20 MHz | -87.5 MHz, -115.5 MHz |

The minimum requirements specified in Table 7.3.2-1 shall be verified with the network signalling value NS_01 configured in Table 6.2.3.1-1.

## 7.4	Maximum input level

<End of change 2>