# 3GPP TSG-RAN WG4 Meeting #112		R4-2413174

Maastricht, Netherlands, 19th – 23rd August, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 7341 | rev |  | Current version: | 17.13.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NB_IOT-Perf) CR on RSRP-ThresholdsNPRACH-InfoList for NB-IoT (Cat-A Rel-17) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOT-Perf |  |  |  |  |  | Date: |  |  | 2024-08-19 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to TS36.331 (RRC spec), RSRP-ThresholdsNPRACH-InfoList-NB in NPRACH-ConfigSIB-NB IE refers to RSRP-Range, of which range is from 0 to 97, instead of NRSRP-Range-NB. The above is in line with TS36.321 (MAC spec), which say ‘(LTE) rsrp-ThresholdsPrachInfoList should be used for the NB-IoT Random Access procedure .’ Since the RSRP threshold values defined in the NB-IoT test cases should remain the same, the indices of RSRP-ThresholdsNPRACH-InfoList should be reduced by 16 according to the RSRP index value difference between Table 9.1.22.9-1 (NRSRP measurement report mapping) and Table 9.1.4-1 (RSRP measurement report mapping). Otherwise, a DUE may send a PRACH in a wrong CE level, which would lead to the test case failure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For the following test cases, correct the indices of RSRP-ThresholdsNPRACH-InfoList and chaned the reference clause, in the corresponding tables, from 9.1.22.9 to 9.1.4-1. - A.6.2.16, A.6.2.17 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | DUE might send a PRACH in a wrong CE level, which would lead to the test case failure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.6.2.16, A.6.2.17 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 36.533 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# <Start of Change 1>

Table A.6.2.16.1-4: NPRACH-Configuration parameters for HD-FDD contention based random access test for UE category NB1 In-Band mode in Normal Coverage

| Field | Value |  |  |  | Comment |  |
| --- | --- | --- | --- | --- | --- | --- |
| Parameters not per NPRACH resource |  |  |  |  |  |  |
| RSRP-ThresholdsNPRACH-InfoList | {24, 39} |  |  |  | Corresponding to {-116, -101} dBm as defined in Table 9.1.4-1 |  |
| nprach-CP-Length | us66dot7 |  |  |  |  |  |
| nrs-Power | -5 dBm/15 kHz |  |  |  | As defined in clause 6.7.3 in TS 36.331. |  |
| Backoff Parameter Index | 1 |  |  |  | As defined in table 7.2-2 in TS 36.321 |  |
| Configured UE transmitted power () | 23 dBm for power class 3, 20 dBm for power class 5, 14 dBm for power class 6 |  |  |  | As defined in clause 6.2.5F in TS 36.101 |  |
| powerRampingStep | dB2 |  |  |  |  |  |
| preambleInitialReceivedTargetPower | dBm-112 |  |  |  |  |  |
| preambleTransMax-CE | n6 |  |  |  |  |  |
| Parameters per NPRACH Resource |  |  |  |  |  |  |
| NPRACH Resource | Level 0 | Level 1 | Level 2 |  |  |
| nprach-Periodicity | ms40 | ms240 | ms1280 |  |  |
| nprach-StartTime | ms8 | ms64 | ms512 |  |  |
| nprach-SubcarrierOffset | n0 | n0 | n0 |  |  |
| nprach-NumSubcarriers | n12 | n12 | n12 |  |  |
| nprach-SubcarrierMSG3-RangeStart | zero | zero | zero |  |  |
| maxNumPreambleAttemptCE | n3 | n6 | n10 |  |  |
| numRepetitionsPerPreambleAttempt | n2 | n8 | n64 |  |  |
| npdcch-NumRepetitions-RA | r4 | r16 | r128 |  |  |
| npdcch-StartSF-CSS-RA | v1dot5 | v1dot5 | v1dot5 |  |  |
| npdcch-Offset-RA | zero | zero | Zero |  |  |
| nprach-NumCBRA-StartSubcarriers | n8 | n8 | n8 |  |  |
| ra-ResponseWindowSize (per NPRACH Resource) | pp2 | pp2 | pp2 |  |  |
| mac-ContentionResolutionTimer (per NPRACH Resource) | pp8 | pp8 | pp8 |  |  |
| Note 1: See Clause 6.7.3 in TS 36.331 for further information on the parameters in this table. |  |  |  |  |  |  |

<End of Change 1>

# <Start of Change 2>

Table A.6.2.17.1-4: NPRACH-Configuration parameters for HD-FDD contention based random access test for UE category NB1 In-Band mode in Enhanced Coverage

| Field | Value |  |  |  | Comment |  |
| --- | --- | --- | --- | --- | --- | --- |
| Parameters not per NPRACH resource |  |  |  |  |  |  |
| RSRP-ThresholdsNPRACH-InfoList | {19, 40} |  |  |  | Corresponding to {-121, -100} dBm as defined in Table 9.1.4-1 |  |
| nprach-CP-Length | us266dot7 |  |  |  |  |  |
| nrs-Power | -5 dBm/15 kHz |  |  |  | As defined in clause 6.7.3 in TS 36.331. |  |
| Backoff Parameter Index | 1 |  |  |  | As defined in table 7.2-2 in TS 36.321 |  |
| Configured UE transmitted power () | 23 dBm for power class 3, 20 dBm for power class 5, 14 dBm for power class 6 |  |  |  | As defined in clause 6.2.5F in TS 36.101 |  |
| powerRampingStep | dB2 |  |  |  |  |  |
| preambleInitialReceivedTargetPower | dBm-120 |  |  |  |  |  |
| preambleTransMax-CE | n6 |  |  |  |  |  |
| Parameters per NPRACH Resource |  |  |  |  |  |  |
| NPRACH Resource | Level 0 | Level 1 | Level 2 |  |  |
| nprach-Periodicity | ms40 | ms240 | ms1280 |  |  |
| nprach-StartTime | ms8 | ms64 | ms512 |  |  |
| nprach-SubcarrierOffset | n0 | n0 | n0 |  |  |
| nprach-NumSubcarriers | n12 | n12 | n12 |  |  |
| nprach-SubcarrierMSG3-RangeStart | zero | zero | zero |  |  |
| maxNumPreambleAttemptCE | n3 | n6 | n10 |  |  |
| numRepetitionsPerPreambleAttempt | n2 | n8 | n64 |  |  |
| npdcch-NumRepetitions-RA | r4 | r16 | r128 |  |  |
| npdcch-StartSF-CSS-RA | v1dot5 | v1dot5 | v1dot5 |  |  |
| npdcch-Offset-RA | zero | zero | Zero |  |  |
| nprach-NumCBRA-StartSubcarriers | n8 | n8 | n8 |  |  |
| ra-ResponseWindowSize (per NPRACH Resource) | pp2 | pp2 | pp2 |  |  |
| mac-ContentionResolutionTimer (per NPRACH Resource) | pp8 | pp8 | pp8 |  |  |
| Note 1: See Clause 6.7.3 in TS 36.331 for further information on the parameters in this table. |  |  |  |  |  |  |

<End of Change 2>