3GPP TSG-RAN WG4 Meeting #112										          R4-2411808

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:			8.1.3

Source:	Moderator (Huawei, HiSilicon)

Title:	Topic summary for [112][213] NR_ENDC_RF_Ph4

Document for:	Information

Introduction

This email thread discusses the RRM requirements of WI on R19 NR_ENDC_RF_Ph4.

Online handling

(Online) Issue 1-3: Cell identification delay, measurement delay, mobility requirements for 6Rx capable UE

(Online) Issue 1-4: Interruption requirements at SRS antenna switching for 6Rx capable UE

(Online) Issue 1-5: RRM performance requirements for 6Rx capable UE

(Online) Issue 1-6: SNR level for RLM and BFD testing for 6Rx capable UE

Topic #1:	RRM requirements for R19 UE RF enhancements for NR FR1/FR2 and EN-DC

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2411568 | Nokia, Nokia Shanghai Bell | Observation #1: In RAN1, 3T6R and 4T6R SRS antenna switching has not been specified. Observation #2: In R17, the interruption requirements for SRS antenna switching were specified considering up to 4Rx and SRS resource(s) is assumed being configured within the last 6 symbols of a slot. Observation #3: In R19, it is not clear SRS resources are always configured within the last 6 symbols of a slot pending on RAN1 discussion. Proposal 1: To wait for RAN1 conclusion on the 6Rx relevant SRS antenna switching before defining the RRM requirements in RAN4. |
| R4-2412223 | Huawei, HiSilicon | Proposal 1: The existing interruption requirements at SRS antenna switching are applicable to 6RX capable UE, and no specification impact is observed. Proposal 2: RAN4 introduce the antenna connection for 6Rx capable UEs in TS 38.133 A.3.6, where the following aspects are specified: Testing principles for 6Rx capable UEs For RLM and BFD testing, reuse SNR levels specified for 4Rx capable UE |
| R4-2412404 | Ericsson | Observation 1: No RRM work is expected due to the WI objective on introducing support for the higher power UE for CA. Observation 2: No RRM work is expected due to the WI objective on power domain enhancements. Observation 3: Impact on RRM performance requirements is expected due to introducing 6 Rx capable UEs in FR1. Proposal 1: The legacy core RRM requirements are reused for 6 Rx capable UEs. Proposal 2: RAN4 to define RRM performance requirements for operating 6 Rx capable UEs in FR1 which includes: New antenna configurations and principle of testing Proposal 3: RAN4 to discuss impact on RLM/BFD tests when operating with 6 Rx UEs. |

Open issues summary

Sub-topic 1: RRM requirements of WI on R19 NR_ENDC_RF_Ph4

Issue 1-1: support for the higher power UE for CA

- Proposals

  - Option 1(Ericsson): No RRM work is expected due to the WI objective on introducing support for the higher power UE for CA.

- Recommended WF

Further discussion.

Issue 1-2: Power boosting and/or MPR reduction

- Proposals

  - Option 1(Ericsson): No RRM work is expected due to the WI objective on power domain enhancements.

- Recommended WF

Further discussion.

(Online) Issue 1-3: Cell identification delay, measurement delay, mobility requirements for 6Rx capable UE

- Proposals

  - Option 1(Ericsson): The legacy core RRM requirements for identification delay, measurement delay, mobility requirements are reused for 6 Rx capable UEs

- Recommended WF

Agree on option 1 and no spec changes.

(Online) Issue 1-4: Interruption requirements at SRS antenna switching for 6Rx capable UE

- Proposals

  - Option 1(Nokia): To wait for RAN1 conclusion on the 6Rx relevant SRS antenna switching before defining the RRM requirements in RAN4.

  - Option 2(Huawei, Ericsson): The existing interruption requirements at SRS antenna switching are applicable to 6RX capable UE, and no specification impact is observed.

- Recommended WF

Further discussion.

(Online) Issue 1-5: RRM performance requirements for 6Rx capable UE

- Proposals

  - Option 1(Huawei, Ericsson): RAN4 introduce the antenna connection for 6Rx capable UEs in TS 38.133 A.3.6, where the following aspects are specified:

    - Testing principles for 6Rx capable UEs

    - RLM and BFD testing

- Recommended WF

Further discussion.

(Online) Issue 1-6: SNR level for RLM and BFD testing for 6Rx capable UE

- Proposals

  - Option 1(Huawei): Reuse SNR levels specified for 4Rx capable UE

  - Option 2 (Ericsson): RAN4 to discuss impact on RLM/BFD tests when operating with 6 Rx UEs

- Recommended WF

Further discussion.