# 3GPP TSG-RAN WG4 Meeting #112	R4-2413026

# Maastricht, Netherlands, 19th – 23rd August, 2024

Source: 	Huawei, HiSilicon

Title:	Discussion on NR CA power class for single configured UL

Agenda Item:	10.2

Document for:	Approval

# 1	Introduction

RAN has assigned a task to RAN4 as shown below [1].

Way Forward

- RAN tasks RAN4 to investigate how to enable HPUE maximum transmit power in downlink CA with a single configured UL.

  - RAN task shall conclude by RAN#106

  - RAN4 online meeting time should be limited to <0.1 TUs

In the following, we share our views on this topic.

# 2	Discussion

### 2.1 Scope of the RAN Task

RAN4 has had lengthy discussions on power class related issue without conclusion, partly due to the complex nature of the problem. The RAN task has significantly narrowed down the scope of problem to be solved. In our understanding, RAN4 is only requested to discuss the use case when a UE is RRC configured as CA in the DL and single-carrier in the UL. And the scenarios when a UE is configured with UL CA but only one carrier is activated or scheduled are NOT in the scope.

Proposal 1: RAN4 to discuss the use case when a UE is RRC configured as CA in the DL and single-carrier in the UL. Other scenarios are not in the scope of the RAN task.

Based on previous discussions, the majority view on the supported power class is that the UE is not required to mandatorily support the power class reported in ue-PowerClass for the UL band when configured with DL CA. In contrast, a UE should be allowed to report the supported power class which may be different from the one in ue-PowerClass.

Proposal 2: UE is not required to mandatorily support the power class reported in ue-PowerClass for the UL band when configured with DL CA.

Within the context described above, the following two sub-issues remain to be resolved:

- How to report the power class for DL CA with single-carrier UL?

- How to treat the MSD requirement if it has not been specified for higher power class for the band combination?

### 2.2 Power Class Reporting

The band combination of DL CA with single-carrier UL is typically viewed as a fallback of a parent BC (i.e. a BC having UL CA). Based on RAN2’s reporting rules, a UE is allowed to report a fallback BC if its power class capability is higher than the parent BC.

As pointed out in our previous contribution [2], RAN2 allows two ways to report higher power class capability for a fallback BC, i.e. either by new entry in the supported BandCombination list or by new entry in the featureSetCombination.

For the purpose of illustration, an example shown in [2] is duplicated below for band combination of CA_n1A-n78A.

It can be seen that PC2 is applicable to UL_n1A, UL_n78A and UL_n1A-n78A, and PC1.5 is also applicable to UL_n78A. A UE may use the following two options to report the power class capabilities.

Table 1: Alt-1 power class capability report for CA_n1A-n78A

| BandCombination List | Remarks |
| --- | --- |
| + BandCombination |  |
| - Band list: n1A, n78A - FeatureSet Combination: - DL_n1A-n78A + UL_n1A-n78A - Power class: PC2 | Can be reported. Note that up to R18 only PC3+PC3/PC2 is allowed for FDD+TDD UL CA as per the HPUE WID. |
| + BandCombination |  |
| - Band list: n1A, n78A - FeatureSet Combination: - DL_n1A-n78A + UL_n1A - DL_n1A-n78A + UL_n78A - Power class: PC2 | Not allowed to report, due to same power class capability as the parent BC with UL_n1A-n78A. How to report PC2 for UL_n1A? |
| + BandCombination |  |
| - Band list: n1A, n78A - FeatureSet Combination: - DL_n1A-n78A + UL_n78A - Power class: PC1.5 | Can be reported, fallback’s capability is higher than the parent BC. If not reported, the power class is PC2 inherited from the parent BC. |

Table 2: Alt-2 power class capability report for CA_n1A-n78A

| BandCombination List | Remarks |
| --- | --- |
| + BandCombination |  |
| - Band list: n1A, n78A - FeatureSet Combination: - DL_n1A-n78A + UL_n1A@PC3-n78A@PC2 - DL_n1A-n78A + UL_n1A@PC2 - DL_n1A-n78A + UL_n78A@PC1.5 - Power class: PC2 | It’s proposed to use the ue-PowerClassPerBandPerBC-r17 capability to report the power class per band for each UL configuration. PC1.5 reported for single-carrier UL_n78A is higher than the BC power class PC2. |

It’s worth noting that both methods are explicitly stated in the RRC spec [3], duplicated as below:

In feature set combinations the UE shall exclude entries with same or lower capabilities, since the network may anyway assume that the UE supports those.

NOTE 1:	The UE may advertise fallback band-combinations in which it supports additional functionality explicitly in two ways: Either by setting FeatureSet IDs to zero (inter-band and intra-band non-contiguous fallback) and by reducing the number of FeatureSet-PerCC Ids in a Feature Set (intra-band contiguous fallback). Or by separate BandCombination entries with associated FeatureSetCombinations.

NOTE 2:	The UE may advertise a FeatureSetCombination containing only fallback band combinations. That means, in a FeatureSetCombination, each group of FeatureSets across the bands may contain at least one pair of FeatureSetUplinkId and FeatureSetDownlinkId which is set to 0/0.

NOTE 3:	The Network configures serving cell(s) and BWP(s) configuration to comply with capabilities derived from the combination of FeatureSets at the same position in the FeatureSetsPerBand, regardless of activated/deactivated serving cell(s) and BWP(s).

Proposal 3: To report the power class for DL CA with single-carrier UL, a UE is allowed to use either powerClass or ue-PowerClassPerBandPerBC-r17.

### 2.3 MSD Requirements

In previous discussions, it was proposed that a UE is allowed to report and operate at PC2/PC1.5 for DL CA with single-carrier UL even if PC2/PC1.5 MSD has not been specified. Since the UE is only needed to verify at PC3, the HPUE MSD requirements would be effectively bypassed. We don’t think this is a fair solution.

A more reasonable solution is to apply the PC3 MSD requirements when the UE is operating at PC2/PC1.5. It means that the UE would have to meet more stringent MSD requirements. If it turns out to be challenging, it could motivate companies to provide contributions to specify new MSD requirements for HPUE.

Proposal 4: If the PC2/PC1.5 MSD has not been specified for the DL CA with single-carrier UL, the UE should meet the PC3 MSD requirements when operating at PC2/PC1.5 in order to report PC2/PC1.5.

# 3	Conclusion

In this paper, we have shared our views on the RAN task and made the following proposals:

Proposal 1: RAN4 to discuss the use case when a UE is RRC configured as CA in the DL and single-carrier in the UL. Other scenarios are not in the scope of the RAN task.

Proposal 2: UE is not required to mandatorily support the power class reported in ue-PowerClass for the UL band when configured with DL CA.

Proposal 3: To report the power class for DL CA with single-carrier UL, a UE is allowed to use either powerClass or ue-PowerClassPerBandPerBC-r17.

Proposal 4: If the PC2/PC1.5 MSD has not been specified for the DL CA with single-carrier UL, the UE should meet the PC3 MSD requirements when operating at PC2/PC1.5 in order to report PC2/PC1.5.

# References

- RP-241687 Moderator's summary for offline discussion on CA Power Class Support, Moderator (AT&T), RAN#104

- R4-2402211 Discussion on power class capability report for NR CA, Huawei, HiSilicon, RAN4#110

- 3GPP TS 38.331